3GPP TSG-SA5 Meeting #164	S5-255243

Dallas, USA, 17 - 21 November 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.561 | CR | 0010 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-19 CR TS 28.561 Fix conflict between attributes |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NDT |  |  |  |  |  | Date: |  |  | 2025-11-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Allowed values for attribute nDTCapability do not match with the allowed values for attribute supportedNDTCapabilities. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Define new enumeration type to ensure that attributes nDTCapability and supportedNDTCapabilities are always aligned. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Requested NDT capabilities are not aligned with supported NDT capabilities. Stage 3 definition is not aligned with stage 2 definition. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.1.x (new), 6.3.1, Forge |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Forge MR link: https://forge.3gpp.org/rep/sa5/MnS/-/merge_requests/1988 at commit c9d183f501a8f312b75583e4ed8c0fdb1b118706 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

#### 6.2.1.x	Enumerations

##### 6.2.1.x.1	NDTCapability <<enumeration>>

Table 6.2.1.x.1-1: <<enumeration>> NDTCapability

| Enumeration value | Description |
| --- | --- |
| RISKY_ACTIONS_PREDICTION | Indicates support for the use case described in clause 5.2.2.2 |
| EVENTS_IMPACTS_VERIFICATION | Indicates support for the use case described in clause 5.2.2.3 |
| NETWORK_ISSUE_INDUCEMENT | Indicates support for the use case described in clause 5.2.2.4 |
| NETWORK_EVENTS_VERIFICATION | Indicates support for the use case described in clause 5.3.2.2 |
| NETWORK_CONFIGURATIONS_VERIFICATION | Indicates support for the use case described in clause 5.3.2.3 |
| AUTOMATION_CONFIGURATION_VERIFICATION | Indicates support for the use case described in clause 5.3.2.4 |
| ML_TRAINING_DATA_GENERATION | Indicates support for the use case described in clause 5.4.2.2 |
| USER_EXPERIENCE_DATA_GENERATION | Indicates support for the use case described in clause 5.4.2.3 |

* * * Next Change * * * *

### 6.3.1	Attribute properties

Table 6.3.1-1

| Attribute Name | Documentation and Allowed Values | Properties |
| --- | --- | --- |
| ndtJobRef | It indicates an DN of a NDTJob Instance. allowedValues: N/A | type: DN multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| ndtFunctionRef | It indicates a DN of NDTFunction Instance(s) that may be involved in the NDT collaboration. allowedValues: N/A | type: DN multiplicity: * isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| ndtReportRefList | It indicates a list of DN for NDTReport Instances. allowedValues: N/A | type: DN multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| supportedNDTCapabilities | It indicates the different types of scenario specific capability which the NDT MnS Producer is capable of undertaking. | type: NDTCapability multiplicity:1..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| nDTFunctionScope | It indicates a scope that can be modelled by an NDT. | type: NDTFunctionScope multiplicity:1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| nDTCapability | It indicates the type of application use cases that is desired to be executed. | type: NDTCapability multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| nDTRANScope | It indicates the scope of the RAN that can be modelled by the NDT function. | type: ScopeDefinition multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| nDTCNScope | It indicates the scope of the CN that can be modelled by the NDT function. | type: ScopeDefinition multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| nDTJobSynchScope | It indicates the scope of the network that should be synchronized into and modelled by the NDT MnS Producer for the specific NDT job. | type: ScopeDefinition multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| ndtJobScenario | It indicates a network scenario that should be modelled in the NDTJob as an extra beyond what is synchronized from the network. The ndtJobScenario can be used for - Verification of network response to one or more events - evaluation of the impact of one or more failure events, e.g. a signalling storm - Evaluating one or more network issues, e.g. a coverage issue. The network issues involve one or more network events. - Evaluation of high-risk network operations which are listed within the planned configuration - Verification of network configurations which are listed within the planned configuration - Generation of ML training data - Generation user experience data - Inducing particular issue in the network | type: NDTInputDescription multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| NDTInputDescriptionId | It indicates the identifier for a specific input to be modelled in the NDTJob | type: string multiplicity: 1 isOrdered: N/A isUnique: True defaultValue: None isNullable: False |
| NDTOutputDescriptionId | It indicates the identifier for a specific output provided as outcomes of the NDTJob. | type: string multiplicity: 1 isOrdered: N/A isUnique: True defaultValue: None isNullable: False |
| networkEventInfo | This defines the information related with a network event (a provisioning, performance measurement, KPI or fault/ alarm event) that can be introduced by the NDT MnS Producer. The networkEventInfo can be used for - Verification of network response to one or more events - evaluation of the impact of one or more failure events, e.g. a signalling storm - Evaluating one or more network issues, e.g. a coverage issue. The network issues involve one or more network events. Editor’s note: The definition and modelling of networkEventInfo is to be clarified. | Type: TBD multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| simulationDataDescriptor | This defines the simulation details for the NDT. | type: SimulationDataDescriptorAttributeValuePair multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| simulationData | This described the management data that need to be updated artificially in order to induce a particular network issue. It also defines the network management data that aims to be simulated/emulated by the NDT. The behaviour can be configurations for verification. The simulationData can be network configurations or automation functionality configurations, such as RAN ES functionality provided by SON. | type: SimulationData multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| condition | This will define the condition that has to be satisfied in order to update the simulation data for the task that is executed by the NDT MnS Producer. This can be defined in terms of location and time. This will be the DN of ConditionMonitor[7]. | type: DN multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| performanceData | This defines the performance data injected in NDT to represent network events. | type: PerformanceData multiplicity: 1 ..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| mDTData | This defines attribute/value pair representing the MDT data name that is to be updated and with what value. | type: AttributeValuePair multiplicity: 1 ..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| configurationData | This defines the configuration updates for the network | type: AttributeValuePair multiplicity: 1 ..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| performanceDataName | It indicates the name of performance measurement or the KPI as defined in 3GPP TS 28.552 [2] and 3GPP TS 28.554 [3]. | type: string multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| performanceDataValue | It indicates the value of performance data. | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| performanceDataScalingFactor | It indicates the percentage of scaling for performance data. A scaling factor less than 100% indicates a reduction in the performance data, a scaling factor greater than 100% indicates an increase in the performance data. | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| ndtJobExecutionRequirements | It describes the performance requirements for simulation/emulation by NDT MnS Producer, e.g., maximum run time for each simulation/emulation job, precision, etc. | type: NdtJobExecutionReqts multiplicity: 1 isOrdered: N/A isUnique: True defaultValue: None isNullable: False |
| nDTJobOutputData | It indicates the list of NDTOutput(s) that are provided by the NDT function as the output for any task executed in an instantiated NDT job. | type: NDTOutputDataPoint multiplicity: 1 ..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| maxRuntime | Maximum run time for each simulation task executed by the NDT MnS Producer. The unit is second. | type: Integer multiplicity: 1 isOrdered: N/A isUnique: True defaultValue: None isNullable: False |
| networkState | It indicates a state of the twin network (the modelled network by the NDT MnS Producer) for which a configuration or reconfiguration is applied. The networkState is the description of what exists in the network at the time when the networkConfiguration is made. | type: NDTOutputDescription multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| networkConfiguration | It indicates a network configuration that is executed according to the NDT Job and being reported in the NDT report. | type: NDTOutputDescription multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| observations | It indicates an impact on the network. It shows the list of network objects that are affected and the effects on the specific objects. | type: NDTOutputDescription multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| ndtJobRef | It indicates the related NDT Job contributing as a collaborator to the executed NDT Job. It describes a relationship to an NDT job, i.e. it indicates the DN of a component NDT which provides input to the NDT job. | type: DN multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| objectInstance | It indicates the MOI contained in the NDT report. | type: DN multiplicity: 1 isOrdered: N/A isUnique: True defaultValue: None isNullable: False |
| objectAttributeList | It indicates the key-value pair of the characteristics of the MOI. | type: AttributeValuePair multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| managedEntitiesScope | A list of DN of managed entities which is the NDT modelling scope. | type: DN multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| areaScope | Geographical area which is the NDT modelling scope. | type: GeoArea multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |

* * * Next Change * * * *

Forge MR link: https://forge.3gpp.org/rep/sa5/MnS/-/merge_requests/1988 at commit c9d183f501a8f312b75583e4ed8c0fdb1b118706

*** START OF CHANGE 1 ***

*** OpenAPI/TS28561_NdtNrm.yaml ***

<CODE BEGINS>

openapi: 3.0.1

info:

title: NDT NRM

version: 19.0.0

description: >-

OAS 3.0.1 definition of the NDT NRM

© 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: 3GPP TS 28.561; Management aspects of Network Digital Twins

url: http://www.3gpp.org/ftp/Specs/archive/28_series/28.561/

paths: {}

components:

schemas:

#-------- Definition of types for name-containments ------

SubNetwork-ncO-NDTNrm:

type: object

properties:

NDTFunction:

$ref: '#/components/schemas/NDTFunction-Multiple'

#-------Definition of generic IOCs ----------#

NDTFunction-Single:

description: >-

This IOC represents the properties of an NDT Function

allOf:

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/Top'

- type: object

properties:

supportedNDTCapabilities:

type: array

uniqueItems: true

items:

$ref: '#/components/schemas/NDTCapability'

description: >-

It indicates the different types of NDT application use cases which the NDT is capable of undertaking..

New values can be added to this list in future releases to support new use cases.

nDTFunctionScope:

$ref: '#/components/schemas/NDTFunctionScope'

nDTFunctionRefList:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/DnList'

NDTJob-Single:

description: >-

This IOC represents the properties of an NDT job demand created by an MnS consumer. An NDT job represents the requirements for a network simulation/emulation task

allOf:

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/Top'

- type: object

properties:

nDTCapability:

$ref: '#/components/schemas/NDTCapability'

description: >-

It indicates the type of application use cases that is desired to be executed.

New values can be added to this list in future releases to support new use cases.

nDTJobSynchScope:

$ref: '#/components/schemas/ScopeDefinition'

nDTJobScenario:

$ref: '#/components/schemas/NDTInputDescription'

nDTJobExecutionRequirements:

$ref: '#/components/schemas/NdtJobExecutionReqts'

collaboratingNDT:

description: >-

It indicates the related NDT Job contributing as a collaborator to the executed NDT Job. It describes the DN of the collaborated NDT Job

$ref: 'TS28623_ComDefs.yaml#/components/schemas/DnListRo'

nDTReportRefList:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/DnListRo'

nDTJobRefList:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/DnListRo'

NDTReport-Single:

description: >-

This IOC represents the properties of an NDT report corresponding to an NDT job

allOf:

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/Top'

- type: object

properties:

nDTJobOutputData:

type: array

uniqueItems: true

items:

$ref: '#/components/schemas/NDTOutputDataPoint'

description: It indicates the list of NDTOutput(s) that are provided by the NDT function as the output for any task executed in an instantiated NDT job.

nDTJobRef:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/DnRo'

#-------Definition of the dataType ----------#

NDTCapability:

type: string

enum:

- RISKY_ACTIONS_PREDICTION

- EVENTS_IMPACTS_VERIFICATION

- NETWORK_ISSUE_INDUCEMENT

- NETWORK_EVENTS_VERIFICATION

- NETWORK_CONFIGURATIONS_VERIFICATION

- AUTOMATION_CONFIGURATION_VERIFICATION

- ML_TRAINING_DATA_GENERATION

- USER_EXPERIENCE_DATA_GENERATION

NDTFunctionScope:

type: object

properties:

nDTRANScope:

$ref: '#/components/schemas/ScopeDefinition'

nDTCNScope:

$ref: '#/components/schemas/ScopeDefinition'

NDTInputDescription:

type: object

properties:

nDTInputDescriptionId:

type: string

simulationData:

type: array

uniqueItems: true

items:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/AttributeNameValuePairSet'

networkEventInfo:

type: string

condition:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/DnRo'

NDTOutputDescription:

type: object

properties:

nDTOutputDescriptionId:

type: string

objectInstance:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/DnRo'

objectAttributeList:

type: array

uniqueItems: true

items:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/AttributeNameValuePairSet'

ScopeDefinition:

type: object

oneOf:

- required: [ managedEntitiesScope ]

- required: [ areaScope ]

properties:

managedEntitiesScope:

type: array

uniqueItems: true

items:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/DnRo'

areaScope:

type: array

uniqueItems: true

items:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/GeoArea'

NdtJobExecutionReqts:

type: object

properties:

maxRuntime:

type: integer

NDTOutputDataPoint:

type: object

properties:

networkState:

type: array

uniqueItems: true

items:

$ref: '#/components/schemas/NDTOutputDescription'

networkConfiguration:

type: array

uniqueItems: true

items:

$ref: '#/components/schemas/NDTOutputDescription'

observations:

type: array

uniqueItems: true

items:

$ref: '#/components/schemas/NDTOutputDescription'

#------Definition of JSON arrays for name-contained IOCs ---------------#

NDTJob-Multiple:

type: array

items:

$ref: '#/components/schemas/NDTJob-Single'

NDTReport-Multiple:

type: array

items:

$ref: '#/components/schemas/NDTReport-Single'

NDTFunction-Multiple:

type: array

items:

$ref: '#/components/schemas/NDTFunction-Single'

#------Definition of JSON arrays for name-contained IOCs ---------------#

#----- Definitions in TS 28.561 for TS 28.532 --------------------------#

resources-ndtNrm:

oneOf:

- $ref: '#/components/schemas/NDTFunction-Single'

- $ref: '#/components/schemas/NDTJob-Single'

- $ref: '#/components/schemas/NDTReport-Single'

<CODE ENDS>

*** END OF CHANGE 1 ***

* * * End of Changes * * * *