3GPP TSG-SA5 Meeting #164	S5-255245

Dallas, USA, 17 – 21 November 2025

Source:	NTT DOCOMO

Title:	Pseudo-CR on Rel-20 TR 28.881 Add potential solution, evaluation and recommendation for Use Case#15

Document for:	Approval

Agenda item:	6.20.1

Spec:	3GPP TR 28.881

Version:	0.2.0

Work Item:	FS_IDMS_MN_Ph4

Comments

This pCR provides potential solution, evaluation and recommendations for Use case#15 for relation and interaction with MnS producers for AI/ML Management

Proposed Changes

* * * First Change * * * *

## 4.15	Use case #15: Relation and Interaction with MnS producers for AI/ML Management

### 4.15.1	Description

This use case describes a scenario where an intent driven MnS producer interacts and coordinates with MnS producers for AI/ML management which is specified in 3GPP TS 28.105 [10]. Such coordination can enable the intent-driven MnS producer to both leverage AI/ML for its own internal intent handling tasks, as well as for when AI/ML processing might be needed in the network as an outcome of processing the intents.

As an example of leveraging AI/ML, in order for the intent-driven MnS producer to fulfil an expectation, the intent driven MnS producer may request to control the inference, e.g., activate/deactivate the ML model/models, configure the allowed ranges of the inference output parameters, request fine-tuning of an ML model etc. These are possible cases whereby the intent driven MnS producer can use an ML model and interact as an MnS consumer with MnS producers described in 3GPP TS 28.105 [10], clause 6 for ML model lifecycle management, such as ML model training, ML testing, ML model deployment, AI/ML inference, AIML Inference emulation.

In the case of AI/ML being leveraged as an outcome of the intent handling, an example can be considered whereby in order for the intent-driven MnS producer to check the feasibility of an intent or exploration of an intent in pre-evaluation phase as specified in 3GPP TS 28.312 [1], the intent-driven MnS producer may request, for example, AIML Inference emulation before the ML model is applied for fulfilment in production network.

### 4.15.2	Potential requirements

REQ-Intent_AIML-1: The intent driven MnS producer should have the capability to interact with MnS producers for ML model lifecycle management in both intent pre-evaluation phase and intent fulfilment phase.

### 4.15.3	Potential solutions

#### 4.15.3.1	Potential solution #1

This potential solution considers enabling an intent-driven MnS producer to interact with MnS producers for ML model lifecycle management. The solution does not require updating the NRM or already specified interfaces, and it only necessitates of updating the 3GPP TS 28.312 [1] to describe that the intent driven MnS producer,  if necessary, can interact with MnS producers for ML model lifecycle management in both intent pre-evaluation phase and intent fulfilment phase, with reference to 3GPP TS 28.105 [10].

### 4.15.4 Evaluation of potential solutions

The potential solution proposed in clause 4.15.3.1 satisfies the requirement identified in clause 4.15.2. The solution does not require updates to the NRM and/or interfaces, only describing the possible interactions between intent-driven MnS producer and MnS producer for ML model lifecycle management in 3GPP TS 28.312 [1], and hence, it is a feasible solution

* * * Second Change * * * *

# 5	Conclusions and Recommendations

Editor's note: this clause will contain conclusions and recommendations for corresponding key issues identified in clause 4.

## 5.X	Use case #15 Relation and Interaction with MnS producers for AI/ML Management

The use case description, requirements and a potential solution for relation and interaction with MnS producers for AI/ML Management are described in clause 4.15. This use case clarifies the relation and interaction of intent-driven MnS producers with MnS producers for ML model lifecycle management in both intent pre-evaluation phase and intent fulfilment phase.

The potential solution described in clause 4.15.3.1, which proposes adding text in 3GPP TS 28.312 [1] in order to describe possible interactions between intent-driven MnS producer and MnS producer for ML model lifecycle management can be used as a baseline for normative work. The exact text and the clause to which it belongs can be decided during the normative phase.

* * * End of Changes * * * *