3GPP TSG-SA5 Meeting #164	S5-255264

Dallas, USA, 17 - 21 November 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.552 | CR | 0754 | rev | - | Current version: | 20.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-20 CR TS 28.552 Add Data Rate Measurements for XRM |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | XRM_Ph2-OAM |  |  |  |  |  | Date: |  |  | 2025-11-04 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-20 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | As specified in the clause 5.45.4 in TS 23.501, data rate monitoring is introduced to allow the measurement of the UL and/or DL data rate per QoS flow at the PSA UPF, which can be applied to a Non-GRB or GRB QoS flow. The data rate is measured over a monitoring averaging window with a standardized value. Therefore, to support the data rate monitoring, new measurements should be specified. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add XRM service related measurements： - Avergae Data Rate between NG-RAN and PSA UPF |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Cannot monitor the Avergae Data Rate for XRM measurement |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.4.X(new), 5.4.X.1(new), 5.4.X.2(new), A.X(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of First change

5.4.X	Data Rate

5.4.X.1	Average DL Data Rate in PSA UPF

a)	This measurement provides the DL Data Rate in PSA UPF. This measurement is intended to measure the data rate over a monitoring averaging window. The measurement is split into subcounters per 5QI.

b)	DER(N=1)

c)	This measurement is obtained according to the following method:

The UPF counts the following information:

- DataVolDl, obtained by counting the number of GTP PDU bits sent from PSA UPF to gNB on the N3 interface in a specified time window.

- MeasTime, the time window for the GTP PDU bits couniting.

The UPF counts the number (N) of data volume measurements and takes the following calculation formula based on the "DataVolDl" and "MeasTime" defined above.

i=1NDataVolDliMeasTimeiN

d)	Each measurement is an integer value representing the data rate in kbit per second.

e)	The measurement name has the form 
GTP.DataRateDlPsaUpf, or optionlally
GTP.DataRateDlPsaUpf.5QI, where 5QI identifies the 5QI;

f)	UPFFunction

g)	Valid for packet switched traffic.

h)	5GS.

5.4.X.2	Average UL Data Rate in PSA UPF

a)	This measurement provides the UL Data Rate in PSA UPF. This measurement is intended to measure the data rate over a monitoring averaging window. The measurement is split into subcounters per 5QI.

b)	DER(N=1)

c)	This measurement is obtained according to the following method:

The UPF counts the following information:

- DataVolUl, obtained by counting the number of GTP PDU bits sent from gNB to PSA UPF on the N3 interface in a specified time window.

- MeasTime, the time window for the GTP PDU bits couniting.

The UPF counts the number (N) of data volume measurements and takes the following calculation formula based on the "DataVolUl" and "MeasTime" defined above.

i=1NDataVolUliMeasTimeiN

d)	Each measurement is an integer value representing the data rate in kbit per second.

e)	The measurement name has the form 
GTP.DataRateUlPsaUpf, or optionlally
GTP.DataRateUlPsaUpf.5QI, where 5QI identifies the 5QI;

f)	UPFFunction

g)	Valid for packet switched traffic.

h)	5GS.

Start of Second change

A.X	Use case of monitoring of Data Rate for XR and Media Service

eXtended Reality (XR) and Media service is considered as one of the key candidate services in 5GA and 6G networks. As specified in the clause 5.37.4 in TS 23.501, 5GS and XR/media services can cooperate to provide a better user experience using External Network Exposure. Based on the AF request, the 5GS can expose the UL and/or DL Data rate information based on the QoS Monitoring

The QoS Monitoring for data rate allows the measurement of the UL and/or DL data rate per QoS flow at the PSA UPF and it can be applied to a Non-GBR or GBR QoS flow. The data rate is measured over a monitoring averaging window with a standardized value.

Start of Second change