TSG SA5 Meeting #164	S5-255285

Dallas, US, 17th – 21th, November, 2025

Source:	NEC, Huawei, Nokia, Samsung, Vivo

Title:	Pseudo-CR on TR 28.882 add Management support use case and requirement for two-sided model training

Document for:	Approval

Agenda item:	6.20.2

Spec:	3GPP TR 28.882

Version:	0.1.0

Work Item:	FS_AIML_MGT_Ph3

Comments

The new SID SP-250867 study on AIML management phase 3 has been approved at the SA #108 meeting. The objectives including:

WT-1: AI/ML Lifecycle Management Enhancements

WT-1.1: Investigate enhancements of AI/ML management capabilities throughout the AI/ML lifecycle in 5GS, including training, testing, emulation, deployment, inference, to support AI/ML-enabled features in the 5GS. This includes:

- ML model transfer/delivery as defined by RAN for Solution 4b: OAM can transfer/delivery AI/ML model(s) to UE.

- NG-RAN use cases including QoE optimization, network energy saving, and mobility use case(as defined in RP-250812).

- 5GC Analytics: Encompasses new 5GC analytics use cases currently under study under WT#2 (see SP-250413) and investigates OAM support for provisioning ML models to relevant 5GC functions to enable AI/ML-based analytics.

- LMF-based AI/ML Positioning including data collection and ML model training by the OAM for UE positioning.

- Study feasibility and potential requirements for data collection for (e.g., UE-side and Network-side) to enable model training.

Item 5 under sub work task WT-1.1 is to study feasibility and potential requirements for data collection to enable model training.

According to the incoming LS in RP-252966, the action for SA5 is to complete the study of dataset/model parameter exchange in Rel-20.

In line with this, this contribution proposes to add a use case on management support to two-sided model training to study management procedures for dataset/model parameter collection and reporting to enable two-side model training supporting the CSI compression features defined in TR 38.843.

Proposed Changes

* * * 1st change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

…

[x1]	3GPP TR 38.843: " Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface ".

* * * Next change * * * *

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

OTT	Over the top

* * * Next change * * * *

# 5	Management capabilities for AI/ML lifecycle

## 5.1	ML model training

### 5.1.1	Use cases

#### 5.1.1.x		Management support to data collection for two-sided model training

##### 5.1.1.x.1	Management support to CSI compression

###### 5.1.1.x.1.1	Description

To support CSI compression defined in TR 38.843 [x1], gNB(s) can share relevant data for two-sided model training (see Note 1) with a UE-side training entity (e.g. a server deployed by an MNO or by an OTT service provider) to enable UE-part model training for two-side CSI compression.

In this use case, the gNB acts as the source of the shared relevant data, while the 3GPP management system performs orchestration and coordination functions. The management system configures the gNB(s) to produce or collect the relevant data and reports them to the authorised UE-side training entity in accordance with operator-defined access control policies (see Note 2).

Figure 5.1.1.x.1.1-1 Illustration of data collection for two-side CSI model training.

Being an SBMA interface consumer, the UE-side training entity requests the 3GPP management system to provide the relevant data and the management system configures the gNB(s) accordingly. After the relevant data is collected, the management system reports it to the UE-side training entity based on access control information specified by the operator (see Note 2).

NOTE 1:	The relevant data are subject to further discussion, pending ongoing correspondence and confirmation by RAN2.

NOTE 2:	Access control information is for further discussion.

###### 5.1.1.x.1.2	Potential requirements

REQ-ML_TWOSIDE-x1: The 3GPP management system should have a capability allowing an authorised UE-side training entity to request and obtain relevant data for two-sided model training.

REQ-ML_TWOSIDE-x2: The 3GPP management system should have a capability of controlling the collection of relevant data by one or more gNBs.

REQ-ML_TWOSIDE-x3: The 3GPP management system should have a capability of reporting the collected relevant data to an authorised UE-side training entity.

NOTE 3:	The interaction between the 3GPP management system and the UE-side training server, including details on the requested information is for further discussion.

* * * End of Changes * * * *