3GPP TSG-SA5 Meeting #164	S5-255297

# Dallas, United States, 17th Nov 2025 - 21st Nov 2025

| CR-Form-v12.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.554 | CR | 0258 | rev | - | Current version: | 19.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at https://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-19 CR TS 28.554 Clarify use of CEF in Estimated carbon emission KPI |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | Energy_OAM_Ph3 |  |  |  |  |  | Date: |  |  | 2025-11-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | gNB Estimated Carbon Emission KPI describes how CEF of the energy supply is obtained. This does not take into account the composition percentage of each energy source, and hence the KPI would evaluate to incorrect values. Hence it needs to be described how the CEF of the energy supply is obtained using the CEF of energy source and the composition percentage of each energy source. ManagedElement is used as the KPI reporting object fot gNB Estimated Carbon Emission KPI, but it is not specified that ManagedElement represents a gNB, but it is possible to use ManagedElement IOC to represent the combined functionality of 3GPP defined gNB. This needs to be clarified. The titles in 6.7.7 and 6.7.7.1 need to be fixed as the 6.7.7 is Estimated Carbon Emission (ECE) KPIs for gNB in 6.7.7.1 and for NG-RAN in 6.7.7.2 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Fix gNB Estimated Carbon Emission (ECE) KPI to describe how CEF of the energy supply is obtained using the CEF of energy source and the composition percentage of each energy source. Add clarification that the KPI is applicable when ManagedElement is used to represent the combined functionality of 3GPP defined gNB Fix the title headers. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Inconsistent specifications cause confusion to implementation, and incorrect KPIs will be implemented |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.7.7, 6.7.7.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

==============First change==============

### 6.7.7	Estimated Carbon Emission (ECE)

#### 6.7.7.1	gNB Estimated Carbon Emission

a)	ECEgNB

b)	This KPI provides the Estimated Carbon Emission of a gNB (ECEgNB) over a time period. The Estimated Carbon Emission of a gNB (ECEgNB) is the Energy Consumption of the gNB (ECgNB) multiplied by the Carbon Emission Factor of the energy supply which powers the gNB (CEF). The CEF of the energy supply is the sum of all CEFs of its energy sources as configured in energySourceCef and proportional to the composition percentage of the energy source configured in energyCompositionPercentage (see TS 28.310 [9]). The KPI object, i.e., ManagedElement is associated with EnergySupplyInfo through IOC EnergyInfoGroup as described in TS 28.310[9].

b-1)	Integer, kg CO2eq

b-2)	CUM

c)	This calculation formula is obtained as:

ECEgNB=ECgNB*CEF

, where:

ECgNB is the Energy Consumption of the gNB as defined in TS 28.554 clause 6.7.3.4.2. Its unit is kWh;

CEFgNB indicates Carbon Emission Factor. The CEF of the energy supply is the sum of all CEFs of its energy sources as configured in energySourceCef proportional to the composition percentage of the energy source configured in energyCompositionPercentage (see TS 28.310 [9]). Its unit is kg CO2eq/kWh.

d) ManagedElement

e) This KPI is applicable when a ManagedElement is used to represent the combined functionality of 3GPP defined gNB.

NOTE: This KPI is applicable for the gNBs that are powered using single energy supply.  The accuracy of this KPI is dependent on the accuracy of the CEF information configured by the operator.

#### 6.7.7.2	NG-RAN Estimated Carbon Emission

a)	ECENG-RAN

b)	This KPI provides the Estimated Carbon Emission (ECE) of the NG-RAN over a time period. It is obtained by summing up the Estimated Carbon Emission of all the gNBs that constitute the NG-RAN.

b-1)	kg CO2eq

b-2)	CUM

c)

d)	Subnetwork

NOTE: ng-eNB is not considered in this KPI.

==============End of change==============