3GPP TSG-SA5 Meeting #164	S5-255346

Dallas, USA, 17 - 21 November 2025

Source:	Ericsson Telecom S.A. de C.V.

Title:	Rel-20 Pseudo-CR TR 28.882 Use case on  enhancing Reinforcement Learning with performance targets

Document for:	Approval

Agenda item:	6.20.2

Spec:	3GPP TR 28.882

Version:	0.1.0

Work Item:	FS_AIML_MGT_Ph3

Comments

Current AI/ML management specification in SA5 does not have any way to specify the performance targets when training a Reinforcement Learning (RL) based policy. In TS 28.105, performance constraints are specified. However, the thresholds specify only safe boundaries. They do not provide the targets necessary for reward design.

In this contribution, we propose a use case, requirements and possible solutions to enhance the RL training request with both performance targets and constraints.

Proposed Changes

* * * First Change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]					3GPP TS 28.105: “Artificial Intelligence / Machine Learning (AI/ML) management”.

[2]					3GPP TS 28.552: “5G performance measurements“

[3]					3GPP TS 28.554: “5G end to end Key Performance Indicators (KPI)”

[4]		3GPP TS 28.622: “Generic Network Resource Model (NRM); Integration Reference Point (IRP);

Information Service (IS)”

* * Next Change * * * *

## 5.1	ML model training

### 5.1.1	Use cases

#### 5.1.1.X	Enhanced RL training with performance targets

#### 5.1.1.X.1	Description

### During RL training, a policy is learnt to maximize a reward aggregated over time. The reward function is defined by the producer from a set of targets set by the consumer. The targets include thresholds such as “Call drop rate(CDR) < 1%”, “Call setup success rate (CSSR) > 90%”, but also optimization instructions such as “minimize CDR”, “maximize CSSR”. Together, these are referred to as “RL performance targets”.

### On the other hand, the policy is also required to ensure that the network performance does not degrade along certain other KPIs. This can be specified as a set of thresholds referred to as RL performance constraints, which should not be violated during training. The constraints may be necessary because otherwise, training the RL model aggressively to optimize the target KPIs may result in degradation of other KPIs.

### During training, the producer trains a policy taking into account the performance targets and constraints set by the consumer, along with constraints derived from the producer’s own knowledge of the network. This is applicable to both offline and online RL.

### It should be clear that performance targets are the primary objects used for reward design while performance constraints, if specified, must be enforced additionally.  Moreover, specifying only the performance constraints does not suffice, because in the absence of performance targets, the producer may learn only to avoid constraint violation. It is possible to treat some of the performance targets as constraints, however, the design of rewards should treat targets and constraints differently due to the completely different semantics. For example, the producer may treat the performance targets as “soft expectations” whereas performance constraints are treated as “hard expectations”.

### In TS 28.105 (Rel 19) [1], only RL performance constraints are specified for online RL training through the attribute RLRequirement.rLPerformanceRequirements of the IOC MLTrainingRequest. RL performance targets are not specified.

### As an additional generalization, it is needed to annotate targets and constraints with context information because the targets/constraints may vary for different conditions. The implication of context for the producer is that the training of the policy will require different rewards for different contexts.

#### 5.1.1.X.2	Potential requirements

REQ-RL_TRAIN_XX: The ML training MnS producer should have a capability to allow an authorized MnS consumer to specify the RL performance targets in the training request.

REQ-RL_TRAIN_YY: The ML training MnS producer should have a capability to allow an authorized MnS consumer to specify the RL performance constraint in the training request.

#### 5.1.1.X.3	Possible solutions

It is proposed to update the rLRequirement attribute of the MLTrainingRequest IOC by adding two new attributes rLPerformanceTargets and rLPerformanceConstraints. The consumer can specify this in the training request while requesting for an RL training.

The attribute rLPerformanceTargets consists of the following attributes:

- Context : a string denoting a Boolean predicate on measurable performance metrics, evaluating to True or False. It denotes whether the targets and constraints are applicable in the current state.

- targetList : which is a string denoting a list of measurable performance metrics that are the targets of training. Performance metrics include measurements defined in TS 28.552 [2] and KPIs defined in TS 28.554 [3].

- targetDirections : which indicates whether a performance metric is required to be maximized (UP) or minimized (DOWN). It is a list of type ENUM {UP, DOWN}.

- thresholdList : which denotes a list of thresholds of type ThresholdInfo (TS 28.622 [4]) for the performance metrics that the training might attempt to achieve.

The attribute rLPerformanceConstraints consists of the following attributes:

- Context : a string denoting a Boolean predicate on measurable performance metrics, evaluating to True or False. It denotes whether the targets and constraints are applicable in the current state.

- thresholdList : which denotes a list of thresholds of type ThresholdInfo (TS 28.622 [4]) for the performance metrics that the training might attempt to achieve.

#### 5.1.1.X.4	Possible solutions evaluation

The solution described in clause 5.1.1.X.4  proposes the addition of two new attributes to the rLRequirement attribute of the MLTrainingRequest IoC, which enables the MnS consumer to indicate the performance targets and performance constraints and satisfies the requirements in clause  5.1.1.X.2. These attributes can be used by the MnS producer to create  the reinforcement learning environment and  specify  the rewards such that the intended semantics of targets and constraints are implemented appropriately.

Therefore, the solution described in clause 5.1.1.X.4 is a feasible solution to be developed further in the normative specifications.

* * End of Changes * * * *