3GPP TSG-SA5 Meeting #164	S5-255568

Dallas, USA, 17 - 21 November 2025

Source:	Nokia

Title:	Pseudo-CR on Add solutions on access control on notifications

Document for:	Approval

Agenda item:	6.20.9

Spec:	3GPP TR 28.888

Version:	0.1.0

Work Item:	FS_EnExpo

Comments

<Proposals, reason for change, abstract, comments if necessary (optional)>

Proposed Changes

* * * First Change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 28.533: "Management and orchestration; Architecture framework".

[3]	3GPP TS 28.579: "Management services exposure to external consumers through CAPIF".

[4]	3GPP TS 28.319: "Access control for management services".

[Y1]	3GPP TS 28.622: "Telecommunication management; Generic Network Resource Model (NRM) Integration   Reference Point (IRP); Information Service (IS)".

…

[x]	<doctype> <#>[ ([up to and including]{yyyy[-mm]|V<a[.b[.c]]>}[onwards])]: "<Title>".

* * * Next Change * * * *

### 5.X.3	Potential solutions

#### 5.X.3.Y Access control for notifications

In this proposed solution, the following enhancement to the AccessRule class (defined in clause 7.3.3 of TS 28.319[4]) is proposed:

| Attribute Name | Documentation and Allowed Values | Properties |
| --- | --- | --- |
| allowedNotificationTypes | It describes the allowed notification types for the MnS consumer for a given scope (as identified by the dataNoteSelector attribute of the AccessRule class). allowedValues: - notifyMOICreation - notifyMOIDeletion - notifyMOIAttributeValueChanges - notifyMOIChanges - notifyEvent - notifyNewAlarm - notifyAckStateChanged - notifyComments - notifyCorrelatedNotificationChanged - notifyChangedAlarmGeneral - notifyClearedAlarm - notifyAlarmListRebuilt - notifyPotentialFaultyAlarmList - notifyFileReady - notifyFilePreparationError - notifyThresholdCrossing - notifyPotentialFaultyDataNodeTree -notifyDataNodeTreeSyncRecommended | type: ENUM multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |

The proposed solution adds a new attribute to AccessRule class called “allowedNotificationTypes”. With this attribute, the allowed notification types on a given set of managed objects (specified by the dataNodeSelector attribute) for the MnS consumer are defined. In the following clauses, it is explained how this proposed attribute is used by the MnS producer when it comes to notification subscriptions and reporting considering different scenarios.

##### 5.X.3.Y.1 Only the notification recepeint address is provided in the notification subscription request

When an MnS consumer subscribes to receive notifications using the NtfSubscriptionControl IOC(see clause 4.3.22 of TS 28.622)  by providing only the notificationRecipientAddress attribute, the MnS producer evaluates the subscription request and responds with a “201 created” status code. When a notification event occurs, the MnS producer checks the following:

- If the managed object instance that produced the notification is part of the scope represented by the “dataNodeSelector” attribute of the AcessRule for a given MnS consumer.

- If the notification event that has occured is part of the configured “allowedNotificationTypes” attribute of the AccessRule classs for the MnS consumer.

If statements (i) and (ii) are true, then the notification event will be sent out to the MnS consumer, however if either statement is false, no notification will be sent out to the MnS consumer.

##### 5.X.3.Y.2 The notification recepeint address and scope or notification types are provided in the notification subscription request

In this case, two different behaviours of the MnS producer are proposed, i.e., the strict scope and partial scope enforcement models.

###### 5.X.3.Y.2.Z1 Access control on notifications based on strict scope enforcement model

When an MnS consumer subscribes to receive notifications using the NtfSubscriptionControl IOC (see clause 4.3.22 of TS 28.622) by providing the notificationRecipientAddress and notificationTypes or scope attributes, the MnS producer checks the following:

- If the scope in the notification subscription request is a subset or corresponds to the “allowed scope” represented by the “dataNodeSelector” attribute of the AcessRule class for a given MnS consumer.

- If the notificationTypes in the the notification subscription request is a subset or corresponds to the “allowed notifications” represented by the allowedNotificationtypes attribute of the AcessRule class for a given MnS consumer

In the strict scope enforcement model, if statements (i) and (ii) are true, then the MnS producer evaluates the subscription request and responds with a “201 created” status code. And once a notification event occurs, the MnS producer behaves as described in clause 5.X.3.1.1.

However, if statement (i) or (ii) is false which means:

- either the MnS consumer is requesting to receive notifications on a scope different or wider than the “allowed scope” represented by the “dataNodeSelector” attribute of the AcessRule class or

- the MnS consumer is requesting to receive a wider set of notifications than the “allowed notifications” represented by the “allowedNotificationTypes” attribute of the AcessRule class

Then in the strict scope enforcement model, the MnS producer responds with a “403 FORBIDDEN” error status code with the appropriate reason indicating that the subscription request wasn’t successful. In this case, the error should be “Unauthorized Scope” with the following reason: "The requested subscription scope exceeds the MnS consumer’s authorized access. Partial subscriptions are not supported." Accordingly, in the strict scope enforcement model, the MnS consumer is not allowed to subscribe to receive notifications on a scope than is authorized for the MnS consumer.

###### 5.X.3.Y.2.Z2 Access control on notifications based on partial scope enforcement model

When an MnS consumer subscribes to receive notifications using the NtfSubscriptionControl IOC (see clause 4.3.22 of TS 28.622) by providing the notificationRecipientAddress and notificationTypes or scope attributes, the MnS producer checks the following:

- If the scope in the notification subscription request is a subset or corresponds to the “allowed scope” represented by the “dataNodeSelector” attribute of the AcessRule class for a given MnS consumer.

- If the notificationTypes in the the notification subscription request is a subset or corresponds to the “allowed notifications” represented by the allowedNotificationtypes attribute of the AcessRule class for a given MnS consumer

In the partial scope enforcement model, if statements (i) and (ii) are true, then the MnS producer behaves similar to the strict scope enforcement model and evaluates the subscription request and responds with a “201 CREATED” status code. And once a notification event occurs, the MnS producer behaves as described in clause 5.X.3.1.1.

However, if statement (i) or (ii) is false which means:

- either the MnS consumer is requesting to receive notifications on a scope different or wider than the “allowed scope” represented by the “dataNodeSelector” attribute of the AcessRule class or

- the MnS consumer is requesting to receive a wider set of notifications than the “allowed notifications” represented by the “allowedNotificationTypes” attribute of the AcessRule class

Then in the partial scope enforcement model, the MnS producer still responds with a “201 CREATED” status code indicating that the subscription request has been successfully created. Optionally, the MnS producer might indicate to the MnS consumer that the subscription is accepted but the notifications will be delivered only for the authorized subset of the requested scope. Subsequently, when a notification event occurs, the MnS producer behaves as described in clause 5.X.3.1.1.

* * * Next Change * * * *

### 5.X.4	Evaluation of potential solutions

In this clause, regarding access control for notifications, a comparison between the strict and partial scope enforcement models is provided in Table 5.X.4-1.

Table 5.X.4-1: Comparison between the strict scope and partial scope enforcement models

| Aspect | Strict Scope Enforcement Model | Partial Scope Enforcement Model |
| --- | --- | --- |
| Subscription Request Handling | Entire request is rejected if any part of the scope is unauthorized. | Request is accepted; only authorized parts of the scope are honored. |
| Partial Subscription Support | Not supported. Subscription must fully comply with authorized scope. | Supported implicitly. Unauthorized parts are silently ignored. |
| Error Handling | Returns an error (e.g., Unauthorized Scope) with reason (e.g., The requested subscription scope exceeds the MnS consumer’s authorized access. Partial subscriptions are not supported."). | No error; may include a warning or informational message (e.g, the subscription is accepted but the notifications will be delivered only for the authorized subset of the requested scope). |
| MnS consumer feedback | Immediate feedback on failure; MnS consumer can adjust request. | No failure; MnS consumer may not be aware of filtered-out scope unless notified. |
| Security | Conservative and strict; minimizes risk of unauthorized access. | More flexible; relies on robust filtering logic to enforce access control. |
| Implementation Complexity | Simpler logic; reject on first unauthorized element of the scope. | A bit more complex; requires per-element scope filtering and dynamic scope evaluation. |
| MnS consumer experience | Strict compliance requirements, might not be considered user-friendly depending on the use case. | MnS consumer experience prioritized over strict rejection. |

* * * End of Changes * * * *