3GPP TSG-SA5 Meeting #164	S5-255584

# Dallas, United States, 17th Nov 2025 - 21st Nov 2025

| CR-Form-v12.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.552 | CR | 0752 | rev | 1 | Current version: | 20.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at https://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-20 CR TS 28.552 Add Number of UE related the SSB beam Index (Maximum) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Unicom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | PM_KPI_Trace_MDT_QoE-OAM |  |  |  |  |  | Date: |  |  | 2025-11-07 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-20 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | During cell search, the UE selects the beam with the strongest signal as its camping beam by measuring the signal strength of each beam. In a certain period of time to analyze the limit number of users of the beam, which is convenient for the differential analysis of the physical space covered by the beam level and optimizes the pattern. Quantifying the peak load on individual beams, which is a key factor for user experience in beam-centric deployments. The current performance measurements in TS 28.552 only provide Number of UE related the SSB beam Index (mean). The proposed change would add a new measurement that number of UE related the SSB beam Index (Maximum). |  |  |  |  |  |  |  |  |
| Summary of change: |  | The proposed change would add a new measurement that number of UE related the SSB beam Index (Maximum) for each individual SSB within a cell. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Lack of Number of UE related the SSB beam Index (Maximum).Based on the number of UE related the SSB beam Index (Maximum) and number of UE related the SSB beam Index (Beam) in the time period, the pattern is optimized and adjusted. After this index is added, it is combined with the original index to make the beam level analysis more refined. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.1.28.x(New clause) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

==============First change==============

5.1.1.28.x	Number of UE related to the SSB beam Index (Maximum)

a)	This measurement provides maximum number of UE related to the SSB beam index.

b)	CC.

c)	The measurement is obtained by sampling the number of UE related to SSB beam index at a pre-defined interval, then taking the maximum sampling value. The UE related to beam index is maintained by UE random access and handover and beam switch in case the beam switch function is enabled (see TS 38.331[20]).

d)	A single integer value.

e)	L1M.SSBBeamRelatedMaxUeNbr.

f)	Beam

g)	Valid for packet switched traffic

h)	5GS

i)	One usage of this performance measurement is for performance assurance and radio resource optimization such as updating the cell coverage shape. This measurement is only applicable when the beam switch function is activated.

==============End of change==============