3GPP TSG-SA5 Meeting #164	S5-255065

# Dallas, United States, 17th Nov 2025 - 21st Nov 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.532 | CR | 0394 | rev | - | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-20 CR TS 28.532 Remove the deprecated subscribe-unsubscribe operations in File reporting service |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI20 |  |  |  |  |  | Date: |  |  | 2025-10-31 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-20 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The subscribe-unsubscribe operations in File data reporting service have been deprecated in Rel-19. According to the lifecycle process it should be removed in the next release (Rel-20). |  |  |  |  |  |  |  |  |
| Summary of change: |  | Remove the deprecated subscribe-unsubscribe operations in File data reporting service. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unneeded duplication in specification and implementations. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.6.1.3, 11.6.1.4, 12.6.1.1.1, 12.6.1.1.3, 12.6.1.1.4, 12.6.1.3.1.1, 12.6.1.3.2.2, 12.6.1.3.2.3, 12.6.1.4.1 Following YAML files normatively defined in the forge are updated: - OpenAPI/TS28532_FileDataReportingMnS.yaml |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Forge MR link: https://forge.3gpp.org/rep/sa5/MnS/-/merge_requests/1959 at commit 51bee16ae280a15c06a94ebf94d55479c7355043 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

==============First change==============

11.6.1.3	Operation subscribe

11.6.1.3.1	Definition

Use createMOI operation specified in clause 11.1.1.1 together with NtfSubscriptionControl IOC specified in TS 28.622 [11] clause 4.3.22.

11.6.1.3.2	Void

11.6.1.3.3	Void

11.6.1.3.4	Void

11.6.1.4	Operation unsubscribe

11.6.1.4.1	Definition

Use deleteMOI operation specified in clause 11.1.1.4 together with NtfSubscriptionControl IOC specified in TS 28.622 [11] clause 4.3.22.

11.6.1.4.2	Void

11.6.1.4.3	Void

11.6.1.4.4	Void

==============Next change==============

12.6.1.1	Mapping of operations

12.6.1.1.1	Introduction

The IS operations are mapped to SS equivalents according to table 12.6.1.1.1-1.

Table 12.6.1.1.1-1: Mapping of IS operations to SS equivalents

| IS operation | HTTP Method | Resource URI | S |
| --- | --- | --- | --- |
| listAvailableFiles | GET | /files | M |

==============Next change==============

12.6.1.1.3	Void

12.6.1.1.4	Void

==============Next change==============

12.6.1.3.1.1	Resource structure on the MnS producer

Figure 12.6.1.3.1.1-1 shows the resource structure of the File Data Reporting MnS on the MnS producer.

Figure 12.6.1.3.1.1-1: Resource URI structure of the File Data Reporting MnS on the MnS producer

Table 12.2.1.3.1.1-1 provides an overview of the resources and applicable HTTP methods.

Table 12.2.1.3.1.1-1: Resources and methods overview

| Resource name | Resource URI | HTTP method | Description |
| --- | --- | --- | --- |
| Files | …/files | GET | Retrieve the information of the available files |
| Notification Target | {notificationTarget} | POST | Send a notification to the notification target |

==============Next change==============

12.6.1.3.2.2	Void

12.6.1.3.2.3	Void

==============Next change==============

12.6.1.4	Data type definitions

12.6.1.4.1	General

Table 12.6.1.4.1-1: Data types defined in this specification

| Data type | Reference | Description |
| --- | --- | --- |
| FileInfo | 12.6.1.4.2.1 | Information describing a file |
| NotifyFileReady | 12.6.1.4.2.2 | Used in the request body of HTTP POST for the notification type notifyFileReady |
| NotifyFilePreparationError | 12.6.1.4.2.3 | Used in the request body of HTTP POST for the notification type notifyFilePreparationError |
| FileDataType | 12.6.1.4.6.3 | File data types |
| FileNotificationTypes | 12.6.1.4.6.4 | File notification types |

Table 12.6.1.4.1-2: Data types imported

| Data type | Reference | Description |
| --- | --- | --- |
| DateTime | TS 28.623 [44] | Date and time |
| Float | TS 28.623 [44] | Float type |
| Uri | TS 28.623 [44] | URI type |
| SystemDN | TS 28.623 [44] | systemDN type |
| NotificationId | TS 28.623 [44] | Notification identifier as defined in ITU-T Rec. X. 733 [4] |
| NotificationHeader | TS 28.623 [44] | Notification header |
| ErrorResponse | TS 28.623 [44] | Used in the response body of multiple HTTP methods in case of error |

==============Next change==============

Forge MR link: https://forge.3gpp.org/rep/sa5/MnS/-/merge_requests/1959 at commit 51bee16ae280a15c06a94ebf94d55479c7355043

*** START OF CHANGE 1 ***

*** OpenAPI/TS28532_FileDataReportingMnS.yaml ***

<CODE BEGINS>

openapi: 3.0.1

info:

title: File Data Reporting MnS

version: 19.1.0

description: >-

OAS 3.0.1 definition of the File Data Reporting MnS

© 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: 3GPP TS 28.532; Generic management services

url: http://www.3gpp.org/ftp/Specs/archive/28_series/28.532/

servers:

- url: '{MnSRoot}/fileDataReportingMnS/{MnSVersion}'

variables:

MnSRoot:

description: See clause 4.4.3 of TS 32.158

default: http://example.com/3GPPManagement

MnSVersion:

description: Version number of the OpenAPI definition

default: XXX

paths:

/files:

get:

summary: Read information about available files

description: >-

Information about available files is read with HTTP GET. The files for

which information shall be returned are identified with the path

component (base resource) and the query component (fileDataType, beginTime,

endTime) of the URI.

parameters:

- name: fileDataType

in: query

description: >-

This parameter selects files based on the file data type.

required: true

schema:

$ref: '#/components/schemas/FileDataType'

- name: beginTime

in: query

description: >-

This parameter selects files based on the earliest time they

became available

required: false

schema:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/DateTime'

- name: endTime

in: query

description: >-

This parameter selects files based on the latest time they

became available

required: false

schema:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/DateTime'

responses:

'200':

description: >-

'Success case ("200 OK").

The resources identified in the request for retrieval are returned

in the response message body.'

content:

application/json:

schema:

type: array

items:

$ref: '#/components/schemas/FileInfo'

default:

description: Error case.

content:

application/json:

schema:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/ErrorResponse'

components:

schemas:

FileDataType:

type: string

enum:

- Performance

- Trace

- Analytics

- Proprietary

FileNotificationTypes:

type: string

enum:

- notifyFileReady

- notifyFilePreparationError

FileInfo:

type: object

properties:

fileLocation:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/Uri'

fileSize:

type: integer

fileReadyTime:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/DateTime'

fileExpirationTime:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/DateTime'

fileCompression:

type: string

fileFormat:

type: string

fileDataType:

$ref: '#/components/schemas/FileDataType'

jobId:

type: string

FileCreationErrorInfoList:

type: object

properties:

fileLocation:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/Uri'

fileDataType:

$ref: '#/components/schemas/FileDataType'

jobId:

type: string

NotifyFileReady:

allOf:

- $ref: 'TS28623_ComDefs.yaml#/components/schemas/NotificationHeader'

- type: object

properties:

fileInfoList:

type: array

items:

$ref: '#/components/schemas/FileInfo'

additionalText:

type: string

NotifyFilePreparationError:

allOf:

- $ref: 'TS28623_ComDefs.yaml#/components/schemas/NotificationHeader'

- type: object

properties:

fileInfoList:

type: array

items:

$ref: '#/components/schemas/FileCreationErrorInfoList'

reason:

type: string

additionalText:

type: string

<CODE ENDS>

*** END OF CHANGE 1 ***

==============End of change==============