3GPP TSG-SA5 Meeting #164	S5-255075

# Dallas, United States, 17th Nov 2025 - 21st Nov 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.312 | CR | 0369 | rev | - | Current version: | 19.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-19 CR TS 28.312 Correct the issue for stage3 YAML definition |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | IDMS_MN_Ph3 |  |  |  |  |  | Date: |  |  | 2025-11-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | 1. The YAML definition for intent is not aligned with Generic Information model definition defined in clause 6.2.1. 2. Several format issues exist in the TS28312_IntentNrm.yaml |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1. Update the YAML definition for intent is not aligned with Generic Information model definition defined in clause 6.2.1. 2. Correct the format issues in the TS28312_IntentNrm.yaml |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The TS28312_IntentNrm.yaml is not aligned with stage 2 definition. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Following YAML files normatively defined in the forge are updated: - OpenAPI/TS28532_FileDataReportingMnS.yaml |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Forge MR link: https://forge.3gpp.org/rep/sa5/MnS/-/merge_requests/1969 at commit 97d8e96aca31b69fa71ab7b0275a400d9c720074 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

==============First change==============

Forge MR link: https://forge.3gpp.org/rep/sa5/MnS/-/merge_requests/1969 at commit 97d8e96aca31b69fa71ab7b0275a400d9c720074

*** START OF CHANGE 1 ***

*** OpenAPI/TS28312_IntentNrm.yaml ***

<CODE BEGINS>

openapi: 3.0.1

info:

title: Intent NRM

version: 19.3.0

description: >-

OAS 3.0.1 definition of the Intent NRM

© 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: 3GPP TS 28.312; Intent driven management services for mobile networks

url: http://www.3gpp.org/ftp/Specs/archive/28_series/28.312/

paths: {}

components:

schemas:

#-------- Definition of types for name-containments ------

SubNetwork-ncO-IntentNrm:

type: object

properties:

IntentHandlingFunction:

$ref: '#/components/schemas/IntentHandlingFunction-Multiple'

#-------Definition of generic IOCs ----------#

Intent-Single:

description: >-

This IOC represents the properties of an Intent driven management information between MnS consumer and MnS producer.

allOf:

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/Top'

- type: object

properties:

userLabel:

type: string

intentExpectations:

type: array

uniqueItems: true

minItems: 1

items:

type: object

oneOf:

- $ref: '#/components/schemas/IntentExpectation'

- $ref: 'TS28312_IntentExpectations.yaml#/components/schemas/RadioNetworkExpectation'

- $ref: 'TS28312_IntentExpectations.yaml#/components/schemas/EdgeServiceSupportExpectation'

- $ref: 'TS28312_IntentExpectations.yaml#/components/schemas/5GCNetworkExpectation'

- $ref: 'TS28312_IntentExpectations.yaml#/components/schemas/RadioServiceExpectation'

- $ref: 'TS28312_IntentExpectations.yaml#/components/schemas/NetworkMaintenanceExpectation'

intentMgmtPurpose:

$ref: '#/components/schemas/IntentMgmtPurpose'

contextSelectivity:

$ref: '#/components/schemas/Selectivity'

consumerSatisfactionIndexThreshold:

type: integer

expectationSelectivity:

$ref: '#/components/schemas/Selectivity'

intentContexts:

type: array

uniqueItems: true

items:

$ref: '#/components/schemas/Context'

description: >-

It describes the list of Context(s) which represents the constraints and conditions that should apply

for the entire intent even if there may be specific contexts defined for specific parts of the intent

intentAdminState:

type: string

enum:

- ACTIVATED

- DEACTIVATED

default: ACTIVATED

description: >-

It describes the intent administrative state.

This attribute is used when MnS consumer-suspension mechanism is supported

intentPriority:

type: integer

minimum: 1

maximum: 100

description: It expresses the priority of the stated intent within a MnS consumer.

intentPreemptionCapability:

type: boolean

intentReportControl:

type: array

uniqueItems: true

items:

$ref: '#/components/schemas/IntentReportControl'

implicitIntentIndex:

type: boolean

default: false

intentReportReference:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/DnRo'

intentUtilityFormulaRef:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/DnRo'

IntentReport-Single:

description: It represents intent report information from MnS producer to MnS consumer.

allOf:

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/Top'

- type: object

properties:

intentFulfilmentReport:

$ref: '#/components/schemas/IntentFulfilmentReport'

intentConflictReports:

type: array

uniqueItems: true

items:

$ref: '#/components/schemas/IntentConflictReport'

intentFeasibilityCheckReport:

$ref: '#/components/schemas/IntentFeasibilityCheckReport'

intentExplorationReport:

$ref: '#/components/schemas/IntentExplorationReport'

intentUtilityReports:

type: array

uniqueItems: true

items:

$ref: '#/components/schemas/IntentUtilityReport'

intentFulfilmentNegotiationReport:

$ref: '#/components/schemas/IntentFulfilmentNegotiationReport'

lastUpdatedTime:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/DateTimeRo'

intentReference:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/DnRo'

IntentHandlingFunction-Single:

description: >-

It represents the intent handling capabilities can be supported by a specific intent

handling function of MnS producer.

allOf:

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/Top'

- type: object

properties:

intentHandlingCapabilityList:

type: array

uniqueItems: true

minItems: 1

items:

$ref: '#/components/schemas/IntentHandlingCapability'

supportedUtilityList:

type: array

items:

$ref: '#/components/schemas/UtilityDefinition'

uniqueItems: true

Intent:

$ref: '#/components/schemas/Intent-Multiple'

IntentReport:

$ref: '#/components/schemas/IntentReport-Multiple'

IntentUtilityFormula:

$ref: '#/components/schemas/IntentUtilityFormula-Multiple'

IntentUtilityFormula-Single:

description: >-

It represents an intent utility function instance.

allOf:

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/Top'

- type: object

properties:

utilityFunctionId:

type: string

utilityParameterList:

type: array

items:

$ref: '#/components/schemas/UtilityParameter'

uniqueItems: true

utilityScale:

type: number

default: 1

utilityOffset:

type: number

default: 0

#-------Definition of generic IOCs ----------#

#-------Definition of the generic IntentExpectation dataType ----------#

IntentExpectation:

description: >-

This data type is the "IntentExpectation" data type without specialisations

It represents MnS consumer's requirements, goals and contexts given to a 3GPP system

type: object

properties:

expectationId:

type: string

description: A unique identifier of the intentExpectation within the intent.

expectationVerb:

$ref: "#/components/schemas/ExpectationVerb"

expectationObject:

$ref: "#/components/schemas/ExpectationObject"

expectationTargets:

type: array

uniqueItems: true

minItems: 1

items:

$ref: '#/components/schemas/ExpectationTarget'

contextSelectivity:

$ref: "#/components/schemas/Selectivity"

expectationContexts:

type: array

uniqueItems: true

items:

$ref: '#/components/schemas/Context'

preferenceWeight:

type: integer

minimum: 0

maximum: 10

description: It represents the preference information of the Consumer on expectations.

required:

- expectationId

- expectationObject

- expectationTargets

#-------Definition of the generic IntentExpectation dataType ----------#

#-------Definition of the generic ExpectationObject dataType ----------#

ExpectationObject:

description: >-

It represents the Object to which the IntentExpectation should apply.

This data type is the "ExpectationObject" data type without specialisations

type: object

properties:

objectType:

type: string

enum:

- RAN_SUBNETWORK  #value for Radio Network Expectation--#

- EDGE_SERVICE_SUPPORT  #value for Edge Service Support Expectation--#

- 5GC_SUBNETWORK  #value for 5GC Network Expectation--#

- RADIO_SERVICE  #value for Radio Service Expectation--#

- SUBNETWORK  #value for Network Maintenance Expectation--#

objectInstance:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/Dn'

objectContexts:

type: array

uniqueItems: true

items:

$ref: '#/components/schemas/Context'

description: >-

It describes the list of Context(s) which represents the constraints and conditions to be

used as filter information to identify the object(s) to which a given intentExpectation should apply.

#-------Definition of the generic ExpectationObject dataType ----------#

#-------Definition of the generic dataType --------------#

Condition:

type: string

enum:

- IS_EQUAL_TO

- IS_LESS_THAN

- IS_GREATER_THAN

- IS_WITHIN_RANGE

- IS_OUTSIDE_RANGE

- IS_ONE_OF

- IS_NOT_ONE_OF

- IS_EQUAL_TO_OR_LESS_THAN

- IS_EQUAL_TO_OR_GREATER_THAN

- IS_ALL_OF

Selectivity:

type: string

enum:

- ALL_OF

- ONE_OF

- ANY_OF

IntentMgmtPurpose:

description: >-

It describes the MnS consumer requirements for the management purpose (required procedures)

of the created or modified intent instance

type: string

enum:

- FEASIBILITYCHECK

- FULFILMENT_WITHOUT_NEGOTIATION

- EXPLORATION

- FULFILMENT_WITH_NEGOTIATION

default: FULFILMENT_WITHOUT_NEGOTIATION

FulfilmentStatus:

type: string

readOnly: true

enum:

- FULFILLED

- NOT_FULFILLED

default: NOT_FULFILLED

description: It describes the current status of the intent fulfilment result.

NotFulfilledState:

type: string

readOnly: true

enum:

- ACKNOWLEDGED

- COMPLIANT

- DEGRADED

- SUSPENDED

- TERMINATED

- FULFILMENTFAILED

default: ACKNOWLEDGED

description: It describes the current progress of or the reason for not achieving fulfilment

for the intent, intentExpectation or expectationTarget.

An attribute which is used when FulfilmentInfo is implemented for IntentFulfilmentInfo

FulfilmentInfo:

description: >-

This dataType represents the properties of a specific fulfilment information for an aspect of

the intent (i.e. either an expectation, a target or the whole intent).

type: object

properties:

fulfilmentStatus:

$ref: '#/components/schemas/FulfilmentStatus'

notFullfilledState:

$ref: "#/components/schemas/NotFulfilledState"

notFulfilledReasons:

type: array

uniqueItems: true

items:

type: string

readOnly: true

description: An attribute which is used when FulfilmentInfo is implemented for IntentFulfilmentInfo

ExpectationVerb:

type: string

enum:

- DELIVER

- ENSURE

- MAINTAIN

description: It describes the characteristic of the intentExpectation and is the property that describes the types of intentExpectations. Vendor extensions are allowed

Frequency:

description: >-

It desribes the RF reference frequency (i.e. Absolute Radio Frequency Channel Number)

and/or the frequency operating band used for a given direction (UL or DL) in FDD or

for both UL and DL directions in TDD.

type: object

properties:

arfcn:

type: integer

description: >-

This attribute shall be supported, when the frequency information represent RF reference frequency.

The allowed values for NR see TS 38.104 subclause 5.4.2.1; The allowed values for EUTRAN see TS 36.104 [X] subclause 5.7.3;

freqband:

type: string

description: >-

This attribute shall be supported, when the frequency information represent frequency operating band.

The allowed values for NR see TS 38.104 subclause 5.4.2.3; The allowed value for EUTRAN see TS 36.104 subclause 5.7.3

ValueRangeType:

oneOf:

- type: number

- type: string

- type: boolean

- type: integer

- $ref: 'TS28623_ComDefs.yaml#/components/schemas/TimeWindow'

- $ref: 'TS28623_ComDefs.yaml#/components/schemas/DateTime'

- $ref: 'TS28623_ComDefs.yaml#/components/schemas/GeoArea'

- $ref: 'TS28623_ComDefs.yaml#/components/schemas/PlmnId'

- $ref: 'TS28623_ComDefs.yaml#/components/schemas/GeoCoordinate'

- $ref: '#/components/schemas/UEGroup'

- $ref: '#/components/schemas/Frequency'

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/SchedulingTime'

UEGroup:

description: >-

It describes the UE Group,

which is represented by specific 5QI, specific S-NSSAI, specific PLMNId or

a specific combination of S-NSSAI, 5QI and PLMNId.

type: object

properties:

pLMNId:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/PlmnId'

fiveQI:

type: integer

minimum: 0

maximum: 255

sNssai:

$ref: 'TS28541_NrNrm.yaml#/components/schemas/Snssai'

#-------Definition of the generic dataType --------------#

#-------Definition of the generic ExpectationTarget dataType----------#

ExpectationTarget:

description: >-

This data type represents the target of the IntentExpectation that are required to be achieved.

This data type is the "ExpectationTarget" data type without specialisations

type: object

properties:

targetName:

type: string

targetCondition:

$ref: '#/components/schemas/Condition'

targetValueRange:

oneOf:

- type: array

uniqueItems: true

minItems: 1

items:

$ref: "#/components/schemas/ValueRangeType"

- $ref: "#/components/schemas/ValueRangeType"

contextSelectivity:

$ref: "#/components/schemas/Selectivity"

targetContexts:

type: array

uniqueItems: true

items:

$ref: '#/components/schemas/Context'

description: It describes the list of constraints and conditions that should apply for a specific expectationTarget.

preferenceWeight:

type: integer

minimum: 0

maximum: 10

description: It represents the preference information of the Consumer on expectationTargets.

#-------Definition of the generic ExpectationTarget  dataType----------#

#-------Definition of the generic Context dataType----------------#

Context:

description: >-

This data type is the "Context" data type without specialisations

type: object

properties:

contextAttribute:

type: string

contextCondition:

$ref: '#/components/schemas/Condition'

contextValueRange:

oneOf:

- type: array

uniqueItems: true

minItems: 1

items:

$ref: "#/components/schemas/ValueRangeType"

- $ref: "#/components/schemas/ValueRangeType"

#-------Definition of the generic Context dataType----------------#

#-------Definition of the generic IntentReportControl dataType----------------#

IntentReportControl:

description: >-

It describes intent report subscription information

type: object

properties:

reportRecipientAddress:

description: >-

It indicates the address of report recipient for MnS consumer

type: string

observationPeriod:

description: >-

It indicates the time period for which the fulfilment process is observed

and at the end of which the fulfilmentInfo for corresponding

ExpectationTargets, IntentExpectations and Intent is updated.

type: integer

expectedReportTypes:

description: >-

It indicates the type of IntentReports.

type: array

uniqueItems: true

items:

$ref: '#/components/schemas/ExpectedReportType'

reportingConditions:

description: >-

It indicates the specified conditions for intent reporting.

type: array

uniqueItems: true

items:

$ref: '#/components/schemas/ReportingCondition'

reportingTargets:

description: >-

It indicates the specified targets needed to be reported.

type: array

items:

type: string

required:

- reportRecipientAddress

- observationPeriod

ExpectedReportType:

type: string

enum:

- INTENT_FULFILMENT_REPORT

- INTENT_CONFLICT_REPORT

- INTENT_FEASIBILITY_CHECK_REPORT

- INTENT_EXPLORATION_REPORT

- INTENT_FULFILMENT_NEGOTIATION_REPORT

- INTENT_UTILITY_REPORT

ReportingCondition:

description: >-

It describes  the specified conditions for intent reporting.

oneOf:

- $ref: '#/components/schemas/TimeCondition'

- $ref: '#/components/schemas/TargetFulfilmentCondition'

TimeCondition:

$ref: 'TS28623_GenericNrm.yaml#/components/schemas/SchedulingTime'

TargetFulfilmentCondition:

description: >-

It indicates the specified conditions of target fulfilment for intent reporting.

type: object

properties:

targetName:

type: string

targetCondition:

$ref: '#/components/schemas/Condition'

targetValueRange:

oneOf:

- type: array

uniqueItems: true

items:

$ref: '#/components/schemas/ValueRangeType'

- $ref: '#/components/schemas/ValueRangeType'

#-------Definition of the concrete IntentReportControl dataType----------------#

#-------Definition of the generic IntentFulfilmentReport dataType----------------#

IntentFulfilmentReport:

description: >-

It includes the intentFulfilmentInfo and expectationFulfilmetResult.

This attribute shall be supported when intent fulfilment information is supported by IntentReport

type: object

properties:

intentFulfilmentInfo:

$ref: '#/components/schemas/FulfilmentInfo'

expectationFulfilmentResult:

type: array

uniqueItems: true

minItems: 1

items:

$ref: '#/components/schemas/ExpectationFulfilmentResult'

additionalFulfilmentInfo:

type: string

#-------Definition of the concrete IntentFulfilmentReport dataType----------------#

#-------Definition of the generic ExpectationFulfilmentResult dataType----------------#

ExpectationFulfilmentResult:

description: >-

It includes the expectationFulfilmentInfo and targetFulfilmentResults for each IntentExpectation.

type: object

properties:

expectaitonId:

type: string

readOnly: true

expectationFulfilmentInfo:

$ref: '#/components/schemas/FulfilmentInfo'

targetFulfilmentResults:

type: array

uniqueItems: true

items:

$ref: '#/components/schemas/TargetFulfilmentResult'

required:

- expectaitonId

- expectationFulfilmentInfo

#-------Definition of the concrete ExpectationFulfilmentResult dataType----------------#

#-------Definition of the generic TargetFulfilmentResult dataType----------------#

TargetFulfilmentResult:

description: >-

This data type includes targetFulfilmentInfo and targetAchievedValue for each ExpectationTarget.

type: object

properties:

targetName:

type: string

readOnly: true

targetFulfilmentInfo:

$ref: '#/components/schemas/FulfilmentInfo'

targetAchievedValue:

description: >-

It describes the value that has been achieved for the expectation target at the time at which

the report is generated.

$ref: "#/components/schemas/ValueRangeType"

targetContexts:

type: array

uniqueItems: true

items:

$ref: '#/components/schemas/Context'

required:

- targetName

- targetFulfilmentInfo

#-------Definition of the concrete TargetFulfilmentResult dataType----------------#

#-------Definition of the generic IntentConflictReport dataType----------------#

IntentConflictReport:

description: >-

It represents the conflict information for the detected conflict

This attribute shall be supported when intent conflict information is supported by IntentReport

type: object

properties:

conflictId:

type: string

readOnly: true

conflictType:

type: string

readOnly: true

enum:

- INTENT_CONFLICT

- EXPECTATION_CONFLICT

- TARGET_CONFLICT

conflictingIntent:

description: >-

This will be present if the value of conflictType is INTENT_CONFLICT. It describes the DN of the conflicting intent

$ref: 'TS28623_ComDefs.yaml#/components/schemas/DnRo'

conflictingExpectation:

description: >-

This will be present if the value of conflictType is EXPECTATION_CONFLICT. It describes the expectationId of the conflicting IntentExpectation with an Intent

type: string

readOnly: true

conflictingTarget:

description: >-

This will be present if the value of conflictType is TARGET_CONFLICT. It describes the targetName of the conflicting ExpectationTarget with an IntentExpectation

type: string

readOnly: true

recommendedSolutions:

type: string

readOnly: true

enum:

- MODIFY

- DELETE

required:

- conflictId

- conflictType

#-------Definition of the concrete IntentConflictReport dataType----------------#

#-------Definition of the generic IntentUtilityReport dataType----------------#

IntentUtilityReport:

description: >-

It represents the utility function results.

type: object

properties:

utilityResultList:

type: array

uniqueItems: true

items:

$ref: '#/components/schemas/UtilityResult'

#-------Definition of the concrete IntentUtilityReport dataType----------------#

#-------Definition of the generic IntentFeasibilityCheckReport dataType----------------#

IntentFeasibilityCheckReport:

description: >-

It represents the intent feasibility check information

This attribute shall be supported when intent feasibility check information information is supported by IntentReport

type: object

properties:

feasibilityCheckResult:

type: string

readOnly: true

enum:

- FEASIBLE

- INFEASIBLE

infeasibilityReasons:

description: An attribute which is used when feasibilityCheckResult is INFEASIBLE

type: array

uniqueItems: true

items:

type: string

readOnly: true

enum:

- INVALID_INTENT_EXPRESSION

- INTENT_CONFLICT

inFeasibleExpectationInfos:

type: array

items:

$ref: '#/components/schemas/InFeasibleExpectationInfo'

required:

- feasibilityCheckResult

- infeasibilityReasons

InFeasibleExpectationInfo:

description: >-

It describes the list of InFeasibleExpectationInfo for all infeasible IntentExpectations

in the intent

type: object

properties:

expectationId:

type: string

readOnly: true

inFeasibleTargets:

type: array

uniqueItems: true

items:

type: string

readOnly: true

description: It describes the list of TargetNames for the InFeasibleTargets

required:

- expectationId

- inFeasibleTargets

#-------Definition of the concrete IntentFeasibilityCheckReport dataType----------------#

#-------Definition of the generic IntentExplorationReport dataType----------------#

IntentExplorationReport:

description: >-

It represents the intent exploration information

This attribute shall be supported when intent exploration information is supported by IntentReport

type: object

properties:

expectationExplorationResults:

type: array

items:

$ref: '#/components/schemas/ExpectationExplorationResult'

uniqueItems: true

minItems: 1

ExpectationExplorationResult:

description: >-

It represents the expectation exploration result for a specific intent expectation

type: object

properties:

expectationId:

type: string

readOnly: true

targetExplorationResults:

type: array

items:

$ref: '#/components/schemas/ExpectationTarget'

uniqueItems: true

minItems: 1

contextExplorationResults:

type: array

items:

$ref: '#/components/schemas/Context'

uniqueItems: true

minItems: 1

required:

- expectationId

- targetExplorationResults

#-------Definition of the generic IntentExplorationReport dataType----------------#

#-------Definition of the generic IntentFulfilmentNegotiationReport dataType----------------#

IntentFulfilmentNegotiationReport:

description: >-

It represents the intent negotiation information during fulfilment phase

This attribute shall be supported when intent fulfilment negotiation information is supported by IntentReport

type: object

properties:

possibleIntentOutcomeList:

type: array

items:

$ref: '#/components/schemas/PossibleIntentOutcome'

uniqueItems: true

minItems: 1

intentFulfilmentNegotiationConsumerFeedback:

$ref: '#/components/schemas/IntentFulfilmentNegotiationFeedback'

implicitIntent:

$ref: '#/components/schemas/ImplicitIntent'

PossibleIntentOutcome:

description: >-

It represents a single outcome evaluated by the MnS producer

type: object

properties:

possibleIntentOutcomeId:

type: integer

readOnly: true

intentFulfilmentInfo:

$ref: '#/components/schemas/FulfilmentInfo'

expectationFulfilmentResults:

type: array

items:

$ref: '#/components/schemas/ExpectationFulfilmentResult'

uniqueItems: true

minItems: 1

possibleImpacts:

type: array

items:

$ref: '#/components/schemas/PossibleImpact'

uniqueItems: true

minItems: 1

required:

- possibleIntentOutcomeId

- intentFulfilmentInfo

PossibleImpact:

description: >-

It represents the possible impact of the possible outcome

type: object

properties:

impactedObjects:

type: array

items:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/DnRo'

minItems: 1

impactedAttributes:

type: array

items:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/AttributeNameValuePairSet'

minItems: 1

IntentFulfilmentNegotiationFeedback:

description: >-

It represents the feedback information that the MnS consumer provides to the

MnS producer as response during intent fulfilment Negotiation.

type: object

properties:

referredIntentOutcomeId:

type: integer

consumerSatisfactionIndex:

type: integer

ImplicitIntent:

description: >-

It represents the implicit intent which includes the information

that MnS consumers have not explicitly pointed out in its intent.

type: object

properties:

implicitIntentExpectations:

type: array

items:

$ref: '#/components/schemas/IntentExpectation'

implicitIntentContexts:

type: array

items:

$ref: '#/components/schemas/Context'

#-------Definition of the generic IntentFulfilmentNegotiationReport dataType----------------#

#-------Definition of the generic IntentHandlingCapability dataType----------------#

IntentHandlingCapability:

description: >-

It represents expectation object information and expectation target information

which can be supported by a specific intent handling function of MnS producer.

type: object

properties:

intentHandlingCapabilityId:

type: string

readOnly: true

supportedExpectationObjectType:

type: string

enum:

- RAN_SUBNETWORK

- EDGE_SERVICE_SUPPORT

- 5GC_SUBNETWORK

- RADIO_SERVICE

readOnly: true

description: It describes the expectation object type which can be supported by a specific intent handling function of MnS producer.

supportedExpectationTargetInfoList:

type: array

items:

$ref: '#/components/schemas/SupportedExpectationTargetInfo'

uniqueItems: true

minItems: 1

required:

- intentHandlingCapabilityId

- supportedExpectationObjectType

- supportedExpectationTargetInfoList

SupportedExpectationTargetInfo:

description: >-

It indicates the detailed information about what the intent driven MnS producer supports for a given supportedExpectationObjectType.

type: object

properties:

supportedTargetName:

type: string

readOnly: true

supportedTargetCondition:

$ref: '#/components/schemas/Condition'

supportedTargetValueRange:

oneOf:

- type: array

uniqueItems: true

minItems: 1

items:

$ref: '#/components/schemas/ValueRangeType'

- $ref: '#/components/schemas/ValueRangeType'

required:

- supportedTargetName

#-------Definition of the concrete IntentHandlingCapability dataType----------------#

#-------Definition of the generic UtilityParameter dataType----------------#

UtilityParameter:

description: >-

It represents the inputs for the specified Intent Utility Function.

type: object

properties:

parameterName:

type: string

parameterWeight:

type: number

#-------Definition of the concrete UtilityParameter dataType----------------#

#-------Definition of the generic UtilityResult dataType----------------#

UtilityResult:

description: >-

It represents the inputs for the specified Intent Utility Function.

type: object

properties:

utilityFunctionId:

type: string

readOnly: true

utilityResult:

type: number

readOnly: true

#-------Definition of the concrete UtilityResult dataType----------------#

#-------Definition of the generic UtilityDefinition dataType----------------#

UtilityDefinition:

description: >-

It represents the inputs for the specified Intent Utility Function.

type: object

properties:

utilityDefinitionId:

type: string

readOnly: true

utilityDescription:

type: string

readOnly: true

utilityParameterList:

type: array

items:

$ref: '#/components/schemas/UtilityParameter'

uniqueItems: true

#-------Definition of the concrete UtilityDefinition dataType----------------#

#------Definition of JSON arrays for name-contained IOCs ---------------#

Intent-Multiple:

type: array

items:

$ref: '#/components/schemas/Intent-Single'

IntentReport-Multiple:

type: array

items:

$ref: '#/components/schemas/IntentReport-Single'

IntentHandlingFunction-Multiple:

type: array

items:

$ref: '#/components/schemas/IntentHandlingFunction-Single'

IntentUtilityFormula-Multiple:

type: array

items:

$ref: '#/components/schemas/IntentUtilityFormula-Single'

#------Definition of JSON arrays for name-contained IOCs ---------------#

#----- Definitions in TS 28.312 for TS 28.532 --------------------------#

resources-intentNrm:

oneOf:

- $ref: '#/components/schemas/IntentHandlingFunction-Single'

- $ref: '#/components/schemas/Intent-Single'

- $ref: '#/components/schemas/IntentReport-Single'

- $ref: '#/components/schemas/IntentUtilityFormula-Single'

#----- Definitions in TS 28.312 for TS 28.532 --------------------------#

<CODE ENDS>

*** END OF CHANGE 1 ***

==============End of change==============