3GPP TSG SA5 Meeting #165	S5-260337

# Goa, India, 9-13 February 2026

| CR-Form-v12.5 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.531 | CR | <CR#> | rev | - | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at https://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel20 CR TS 28.531 Procedure for NF instance modification on a generic orchestration and management system |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NF_Deployment_LCM-OAM |  |  |  |  |  | Date: |  |  | 2026-01-30 |
| Category: | B |  |  |  |  |  | Release: |  |  | 20 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) Rel-21 (Release 21) |  |
| Reason for change: |  | Currently clause 7.11 of TS 28.531 specifies the procedure for NF instance modification when the orchestration and management system is ETSI-NFV MANO. This CR adds another clause specifying the procedure for NF instance modification on a generic orchestration and management system not limited to only ETSI-NFV MANO. |  |  |  |  |  |  |  |  |
| Summary of change: |  | A new clause adding a procedure for NF instance modification on a generic orchestration and management entity |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The current specification leads the industry to perceive the 3GPP management system as being able to interact exclusively with ETSI NFV-MANO for the NF instance modification, and not with other O&M systems commonly used in telco environments. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.11 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

==============First change==============

## 7.11	Procedure of NF instance modification

The Figure 7.11-1 illustrates the procedure of modify NF instance.

Figure 7.11-1: Network Function Instance Modify procedure

1)	Network Function Management Service Provider (NFMS_P) receives a modifyMOIAttributes operation defined in TS 28.532 [8] from Network Function Management Service Consumer (NFMS_C) with DN of MOI for ManagedFunction and network function related requirements.

Note:	The network funcation related requirements see information model definition for NR NRM in clause 4 and information model definition for 5GC NRM in clause 5 in TS 28.541[6].

2)	If ManagedFunction instance contains virtualized part, NFMS_P checks whether corresponding VNF instance needs to be scaled to satisfy the network function related requirements.

3)	If corresponding VNF instance needs to be scaled, NFMS_P invokes corresponding VNF instance scaling procedure. For the interaction with ETSI NFV MANO, the procedure is described in clause 7.2.4 in ETSI GS NFV-IFA008 [15] or clause 7.3.4 in ETSI GS NFV-IFA013 [3].

4)	NFMS_P reconfigures corresponding ManagedFunction MOI.

5)	The NFMS_P sends the response (see modifyMOIAttributes operation defined in TS 28.532 [8]) to NFMS_C.

## 7.11a	Procedure of NF instance modification on a generic orchestration and management system

The Figure 7.11a-1 illustrates the procedure of modify NF instance on a generic orchestration and management system.

Figure 7.11a-1: Network Function Instance Modify procedure on a generic orchestration and management system

1)	Network Function Management Service Provider (NFMS_P) receives a modifyMOIAttributes operation defined in TS 28.532 [8] from Network Function Management Service Consumer (NFMS_C) with DN of MOI for ManagedFunction and network function related requirements.

Note:	For the network function related requirements, see information model definition for NR NRM in clause 4 and information model definition for 5GC NRM in clause 5 in TS 28.541[6].

2)	If ManagedFunction instance contains virtualized part, NFMS_P checks whether corresponding NF Deployment instance needs to be modified to satisfy the network function related requirements.

3)	If corresponding NF Deployment instance needs to be modified, NFMS_P interacts with the generic orchestration and management entity over the deployment management reference point to enable the NF Deployment instance modification.

4)	NFMS_P reconfigures corresponding ManagedFunction MOI.

5)	The NFMS_P sends the response (see modifyMOIAttributes operation defined in TS 28.532 [8]) to NFMS_C.

==============End of change==============