3GPP TSG-SA5 (Telecom Management)	S5-070146

- Meeting SA5#51, 22 - 26 Jan 2007, Seville, SPAIN

| CR-Form-v9.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.405 | CR | 0023 |  rev | - |  Current version: | 7.3.0 |  |
| For HELP on using this form look at the pop-up text over the  symbols. Comprehensive instructions on how to use this form can be found at http://www.3gpp.org/specs/CR.htm. |  |  |  |  |  |  |  |  |

| Proposed change affects:  | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title:  | Distinguish Hard Handover counters for FDD and TDD mode |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG:  | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG:  | SA5 |  |  |  |  |  |  |  |  |  |
| Work item code:  | OAM7-PM |  |  |  |  |  |  | Date:  |  | 26/01/2007 |
| Category:  | F |  |  |  |  |  |  | Release:  |  | Rel-7 |
|  | Use one of the following categories: F (correction) A (corresponds to a correction in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: R97 (Release 1997) R98 (Release 1998) R99 (Release 1999) Rel-4 (Release 4) Rel-5 (Release 5) Rel-6 (Release 6) Rel-7 (Release 7) Rel-8 (Release 8) |  |
| Reason for change:  |  | Inter-RNC hard handover in TDD network is switched in CN associated with SRNS relocation, and it will not proceed through Iur interface. |  |  |  |  |  |  |  |  |
| Summary of change:  |  | Constrain the applicability of hard handover performance measurements related to Iur interface to FDD mode only. |  |  |  |  |  |  |  |  |
| Consequences if  not approved: |  | Some performance measurements in the TS are not applicable to TDD mode, while there is no explicit constraint. |  |  |  |  |  |  |  |  |
| Clauses affected:  |  | Clause 4.10.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs  |  |  | x | Other core specifications  |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
|  |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments:  |  |  |  |  |  |  |  |  |  |  |

1st Modified Section

### 4.10.4	Outgoing inter-RNC hard handovers via Iur

The three measurement types defined in the subclause 4.10.4 for outgoing inter-RNC hard handovers are subject to the "2 out of 3 approach".

#### 4.10.4.1	Attempted outgoing inter-RNC hard handovers via Iur

- This measurement provides the number of attempted outgoing inter-RNC hard handovers via Iur. This measurement is only valid for FDD mode.

- CC.

- Transmission of a RRC message PHYSICAL CHANNEL RECONFIGURATION, RADIO BEARER SETUP, RADIO BEARER RECONFIGURATION, RADIO BEARER RELEASE, or TRANSPORT CHANNEL RECONFIGURATION from the source RNC to the UE, indicating the attempt of an outgoing inter-RNC hard handover via Iur (see TS 25.331 [4]).

- A single integer value.

- HHO.AttOutInterRNCIur.

- UtranCell.

- Valid for circuit switched and packet switched traffic.

- UMTS.

#### 4.10.4.2	Successful outgoing inter-RNC hard handovers via Iur

- This measurement provides the number of successful outgoing inter-RNC hard handovers via Iur. This measurement is only valid for FDD mode.

- CC.

- Receipt of a RRC message PHYSICAL CHANNEL RECONFIGURATION COMPLETE, RADIO BEARER SETUP COMPLETE, RADIO BEARER RECONFIGURATION COMPLETE, RADIO BEARER RELEASE COMPLETE, or TRANSPORT CHANNEL RECONFIGURATION COMPLETE sent from the UE to the source RNC, indicating a successful outgoing inter-RNC hard handover via Iur (see TS 25.331 [4]).

- A single integer value.

- HHO.SuccOutInterRNCIur.

- UtranCell.

- Valid for circuit switched and packet switched traffic.

- UMTS.

#### 4.10.4.3	Failed outgoing inter-RNC hard handovers via Iur

- This measurement provides the number of failed outgoing inter-RNC hard handovers via Iur per cause. This measurement is only valid for FDD mode.

- CC.

- Receipt of a RRC message PHYSICAL CHANNEL RECONFIGURATION FAILURE, RADIO BEARER SETUP FAILURE, RADIO BEARER RECONFIGURATION FAILURE, RADIO BEARER RELEASE FAILURE, or TRANSPORT CHANNEL RECONFIGURATION FAILURE sent from the UE to the source RNC, indicating a failed outgoing inter-RNC hard handover via Iur. Failure causes are defined within TS 25.331 [4].

Each expected RRC message PHYSICAL CHANNEL RECONFIGURATION COMPLETE, RADIO BEARER SETUP COMPLETE, RADIO BEARER RECONFIGURATION COMPLETE, RADIO BEARER RELEASE COMPLETE, TRANSPORT CHANNEL RECONFIGURATION COMPLETE, PHYSICAL CHANNEL RECONFIGURATION FAILURE, RADIO BEARER SETUP FAILURE, RADIO BEARER RECONFIGURATION FAILURE, RADIO BEARER RELEASE FAILURE, or TRANSPORT CHANNEL RECONFIGURATION FAILURE not received by the source RNC is added to the measurement cause 'No Reply' (not specified in TS 25.331 [4]).

The sum of all supported per cause measurements shall equal the total number of failed events. In case only a subset of per cause measurements is supported, a sum subcounter will be provided first.

- Each measurement is an integer value. The number of measurements is equal to the number of causes supported plus a possible sum value identified by the .sum suffix.

- The measurement name has the form HHO.FailOutInterRNCIur.Cause 
where Cause identifies the failure cause.
The cause 'No Reply' is identified by the .NoReply suffix.

- UtranCell.

- Valid for circuit switched and packet switched traffic.

- UMTS

End of modifications

Annex A (informative):
Change history

| Change history |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Date | TSG # | TSG Doc. | CR | Rev | Subject/Comment | Cat | Old | New |
| Mar 2006 | SA_31 | SP-060108 | -- | -- | Split of 32.403-710 into four new TSs 32.404, 32.405, 32.406, 32.408. Submitted to TSG SA #31 for Approval. | -- | 1.0.0 | 7.0.0 |
| Jun 2006 | SA_32 | SP-060260 | 0001 | -- | Add TDD Counters to Measure Mean Code Resources Used | B | 7.0.0 | 7.1.0 |
| Jun 2006 | SA_32 | SP-060260 | 0002 | -- | Add ATM layer performance measurements | B | 7.0.0 | 7.1.0 |
| Jun 2006 | SA_32 | SP-060260 | 0003 | -- | Add Power Counters for radio channel monitoring | B | 7.0.0 | 7.1.0 |
| Jun 2006 | SA_32 | SP-060260 | 0004 | -- | Add measurements about Radio Access Bearer (RAB) related to Iu release request | B | 7.0.0 | 7.1.0 |
| Jun 2006 | SA_32 | SP-060260 | 0005 | -- | Add paging counters for UTRAN | B | 7.0.0 | 7.1.0 |
| Jun 2006 | SA_32 | SP-060260 | 0006 | -- | Correction for wrong reference to TS 25.331 | F | 7.0.0 | 7.1.0 |
| Jun 2006 | SA_32 | SP-060260 | 0007 | -- | Correction for TDD Counters to Measure Max Code Resources Used | F | 7.0.0 | 7.1.0 |
| Jun 2006 | SA_32 | SP-060260 | 0008 | -- | Add measurements about SHO radio link performance | B | 7.0.0 | 7.1.0 |
| Sep 2006 | SA_33 | SP-060663 | 0009 | 1 | Add RRC Connection Usage Measurements to fully gauge the usage of the UTRAN | B | 7.1.0 | 7.2.0 |
| Sep 2006 | SA_33 | SP-060663 | 0010 | 1 | Correction to TDD measurements for Received Total Wideband Power and Transmitted Carrier Power | F | 7.1.0 | 7.2.0 |
| Sep 2006 | SA_33 | SP-060550 | 0011 | -- | Add measurements about RAB with different data rates of conversational service - Align with RAN2's 25.993 | B | 7.1.0 | 7.2.0 |
| Sep 2006 | SA_33 | SP-060550 | 0012 | -- | Add measurements about the non-HS carrier power - Align with RAN3's 25.433 | B | 7.1.0 | 7.2.0 |
| Sep 2006 | SA_33 | SP-060550 | 0013 | -- | Add Uplink ISCP measurements in PM for TDD | B | 7.1.0 | 7.2.0 |
| Sep 2006 | SA_33 | SP-060550 | 0014 | -- | Add measurements about the non-HS carrier power for TDD | B | 7.1.0 | 7.2.0 |
| Sep 2006 | SA_33 | SP-060550 | 0015 | -- | Constrain the applicability of performance measurements related to Iur interface | C | 7.1.0 | 7.2.0 |
| Sep 2006 | SA_33 | SP-060550 | 0016 | -- | Add measurements about call duration in utran cell for CS domain | B | 7.1.0 | 7.2.0 |
| Sep 2006 | SA_33 | SP-060550 | 0017 | -- | Add measurements related to channel switches for HSDPA | B | 7.1.0 | 7.2.0 |
| Sep 2006 | SA_33 | SP-060550 | 0018 | -- | Add measurements about the HSDPA mobility management | B | 7.1.0 | 7.2.0 |
| Sep 2006 | SA_33 | SP-060550 | 0019 | -- | Add HSDPA setup related measurements | B | 7.1.0 | 7.2.0 |
| Dec 2006 | SA_34 | SP-060726 | 0020 | -- | Correction of the HHO measurements | C | 7.2.0 | 7.3.0 |
| Dec 2006 | SA_34 | SP-060726 | 0021 | -- | Add Measurements related to MAC-hs | B | 7.2.0 | 7.3.0 |
| Dec 2006 | SA_34 | SP-060726 | 0022 | -- | Correct the measurements related to RAB release request by CN - Align with 25.413 | F | 7.2.0 | 7.3.0 |