3GPP TSG-SA5 (Telecom Management)	S5-070201

- Meeting SA5#51, 22 - 26 Jan 2007, Seville, SPAIN

| CR-Form-v9.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.298 | CR | 0063 |  rev | - |  Current version: | 7.1.0 |  |
| For HELP on using this form look at the pop-up text over the  symbols. Comprehensive instructions on how to use this form can be found at http://www.3gpp.org/specs/CR.htm. |  |  |  |  |  |  |  |  |

| Proposed change affects:  | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title:  | Add PoC session initiation type into PoC-information - Align with 32.272 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG:  | Huawei (Cloud.zhao@huawei.com) |  |  |  |  |  |  |  |  |  |
| Source to TSG:  | S5 |  |  |  |  |  |  |  |  |  |
| Work item code:  | CH7-PoC-CH |  |  |  |  |  |  | Date:  |  | 26/01/2007 |
| Category:  | C |  |  |  |  |  |  | Release:  |  | Rel-7 |
|  | Use one of the following categories: F (correction) A (corresponds to a correction in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: R97 (Release 1997) R98 (Release 1998) R99 (Release 1999) Rel-4 (Release 4) Rel-5 (Release 5) Rel-6 (Release 6) Rel-7 (Release 7) Rel-8 (Release 8) |  |
| Reason for change:  |  | As described in 32.272 Sec.5.1.1 (“PoC session related charging”), two session establishment types are supported, which can provide different call latency, QoE, etc. This should be considered in charging. See details in OMA-TS-PoC-ControlPlane-V1_0. |  |  |  |  |  |  |  |  |
| Summary of change:  |  | Add PoC Session Initiation Type to identify the initiation type each participant chooses when initiating session.The value type can be: Pre-established or On-demand. |  |  |  |  |  |  |  |  |
| Consequences if  not approved: |  | Inability to identify which type of session a PoC subscriber uses to initiate a PoC session. Misalignment with 32.272 |  |  |  |  |  |  |  |  |
| Clauses affected:  |  | 5.1.4.3.7A (new), 5.2.4.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs  |  |  | X | Other core specifications  |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
|  |  | X |  | O&M Specifications |  |  | 32.272, 32.299 |  |  |  |
| Other comments:  |  | Related R7 CR 32.272 in S5-070200 Related R7CR 32.299 in S5-070203 |  |  |  |  |  |  |  |  |

1st Modified Section

##### 5.1.4.3.7A	PoC session initiation type

The field is of type Enumerated. It identifies the type of the PoC session initiation.

The identifier can be one of the following:

- Pre-established

- On-demand

Next Modified Section

#### 5.2.4.3	PoC CDRs

This subclause contains the abstract syntax definitions that are specific to the CDR types defined in TS 32.272 [32].

POCChargingDataTypes {itu-t (0) identified-organization (4) etsi(0) mobileDomain (0) charging (5) pocChargingDataTypes (7) asn1Module (0) version1 (0)}

DEFINITIONS IMPLICIT TAGS	::=

BEGIN

-- EXPORTS everything

IMPORTS

CallDuration, RecordType, Diagnostics, IMSI, IMEI, IPAddress, ISDN-AddressString, LocalSequenceNumber, ManagementExtensions, MSISDN, NodeAddress, RecordingEntity, ServiceKey, TimeStamp, ServiceContextID

FROM GenericChargingDataTypes {itu-t (0) identified-organization (4) etsi(0) mobileDomain (0) charging (5) genericChargingDataTypes (0) asn1Module (0) version1 (0)}

GSNAddress, NodeID, SGSNPLMNIdentifier

FROM GPRSChargingDataTypes {itu-t (0) identified-organization (4) etsi(0) mobileDomain (0) charging (5) genericChargingDataTypes (2) asn1Module (0) version1 (0)}

IMS-Charging-Identifier, Incomplete-CDR-Indication, InterOperatorIdentifiers, InvolvedParty, MessageBody, Media-Components-List, Session-Id, SIP-Method

FROM IMSChargingDataTypes {itu-t (0) identified-organization (4) etsi(0) mobileDomain (0) charging (5) imsChargingDataTypes (4) asn1Module (0) version1 (0)}

;

------------------------------------------------------------------------------

--

--  POC RECORDS

--

------------------------------------------------------------------------------

POCRecord	::= CHOICE

{

--

-- Record values 80..81 are PoC specific

--

pPFRecord			[80] PPFRecord,

cPFRecord			[81] CPFRecord

}

PPFRecord 	::= SET

{

recordType						[0] RecordType,

retransmission					[1] NULL OPTIONAL,

sIP-Method						[2] SIP-Method	OPTIONAL,

nodeAddress						[3] NodeAddress	OPTIONAL,

session-Id						[4] Session-Id	OPTIONAL,

calling-Party-Address			[5] InvolvedParty	OPTIONAL,

called-Party-Address			[6] InvolvedParty	OPTIONAL,

servedParty					[7] GraphicString	OPTIONAL,

serviceRequestTimeStamp			[8] TimeStamp	OPTIONAL,

serviceDeliveryStartTimeStamp	[9] TimeStamp	OPTIONAL,

serviceDeliveryEndTimeStamp		[10] TimeStamp	OPTIONAL,

recordOpeningTime				[11] TimeStamp	OPTIONAL,

recordClosureTime				[12] TimeStamp OPTIONAL,

interOperatorIdentifiers		[13] InterOperatorIdentifiers OPTIONAL,

localRecordSequenceNumber		[14] LocalSequenceNumber OPTIONAL,

recordSequenceNumber			[15] INTEGER	OPTIONAL,

causeForRecordClosing			[16] CauseForRecordClosing	OPTIONAL,

incomplete-CDR-Indication		[17] Incomplete-CDR-Indication	OPTIONAL,

iMS-Charging-Identifier			[18] IMS-Charging-Identifier	OPTIONAL,

list-Of-SDP-Media-Components	[19] SEQUENCE	OF	Media-Components-List	OPTIONAL,

gGSNaddress						[20] NodeAddress	OPTIONAL,

serviceReasonReturnCode			[21] UTF8String	OPTIONAL,

list-Of-Message-Bodies			[22] SEQUENCE	OF	MessageBody	OPTIONAL,

userLocationInfo				[23] OctetString	OPTIONAl,

poCInformation					[24] POCInformation	OPTIONAL,

recordExtensions				[25] RecordExtensions	OPTIONAL,

serviceContextID				[26] ServiceContextID OPTIONAL

}

CPFRecord		::=	SET

{

recordType						[0] RecordType,

retransmission					[1] NULL	OPTIONAL,

sIP-Method						[2] SIP-Method	OPTIONAL,

nodeAddress						[3] NodeAddress	OPTIONAL,

session-Id						[4] Session-Id	OPTIONAL,

calling-Party-Address			[5] InvolvedParty	OPTIONAL,

called-Party-Address			[6] InvolvedParty	OPTIONAL,

servedParty					[7] GraphicString	OPTIONAL,

serviceRequestTimeStamp			[8] TimeStamp	OPTIONAL,

serviceDeliveryStartTimeStamp	[9] TimeStamp	OPTIONAL,

serviceDeliveryEndTimeStamp		[10] TimeStamp	OPTIONAL,

recordOpeningTime				[11] TimeStamp	OPTIONAL,

recordClosureTime				[12] TimeStamp	OPTIONAL,

interOperatorIdentifiers		[13] InterOperatorIdentifiers	OPTIONAL,

localRecordSequenceNumber		[14] LocalSequenceNumber	OPTIONAL,

recordSequenceNumber			[15] INTEGER	OPTIONAL,

causeForRecordClosing			[16] CauseForRecordClosing	OPTIONAL,

incomplete-CDR-Indication		[17] Incomplete-CDR-Indication	OPTIONAL,

iMS-Charging-Identifier			[18] IMS-Charging-Identifier	OPTIONAL,

list-Of-SDP-Media-Components	[19] SEQUENCE	OF	Media-Components-List	OPTIONAL,

gGSNaddress						[20] NodeAddress	OPTIONAL,

serviceReasonReturnCode			[21] UTF8String	OPTIONAL,

list-Of-Message-Bodies			[22] SEQUENCE	OF	MessageBody	OPTIONAL,

userLocationInfo				[23] OctetString	OPTIONAl,

poCInformation					[24] POCInformation	OPTIONAL,

recordExtensions				[25] RecordExtensions	OPTIONAL,

serviceContextID				[26] ServiceContextID OPTIONAL

}

------------------------------------------------------------------------------

--

--  PoC DATA TYPES

--

------------------------------------------------------------------------------

CauseForRecordClosing	::= ENUMERATED

{

normalRelease					(0),

abnormalRelease					(1),

serviceChange					(2), -- e.g. change in media due to Re-Invite

volumeLimit						(3),

timeLimit						(4),

numberofTalkBurstLimit			(5),

maxChangeCond					(6),

sessionTypeChange				(7),

managementIntervention			(8)

}

ChangeCondition			::= ENUMERATED

{

serviceChange					(0), -- e.g. change in media due to Re-Invite

volumeLimit						(1),

timeLimit						(2),

numberofTalkBurstLimit			(3),

numberofActiveParticipants		(4),

tariffTime						(5)

}

ListofTalkBurstExchange			::= SET

{

number-Of-Talk-Bursts				 [1] INTEGER OPTIONAL,

talk-Burst-Volume					 [2] INTEGER OPTIONAL,		-- measured in octets

talk-Bursts-Time					 [3] CallDuration OPTIONAL,

number-Of-Received-Talk-Bursts		 [4] INTEGER OPTIONAL,

received-Talk-Burst-Volume			 [5] INTEGER OPTIONAL,		-- measured in octets

received-Talk-Burst-Time			 [6] CallDuration OPTIONAL,

changeCondition						 [7] ChangeCondition OPTIONAL,

changeTime							 [8] TimeStamp,

numberofParticipants				 [9] INTEGER OPTIONAL

}

POCInformation			::= SET

{

pOCSessionType					 [1] POCSessionType OPTIONAL,

numberofParticipants			 [2] INTEGER OPTIONAL,

listofParticipants				 [3] SEQUENCE OF InvolvedParty OPTIONAL,

listofTalkBurstExchange			 [4] SEQUENCE OF ListofTalkBurstExchange OPTIONAL,

pOCControllingAddress			 [5] UTF8String OPTIONAL,

pOCGroupName					 [6] UTF8String OPTIONAL,

pOCSessionId					 [7] UTF8String OPTIONAL,

pOCSessionInitiationType		 [8] POCSessionInitType OPTIONAL

}

POCSessionInitType		::= ENUMERATED

{

pre-established			   (0),

on-demand	  			   (1)

}

POCSessionType			::= ENUMERATED

{

1-to-1session              (0),

chat-group-session         (1),

pre-arranged-group-session (2),

ad-hoc-group-session       (3)

}

End of modifications