3GPP TSG-SA5 (Telecom Management)	 S5-070213

- Meeting SA5#51, 22 - 26 Jan 2007, Seville, SPAIN

| CR-Form-v9.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.615 | CR | 0027 |  rev | - |  Current version: | 7.0.0 |  |
| For HELP on using this form look at the pop-up text over the  symbols. Comprehensive instructions on how to use this form can be found at http://www.3gpp.org/specs/CR.htm. |  |  |  |  |  |  |  |  |

| Proposed change affects:  | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title:  | Add Notification XML definition in BulkCMIRP |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG:  | Huawei, Yangli (afi@huawei.com) |  |  |  |  |  |  |  |  |  |
| Source to TSG:  | S5 |  |  |  |  |  |  |  |  |  |
| Work item code:  | OAM7-NIM |  |  |  |  |  |  | Date:  |  | 26/01/2007 |
| Category:  | B |  |  |  |  |  |  | Release:  |  | Rel-7 |
|  | Use one of the following categories: F (correction) A (corresponds to a correction in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: R97 (Release 1997) R98 (Release 1998) R99 (Release 1999) Rel-4 (Release 4) Rel-5 (Release 5) Rel-6 (Release 6) Rel-7 (Release 7) Rel-8 (Release 8) |  |
| Reason for change:  |  | Notification XML definition is missing in 32.615. |  |  |  |  |  |  |  |  |
| Summary of change:  |  | Add BulkCMIRP Notification XML Definitions |  |  |  |  |  |  |  |  |
| Consequences if  not approved: |  | Notification of BulkCMIRP can’t be logged. |  |  |  |  |  |  |  |  |
| Clauses affected:  |  | 6, Annex F |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs  |  |  | x | Other core specifications  |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
|  |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments:  |  |  |  |  |  |  |  |  |  |  |

1st Modified Section

# 6	BulkCMIRP Notification XML Definitions

This clause provides XML definitions of Notification Log IRP notifications as defined in [1].

The structure of the BulkCMIRP Notification XML Definitions is shown in graphical depiction below:

Note: The use of XML schema key word “sequence” to support IS-defined set (not sequence) is for the purpose of XML processor efficiency. This shall not imply the use of “sequence” in other technology.

Next Modified Section

Annex F (normative):
BulkCMIRP Notification XML schema (file name "bulkCMIRPNotif.xsd")

<?xml version="1.0" encoding="UTF-8"?>

<!--

3GPP TS 32.615 BulkCMIRP Notification XML Schema

bulkCMIRPNotif.xsd

-->

<schema xmlns:xb="http://www.3gpp.org/ftp/specs/archive/32_series/32.615#bulkCMIRPNotif" xmlns:xe="http://www.3gpp.org/ftp/specs/archive/32_series/32.305#notification" xmlns="http://www.w3.org/2001/XMLSchema" targetNamespace="http://www.3gpp.org/ftp/specs/archive/32_series/32.615#bulkCMIRPNotif" elementFormDefault="qualified" attributeFormDefault="unqualified">

<import namespace="http://www.3gpp.org/ftp/specs/archive/32_series/32.305#notification"/>

<simpleType name="SessionId">

<restriction base="string">

<minLength value="1"/>

</restriction>

</simpleType>

<simpleType name="SourceIndicator">

<restriction base="string">

<enumeration value="Resource Operation"/>

<enumeration value="management Operation"/>

<enumeration value="unknown"/>

</restriction>

</simpleType>

<simpleType name="SessionState">

<restriction base="string">

<enumeration value="Upload Failed"/>

<enumeration value="Upload Completed"/>

<enumeration value="Download Failed"/>

<enumeration value="Download Completed"/>

<enumeration value="Validation Failed"/>

<enumeration value="Validation Completed"/>

<enumeration value="Preactivation Failed"/>

<enumeration value="Preactivation Partly Realised"/>

<enumeration value="Preactivation Completed"/>

<enumeration value="Activation Failed"/>

<enumeration value="Activation Partly Realised"/>

<enumeration value="Activation Completed"/>

<enumeration value="Fallback Failed"/>

<enumeration value="Fallback Partly Realised"/>

<enumeration value="Fallback Completed"/>

</restriction>

</simpleType>

<simpleType name="SessionLogStatus">

<restriction base="string">

<enumeration value="GetSessionLog completed successfully"/>

<enumeration value="GetSessionLog completed unsuccessfully"/>

</restriction>

</simpleType>

<complexType name="NotifySessionStateChanged">

<complexContent>

<extension base="xe:Notification">

<sequence>

<element name="body">

<complexType>

<sequence>

<element name="SessionId" type="xb:SessionId"/>

<element name="SessionState" type="xb:SessionState"/>

<element name="SourceIndicator" type="xb:SourceIndicator" minOccurs="0"/>

</sequence>

</complexType>

</element>

</sequence>

</extension>

</complexContent>

</complexType>

<complexType name="NotifyGetSessionLogEnded">

<complexContent>

<extension base="xe:Notification">

<sequence>

<element name="body">

<complexType>

<sequence>

<element name="SessionId" type="xb:SessionId"/>

<element name="SessionLogStatus" type="xb:SessionLogStatus"/>

<element name="SourceIndicator" type="xb:SourceIndicator" minOccurs="0"/>

</sequence>

</complexType>

</element>

</sequence>

</extension>

</complexContent>

</complexType>

<element name="NotifySessionStateChanged" type="xb:NotifySessionStateChanged"/>

<element name="NotifyGetSessionLogEnded" type="xb:NotifyGetSessionLogEnded"/>

</schema>

End of modifications

Annex F (informative):
Change history

| Change history |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Date | TSG # | TSG Doc. | CR | Rev | Subject/Comment | Cat | Old | New |
| Dec 2006 | SA_34 | SP-060730 | 0026 | -- | Include IMS NRM IRP in the scope for Bulk CM IRP - Align with 32.612 | C | 6.3.0 | 7.0.0 |