3GPP TSG-SA5 (Telecom Management)	S5-070239

- Meeting SA5#51, 22 - 26 Jan 2007, Seville, SPAIN

| CR-Form-v9.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.298 | CR | 0065 |  rev | - |  Current version: | 7.1.0 |  |
| For HELP on using this form look at the pop-up text over the  symbols. Comprehensive instructions on how to use this form can be found at http://www.3gpp.org/specs/CR.htm. |  |  |  |  |  |  |  |  |

| Proposed change affects:  | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title:  | Correction of ASN.1 tags |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG:  | Ericsson, Alcatel-Lucent (patrik.teppo@ericsson.com, njmazzarella@alcatel-lucent.com) |  |  |  |  |  |  |  |  |  |
| Source to TSG:  | S5 |  |  |  |  |  |  |  |  |  |
| Work item code:  | CH |  |  |  |  |  |  | Date:  |  | 26/01/2007 |
| Category:  | A |  |  |  |  |  |  | Release:  |  | Rel-7 |
|  | Use one of the following categories: F (correction) A (corresponds to a correction in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: R97 (Release 1997) R98 (Release 1998) R99 (Release 1999) Rel-4 (Release 4) Rel-5 (Release 5) Rel-6 (Release 6) Rel-7 (Release 7) Rel-8 (Release 8) |  |
| Reason for change:  |  | Some ASN.1 tags are not inline with the tag numbers in Rel-5 and earlier. |  |  |  |  |  |  |  |  |
| Summary of change:  |  | Changes Rel-6 tags to the values in Rel-5 |  |  |  |  |  |  |  |  |
| Consequences if  not approved: |  | No backward compatibility. |  |  |  |  |  |  |  |  |
| Clauses affected:  |  | 5.2.2.1, 5.2.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs  |  |  | X | Other core specifications  |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
|  |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments:  |  | R7 Mirror CR of S5-070238 |  |  |  |  |  |  |  |  |

1st Modified Section

#### 5.2.2.1	CS domain CDRs

This subclause contains the abstract syntax definitions that are specific to the CDR types defined in TS 32.250 [10].

CSChargingDataTypes {itu-t (0) identified-organization (4) etsi(0) mobileDomain (0) charging (5) csChargingDataTypes (1) asn1Module (0) version1 (0)}  DEFINITIONS IMPLICIT TAGS	::=

BEGIN

-- EXPORTS everything

IMPORTS

CallDuration, CalledNumber, RecordType, CallingNumber, CallReferenceNumber, CellId, DefaultSMS-Handling, Diagnostics, Ext-GeographicalInformation, IMSI, IMEI, IPAddress, ISDN-AddressString, LCSCause, LCSClientExternalID, LCSClientIdentity, LCSClientInternalID, LCSClientType, LCS-Priority, LCSQoSInfo, LevelOfCAMELService, LocalSequenceNumber, LocationAreaAndCell, LocationAreaCode, LocationType, ManagementExtensions, MessageReference, MSISDN, NotificationToMSUser, PositioningData, RecordingEntity, ServiceKey, SMSResult, SmsTpDestinationNumber, TimeStamp

FROM GenericChargingDataTypes {itu-t (0) identified-organization (4) etsi(0) mobileDomain (0) charging (5) genericChargingDataTypes (0) asn1Module (0) version1 (0)}

BearerServiceCode

FROM MAP-BS-Code { itu-t identified-organization (4) etsi (0) mobileDomain (0) gsm-Network (1) modules (3) map-BS-Code (20) version6 (6) }

-- from TS 29.002 [60]

TeleserviceCode

FROM MAP-TS-Code { itu-t identified-organization (4) etsi (0) mobileDomain (0) gsm-Network (1) modules (3) map-TS-Code (19) version2 (2) }

-- from TS 29.002 [60]

SS-Code

FROM MAP-SS-Code { itu-t identified-organization (4) etsi (0) mobileDomain (0) gsm-Network (1) modules (3) map-SS-Code (15) version6 (6) }

-- from TS 29.002 [60]

MOLR-Type

FROM SS-DataTypes { itu-t identified-organization (4) etsi (0) mobileDomain (0) gsm-Access (2) modules (3) ss-DataTypes (2) version7 (7)}

-- from TS 24.080 [61]

;

------------------------------------------------------------------------------

--

--  CS CALL AND EVENT RECORDS

--

------------------------------------------------------------------------------

CSRecord	::= CHOICE

--

-- Record values 0..19 are circuit switch specific

--

{

moCallRecord			[0] MOCallRecord,

mtCallRecord			[1] MTCallRecord,

roamingRecord			[2] RoamingRecord,

incGatewayRecord		[3] IncGatewayRecord,

outGatewayRecord		[4] OutGatewayRecord,

transitRecord			[5] TransitCallRecord,

moSMSRecord				[6] MOSMSRecord,

mtSMSRecord				[7] MTSMSRecord,

moSMSIWRecord			[8] MOSMSIWRecord,

mtSMSGWRecord			[9] MTSMSGWRecord,

ssActionRecord			[10] SSActionRecord,

hlrIntRecord			[11] HLRIntRecord,

locUpdateHLRRecord		[12] LocUpdateHLRRecord,

locUpdateVLRRecord		[13] LocUpdateVLRRecord,

commonEquipRecord		[14] CommonEquipRecord,

recTypeExtensions		[15] ManagementExtensions,

termCAMELRecord			[16] TermCAMELRecord,

mtLCSRecord				[17] MTLCSRecord,

moLCSRecord				[18] MOLCSRecord,

niLCSRecord				[19] NILCSRecord

}

MOCallRecord	::= SET

{

recordType					[0] RecordType,

servedIMSI					[1] IMSI OPTIONAL,

servedIMEI					[2] IMEI OPTIONAL,

servedMSISDN				[3] MSISDN OPTIONAL,

callingNumber				[4] CallingNumber OPTIONAL,

calledNumber				[5] CalledNumber OPTIONAL,

translatedNumber			[6] TranslatedNumber OPTIONAL,

connectedNumber				[7] ConnectedNumber OPTIONAL,

roamingNumber				[8] RoamingNumber OPTIONAL,

recordingEntity				[9] RecordingEntity,

mscIncomingTKGP				[10] TrunkGroup OPTIONAL,

mscOutgoingTKGP				[11] TrunkGroup OPTIONAL,

location					[12] LocationAreaAndCell OPTIONAL,

changeOfLocation			[13] SEQUENCE OF LocationChange OPTIONAL,

basicService				[14] BasicServiceCode OPTIONAL,

transparencyIndicator		[15] TransparencyInd OPTIONAL,

changeOfService				[16] SEQUENCE OF ChangeOfService OPTIONAL,

supplServicesUsed			[17] SEQUENCE OF SuppServiceUsed OPTIONAL,

aocParameters				[18] AOCParameters OPTIONAL,

changeOfAOCParms			[19] SEQUENCE OF AOCParmChange OPTIONAL,

msClassmark					[20] Classmark OPTIONAL,

changeOfClassmark			[21] ChangeOfClassmark OPTIONAL,

seizureTime					[22] TimeStamp OPTIONAL,

answerTime					[23] TimeStamp OPTIONAL,

releaseTime					[24] TimeStamp OPTIONAL,

callDuration				[25] CallDuration,

dataVolume					[26] DataVolume OPTIONAL,

radioChanRequested			[27] RadioChanRequested OPTIONAL,

radioChanUsed				[28] TrafficChannel OPTIONAL,

changeOfRadioChan			[29] ChangeOfRadioChannel OPTIONAL,

causeForTerm				[30] CauseForTerm,

diagnostics					[31] Diagnostics OPTIONAL,

callReference				[32] CallReference,

sequenceNumber				[33] INTEGER OPTIONAL,

additionalChgInfo			[34] AdditionalChgInfo OPTIONAL,

recordExtensions			[35] ManagementExtensions OPTIONAL,

gsm-SCFAddress				[36] Gsm-SCFAddress OPTIONAL,

serviceKey					[37] ServiceKey OPTIONAL,

networkCallReference		[38] NetworkCallReference OPTIONAL,

mSCAddress					[39] MSCAddress OPTIONAL,

cAMELInitCFIndicator		[40] CAMELInitCFIndicator OPTIONAL,

defaultCallHandling			[41] DefaultCallHandling OPTIONAL,

hSCSDChanRequested			[42] NumOfHSCSDChanRequested OPTIONAL,

hSCSDChanAllocated			[43] NumOfHSCSDChanAllocated OPTIONAL,

changeOfHSCSDParms			[44] SEQUENCE OF HSCSDParmsChange OPTIONAL,

fnur						[45] Fnur OPTIONAL,

aiurRequested				[46] AiurRequested OPTIONAL,

chanCodingsAcceptable		[47] SEQUENCE OF ChannelCoding OPTIONAL,

chanCodingUsed				[48] ChannelCoding OPTIONAL,

speechVersionSupported		[49] SpeechVersionIdentifier OPTIONAL,

speechVersionUsed			[50] SpeechVersionIdentifier OPTIONAL,

numberOfDPEncountered		[51] INTEGER OPTIONAL,

levelOfCAMELService			[52] LevelOfCAMELService OPTIONAL,

freeFormatData				[53] FreeFormatData OPTIONAL,

cAMELCallLegInformation		[54] SEQUENCE OF CAMELInformation OPTIONAL,

freeFormatDataAppend		[55] BOOLEAN OPTIONAL,

defaultCallHandling-2		[56] DefaultCallHandling OPTIONAL,

gsm-SCFAddress-2			[57] Gsm-SCFAddress OPTIONAL,

serviceKey-2				[58] ServiceKey OPTIONAL,

freeFormatData-2			[59] FreeFormatData OPTIONAL,

freeFormatDataAppend-2		[60] BOOLEAN OPTIONAL,

systemType					[61] SystemType OPTIONAL,

rateIndication				[62] RateIndication OPTIONAL,

locationRoutNum				[63] LocationRoutingNumber OPTIONAL,

lrnSoInd					[64] LocationRoutingNumberSourceIndicator OPTIONAL,

lrnQuryStatus				[65] LocationRoutingNumberQueryStatus OPTIONAL,

jIPPara						[66] JurisdictionInformationParameter OPTIONAL,

jIPSoInd					[67] JurisdictionInformationParameterSourceIndicator OPTIONAL,

jIPQuryStatus				[68] JurisdictionInformationParameterQueryStatus OPTIONAL,

partialRecordType			[69] PartialRecordType OPTIONAL,

guaranteedBitRate			[70] GuaranteedBitRate OPTIONAL,

maximumBitRate				[71] MaximumBitRate OPTIONAL,

redial						[72] BOOLEAN OPTIONAL,	-- set indicates redial attempt

reasonForServiceChange		[73] ReasonForServiceChange OPTIONAL,

serviceChangeInitiator		[74] BOOLEAN OPTIONAL

}

MTCallRecord			::= SET

{

recordType				[0] RecordType,

servedIMSI				[1] IMSI,

servedIMEI				[2] IMEI OPTIONAL,

servedMSISDN			[3] CalledNumber OPTIONAL,

callingNumber			[4] CallingNumber OPTIONAL,

connectedNumber			[5] ConnectedNumber OPTIONAL,

recordingEntity			[6] RecordingEntity,

mscIncomingTKGP			[7] TrunkGroup OPTIONAL,

mscOutgoingTKGP			[8] TrunkGroup OPTIONAL,

location				[9] LocationAreaAndCell OPTIONAL,

changeOfLocation		[10] SEQUENCE OF LocationChange OPTIONAL,

basicService			[11] BasicServiceCode OPTIONAL,

transparencyIndicator	[12] TransparencyInd OPTIONAL,

changeOfService			[13] SEQUENCE OF ChangeOfService OPTIONAL,

supplServicesUsed		[14] SEQUENCE OF SuppServiceUsed OPTIONAL,

aocParameters			[15] AOCParameters OPTIONAL,

changeOfAOCParms		[16] SEQUENCE OF AOCParmChange OPTIONAL,

msClassmark				[17] Classmark OPTIONAL,

changeOfClassmark		[18] ChangeOfClassmark OPTIONAL,

seizureTime				[19] TimeStamp OPTIONAL,

answerTime				[20] TimeStamp OPTIONAL,

releaseTime				[21] TimeStamp OPTIONAL,

callDuration			[22] CallDuration,

dataVolume				[23] DataVolume OPTIONAL,

radioChanRequested		[24] RadioChanRequested OPTIONAL,

radioChanUsed			[25] TrafficChannel OPTIONAL,

changeOfRadioChan		[26] ChangeOfRadioChannel OPTIONAL,

causeForTerm			[27] CauseForTerm,

diagnostics				[28] Diagnostics OPTIONAL,

callReference			[29] CallReference,

sequenceNumber			[30] INTEGER OPTIONAL,

additionalChgInfo		[31] AdditionalChgInfo OPTIONAL,

recordExtensions		[32] ManagementExtensions OPTIONAL,

networkCallReference	[33] NetworkCallReference OPTIONAL,

mSCAddress				[34] MSCAddress OPTIONAL,

hSCSDChanRequested		[35] NumOfHSCSDChanRequested OPTIONAL,

hSCSDChanAllocated		[36] NumOfHSCSDChanAllocated OPTIONAL,

changeOfHSCSDParms		[37] SEQUENCE OF HSCSDParmsChange OPTIONAL,

fnur					[38] Fnur OPTIONAL,

aiurRequested			[39] AiurRequested OPTIONAL,

chanCodingsAcceptable	[40] SEQUENCE OF ChannelCoding OPTIONAL,

chanCodingUsed			[41] ChannelCoding OPTIONAL,

speechVersionSupported	[42] SpeechVersionIdentifier OPTIONAL,

speechVersionUsed		[43] SpeechVersionIdentifier OPTIONAL,

gsm-SCFAddress			[44] Gsm-SCFAddress OPTIONAL,

serviceKey				[45] ServiceKey OPTIONAL,

systemType				[46] SystemType OPTIONAL,

rateIndication			[47] RateIndication OPTIONAL,

locationRoutNum			[48] LocationRoutingNumber OPTIONAL,

lrnSoInd				[49] LocationRoutingNumberSourceIndicator OPTIONAL,

lrnQuryStatus			[50] LocationRoutingNumberQueryStatus OPTIONAL,

jIPPara					[51] JurisdictionInformationParameter OPTIONAL,

jIPSoInd				[52] JurisdictionInformationParameterSourceIndicator OPTIONAL,

jIPQuryStatus			[53] JurisdictionInformationParameterQueryStatus OPTIONAL,

partialRecordType		[54] PartialRecordType OPTIONAL,

guaranteedBitRate		[55] GuaranteedBitRate OPTIONAL,

maximumBitRate			[56] MaximumBitRate OPTIONAL,

reasonForServiceChange	[57] ReasonForServiceChange OPTIONAL,

serviceChangeInitiator	[58] BOOLEAN OPTIONAL

}

RoamingRecord			::= SET

{

recordType				[0] RecordType,

servedIMSI				[1] IMSI,

servedMSISDN			[2] MSISDN OPTIONAL,

callingNumber			[3] CallingNumber OPTIONAL,

roamingNumber			[4] RoamingNumber OPTIONAL,

recordingEntity			[5] RecordingEntity,

mscIncomingTKGP			[6] TrunkGroup OPTIONAL,

mscOutgoingTKGP			[7] TrunkGroup OPTIONAL,

basicService			[8] BasicServiceCode OPTIONAL,

transparencyIndicator	[9] TransparencyInd OPTIONAL,

changeOfService			[10] SEQUENCE OF ChangeOfService OPTIONAL,

supplServicesUsed		[11] SEQUENCE OF  SuppServiceUsed OPTIONAL,

seizureTime				[12] TimeStamp OPTIONAL,

answerTime				[13] TimeStamp OPTIONAL,

releaseTime				[14] TimeStamp OPTIONAL,

callDuration			[15] CallDuration,

dataVolume				[16] DataVolume OPTIONAL,

causeForTerm			[17] CauseForTerm,

diagnostics				[18] Diagnostics OPTIONAL,

callReference			[19] CallReference,

sequenceNumber			[20] INTEGER OPTIONAL,

recordExtensions		[21] ManagementExtensions OPTIONAL,

networkCallReference	[22] NetworkCallReference OPTIONAL,

mSCAddress				[23] MSCAddress OPTIONAL,

locationRoutNum			[24] LocationRoutingNumber OPTIONAL,

lrnSoInd				[25] LocationRoutingNumberSourceIndicator OPTIONAL,

lrnQuryStatus			[26] LocationRoutingNumberQueryStatus OPTIONAL,

jIPPara					[27] JurisdictionInformationParameter OPTIONAL,

jIPSoInd				[28] JurisdictionInformationParameterSourceIndicator OPTIONAL,

jIPQuryStatus			[29] JurisdictionInformationParameterQueryStatus OPTIONAL,

partialRecordType		[30] PartialRecordType OPTIONAL

}

TermCAMELRecord	::= SET

{

recordtype					[0] RecordType,

servedIMSI					[1] IMSI,

servedMSISDN				[2] MSISDN OPTIONAL,

recordingEntity				[3] RecordingEntity,

interrogationTime			[4] TimeStamp,

destinationRoutingAddress	[5] DestinationRoutingAddress,

gsm-SCFAddress				[6] Gsm-SCFAddress,

serviceKey					[7] ServiceKey,

networkCallReference		[8] NetworkCallReference OPTIONAL,

mSCAddress					[9] MSCAddress OPTIONAL,

defaultCallHandling			[10] DefaultCallHandling OPTIONAL,

recordExtensions			[11] ManagementExtensions OPTIONAL,

calledNumber				[12] CalledNumber,

callingNumber				[13] CallingNumber OPTIONAL,

mscIncomingTKGP				[14] TrunkGroup OPTIONAL,

mscOutgoingTKGP				[15] TrunkGroup OPTIONAL,

seizureTime					[16] TimeStamp OPTIONAL,

answerTime					[17] TimeStamp OPTIONAL,

releaseTime					[18] TimeStamp OPTIONAL,

callDuration				[19] CallDuration,

dataVolume					[20] DataVolume OPTIONAL,

causeForTerm				[21] CauseForTerm,

diagnostics					[22] Diagnostics OPTIONAL,

callReference				[23] CallReference,

sequenceNumber				[24] INTEGER OPTIONAL,

numberOfDPEncountered		[25] INTEGER OPTIONAL,

levelOfCAMELService			[26] LevelOfCAMELService OPTIONAL,

freeFormatData				[27] FreeFormatData OPTIONAL,

cAMELCallLegInformation		[28] SEQUENCE OF CAMELInformation OPTIONAL,

freeFormatDataAppend		[29] BOOLEAN OPTIONAL,

defaultCallHandling-2		[30] DefaultCallHandling OPTIONAL,

gsm-SCFAddress-2			[31] Gsm-SCFAddress OPTIONAL,

serviceKey-2				[32] ServiceKey OPTIONAL,

freeFormatData-2			[33] FreeFormatData OPTIONAL,

freeFormatDataAppend-2		[34] BOOLEAN OPTIONAL,

mscServerIndication			[35] BOOLEAN OPTIONAL,

locationRoutNum				[36] LocationRoutingNumber OPTIONAL,

lrnSoInd					[37] LocationRoutingNumberSourceIndicator OPTIONAL,

lrnQuryStatus				[38] LocationRoutingNumberQueryStatus OPTIONAL,

jIPPara						[39] JurisdictionInformationParameter OPTIONAL,

jIPSoInd					[40] JurisdictionInformationParameterSourceIndicator OPTIONAL,

jIPQuryStatus				[41] JurisdictionInformationParameterQueryStatus OPTIONAL,

partialRecordType			[42] PartialRecordType OPTIONAL

}

IncGatewayRecord		::= SET

{

recordType				[0] RecordType,

callingNumber			[1] CallingNumber OPTIONAL,

calledNumber			[2] CalledNumber,

recordingEntity			[3] RecordingEntity,

mscIncomingTKGP			[4] TrunkGroup OPTIONAL,

mscOutgoingTKGP			[5] TrunkGroup OPTIONAL,

seizureTime				[6] TimeStamp OPTIONAL,

answerTime				[7] TimeStamp OPTIONAL,

releaseTime				[8] TimeStamp OPTIONAL,

callDuration			[9] CallDuration,

dataVolume				[10] DataVolume OPTIONAL,

causeForTerm			[11] CauseForTerm,

diagnostics				[12] Diagnostics OPTIONAL,

callReference			[13] CallReference,

sequenceNumber			[14] INTEGER OPTIONAL,

recordExtensions		[15] ManagementExtensions OPTIONAL,

locationRoutNum			[16] LocationRoutingNumber OPTIONAL,

lrnSoInd				[17] LocationRoutingNumberSourceIndicator OPTIONAL,

lrnQuryStatus			[18] LocationRoutingNumberQueryStatus OPTIONAL,

jIPPara					[19] JurisdictionInformationParameter OPTIONAL,

jIPSoInd					[20] JurisdictionInformationParameterSourceIndicator OPTIONAL,

jIPQuryStatus			[21] JurisdictionInformationParameterQueryStatus OPTIONAL,

reasonForServiceChange	[22] ReasonForServiceChange OPTIONAL,

serviceChangeInitiator	[23] BOOLEAN OPTIONAL}

OutGatewayRecord		::= SET

{

recordType				[0] RecordType,

callingNumber			[1] CallingNumber OPTIONAL,

calledNumber			[2] CalledNumber,

recordingEntity			[3] RecordingEntity,

mscIncomingTKGP			[4] TrunkGroup OPTIONAL,

mscOutgoingTKGP			[5] TrunkGroup OPTIONAL,

seizureTime				[6] TimeStamp OPTIONAL,

answerTime				[7] TimeStamp OPTIONAL,

releaseTime				[8] TimeStamp OPTIONAL,

callDuration			[9] CallDuration,

dataVolume				[10] DataVolume OPTIONAL,

causeForTerm			[11] CauseForTerm,

diagnostics				[12] Diagnostics OPTIONAL,

callReference			[13] CallReference,

sequenceNumber			[14] INTEGER OPTIONAL,

recordExtensions		[15] ManagementExtensions OPTIONAL,

locationRoutNum			[16] LocationRoutingNumber OPTIONAL,

lrnSoInd				[17] LocationRoutingNumberSourceIndicator OPTIONAL,

lrnQuryStatus			[18] LocationRoutingNumberQueryStatus OPTIONAL,

jIPPara					[19] JurisdictionInformationParameter OPTIONAL,

jIPSoInd				[20] JurisdictionInformationParameterSourceIndicator OPTIONAL,

jIPQuryStatus			[21] JurisdictionInformationParameterQueryStatus OPTIONAL,

reasonForServiceChange	[22] ReasonForServiceChange OPTIONAL,

serviceChangeInitiator	[23] BOOLEAN OPTIONAL
}

TransitCallRecord		::= SET

{

recordType				[0] RecordType,

recordingEntity			[1] RecordingEntity,

mscIncomingTKGP			[2] TrunkGroup OPTIONAL,

mscOutgoingTKGP			[3] TrunkGroup OPTIONAL,

callingNumber			[4] CallingNumber OPTIONAL,

calledNumber			[5] CalledNumber,

isdnBasicService		[6] BasicService OPTIONAL,

seizureTimestamp		[7] TimeStamp OPTIONAL,

answerTimestamp			[8] TimeStamp OPTIONAL,

releaseTimestamp		[9] TimeStamp OPTIONAL,

callDuration			[10] CallDuration,

dataVolume				[11] DataVolume OPTIONAL,

causeForTerm			[12] CauseForTerm,

diagnostics				[13] Diagnostics OPTIONAL,

callReference			[14] CallReference,

sequenceNumber			[15] INTEGER OPTIONAL,

recordExtensions		[16] ManagementExtensions OPTIONAL,

locationRoutNum			[17] LocationRoutingNumber OPTIONAL,

lrnSoInd				[18] LocationRoutingNumberSourceIndicator OPTIONAL,

lrnQuryStatus			[19] LocationRoutingNumberQueryStatus OPTIONAL,

jIPPara					[20] JurisdictionInformationParameter OPTIONAL,

jIPSoInd				[21] JurisdictionInformationParameterSourceIndicator OPTIONAL,

jIPQuryStatus			[22] JurisdictionInformationParameterQueryStatus OPTIONAL

}

MOSMSRecord				::= SET

{

recordType				[0] RecordType,

servedIMSI				[1] IMSI,

servedIMEI				[2] IMEI OPTIONAL,

servedMSISDN			[3] MSISDN OPTIONAL,

msClassmark				[4] Classmark,

serviceCentre			[5] AddressString,

recordingEntity			[6] RecordingEntity,

location				[7] LocationAreaAndCell OPTIONAL,

messageReference		[8] MessageReference,

originationTime			[9] TimeStamp,

smsResult				[10] SMSResult OPTIONAL,

recordExtensions		[11] ManagementExtensions OPTIONAL,

destinationNumber		[12] SmsTpDestinationNumber OPTIONAL,

cAMELSMSInformation		[13] CAMELSMSInformation OPTIONAL,

systemType				[14] SystemType OPTIONAL

}

MTSMSRecord				::= SET

{

recordType				[0] RecordType,

serviceCentre			[1] AddressString,

servedIMSI				[2] IMSI,

servedIMEI				[3] IMEI OPTIONAL,

servedMSISDN			[4] MSISDN OPTIONAL,

msClassmark				[5] Classmark,

recordingEntity			[6] RecordingEntity,

location				[7] LocationAreaAndCell OPTIONAL,

deliveryTime			[8] TimeStamp,

smsResult				[9] SMSResult OPTIONAL,

recordExtensions		[10] ManagementExtensions OPTIONAL,

systemType				[11] SystemType OPTIONAL,

cAMELSMSInformation		[12] CAMELSMSInformation OPTIONAL

}

MOSMSIWRecord			::= SET

{

recordType			[0] RecordType,

serviceCentre		[1] AddressString,

servedIMSI			[2] IMSI,

recordingEntity		[3] RecordingEntity,

eventTime			[4] TimeStamp,

smsResult			[5] SMSResult OPTIONAL,

recordExtensions	[6] ManagementExtensions OPTIONAL

}

MTSMSGWRecord			::= SET

{

recordType			[0] RecordType,

serviceCentre		[1] AddressString,

servedIMSI			[2] IMSI,

servedMSISDN		[3] MSISDN OPTIONAL,

recordingEntity		[4] RecordingEntity,

eventTime			[5] TimeStamp,

smsResult			[6] SMSResult OPTIONAL,

recordExtensions	[7] ManagementExtensions OPTIONAL

}

SSActionRecord			::= SET

{

recordType			[0] RecordType,

servedIMSI			[1] IMSI,

servedIMEI			[2] IMEI OPTIONAL,

servedMSISDN		[3] MSISDN OPTIONAL,

msClassmark			[4] Classmark,

recordingEntity		[5] RecordingEntity,

location			[6] LocationAreaAndCell OPTIONAL,

basicServices		[7] BasicServices OPTIONAL,

supplService		[8] SS-Code OPTIONAL,

ssAction			[9] SSActionType OPTIONAL,

ssActionTime		[10] TimeStamp,

ssParameters		[11] SSParameters OPTIONAL,

ssActionResult		[12] SSActionResult OPTIONAL,

callReference		[13] CallReference,

recordExtensions	[14] ManagementExtensions OPTIONAL,

systemType			[15] SystemType OPTIONAL

}

HLRIntRecord			::= SET

{

recordType				[0] RecordType,

servedIMSI				[1] IMSI,

servedMSISDN			[2] MSISDN,

recordingEntity			[3] RecordingEntity,

basicService			[4] BasicServiceCode OPTIONAL,

routingNumber			[5] RoutingNumber,

interrogationTime		[6] TimeStamp,

numberOfForwarding		[7] NumberOfForwarding OPTIONAL,

interrogationResult		[8] HLRIntResult OPTIONAL,

recordExtensions		[9] ManagementExtensions OPTIONAL

}

Editor’s note:  clarify if the above is really still a CS specific record

LocUpdateHLRRecord 		::= SET

{

recordType				[0] RecordType,

servedIMSI				[1] IMSI,

recordingEntity			[2] RecordingEntity,

oldLocation				[3] Visited-Location-info OPTIONAL,

newLocation				[4] Visited-Location-info,

updateTime				[5] TimeStamp,

updateResult			[6] LocUpdResult OPTIONAL,

recordExtensions		[7] ManagementExtensions OPTIONAL

}

LocUpdateVLRRecord 		::= SET

{

recordType				[0] RecordType,

servedIMSI				[1] IMSI,

servedMSISDN			[2] MSISDN OPTIONAL,

recordingEntity			[3] RecordingEntity,

oldLocation				[4] Location-info OPTIONAL,

newLocation				[5] Location-info,

msClassmark				[6] Classmark,

updateTime				[7] TimeStamp,

updateResult			[8] LocUpdResult OPTIONAL,

recordExtensions		[9] ManagementExtensions OPTIONAL

}

CommonEquipRecord 		::= SET

{

recordType				[0] RecordType,

equipmentType			[1] EquipmentType,

equipmentId				[2] EquipmentId,

servedIMSI				[3] IMSI,

servedMSISDN			[4] MSISDN OPTIONAL,

recordingEntity			[5] RecordingEntity,

basicService			[6] BasicServiceCode OPTIONAL,

changeOfService			[7] SEQUENCE OF ChangeOfService OPTIONAL,

supplServicesUsed		[8] SEQUENCE OF SuppServiceUsed OPTIONAL,

seizureTime				[9] TimeStamp,

releaseTime				[10] TimeStamp OPTIONAL,

callDuration			[11] CallDuration,

callReference			[12] CallReference,

sequenceNumber			[13] INTEGER OPTIONAL,

recordExtensions		[14] ManagementExtensions OPTIONAL,

systemType				[15] SystemType OPTIONAL,

rateIndication			[16] RateIndication OPTIONAL,

fnur					[17] Fnur OPTIONAL

}

------------------------------------------------------------------------------

--

--  OBSERVED IMEI TICKETS

--

------------------------------------------------------------------------------

ObservedIMEITicket		::= SET

{

servedIMEI			[0] IMEI,

imeiStatus			[1] IMEIStatus,

servedIMSI			[2] IMSI,

servedMSISDN		[3] MSISDN OPTIONAL,

recordingEntity		[4] RecordingEntity,

eventTime			[5] TimeStamp,

location			[6] LocationAreaAndCell	,

imeiCheckEvent		[7] IMEICheckEvent OPTIONAL,

callReference		[8] CallReference OPTIONAL,

recordExtensions	[9] ManagementExtensions OPTIONAL

}

------------------------------------------------------------------------------

--

--  CS LOCATION SERVICE RECORDS

--

------------------------------------------------------------------------------

MTLCSRecord				::= SET

{

recordType				[0] RecordType,

recordingEntity			[1] RecordingEntity,

lcsClientType			[2] LCSClientType,

lcsClientIdentity		[3] LCSClientIdentity,

servedIMSI				[4] IMSI,

servedMSISDN			[5] MSISDN OPTIONAL,

locationType			[6] LocationType,

lcsQos					[7] LCSQoSInfo OPTIONAL,

lcsPriority				[8] LCS-Priority OPTIONAL,

mlc-Number				[9] ISDN-AddressString,

eventTimeStamp			[10] TimeStamp,

measureDuration			[11] CallDuration OPTIONAL,

notificationToMSUser	[12] NotificationToMSUser OPTIONAL,

privacyOverride			[13] NULL OPTIONAL,

location				[14] LocationAreaAndCell OPTIONAL,

locationEstimate		[15] Ext-GeographicalInformation OPTIONAL,

positioningData			[16] PositioningData OPTIONAL,

lcsCause				[17] LCSCause OPTIONAL,

diagnostics				[18] Diagnostics OPTIONAL,

systemType				[19] SystemType OPTIONAL,

recordExtensions		[20] ManagementExtensions OPTIONAL,

causeForTerm			[21] CauseForTerm

}

MOLCSRecord				::= SET

{

recordType				[0] RecordType,

recordingEntity			[1] RecordingEntity,

lcsClientType			[2] LCSClientType OPTIONAL,

lcsClientIdentity		[3] LCSClientIdentity OPTIONAL,

servedIMSI				[4] IMSI,

servedMSISDN			[5] MSISDN OPTIONAL,

molr-Type				[6] MOLR-Type,

lcsQos					[7] LCSQoSInfo OPTIONAL,

lcsPriority				[8] LCS-Priority OPTIONAL,

mlc-Number				[9] ISDN-AddressString OPTIONAL,

eventTimeStamp			[10] TimeStamp,

measureDuration			[11] CallDuration OPTIONAL,

location				[12] LocationAreaAndCell OPTIONAL,

locationEstimate		[13] Ext-GeographicalInformation OPTIONAL,

positioningData			[14] PositioningData OPTIONAL,

lcsCause				[15] LCSCause OPTIONAL,

diagnostics				[16] Diagnostics OPTIONAL,

systemType				[17] SystemType OPTIONAL,

recordExtensions		[18] ManagementExtensions OPTIONAL,

causeForTerm			[19] CauseForTerm

}

NILCSRecord				::= SET

{

recordType				[0] RecordType,

recordingEntity			[1] RecordingEntity,

lcsClientType			[2] LCSClientType OPTIONAL,

lcsClientIdentity		[3] LCSClientIdentity OPTIONAL,

servedIMSI				[4] IMSI OPTIONAL,

servedMSISDN			[5] MSISDN OPTIONAL,

servedIMEI				[6] IMEI OPTIONAL,

emsDigits				[7] ISDN-AddressString OPTIONAL,

emsKey					[8] ISDN-AddressString OPTIONAL,

lcsQos					[9] LCSQoSInfo OPTIONAL,

lcsPriority				[10] LCS-Priority OPTIONAL,

mlc-Number				[11] ISDN-AddressString OPTIONAL,

eventTimeStamp			[12] TimeStamp,

measureDuration			[13] CallDuration OPTIONAL,

location				[14] LocationAreaAndCell OPTIONAL,

locationEstimate		[15] Ext-GeographicalInformation OPTIONAL,

positioningData			[16] PositioningData OPTIONAL,

lcsCause				[17] LCSCause OPTIONAL,

diagnostics				[18] Diagnostics OPTIONAL,

systemType				[19] SystemType OPTIONAL,

recordExtensions		[20] ManagementExtensions OPTIONAL,

causeForTerm			[21] CauseForTerm

}

------------------------------------------------------------------------------

--

--  NP Fields

--

------------------------------------------------------------------------------

LocationRoutingNumber			::= OCTET STRING (SIZE (5))

--

--  The format is selected to meet the existing standards for the wireline in Telcordia

--   Belcore GR-1100-CORE, BAF Module 720.

--

LocationRoutingNumberSourceIndicator				::=	INTEGER

{

lRN-NP-Database				(1),

switchingSystemData			(2),

incomingsignaling			(3),

unknown						(9)

}

LocationRoutingNumberQueryStatus 			::=	INTEGER

{

successfulQuery				(1),

noQueryResponseMsg			(2),

queryProtocolErr			(4),

queryResponseDataErr		(5),

queryRejected				(6),

queryNotPerformed			(9),

queryUnsuccessful			(99)

}

JurisdictionInformationParameter 	::= OCTET STRING (SIZE (5))

--

-- /* JIP Parameter */

JurisdictionInformationParameterSourceIndicator 				::=	INTEGER

-- Identical to LocationRoutingNumberSourceIndicator

{

lRN-NP-Database				(1),

switchingSystemData			(2),

incomingsignaling			(3),

unknown						(9)

}

JurisdictionInformationParameterQueryStatus 			::=	INTEGER

{

successfulQuery				(1),

noQueryResponseMsg			(2),

queryProtocolErr			(4),

queryResponseDataErr		(5),

queryRejected				(6),

queryNotPerformed			(9),

queryUnsuccessful			(99)

}

------------------------------------------------------------------------------

--

--  CS DATA TYPES

--

------------------------------------------------------------------------------

AdditionalChgInfo		::= SEQUENCE

{

chargeIndicator		[0] ChargeIndicator OPTIONAL,

chargeParameters	[1] OCTET STRING OPTIONAL

}

AiurRequested			::= ENUMERATED

{

--

-- See Bearer Capability TS 24.008 [64]

-- (note that value "4" is intentionally missing

--  because it is not used in TS 24.008 [64])

--

aiur09600BitsPerSecond		(1),

aiur14400BitsPerSecond		(2),

aiur19200BitsPerSecond		(3),

aiur28800BitsPerSecond		(5),

aiur38400BitsPerSecond		(6),

aiur43200BitsPerSecond		(7),

aiur57600BitsPerSecond		(8),

aiur38400BitsPerSecond1		(9),

aiur38400BitsPerSecond2		(10),

aiur38400BitsPerSecond3		(11),

aiur38400BitsPerSecond4		(12)

}

AOCParameters			::= SEQUENCE

{

--

-- See TS 22.024 [63].

--

e1					[1] EParameter OPTIONAL,

e2					[2] EParameter OPTIONAL,

e3					[3] EParameter OPTIONAL,

e4					[4] EParameter OPTIONAL,

e5					[5] EParameter OPTIONAL,

e6					[6] EParameter OPTIONAL,

e7					[7] EParameter OPTIONAL

}

AOCParmChange			::= SEQUENCE

{

changeTime			[0] TimeStamp,

newParameters		[1] AOCParameters

}

BasicServices			::= SET OF BasicServiceCode

CallingPartyCategory	::= Category

CallType				::= INTEGER

{

mobileOriginated	(0),

mobileTerminated	(1)

}

CallTypes 				::= SET OF CallType

CAMELDestinationNumber	::= DestinationRoutingAddress

CAMELInformation		::= SET

{

cAMELDestinationNumber		[1] CAMELDestinationNumber OPTIONAL,

connectedNumber				[2] ConnectedNumber OPTIONAL,

roamingNumber				[3] RoamingNumber OPTIONAL,

mscOutgoingTKGP				[4] TrunkGroup OPTIONAL,

seizureTime					[5] TimeStamp OPTIONAL,

answerTime					[6] TimeStamp OPTIONAL,

releaseTime					[7] TimeStamp OPTIONAL,

callDuration				[8] CallDuration OPTIONAL,

dataVolume					[9] DataVolume OPTIONAL,

cAMELInitCFIndicator		[10] CAMELInitCFIndicator OPTIONAL,

causeForTerm				[11] CauseForTerm OPTIONAL,

cAMELModification			[12] ChangedParameters OPTIONAL,

freeFormatData				[13] FreeFormatData OPTIONAL,

diagnostics					[14] Diagnostics OPTIONAL,

freeFormatDataAppend		[15] BOOLEAN OPTIONAL,

freeFormatData-2			[16] FreeFormatData OPTIONAL,

freeFormatDataAppend-2		[17] BOOLEAN OPTIONAL

}

CAMELInitCFIndicator	::= ENUMERATED

{

noCAMELCallForwarding		(0),

cAMELCallForwarding			(1)

}

CAMELModificationParameters	::= SET

--

-- The list contains only parameters changed due to CAMEL call

-- handling.

--

{

callingPartyNumber			[0] CallingNumber OPTIONAL,

callingPartyCategory		[1] CallingPartyCategory OPTIONAL,

originalCalledPartyNumber	[2] OriginalCalledNumber OPTIONAL,

genericNumbers				[3] GenericNumbers OPTIONAL,

redirectingPartyNumber		[4] RedirectingNumber OPTIONAL,

redirectionCounter			[5] NumberOfForwarding OPTIONAL

}

CAMELSMSInformation		::= SET

{

gsm-SCFAddress						[1] Gsm-SCFAddress OPTIONAL,

serviceKey							[2] ServiceKey OPTIONAL,

defaultSMSHandling			 		[3] DefaultSMS-Handling OPTIONAL,

freeFormatData       				[4] FreeFormatData OPTIONAL,

callingPartyNumber 					[5] CallingNumber OPTIONAL,

destinationSubscriberNumber			[6] SmsTpDestinationNumber OPTIONAL,

cAMELSMSCAddress					[7] AddressString OPTIONAL,

smsReferenceNumber					[8] CallReferenceNumber OPTIONAL

}

Category		::= OCTET STRING (SIZE(1))

--

-- The internal structure is defined in ITU-T Recommendation Q.763.

--

CauseForTerm			::= INTEGER

--

-- Cause codes from 16 up to 31 are defined in GSM12.15 as ‘CauseForRecClosing’

-- (cause for record closing).

-- There is no direct correlation between these two types.

-- LCS related causes belong to the MAP error causes acc. TS 29.002 [60].

--

{

normalRelease						(0),

partialRecord						(1),

partialRecordCallReestablishment	(2),

unsuccessfulCallAttempt				(3),

stableCallAbnormalTermination		(4),

cAMELInitCallRelease				(5),

unauthorizedRequestingNetwork		(52),

unauthorizedLCSClient				(53),

positionMethodFailure				(54),

unknownOrUnreachableLCSClient		(58)

}

ChangedParameters		::= SET

{

changeFlags		[0] ChangeFlags,

changeList		[1] CAMELModificationParameters OPTIONAL

}

ChangeFlags				::= BIT STRING

{

callingPartyNumberModified			(0),

callingPartyCategoryModified		(1),

originalCalledPartyNumberModified	(2),

genericNumbersModified				(3),

redirectingPartyNumberModified		(4),

redirectionCounterModified			(5)

}

ChangeOfClassmark 		::= SEQUENCE

{

classmark			[0] Classmark,

changeTime			[1] TimeStamp

}

ChangeOfRadioChannel 	::= SEQUENCE

{

radioChannel		[0] TrafficChannel,

changeTime			[1] TimeStamp,

speechVersionUsed	[2] SpeechVersionIdentifier OPTIONAL

}

ChangeOfService 		::= SEQUENCE

{

basicService		[0] BasicServiceCode,

transparencyInd		[1] TransparencyInd OPTIONAL,

changeTime			[2] TimeStamp,

rateIndication		[3] RateIndication OPTIONAL,

fnur				[4] Fnur OPTIONAL

}

ChannelCoding				::= ENUMERATED

{

tchF4800					(1),

tchF9600					(2),

tchF14400				(3)

}

Classmark				::= OCTET STRING

--

-- See Mobile station classmark  2,  TS 24.008 [64]

--

ConnectedNumber			::= BCDDirectoryNumber

DataVolume				::= INTEGER

--

-- The volume of data transferred in segments of 64 octets.

--

Day						::= INTEGER (1..31)

DayClass				::= ObjectInstance

DayClasses				::= SET OF DayClass

DayDefinition			::= SEQUENCE

{

day					[0] DayOfTheWeek,

dayClass			[1] ObjectInstance

}

DayDefinitions			::= SET OF DayDefinition

DateDefinition			::= SEQUENCE

{

month				[0] Month,

day					[1] Day,

dayClass			[2] ObjectInstance

}

DateDefinitions			::= SET OF DateDefinition

DayOfTheWeek			::= ENUMERATED

{

allDays				(0),

sunday				(1),

monday				(2),

tuesday				(3),

wednesday			(4),

thursday			(5),

friday				(6),

saturday			(7)

}

Destinations			::= SET OF AE-title

EmergencyCallIndEnable	::= BOOLEAN

EmergencyCallIndication	::= SEQUENCE

{

cellId				[0] CellId,

callerId			[1] IMSIorIMEI

}

EParameter	::= INTEGER (0..1023)

--

-- Coded according to  TS 22.024 [63] and TS 24.080 [61]

--

EquipmentId				::= INTEGER

EquipmentType			::= INTEGER

{

conferenceBridge	(0)

}

FileType				::= INTEGER

{

callRecords			(1),

traceRecords		(9),

observedIMEITicket	(14)

}

Fnur							::= ENUMERATED

{

--

-- See Bearer Capability TS 24.008 [64]

--

fnurNotApplicable			(0),

fnur9600-BitsPerSecond		(1),

fnur14400BitsPerSecond		(2),

fnur19200BitsPerSecond		(3),

fnur28800BitsPerSecond		(4),

fnur38400BitsPerSecond		(5),

fnur48000BitsPerSecond		(6),

fnur56000BitsPerSecond		(7),

fnur64000BitsPerSecond		(8),

fnur33600BitsPerSecond		(9),

fnur32000BitsPerSecond			(10),

fnur31200BitsPerSecond			(11)

}

ForwardToNumber 		::= AddressString

FreeFormatData			::= OCTET STRING (SIZE(1..160))

--

-- Free formated data as sent in the FCI message

-- See TS 29.078 [66]

--

GenericNumber			::= BCDDirectoryNumber

GenericNumbers			::= SET OF GenericNumber

Gsm-SCFAddress			::= ISDN-AddressString

--

-- See TS 29.002 [60]

--

GuaranteedBitRate ::= ENUMERATED

{
     GBR14400BitsPerSecond (1),		-- BS20 non-transparent 
     GBR28800BitsPerSecond (2),		-- BS20 non-transparent and transparent,

-- BS30 transparent and multimedia
     GBR32000BitsPerSecond (3),		-- BS30 multimedia
     GBR33600BitsPerSecond (4),		-- BS30 multimedia 
     GBR56000BitsPerSecond (5),		-- BS30 transparent and multimedia 
     GBR57600BitsPerSecond (6),		-- BS20 non-transparent
     GBR64000BitsPerSecond (7)		-- BS30 transparent and multimedia
}

HLRIntResult			::= Diagnostics

HSCSDParmsChange		::= SEQUENCE

{

changeTime				[0] TimeStamp,

hSCSDChanAllocated		[1] NumOfHSCSDChanAllocated,

initiatingParty			[2] InitiatingParty OPTIONAL,

aiurRequested			[3] AiurRequested OPTIONAL,

chanCodingUsed			[4] ChannelCoding,

hSCSDChanRequested		[5] NumOfHSCSDChanRequested OPTIONAL

}

IMEICheckEvent			::= INTEGER

{

mobileOriginatedCall	(0),

mobileTerminatedCall	(1),

smsMobileOriginating	(2),

smsMobileTerminating	(3),

ssAction				(4),

locationUpdate			(5)

}

IMEIStatus				::= ENUMERATED

{

greyListedMobileEquipment		(0),

blackListedMobileEquipment		(1),

nonWhiteListedMobileEquipment	(2)

}

IMSIorIMEI				::= CHOICE

{

imsi				[0] IMSI,

imei				[1] IMEI

}

InitiatingParty			::= ENUMERATED

{

network				(0),

subscriber			(1)

}

LocationChange			::= SEQUENCE

{

location			[0] LocationAreaAndCell,

changeTime			[1] TimeStamp

}

Location-info			::= SEQUENCE

{

mscNumber			[1] MscNo OPTIONAL,

location-area		[2] LocationAreaCode,

cell-identification	[3] CellId OPTIONAL

}

LocUpdResult			::= Diagnostics

MaximumBitRate ::= ENUMERATED

{
     MBR14400BitsPerSecond (1),		-- BS20 non-transparent 
     MBR28800BitsPerSecond (2),		-- BS20 non-transparent and transparent,

-- BS30 transparent and multimedia
     MBR32000BitsPerSecond (3),		-- BS30 multimedia
     MBR33600BitsPerSecond (4),		-- BS30 multimedia 
     MBR56000BitsPerSecond (5),		-- BS30 transparent and multimedia 
     MBR57600BitsPerSecond (6),		-- BS20 non-transparent
}

MCCMNC	::= GraphicString (SIZE(6))

--

-- This type contains the mobile country code (MCC) and the mobile

-- network code (MNC) of a PLMN.

--

Month					::= INTEGER (1..12)

MSCAddress				::= AddressString

MSPowerClasses			::= SET OF RFPowerCapability

NetworkCallReference	::= CallReferenceNumber	--

-- See TS 29.002 [60]

--

NetworkSpecificCode		::= INTEGER

--

-- To be defined by network operator

--

NetworkSpecificServices	::= SET OF NetworkSpecificCode

NumOfHSCSDChanRequested		::= INTEGER

NumOfHSCSDChanAllocated		::= INTEGER

ObservedIMEITicketEnable	::= BOOLEAN

OriginalCalledNumber		::= BCDDirectoryNumber

OriginDestCombinations		::= SET OF OriginDestCombination

OriginDestCombination		::= SEQUENCE

{

origin					[0] INTEGER OPTIONAL,

destination				[1] INTEGER OPTIONAL

--

-- Note that these values correspond to the contents

-- of the attributes originId and destinationId

-- respectively. At least one of the two must be present.

--

}

PartialRecordTimer		::= INTEGER

PartialRecordType		::= ENUMERATED

{

timeLimit					(0),

serviceChange				(1),

locationChange				(2),

classmarkChange				(3),

aocParmChange				(4),

radioChannelChange			(5),

hSCSDParmChange				(6),

changeOfCAMELDestination	(7)

}

PartialRecordTypes		::= SET OF PartialRecordType

RadioChannelsRequested	::= SET OF RadioChanRequested

RadioChanRequested		::= ENUMERATED

{

--

-- See Bearer Capability TS 24.008 [64]

--

halfRateChannel				(0),

fullRateChannel				(1),

dualHalfRatePreferred		(2),

dualFullRatePreferred		(3)

}

RateIndication ::= OCTET STRING(SIZE(1))

ReasonForServiceChange		::= ENUMERATED

{

msubInitiated				(0),

mscInitiated				(1),

callSetupFallBack			(2),

callSetupChangeOrder		(3)

}

RecordClassDestination	::= CHOICE

{

osApplication		[0] AE-title,

fileType			[1] FileType

}

RecordClassDestinations	::= SET OF RecordClassDestination

RecordingMethod			::= ENUMERATED

{

inCallRecord		(0),

inSSRecord			(1)

}

RedirectingNumber		::= BCDDirectoryNumber

RFPowerCapability		::= INTEGER

--

-- This field contains the RF power capability of the

-- Mobile station

-- classmark 1 and 2 of TS 24.008 [64] expressed as an integer.

--

RoamingNumber			::= ISDN-AddressString

--

-- See TS 23.003 [68]

--

RoutingNumber			::= CHOICE

{

roaming				[1] RoamingNumber,

forwarded			[2] ForwardToNumber

}

Service						::= CHOICE

{

teleservice					[1] TeleserviceCode,

bearerService				[2] BearerServiceCode,

supplementaryService		[3] SS-Code,

networkSpecificService		[4] NetworkSpecificCode

}

ServiceDistanceDependencies	::= SET OF ServiceDistanceDependency

ServiceDistanceDependency	::= SEQUENCE

{

aocService				[0] INTEGER,

chargingZone			[1] INTEGER OPTIONAL

--

-- Note that these values correspond to the contents

-- of the attributes aocServiceId and zoneId

-- respectively.

--

}

SimpleIntegerName			::= INTEGER

SimpleStringName			::= GraphicString

SpeechVersionIdentifier	::= OCTET STRING (SIZE(1))

--

--	see GSM 08.08

--

--	000 0001	GSM speech full rate version 1

--	001 0001	GSM speech full rate version 2 	 used for enhanced full rate

--	010 0001	GSM speech full rate version 3	 for future use

--	000 0101	GSM speech half rate version 1

--	001 0101	GSM speech half rate version 2 	for future use

--	010 0101	GSM speech half rate version 3	for future use

--

SSActionResult				::= Diagnostics

SSActionType				::= ENUMERATED

{

registration			(0),

erasure					(1),

activation				(2),

deactivation			(3),

interrogation			(4),

invocation				(5),

passwordRegistration	(6)

}

SSParameters				::= CHOICE

{

forwardedToNumber		[0] ForwardToNumber,

unstructuredData		[1] OCTET STRING

}

SupplServices				::= SET OF SS-Code

SuppServiceUsed				::= SEQUENCE

{

ssCode					[0] SS-Code,

ssTime					[1] TimeStamp OPTIONAL

}

SwitchoverTime				::= SEQUENCE

{

hour					INTEGER (0..23),

minute					INTEGER (0..59),

second					INTEGER (0..59)

}

TariffId					::= INTEGER

TariffPeriod				::= SEQUENCE

{

switchoverTime			[0] SwitchoverTime,

tariffId				[1] INTEGER

--

-- Note that the value of tariffId corresponds

-- to the attribute tariffId.

--

}

TariffPeriods				::= SET OF TariffPeriod

TariffSystemStatus			::= ENUMERATED

{

available				(0),	-- available for modification

checked					(1),	-- "frozen" and checked

standby					(2),	-- "frozen" awaiting activation

active					(3)	-- "frozen" and active

}

TrafficChannel			::=	ENUMERATED

{

fullRate			(0),

halfRate			(1)

}

TranslatedNumber		::= 	BCDDirectoryNumber

TransparencyInd			::=	ENUMERATED

{

transparent			(0),

nonTransparent		(1)

}

TrunkGroup				::=	 CHOICE

{

tkgpNumber			[0] INTEGER,

tkgpName			[1] GraphicString

}

TSChangeover			::=	SEQUENCE

{

newActiveTS			[0] INTEGER,

newStandbyTS		[1] INTEGER,

changeoverTime		[2] GeneralizedTime OPTIONAL,

authkey				[3] OCTET STRING OPTIONAL,

checksum			[4] OCTET STRING OPTIONAL,

versionNumber		[5] OCTET STRING OPTIONAL

--

-- Note that if the changeover time is not

-- specified then the change is immediate.

--

}

TSCheckError			::=	SEQUENCE

{

errorId				[0] TSCheckErrorId,

fail				[1] ANY DEFINED BY errorId OPTIONAL

}

TSCheckErrorId			::=	CHOICE

{

globalForm			[0] OBJECT IDENTIFIER,

localForm			[1] INTEGER

}

TSCheckResult			::=	CHOICE

{

success				[0] NULL,

fail				[1] SET OF TSCheckError

}

TSCopyTariffSystem		::=	SEQUENCE

{

oldTS				[0] INTEGER,

newTS				[1] INTEGER

}

TSNextChange			::=	CHOICE

{

noChangeover		[0] NULL,

tsChangeover		[1] TSChangeover

}

TypeOfSubscribers		::= ENUMERATED

{

home				(0),	-- HPLMN subscribers

visiting			(1),	-- roaming subscribers

all	(2)

}

TypeOfTransaction		::=	ENUMERATED

{

successful			(0),

unsuccessful		(1),

all					(2)

}

Visited-Location-info			::= SEQUENCE

{

mscNumber			[1] MscNo,

vlrNumber			[2] VlrNo

}

VlrNo					::= ISDN-AddressString

--

-- See TS 23.003 [68]

--

END

Next Modified Section

#### 5.2.2.2	PS domain CDRs

This subclause contains the abstract syntax definitions that are specific to the GPRS CDR types defined in TS 32.251 [11].

GPRSChargingDataTypes {itu-t (0) identified-organization (4) etsi (0) mobileDomain (0) charging (5) gprsChargingDataTypes (2) asn1Module (0) version1 (0)}

DEFINITIONS IMPLICIT TAGS	::=

BEGIN

-- EXPORTS everything

IMPORTS

CallDuration, CalledNumber, RecordType, CallingNumber, CallReferenceNumber, CellId, DefaultSMS-Handling, Diagnostics, Ext-GeographicalInformation, IMSI, IMEI, IPAddress, ISDN-AddressString, LCSCause, LCSClientExternalID, LCSClientIdentity, LCSClientInternalID, LCSClientType, LCS-Priority, LCSQoSInfo, LevelOfCAMELService, LocalSequenceNumber, LocationAreaAndCell, LocationAreaCode, LocationType, ManagementExtensions, MessageReference, MSISDN, NotificationToMSUser, PositioningData, RecordingEntity, ServiceKey, SMSResult, SmsTpDestinationNumber, TimeStamp

FROM GenericChargingDataTypes {itu-t (0) identified-organization (4) etsi(0) mobileDomain (0) charging (5) genericChargingDataTypes (0) asn1Module (0) version1 (0)}

DefaultGPRS-Handling, RAIdentity

FROM MAP-MS-DataTypes { itu-t identified-organization (4) etsi (0) mobileDomain (0)

gsm-Network (1) modules (3) map-MS-DataTypes (11) version6 (6)}

--

-- from TS 29.002 [60]

LocationMethod

FROM SS-DataTypes { itu-t identified-organization (4) etsi (0) mobileDomain (0) gsm-Access (2) modules (3) ss-DataTypes (2) version7 (7)}

--

-- from TS 24.080 [61]

MBMS2G3GIndicator, FileRepairSupported, MBMSServiceType, MBMSUserServiceType, RequiredMBMSBearerCapabilities, MBMSSessionIdentity, TMGI, MBMSInformation

FROM MBMSChargingDataTypes {itu-t (0) identified-organization (4) etsi(0) mobileDomain (0) charging (5) mbmsChargingDataTypes (8) asn1Module (0) version1 (0)}

Editor’s note:  consider moving the above 2 items also into the generic module in order to avoid again copying from external sources.

;

------------------------------------------------------------------------------

--

--  GPRS RECORDS

--

------------------------------------------------------------------------------

GPRSRecord	::= CHOICE

--

-- Record values 20..27 are GPRS specific

-- Record value 70 is FBC specific

-- Record values 76..77 are GPRS and MBMS specific

{

sgsnPDPRecord			[20] SGSNPDPRecord,

ggsnPDPRecord			[21] GGSNPDPRecord,

sgsnMMRecord			[22] SGSNMMRecord,

sgsnSMORecord			[23] SGSNSMORecord,

sgsnSMTRecord			[24] SGSNSMTRecord,

sgsnLCTRecord			[25] SGSNLCTRecord,

sgsnLCORecord			[26] SGSNLCORecord,

sgsnLCNRecord			[27] SGSNLCNRecord,

--

egsnPDPRecord			[70] EGSNPDPRecord,

--

sgsnMBMSRecord			[76] SGSNMBMSRecord,

ggsnMBMSRecord			[77] GGSNMBMSRecord

}

…

End of modifications