3GPP TSG-SA5 (Telecom Management)	S5-070265

- Meeting SA5#51, 22 - 26 Jan 2007, Seville, SPAIN

| CR-Form-v9.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.151 | CR | 0007 |  rev | - |  Current version: | 7.1.0 |  |
| For HELP on using this form look at the pop-up text over the  symbols. Comprehensive instructions on how to use this form can be found at http://www.3gpp.org/specs/CR.htm. |  |  |  |  |  |  |  |  |

| Proposed change affects:  | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title:  | Improve specification of Common notifications |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG:  | Ericsson (thomas.tovinger@ericsson.com) |  |  |  |  |  |  |  |  |  |
| Source to TSG:  | S5 |  |  |  |  |  |  |  |  |  |
| Work item code:  | OAM7-NIM |  |  |  |  |  |  | Date:  |  | 26/01/2007 |
| Category:  | F |  |  |  |  |  |  | Release:  |  | Rel-7 |
|  | Use one of the following categories: F (correction) A (corresponds to a correction in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: R97 (Release 1997) R98 (Release 1998) R99 (Release 1999) Rel-4 (Release 4) Rel-5 (Release 5) Rel-6 (Release 6) Rel-7 (Release 7) Rel-8 (Release 8) |  |
| Reason for change:  |  | The template specification of Common notifications for more than one IOC is not clear enough and does not cover all needed combinations in an efficient way. |  |  |  |  |  |  |  |  |
| Summary of change:  |  | Clarify the specification of Common notifications and improve the possibility to allow for all combinations, including exceptions and additional/specific notifications to the common ones. |  |  |  |  |  |  |  |  |
| Consequences if  not approved: |  | The template would remain unclear and inefficient for Common notifications, causing confusion for the NRM IRP specification authors and also potentially inconsistent NRM IRP specifications. |  |  |  |  |  |  |  |  |
| Clauses affected:  |  | X.3.a.6, X.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs  |  |  | X | Other core specifications  |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
|  |  | X |  | O&M Specifications |  |  | 32.622, 32.632, 32.732 |  |  |  |
| Other comments:  |  | Note: An important change of principle for the Common notifications is that compared to the current TS version, now subclause X.6 will not anymore define Common notifications which automatically apply to all IOCs in the IRP IS, but instead they are defined as a “library” to which each IOC subclause can refer to as applicable. Any additions or exceptions from that common list will then be specified per IOC, which is much clearer and also much more flexible. |  |  |  |  |  |  |  |  |

1st Modified Section

#### X.3.a.6	Notifications

The < Notifications> subclause, for this IOC, presents:

- optionally, a reference to the common notifications defined in subclause X.6 as valid for this IOC, and

- optionally, a list of notifications that shall be excluded from the list of common notifications (defined in subclause X.6) for this IOC (note: inherited notifications from the parent IOC(s) can not be excluded), and

- optionally, a list of notifications applicable to this IOC, and which may or may not be defined in the common notifications in subclause X.6.

The notifications identified in this subclause are notifications that can be emitted across the Itf-N, where the "object class" and "object instance" parameters of the notification header (see note 2) of these notifications identifies an instance of the IOC defined by the encapsulating subclause (i.e. clause X.3.a).

The notifications identified in this subclause, may originate from implementation object(s) whose identifier is mapped in the implementation, to the object instance identifier used over the Itf-N. Hence the presence of notifications in this clause (i.e. clause X.3.a.6) does not imply nor identify those notifications as being originated from an instance of the IOC defined by the encapsulating subclause (i.e. clause X.3.a).

The information related to option c) above  is provided in a table. An example of such a table is given below:

| Name | Qualifier | Notes |
| --- | --- | --- |
| notifyCMSynchronizationRecommended | O | Example notification. |

NOTE 1:	This subclause and table dos not need to be present when there are no notifications to define.

NOTE 2:	The notification header is defined in the notification IRP Information service TS 32.302 [7].

Next Modified Section

## X.6	Common Notifications

This <Common Notifications> subclause presents a list of notifications that can be referred to by any IOC defined by this IRP specification. These notifications are only applicable to IOCs referring  to this subclause in  subclause “X.3.a.6”.

This information is provided in a table. An example of such a table is given below:

| Name | Qualifier | Notes |
| --- | --- | --- |
| notifyAttributeValueChange | O | Example common notification |
| notifyObjectCreation | O | Example |
| notifyObjectDeletion | O | Example |

NOTE 1:	This subclause does not need to be present when there are no common notifications.

End of Modifications

Annex A (informative):
Change history

| Change history |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Date | TSG # | TSG Doc. | CR | Rev | Subject/Comment | Cat | Old | New |
| Sep 2006 | SA_33 | SP-060548 | 0004 | -- | Update of 32.151 to use a MCC supported font | F | 6.3.0 | 7.0.0 |
| Dec 2006 | SA_34 | SP-060724 | 0005 | -- | Correct the clause numbering instructions for Operations and Notifications | F | 7.0.0 | 7.1.0 |