3GPP TS 28.673 V12.0.0 (2014-09)

Technical Specification

3rd Generation Partnership Project;

Technical Specification Group Services and System Aspects;

Telecommunication management;

Home Node B (HNB) Subsystem (HNS)

Network Resource Model (NRM)

Integration Reference Point (IRP);

Solution Set (SS) definitions

(Release 12)

The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. 
The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented.	 
This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification.
Specifications and reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices.

Keywords

NRM, IRP, Converged Management,State

3GPP

Postal address

3GPP support office address

650 Route des Lucioles - Sophia Antipolis

Valbonne - FRANCE

Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16

Internet

http://www.3gpp.org

Copyright Notification

No part may be reproduced except as authorized by written permission.
The copyright and the foregoing restriction extend to reproduction in all media.

© 2014, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TTA, TTC).

All rights reserved.

UMTS™ is a Trade Mark of ETSI registered for the benefit of its members

3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners
LTE™ is a Trade Mark of ETSI currently being registered for the benefit of its Members and of the 3GPP Organizational Partners

GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	4

Introduction	4

1	Scope	5

2	References	5

3	Definitions and abbreviations	6

3.1	Definitions	6

3.2	Abbreviations	6

4	Solution Set Definitions	7

Annex A (normative):	CORBA Solution Set	8

A.0	General	8

A.1	Architectural features	8

A.1.1	Syntax for Distinguished Names	8

A.1.2	Rules for NRM extensions	8

A.2	Mapping	9

A.2.1	General mappings	9

A.2.2	Information Object Class (IOC) mapping	9

A.2.2.1	IOC HNBGWFunction	9

A.2.2.2	 IOC HNBProfile	9

A.2.2.3	IOC HMSFunction	9

A.2.2.4	IOC IuhSignLinkTp	9

A.2.2.5	IOC EP_Iuh	9

A.3	Solution Set definitions	10

A.3.1	IDL definition structure	10

A.3.2	IDL specification “HnsNetworkResourcesNRMDefs.idl”	11

Annex B (normative):	XML Definitions	13

B.0	General	13

B.1	Architectural features	13

B.1.1	Syntax for Distinguished Names	13

B.2	Mapping	13

B.2.1	General mapping	13

B.2.2	Information Object Class (IOC) mapping	13

B.3	Solution Set definitions	14

B.3.1	XML definition structure	14

B.3.2	Graphical Representation	14

B.3.3	XML schema "hnsNrm.xsd"	15

Annex C (informative):	Change history	18

# Foreword

This Technical Specification has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

# Introduction

The present document is part of a TS-family covering the 3rd Generation Partnership Project: Technical Specification Group Services and System Aspects; Telecommunication management; Configuration Management (CM); as identified below:

28.671:	Telecommunication management; Home Node B Subsystem (HNS) Network Resource Model (NRM) Integration Reference Point (IRP); Requirements

28.672:	Telecommunication management; Home Node B Subsystem (HNS) Network Resource Model (NRM) Integration Reference Point (IRP); Information Service (IS)

28.673:	Telecommunication management; Home Node B (HNB) Subsystem (HNS) Network Resource Model (NRM) Integration Reference Point (IRP); Solution Set (SS) definitions

# 1	Scope

The present document is part of an Integration Reference Point (IRP) named HNS Network Resource Model (NRM) IRP, through which an IRPAgent can communicate configuration management information to one or several IRPManagers concerning HNS resources. The HNS NRM IRP comprises a set of specifications defining Requirements, a protocol neutral Information Service and one or more Solution Set(s).

- The present document specifies the Solution Sets for the HNS NRM IRP.

This specification is related to 3GPP TS 28.672V12.0.X.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

- References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

- For a specific reference, subsequent revisions do not apply.

- For a non-specific reference, the latest version applies.  In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 32.101: "Telecommunication management; Principles and high level requirements".

[3]	3GPP TS 32.102: "Telecommunication management; Architecture".

[4]	3GPP TS 32.600: "Telecommunication management; Configuration Management (CM); Concept and high-level requirements".

[5]	3GPP TS 28.672: "Telecommunication management; Home Node B (HNB) Subsystem (HNS) Network Resource Model (NRM) Integration Reference Point (IRP); Information Service (IS)

[6]	3GPP TS 32.616: "Telecommunication management; Configuration Management (CM); Bulk CM Integration Reference Point (IRP): Solution Set (SS) definitions".

[7]	3GPP TS 32.606: "Telecommunication management; Configuration Management (CM); Basic CM Integration Reference Point (IRP): Solution Set (SS) definitions".

[8]	W3C REC-xml11-20060816: "Extensible Markup Language (XML) 1.1 (Second Edition)".

[9]	Void

[10]	W3C XML Schema Definition Language (XSD) 1.1 Part 1: Structures.

[11]	W3C XML Schema Definition Language (XSD) 1.1 Part 2: Datatypes.

[12]	W3C REC-xml-names-20060816: "Namespaces in XML 1.1 (Second Edition)".

[13]	3GPP TS 32.300: "Telecommunication management; Configuration Management (CM); Name convention for Managed Objects"

[14]	3GPP TS 28.623: “Generic Network Resource Model (NRM) Integration Reference Point (IRP); Solution Set (SS) definitions”.

# 3	Definitions and abbreviations

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

For terms and definitions please refer to 3GPP TS 32.101 [2], 32.102 [3], 32.600 [4] and 32.772 [5].

XML file: See definition of [14].

XML document: See definition of [14].

XML declaration: See definition of [14].

XML element: See definition of [14].

empty XML element: See definition of [14].

XML content (of an XML element): See definition of [14].

XML start-tag: See definition of [14].

XML end-tag: See definition of [14].

XML empty-element tag: See definition of [14].

XML attribute specification: See definition of [14].

DTD: See definition of [14].

XML schema: See definition of [14].

XML namespace: See definition of [14].

XML complex type: See definition of [14].

XML element type: See definition of [14].

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

CM	Configuration Management

CORBA	Common Object Request Broker Architecture

DN	Distinguished Name

DTD	Document Type Definition

HNB	Home Node B

HNS	Home Node B Subsystem

HNB-GW	Home Node B Gateway

IDL	Interface Definition Language

IRP	Integration Reference Point

IS	Information Service

MO	Managed Object

MOC	Managed Object Class

NRM	Network Resource Model

OMG	Object Management Group

SS	Solution Set

XML	eXtensible Markup Language

# 4	Solution Set Definitions

This specification defines the following 3GPP HNS NRM IRP Solution Set Definitions:

-	3GPP HNS NRM IRP CORBA SS (Annex A)

-	3GPP HNS NRM IRP XML Definitions (Annex B)

Annex A (normative):
CORBA Solution Set

# A.0	General

This annex contains the CORBA Solution Set for the IRP whose semantics is specified in HNS NRM IRP: Information Service (TS 32.772 [5]).

# A.1	Architectural features

The overall architectural feature of HNS Network Resources IRP is specified in 3GPP TS 28.672 [5].

This clause specifies features that are specific to the CORBA SS.

## A.1.1	Syntax for Distinguished Names

See clause A.1.1 of [14]

## A.1.2	Rules for NRM extensions

See clause A.1.1 of [14].

### A.2	Mapping

## A.2.1	General mappings

See clause A.2.1 of [14].

## A.2.2	Information Object Class (IOC) mapping

This SS supports reference attributes for relations other than containment relations between objects. Reference attributes are therefore introduced in each MOC where needed.

### A.2.2.1	IOC HNBGWFunction

| IS Attributes | SS Attributes | SS Type |
| --- | --- | --- |
| id | hnbgwFunctionId | string |
| hnbgwId | hnbgwId | string |
| userLabel | userLabel | string |
| ipConfigInfo | ipConfigInfo | string |
| maxNbrHNBRegistered | maxNbrHNBRegistered | Integer |
| maxPacketCapability | maxPacketCapability | integer |

### A.2.2.2		IOC HNBProfile

| IS Attributes | SS Attributes | SS Type |
| --- | --- | --- |
| Id | hnbProfileId | string |
| userLabel | userLabel | string |
| Configuration | configuration | string |
| Criterion | criterion | string |

### A.2.2.3	IOC HMSFunction

| IS Attributes | SS Attributes | SS Type |
| --- | --- | --- |
| userLabel | userLabel | string |

### A.2.2.4	IOC IuhSignLinkTp

| IS Attributes | SS Attributes | SS Type |
| --- | --- | --- |
| Id | iuhSignLinkTpId | string |
| userLabel | userLabel | string |
| farEndEntity | farEndEntity | string |
| sctpAssocLocalAddr | sctpAssocLocalAddr | string |
| sctpAssocRemoteAddr | sctpAssocRemoteAddr | string |

### A.2.2.5	IOC EP_Iuh

| IS Attributes | SS Attributes | SS Type |
| --- | --- | --- |
| Id | epIuhId | string |
| userLabel | userLabel | string |
| farEndEntity | farEndEntity | string |
| farEndNEIPAddr | farEndNEIPAddr | string |

# A.3	Solution Set definitions

## A.3.1	IDL definition structure

Clause A.3.2 defines the MO classes for the HNS NRM IRP.

## A.3.2	IDL specification “HnsNetworkResourcesNRMDefs.idl”

//File: HnsNetworkResourcesNRMDefs.idl

#ifndef _HNSNETWORKRESOURCESNRMDEFS_IDL_

#define _HNSNETWORKRESOURCESNRMDEFS_IDL_

#include "GenericNetworkResourcesNRMDefs.idl"

#pragma prefix "3gppsa5.org"

/**

* This module defines constants for each MO class name and

* the attribute names for each defined MO class.

*/

module HnsNetworkResourcesNRMDefs

{

/**

*  Definitions for MO class HnbgwFunction

*/

interface HNBGWFunction : GenericNetworkResourcesNRMDefs::ManagedFunction

{

const string CLASS = "HNBGWFunction";

// Attribute Names

//

const string hnbgwFunctionId = "hnbgwFunctionId";

const string hnbgwId = "hnbgwId";

const string ipConfigInfo = "ipConfigInfo";

const string maxNbrHNBRegistered = "maxNbrHNBRegistered";

const string maxPacketCapability = "maxPacketCapability";

};

/**

*  Definitions for MO class IuhSignLinkTp

*/

interface IuhSignLinkTp : GenericNetworkResourcesNRMDefs::EP_RP

{

const string CLASS = "IuhSignLinkTp";

// Attribute Names

//

const string sctpAssocLocalAddr = "sctpAssocLocalAddr";

const string sctpAssocRemoteAddr = "sctpAssocRemoteAddr";

};

/**

*  Definitions for MO class EP_Iuh

*/

interface EP_Iuh : GenericNetworkResourcesNRMDefs::EP_RP

{

const string CLASS= "EP_Iuh";

// Attribute Names

//

const string farEndNEIPAddr= "farEndNEIPAddr";

};

/**

*  Definitions for MO class HNBProfile

*/

interface HNBProfile : GenericNetworkResourcesNRMDefs::ManagedFunction

{

const string CLASS= "HNBProfile";

// Attribute Names

//

const string hnbProfileId = "hnbProfileId";

const string configuration = "configuration";

const string criterion = "criterion";

};

/**

*  Definitions for MO class HMSFunction

*/

interface HMSFunction : GenericNetworkResourcesNRMDefs::ManagedFunction

{

const string CLASS= "HMSFunction";

// Attribute Names

//

};

};

#endif //_HNSETWORKRESOURCESNRMDEFS_IDL_

Annex B (normative):
XML Definitions

# B.0	General

This annex contains the XML Definitions for the HNS NRM IRP as it applies to Itf-N, in accordance with HNS NRM IRP Information Service (TS 28.672 [5]).

The XML file formats are based on XML [8], XML Schema [10] [11] and XML Namespace [12] standards.

# B.1	Architectural features

The overall architectural feature of HNS Network Resources IRP is specified in 3GPP TS 28.672 [5]. 
This clause specifies features that are specific to the Schema definitions.

## B.1.1	Syntax for Distinguished Names

The syntax of a Distinguished Name is defined in 3GPP TS 32.300 [13].

# B.2	Mapping

## B.2.1	General mapping

An IOC maps to an XML element of the same name as the IOC's name in the IS. An IOC attribute maps to a sub-element of the corresponding IOC's XML element, and the name of this sub-element is the same as the attribute's name in the IS.

## B.2.2	Information Object Class (IOC) mapping

The mapping is not present in the current version of this specification.

# B.3	Solution Set definitions

## B.3.1	XML definition structure

Annex B.3.3 of the present document defines the NRM-specific XML schema hnsNrm.xsd for the HNS Network Resources IRP NRM defined in 3GPP TS 28.672 [5].

XML schema hnsNrm.xsd explicitly declares NRM-specific XML element types for the related NRM.

The definition of those NRM-specific XML element types complies with the generic mapping rules defined in 3GPP TS 32.616 [6].

## B.3.2	Graphical Representation

The graphical representation is not present in the current version of this specification.

## B.3.3	XML schema "hnsNrm.xsd"

<?xml version="1.1" encoding="UTF-8"?>

<!--

3GPP TS 28.673 HNS Network Resources IRP

Bulk CM Configuration data file NRM-specific XML schema

hnsNrm.xsd

-->

<schema

targetNamespace=

"http://www.3gpp.org/ftp/specs/archive/28_series/28.676#hnsNrm"

elementFormDefault="qualified"

xmlns="http://www.w3.org/2001/XMLSchema"

xmlns:xn=

"http://www.3gpp.org/ftp/specs/archive/28_series/28.623#genericNrm"

xmlns:un=

"http://www.3gpp.org/ftp/specs/archive/28_series/28.673#hnsNrm"

xmlns:gn=

"http://www.3gpp.org/ftp/specs/archive/28_series/28.656#geranNrm"

xmlns:sm=

"http://www.3gpp.org/ftp/specs/archive/28_series/28.626#stateManagementIRP"

>

<import namespace="http://www.3gpp.org/ftp/specs/archive/28_series/28.623#genericNrm" schemaLocation="genericNrm.xsd"/>

<import namespace="http://www.3gpp.org/ftp/specs/archive/28_series/28.656#geranNrm" schemaLocation="geranNrm.xsd"/>

<import namespace="http://www.3gpp.org/ftp/specs/archive/28_series/28.626#stateManagementIRP" schemaLocation="stateManagementIRP.xsd"/>

<!-- HNS Network Resources IRP NRM attribute related XML types -->

<!-- HNS network Resources IRP NRM class associated XML elements -->

<element

name="HNBGWFunction"

substitutionGroup="xn:ManagedElementOptionallyContainedNrmClass"

>

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes" minOccurs="0">

<complexType>

<all>

<element name="hnbgwId" type="string"/>

<element name="userLabel" type="string"/>

<element name="iPConfigInfo" type="string"/>

<element name="maxNbrHNBRegistered" type="integer"/>

<element name="maxPacketCapability" type="integer"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="xn:VsDataContainer"/>

<element ref="un:IuhSignLinkTp"/>

<element ref="un:EP_Iuh"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="IuhSignLinkTp">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes" minOccurs="0">

<complexType>

<all>

<element name="userLabel" type="string" minOccurs="0"/>

<element name="farEndEntity" type="string" minOccurs="0"/>

<element name="sctpAssocLocalAddr" type="string"/>

<element name="sctpAssocRemoteAddr" type="string"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="xn:VsDataContainer"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="EP_Iuh">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes" minOccurs="0">

<complexType>

<all>

<element name="userLabel" type="string" minOccurs="0"/>

<element name="farEndEntity" type="string" minOccurs="0"/>

<element name="farEndNEIPAddr" type="string" minOccurs="0"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="xn:VsDataContainer"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="HNBProfile">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes" minOccurs="0">

<complexType>

<all>

<element name="userLabel" type="string"/>

<element name="configuration" type="string"/>

<element name="criterion" type="string" minOccurs="0"/>

</all>

</complexType>

</element>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

<element name="HMSFunction">

<complexType>

<complexContent>

<extension base="xn:NrmClass">

<sequence>

<element name="attributes" minOccurs="0">

<complexType>

<all>

<element name="userLabel" type="string"/>

</all>

</complexType>

</element>

<choice minOccurs="0" maxOccurs="unbounded">

<element ref="xn:VsDataContainer"/>

<element ref="un:HNBProfile"/>

</choice>

</sequence>

</extension>

</complexContent>

</complexType>

</element>

</schema>

Annex C (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | TSG # | TSG Doc. | CR | Rev | Subject/Comment | Old | New |
| 2014-06 | SA#64 | SP-140332 | 001 | - | Upgrade W3C XML Schema version from 1.0 to 1.1 | 11.0.0 | 11.1.0 |
|  |  | SP-140360 | 002 | - | remove the feature support statements |  |  |
| 2014-09 | SA#65 | SP-140560 | 003 | - | Update the link from Solution Set to Information Service due to the end of Release 12 | 11.1.0 | 12.0.0 |