3GPP TS 32.406 V9.1.0 (2014-09)

Technical Specification

3rd Generation Partnership Project;

Technical Specification Group Services and System Aspects;

Telecommunication management;

Performance Management (PM);

Performance measurements

Core Network (CN) Packet Switched (PS) domain

(Release 9)

The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP.	 
The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented.	 
This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification.
Specifications and reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices.

Keywords

GSM, UMTS, management, performance

3GPP

Postal address

3GPP support office address

650 Route des Lucioles - Sophia Antipolis

Valbonne - FRANCE

Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16

Internet

http://www.3gpp.org

Copyright Notification

No part may be reproduced except as authorized by written permission.
The copyright and the foregoing restriction extend to reproduction in all media.

©2014, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TTA, TTC).

All rights reserved.

UMTS™ is a Trade Mark of ETSI registered for the benefit of its members

3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners
LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners

GSM® and the GSM logo are registered and owned by the GSM Association

# Contents

Foreword	10

Introduction	10

1	Scope	12

2	References	12

3	Measurement family and abbreviations	13

3.1	Measurement family	13

3.2	Abbreviations	14

4	Measurements related to the SGSN	16

4.1	Mobility Management	16

4.1.1	GPRS attach procedures	16

4.1.1.1	Attempted GPRS attach procedures	16

4.1.1.2	Successful GPRS attach procedures	16

4.1.1.3	Failed GPRS attach procedures	17

4.1.1.4	Aborted GPRS attach procedures	17

4.1.2	Intra-SGSN Routing Area update procedures	17

4.1.2.1	Attempted intra-SGSN Routing Area update procedures	17

4.1.2.2	Successful intra-SGSN Routing Area update procedures	18

4.1.2.3	Failed intra-SGSN Routing Area update procedures	18

4.1.3	GPRS detach procedures initiated by MS	19

4.1.3.1	Attempted GPRS detach procedures initiated by MS	19

4.1.4	GPRS detach procedures initiated by SGSN	19

4.1.4.1	Attempted GPRS detach procedures initiated by SGSN	19

4.1.4.2	Successful GPRS detach procedures initiated by SGSN	20

4.1.5	Inter-SGSN Routing Area update procedures	20

4.1.5.1	Attempted inter-SGSN Routing Area update procedures	20

4.1.5.2	Successful inter-SGSN Routing Area update procedures	20

4.1.5.3	Failed inter-SGSN Routing Area update procedures	21

4.1.6	GPRS attach procedures with IMSI already attached	21

4.1.6.1	Attempted GPRS attach procedures with IMSI already attached	21

4.1.6.2	Successful GPRS attach procedures with IMSI already attached	22

4.1.6.3	Failed GPRS attach procedures with IMSI already attached	22

4.1.6.4	Aborted GPRS attach procedures with IMSI already attached	23

4.1.7	IMSI detach procedures initiated by MS	23

4.1.7.1	Attempted IMSI detach procedures initiated by MS	23

4.1.8	Combined GPRS/IMSI attach procedures	24

4.1.8.1	Attempted combined GPRS/IMSI attach procedures	24

4.1.8.2	Successful combined GPRS/IMSI attach procedures	24

4.1.8.3	Failed combined GPRS/IMSI attach procedures	25

4.1.8.4	Aborted combined GPRS/IMSI attach procedures	25

4.1.9	Combined GPRS/IMSI detach procedures initiated by MS	26

4.1.9.1	Attempted combined GPRS/IMSI detach procedures initiated by MS	26

4.1.10	Combined RA/LA intra-SGSN Routing Area update procedures	26

4.1.10.1	Attempted combined RA/LA intra-SGSN Routing Area update procedures	26

4.1.10.2	Successful combined RA/LA intra-SGSN Routing Area update procedures	26

4.1.10.3	Failed combined RA/LA intra-SGSN Routing Area update procedures	27

4.1.11	"Combined RA/LA with IMSI Attach" intra-SGSN Routing Area update procedures	27

4.1.11.1	Attempted "combined RA/LA with IMSI Attach" intra-SGSN Routing Area update procedures	27

4.1.11.2	Failed "combined RA/LA with IMSI Attach"  intra-SGSN Routing Area update procedures	28

4.1.12	Combined RA/LA inter-SGSN Routing Area update procedures	28

4.1.12.1	Attempted combined RA/LA inter-SGSN Routing Area update procedures	28

4.1.12.2	Successful combined RA/LA inter-SGSN Routing Area update procedures	29

4.1.12.3	Failed combined RA/LA inter-SGSN Routing Area update procedures	29

4.1.13	"Combined RA/LA with IMSI Attach" inter-SGSN Routing Area update procedures	30

4.1.13.1	Attempted "combined RA/LA with IMSI Attach" inter-SGSN Routing Area update procedures	30

4.1.13.2	Failed "combined RA/LA with IMSI Attach" inter-SGSN Routing Area update procedures	30

4.1.14	Number of received invalid P-TMSI's during detach	31

4.1.15	GSM PS paging procedures	31

4.1.15.1	Attempted GSM PS paging procedures	31

4.1.15.2	Successful GSM PS paging procedures	31

4.1.15.3	Failed GSM PS paging procedures	32

4.1.16	UMTS PS paging procedures	32

4.1.16.1	Attempted UMTS PS paging procedures	32

4.1.16.2	Successful UMTS PS paging procedures	32

4.1.16.3	Failed UMTS PS paging procedures	33

4.1.17	PS paging procedures with unknown access type	33

4.1.17.1	Attempted PS paging procedures with unknown access type	33

4.1.18	Number of PS paging message sends from 2G-SGSN to the MS	33

4.1.19	Number of PS paging message sends from 3G-SGSN to the MS	34

4.1.20	GSM subscribers state	34

4.1.20.1	Subscribers in STANDBY state	34

4.1.20.1.1	Number of subscribers in STANDBY state	34

4.1.20.1.2	Mean number of subscribers in STANDBY state	34

4.1.20.1.3	Max number of subscribers in STANDBY state	34

4.1.20.2	Subscribers in READY state	35

4.1.20.2.1	Number of subscribers in READY state	35

4.1.20.2.2	Mean number of subscribers in READY state	35

4.1.20.2.3	Max number of subscribers in READY state	35

4.1.21	UMTS subscribers state	36

4.1.21.1	Subscribers in PMM-IDLE state	36

4.1.21.1.1	Number of subscribers in PMM-IDLE state	36

4.1.21.1.2	Mean number of subscribers in PMM-IDLE state	36

4.1.21.1.3	Max number of subscribers in PMM-IDLE state	36

4.1.21.2	Subscribers in PMM-CONNECTED state	37

4.1.21.2.1	Number of subscribers in PMM-CONNECTED state	37

4.1.21.2.2	Mean number of subscribers in PMM-CONNECTED state	37

4.1.21.2.3	Max number of subscribers in PMM-CONNECTED state	37

4.1.22	Attached subscribers	37

4.1.22.1	Number of attached subscribers	37

4.1.22.2	Mean number of attached subscribers	38

4.1.22.3	Max number of attached subscribers	38

4.1.23	Home subscribers	39

4.1.23.1	Number of home subscribers	39

4.1.23.2	Mean number of home subscribers	39

4.1.23.3	Max number of home subscribers	40

4.1.24	Visiting national subscribers	40

4.1.24.1	Number of visiting national subscribers	40

4.1.24.2	Mean number of visiting national subscribers	40

4.1.24.3	Max number of visiting national subscribers	41

4.1.25	Visiting foreign subscribers	41

4.1.25.1	Number of visiting foreign subscribers	41

4.1.25.2	Mean number of visiting foreign subscribers	42

4.1.25.3	Max number of visiting foreign subscribers	42

4.1.26	Void	43

4.1.27	Void	43

4.1.28	Void	43

4.1.29	Void	43

4.1.30	CAMEL subscribers	43

4.1.30.1	Number of CAMEL subscribers	43

4.1.30.2	Mean Number of CAMEL subscribers	43

4.1.30.3	Max number of CAMEL subscribers	44

4.1.31	Void	44

4.1.32	InsertSubscriberData requests received from a HLR during GPRS Update Location procedure	44

4.1.32.1	Attempted InsertSubscriberData requests received from a HLR during GPRS Update Location procedure	44

4.1.33	GPRS Update Locations sent to the HLR	44

4.1.33.1	Attempted GPRS Update Locations sent to the HLR	44

4.1.33.2	Successful GPRS Update Locations returned from the HLR	45

4.1.34	CancelLocation requests received from an HLR-operator, in case of a HLR-initiated Detach	45

4.1.34.1	Attempted CancelLocation requests received from an HLR-operator, in case of a HLR-initiated Detach	45

4.1.35	CancelLocation requests received from a HLR due to a SGSN-change (previous SGSN)	45

4.1.35.1	Attempted CancelLocation requests received from a HLR due to a SGSN-change (previous SGSN)	45

4.1.36	Reset requests received from a HLR due to an HLR restart, indicating that a failure occurred	46

4.1.36.1	Attempted Reset requests received from a HLR due to an HLR restart, indicating that a failure occurred	46

4.1.37	Periodic Routing Area update procedures	46

4.1.37.1	Attempted periodic Routing Area update procedures	46

4.1.37.2	Successful periodic Routing Area update procedures	46

4.1.37.3	Failed Periodic Routing Area update procedures	47

4.2	Subscriber Management	47

4.2.1	Insert Subscriber Data requests received from a HLR due to an HLR-operator intervention	47

4.2.1.1	Attempted Insert Subscriber Data requests received from a HLR due to an HLR-operator intervention	47

4.2.2	Delete Subscriber Data requests received from a HLR due to an HLR-operator intervention	48

4.2.2.1	Attempted Delete Subscriber Data requests received from a HLR due to an HLR-operator intervention	48

4.3	SRNS Relocation	48

4.3.1	Intra 3G-SGSN SRNS Relocations	48

4.3.1.1	Attempted intra 3G-SGSN SRNS Relocations	48

4.3.1.2	Successful intra 3G-SGSN SRNS Relocations	49

4.3.1.3	Failed intra 3G-SGSN SRNS Relocations, due to internal reasons	49

4.3.1.4	Failed intra 3G-SGSN SRNS Relocations, due to external reasons	49

4.3.2	Inter 3G-SGSN SRNS Relocations	50

4.3.2.1	Attempted inter 3G-SGSN SRNS Relocations, counted in the old 3G-SGSN	50

4.3.2.2	Successful inter 3G-SGSN SRNS Relocations, counted in the old 3G-SGSN	50

4.3.2.3	Failed inter 3G-SGSN SRNS Relocations, due to internal reasons, counted in the old 3G-SGSN	50

4.3.2.4	Failed inter 3G-SGSN SRNS Relocations, due to external reasons, counted in the old 3G-SGSN	51

4.3.3	Inter 3G-SGSN SRNS Relocations, counted in the new 3G-SGSN	51

4.3.3.1	Attempted inter 3G-SGSN SRNS Relocations, counted in the new 3G-SGSN	51

4.3.3.2	Successful Inter 3G-SGSN SRNS Relocations, counted in the new 3G-SGSN	51

4.4	Security	52

4.4.1	P-TMSI reallocation procedures	52

4.4.1.1	Attempted P-TMSI reallocation procedures	52

4.4.1.2	Successful P-TMSI reallocation procedures	52

4.4.2	Identity Request procedures initiated by this SGSN	52

4.4.2.1	Attempted Identity Request procedures initiated by this SGSN	52

4.4.2.2	Successful completed Identity Request procedures initiated by this SGSN	53

4.4.3	Identification information requests sent to a partner (previous) SGSN for subscribers registering afresh in this SGSN	53

4.4.3.1	Attempted identification information requests sent to a partner (previous) SGSN for subscribers registering afresh in this SGSN	53

4.4.3.2	Successful replied identification information requests that were sent to a partner (previous) SGSN	54

4.4.4	Attempted Identity Requests sent to the MS	54

4.4.4.1	Attempted Identity Requests sent to the MS	54

4.4.4.2	Successful replied Identity Requests from the MS	54

4.4.5	Authentication procedures that are started within this SGSN area for a subscriber using a SIM	55

4.4.5.1	Attempted authentication procedures that are started within this SGSN area for a subscriber using a SIM	55

4.4.5.2	Successful authentication procedures within this SGSN area, for a subscriber using a SIM	55

4.4.6	Authentication procedures that are started within this SGSN area for a subscriber using a USIM	56

4.4.6.1	Attempted authentication procedures that are started within this SGSN area for a subscriber using a USIM	56

4.4.6.2	Successful authentication procedures within this SGSN area, for a subscriber using a USIM	56

4.4.6.3	Received ciphering and Authentication failures within this SGSN area	57

4.4.7	Identification information requests that were received from a partner (new) SGSN for subscribers de-registering from this SGSN	57

4.4.7.1	Attempted identification information requests that were received from a partner (new) SGSN for subscribers de-registering from this SGSN	57

4.4.7.2	Successfully replied identification information requests that were received from a partner (new) SGSN	57

4.4.8	SGSN context requests sent to a partner (previous) SGSN for subscribers registering afresh in this SGSN	58

4.4.8.1	Attempted SGSN context requests sent to a partner (previous) SGSN for subscribers registering afresh in this SGSN	58

4.4.8.2	Successfully replied SGSN context requests that were sent to a partner (previous) SGSN	58

4.4.9	SGSN context requests received from a partner (new) SGSN for a subscriber de-registering from this SGSN	59

4.4.9.1	Attempted SGSN context requests received from a partner (new) SGSN for a subscriber de-registering from this SGSN	59

4.4.9.2	Successfully replied SGSN context requests received from a partner (new) SGSN	59

4.4.10	Number of P-TMSI - IMSI correlation failures (User Identity Confidentiality (TS 23.060 [2]))	60

4.4.11	Security mode control procedures started by the SGSN	60

4.4.11.1	Attempted security mode control procedures started by the SGSN	60

4.4.11.2	Successful security mode procedures	60

4.4.12	Ciphering procedures started by the SGSN	61

4.4.12.1	Attempted GSM ciphering procedures started by the SGSN	61

4.4.12.2	Successful GSM ciphering procedures started by the SGSN	61

4.4.12.3	Attempted UMTS ciphering procedures started by the SGSN	61

4.4.12.4	Successful UMTS ciphering procedures started by the SGSN	61

4.4.13	MAP V1 requests for authentication sets	62

4.4.13.1	Attempted MAP V1 requests for authentication sets, sent to the HLR by SGSN	62

4.4.13.2	Successful MAP V1 requests for authentication sets that were sent to the HLR	62

4.4.13.3	Number of empty responses to the MAP V1 request for authentication sets that were sent to the HLR	62

4.4.14	MAP V3 requests for Authentication sets	63

4.4.14.1	Attempted MAP V3 requests for Authentication sets sent to the HLR by SGSN	63

4.4.14.2	Successful MAP V3 requests for authentication sets that were sent to the HLR	63

4.4.14.3	Number of empty responses to the MAP V3 request for authentication sets that were sent to the HLR	63

4.5	Void	64

4.6	Session Management	64

4.6.1	PDP context activation procedures initiated by MS	64

4.6.1.1	Attempted PDP context activation procedures initiated by MS	64

4.6.1.2	Successful PDP context activation procedures initiated by MS	64

4.6.1.3	Failed PDP context activation procedures initiated by MS	65

4.6.2	dynamic PDP context activation procedures initiated by MS	65

4.6.2.1	Attempted dynamic PDP context activation procedures initiated by MS	65

4.6.2.2	Successful dynamic PDP context activation procedures initiated by MS	65

4.6.3	Mean number of activated PDP contexts	66

4.6.4	PDP context deactivation procedures initiated by the MS	66

4.6.4.1	Attempted PDP context deactivation procedures initiated by the MS	66

4.6.4.2	Successful PDP context deactivation procedures initiated by the MS	67

4.6.5	Number of active PDP context	67

4.6.6	Subscribers with activated PDP context	67

4.6.6.1	Number of subscribers with activated PDP context	67

4.6.6.2	Mean number of subscribers with activated PDP context	68

4.6.6.3	Max number of subscribers with activated PDP context	68

4.6.7	Void	69

4.6.8	PDP context deactivation procedures initiated by the GGSN	69

4.6.8.1	Attempted PDP context deactivation procedures initiated by the GGSN	69

4.6.8.2	Successful PDP context deactivation procedures initiated by the GGSN	69

4.6.9	PDP context deactivation procedures initiated by the SGSN	69

4.6.9.1	Attempted PDP context deactivation procedures initiated by the SGSN	69

4.6.9.2	Successful PDP context deactivations initiated by the SGSN	70

4.6.9.3	Abnormal PDP context Deactivation procedures	70

4.6.10	SGSN-Initiated PDP context update procedures	71

4.6.10.1	Attempted SGSN-Initiated PDP context update procedures	71

4.6.10.2	Successful SGSN-Initiated PDP context update procedures	71

4.6.11	GGSN-Initiated PDP context update procedures	71

4.6.11.1	Attempted GGSN-Initiated PDP context update procedures	71

4.6.11.2	Successful GGSN-Initiated PDP context update procedures	72

4.6.12	SGSN-Initiated PDP context modifications procedures	72

4.6.12.1	Attempted SGSN-Initiated PDP context modifications procedures	72

4.6.12.2	Successfully SGSN-Initiated PDP context modifications procedures	73

4.6.13	MS-Initiated PDP context modifications procedures	73

4.6.13.1	Attempted MS-Initiated PDP context modifications procedures	73

4.6.13.2	Successfully MS-Initiated PDP context modifications procedures	73

4.6.14	Secondary PDP context activation procedures	74

4.6.14.1	Attempted Secondary PDP context activation procedures	74

4.6.14.2	Successful Secondary PDP context activations	74

4.6.15	PDP context activation procedures initiated by Network	75

4.6.15.1	Attempted PDP context activation procedures initiated by Network	75

4.6.15.2	Successful PDP context activation procedures initiated by Network	75

4.6.15.3	Failed PDP context activation procedures initiated by Network	75

4.6.16	PDP Context set-up time, initiated by MS (Mean)	76

4.6.17	PDP Context set-up time, initiated by MS (Max)	76

4.6.18	PDP Context set-up time, initiated by Network (Mean)	77

4.6.19	PDP Context set-up time, initiated by Network (Max)	77

4.7	CAMEL Measurements	78

4.7.1	CAMEL dialogues	78

4.7.1.1	Attempted CAMEL dialogues	78

4.7.1.2	Failed CAMEL dialogues, aborted locally by gprsSSF	78

4.7.1.3	Failed CAMEL dialogues, error or reject from gsmSCF	78

4.8	UMTS-GSM Intersystem Change	79

4.8.1	Intra SGSN inter system changes from UMTS to GSM	79

4.8.1.1	Attempted intra SGSN inter system changes from UMTS to GSM	79

4.8.1.2	Successful intra SGSN inter system changes from UMTS to GSM	79

4.8.1.3	Failed intra SGSN inter system changes UMTS to GSM RAU, due to internal reasons	79

4.8.1.4	Failed intra SGSN inter system changes UMTS to GSM RAU, due to external reasons	80

4.8.2	Intra SGSN inter system changes from GSM to UMTS	80

4.8.2.1	Attempted intra SGSN inter system changes from GSM to UMTS	80

4.8.2.2	Successful intra SGSN inter system changes from GSM to UMTS	80

4.8.2.3	Failed intra SGSN inter system changes GSM to UMTS RAU, due to internal reasons	81

4.8.2.4	Failed intra SGSN inter system changes GSM to UMTS RAU, due to external reasons	81

4.9	UMTS GTP Measurements	81

4.9.1	GTP-U Iu	81

4.9.1.1	Number of outgoing GTP data packets on the Iu interface	81

4.9.1.2	Number of incoming GTP data packets on the Iu interface	82

4.9.1.3	Number of octets of outgoing GTP data packets on the Iu interface	82

4.9.1.4	Number of octets of incoming GTP data packets on the Iu interface	82

4.9.2	GTP Gn	82

4.9.2.1	Number of outgoing GTP data packets on the Gn interface, from SGSN to GGSN	82

4.9.2.2	Number of incoming GTP data packets on the Gn interface, from GGSN to SGSN	83

4.9.2.3	Number of octets of outgoing GTP data packets on the Gn interface, from SGSN to GGSN	83

4.9.2.4	Number of octets of incoming GTP data packets on the Gn interface, from GGSN to SGSN	84

4.9.2.5	Number of outgoing GTP signalling packets on the Gn interface, from SGSN to GGSN	84

4.9.2.6	Number of incoming GTP signalling packets on the Gn interface, from GGSN to SGSN	84

4.9.2.7	Number of octets of outgoing GTP signalling packets on the Gn interface, from SGSN to GGSN	85

4.9.2.8	Number of octets of incoming GTP signalling packets on the Gn interface, from GGSN to SGSN	85

4.9.2.9	Number of outgoing GTP data packets on the Gn interface, from SGSN to SGSN	86

4.9.2.10	Number of incoming GTP data packets on the Gn interface, from SGSN to SGSN	86

4.9.2.11	Number of octets of outgoing GTP data packets on the Gn interface, from SGSN to SGSN	86

4.9.2.12	Number of octets of incoming GTP data packets on the Gn interface, from SGSN to SGSN	87

4.9.2.13	Number of outgoing GTP signalling packets on the Gn interface, from SGSN to SGSN	87

4.9.2.14	Number of incoming GTP signalling packets on the Gn interface, from SGSN to SGSN	88

4.9.2.15	Number of octets of outgoing GTP signalling packets on the Gn interface, from SGSN to SGSN	88

4.9.2.16	Number of octets of incoming GTP signalling packets on the Gn interface, from SGSN to SGSN	88

4.10	UMTS Bearer Service	89

4.10.1	UMTS Bearer Service CS time to register (Mean)	89

4.10.2	UMTS Bearer Service CS time to register (Max)	89

4.10.3	UMTS Bearer Service PS time to register (Mean)	90

4.10.4	UMTS Bearer Service PS time to register (Max)	90

4.10.5	UMTS Bearer Service time to establish Communications Management (CM) radio access connectivity (Mean)	90

4.10.6	UMTS Bearer Service time to establish Communications Management (CM) radio access connectivity (Max)	91

4.11	LLC frames	91

4.11.1	Number of LLC frames sent	91

4.11.2	Number of LLC frames received	91

4.11.3	Number of erroneous LLC frames received	92

4.11.4	Number of LLC frames retransmitted	92

4.12	SNDCP N-PDUs	92

4.12.1	Number of SNDCP N-PDUs sent	92

4.12.2	Number of SNDCP N-PDU octets sent	92

4.12.3	Number of SNDCP N-PDUs received	93

4.12.4	Number of SNDCP N-PDU octets received	93

4.13	IMEI checking procedure	93

4.13.1	Number of check IMEI requests	93

4.13.2	Number of check IMEI white list responses	94

4.13.3	Number of check IMEI grey list responses	94

4.13.4	Number of check IMEI black list responses	94

4.13.5	Number of check IMEI unknown equipment responses	94

4.14	DNS	95

4.14.1	Attempted DNS Query Procedures	95

4.14.2	Successful DNS Query Procedures	95

5	Measurements related to the GGSN	96

5.1	Session Management	96

5.1.1	Session establishments	96

5.1.1.1	Attempted session establishments	97

5.1.1.2	Successful session establishments	97

5.1.1.3	Failed session establishments	98

5.1.2	Network-initiated session establishments	98

5.1.2.1	Number of routing information requests for network-initiated session establishment attempts	99

5.1.2.2	Number of routing information successful responses for network-initiated session establishment attempts	99

5.1.2.3	Attempted Network-initiated session establishments	100

5.1.2.3.1	Attempted Network-initiated session establishments	100

5.1.2.3.2	Failed Network-initiated session establishments - failures occurred before sending PDP context activation request to the MS	100

5.1.2.3.3	Failed Network-initiated session establishments - failures occurred after sending PDP context activation request to the MS	100

5.1.3	Subscribers with activated PDP context	101

5.1.3.1	Number of subscribers with activated PDP context	101

5.1.3.2	Mean number of subscribers with activated PDP context	101

5.1.3.3	Max number of subscribers with activated PDP context	102

5.1.4	Session conclusions	102

5.1.4.1	MS & SGSN-initiated session conclusions	103

5.1.4.1.1	Attempted MS & SGSN-initiated session conclusions	103

5.1.4.1.2	Successful MS & SGSN-initiated session conclusions	103

5.1.4.2	GGSN-initiated session conclusions	104

5.1.4.2.1	Attempted GGSN-initiated session conclusions	104

5.1.4.2.2	Successful GGSN-initiated session conclusions	104

5.2	Per APN measurements	104

5.2.1	Session establishments	104

5.2.1.1	Session establishments, per APN	106

5.2.1.1.1	Attempted session establishments, per APN	106

5.2.1.1.2	Successfully established sessions, per APN	106

5.2.1.2	Session establishments with dynamic PDP address allocation required, per APN	106

5.2.1.2.1	Attempted session establishments with dynamic PDP address allocation required, per APN	106

5.2.1.2.2	Successfully established sessions with dynamic PDP address allocation required, per APN	107

5.2.1.3	Session establishments with user authentication required, per APN	107

5.2.1.3.1	Attempted session establishments with user authentication required, per APN	107

5.2.1.3.2	Failed session establishments due to user authentication failure, per APN	107

5.2.2	Active sessions	108

5.2.2.1	Number of simultaneous active sessions, per APN	108

5.2.2.2	Peak number of simultaneous active sessions, per APN	108

5.2.2.3	MS & SGSN-initiated session modifications, per APN	109

5.2.2.3.1	Attempted MS & SGSN-initiated session modifications, per APN	109

5.2.2.3.2	Successfully performed MS & SGSN-initiated session modifications, per APN	109

5.2.3	Session conclusions	110

5.2.3.1	MS-initiated session conclusions, per APN	110

5.2.3.1.1	Attempted MS-initiated session conclusions, per APN	110

5.2.3.1.2	Successful MS-initiated session conclusions, per APN	110

5.2.3.2	GGSN-initiated session conclusions, per APN	110

5.2.3.2.1	Attempted GGSN-initiated session conclusions, per APN	110

5.2.3.2.2	Successful GGSN-initiated session conclusions, per APN	111

5.3	GTP measurements	111

5.3.1	Number of incoming GTP data packets on the Gn interface	111

5.3.2	Number of outgoing GTP data packets on the Gn interface	112

5.3.3	Number of discarded GTP data packets	112

5.3.4	Number of octets of incoming GTP data packets on the Gn interface	112

5.3.5	Number of octets of outgoing GTP data packets on the Gn interface	113

5.3.6	Number of incoming GTP signalling packets on the Gn interface	113

5.3.7	Number of outgoing GTP signalling packets on the Gn interface	114

5.3.8	Number of discarded GTP signalling packets	114

5.3.9	Number of octets of incoming GTP signalling packets on the Gn interface	115

5.3.10	Number of octets of outgoing GTP signalling packets on the Gn interface	115

5.3.11	Number of GTP tunnels on the Gn interface	116

5.3.12	Number of GTP tunnels created on the Gn interface	116

5.4	GTP' measurements	116

5.4.1	Attempted CDR information transfers	117

5.4.2	Successful CDR information transfers	117

5.4.3	Failed CDR information transfers	118

5.5	IP measurements	118

5.5.1	Number of incoming IP data packets on the Gi interface	119

5.5.2	Number of outgoing IP data packets on the Gi interface	120

5.5.3	Number of IP data packets discarded due to node congestion	120

5.5.4	Number of octets of incoming IP data packets on the Gi interface	120

5.5.5	Number of octets of outgoing IP data packets on the Gi interface	121

5.6	AAA Authentication measurements	121

5.6.1	Attempted Authentication Procedures, per APN	122

5.6.2	Successful Authentication Procedures, per APN	122

Annex A (informative):	Change history	123

# Foreword

This Technical Specification has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

# Introduction

The present document is part of a TS-family covering the 3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Telecommunication management; as identified below:

32.401		Performance Management (PM); Concept and requirements

52.402		Performance Management (PM); Performance measurements – GSM

32.404		Performance Management (PM); Performance measurements - Definitions and template

32.405		Performance Management (PM); Performance measurements Universal Terrestrial Radio Access Network (UTRAN)

32.406		Performance Management (PM); Performance measurements Core Network (CN) Packet Switched (PS) domain

32.407		Performance Management (PM); Performance measurements Core Network (CN) Circuit Switched (CS) domain

32.408		Performance Management (PM); Performance measurements Teleservice

32.409		Performance Management (PM); Performance measurements IP Multimedia Subsystem (IMS)

The present document is part of a set of specifications, which describe the requirements and information model necessary for the standardised Operation, Administration and Maintenance (OA&M) of a multi-vendor 3G-system.

During the lifetime of a 3G network, its logical and physical configuration will undergo changes of varying degrees and frequencies in order to optimise the utilisation of the network resources. These changes will be executed through network configuration management activities and/or network engineering, see TS 32.600 [3].

Many of the activities involved in the daily operation and future network planning of a 3G network require data on which to base decisions. This data refers to the load carried by the network and the grade of service offered. In order to produce this data performance measurements are executed in the NEs, which comprise the network. The data can then be transferred to an external system, e.g. an Operations System (OS) in TMN terminology, for further evaluation. The purpose of the present document is to describe the mechanisms involved in the collection of the data and the definition of the data itself.

Annex B of TS 32.404 helps in the definition of new performance measurements that can be submitted to 3GPP for potential adoption and inclusion in the present document. Annex B of TS 32.404 discusses a top-down performance measurement definition methodology that focuses on how the end-user of performance measurements can use the measurements.

# 1	Scope

The present document describes the measurements for Core Network (CN) Packet Switched (PS) domain of UMTS or combined UMTS/GSM network.

TS 32.401 [1] describes Performance Management concepts and requirements.

The present document is valid for all measurement types provided by an implementation of a CN PS domain of UMTS network or combined UMTS/GSM network.

Only measurement types that are specific to CN PS domain of UMTS or combined UMTS/GSM network are defined within the present documents.

Vendor specific measurement types used in CN PS domain of UMTS or combined UMTS/GSM networks are not covered. Instead, these could be applied according to manufacturer's documentation.

Measurements related to "external" technologies (such as ATM or IP) as described by "external" standards bodies (e.g. ITU-T or IETF) shall only be referenced within this specification, wherever there is a need identified for the existence of such a reference.

The definition of the standard measurements is intended to result in comparability of measurement data produced in a multi-vendor network, for those measurement types that can be standardised across all vendors' implementations.

The structure of the present document is as follows:

-	Header 1: Network Element (e.g. RNC related measurements);

-	Header 2: Measurement function (e.g. soft handover measurements);

-	Header 3: Measurements.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

- References are either specific (identified by date of publication and/or edition number or version number) or non-specific.

- For a specific reference, subsequent revisions do not apply.

- For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TS 32.401: "Telecommunication management; Performance Management (PM); Concept and requirements".

[2]	3GPP TS 23.060: "General Packet Radio Service (GPRS) Service description; Stage 2".

[3]	3GPP TS 32.600: "Telecommunication management; Configuration Management (CM); Concept and high-level requirements".

[4]	3GPP TS 29.002: "Mobile Application Part (MAP) specification".

[5]	3GPP TS 25.413: "UTRAN Iu Interface RANAP signalling".

[6]	3GPP TS 29.060: "General Packet Radio Service (GPRS); GPRS Tunnelling Protocol (GTP) across the Gn and Gp interface".

[7]	Void.

[8]	3GPP TS 23.107: "Quality of Service (QoS) concept and architecture".

[9]	3GPP TS 23.003: "Numbering, Addressing and Identification".

[10]	3GPP TS 29.061: "Interworking between the Public Land Mobile Network (PLMN) supporting packet based services and Packet Data Networks (PDN)".

[11]	3GPP TS 44.064: "Digital cellular telecommunications system (Phase 2+); General Packet Radio Service (GPRS); Mobile Station - Serving GPRS Support Node (MS-SGSN) Logical Link Control (LLC) layer specification".

[12]	3GPP TS 44.065: "Digital cellular telecommunications system (Phase 2+); General Packet Radio Service (GPRS); Mobile Station (MS) - Serving GPRS Support Node (SGSN); Subnetwork Dependent Convergence Protocol (SNDCP)".

[13]	IETF RFC 1034: "DOMAIN NAMES - CONCEPTS AND FACILITIES".

[14]	IETF RFC 1035: "DOMAIN NAMES - IMPLEMENTATION AND SPECIFICATION".

[15]	3GPP TS 24.008: "Mobile Radio Interface Layer 3 specification; Core Network Protocols; Stage 3".

[16]	GSM 08.18: "Digital cellular telecommunication system (Phase 2); General Packet Radio Service (GPRS); Base Station System (BSS) - Serving GPRS Support Node (SGSN); BSS GPRS Protocol (BSSGP)".

[17]	IETF RFC 2903: "Generic AAA Architecture".

[18]	IETF RFC 2138: "Remote Authentication Dial In User Service (RADIUS) ".

# 3	Measurement family and abbreviations

## 3.1	Measurement family

- The measurement names defined in the present document are all beginning with a prefix containing the measurement family name (e.g. MM.AttGprsAttach). This family name identifies all measurements which relate to a given functionality and it may be used for measurement administration (see TS 32.401 [1]).

The list of families currently used in the present document is as follows:

-	AAA (measurements related to AAA)

-	CAM (measurements related to CAMEL)

-	DNS (measurements related to DNS)

-	GTP (measurements related to GTP)

-	IMEI (measurements related to IMEI verification)

-	ISYSC (measurements related to GSM/UMTS Intersystem changes)

-	LLC (measurements related to Logical Link Control)

-	MM (measurements related to Mobility Management)

-	RELOC (measurements related to SRNS Relocation)

-	RRC (measurements related to Radio Resource Control)

-	SEC (measurements related to Security)

-	SIG (measurements related to Signalling)

-	SM (measurements related to Session Management)

-	SNDCP (measurements related to Sub-Network Dependent Convergence Protocol)

-	SUB (measurements related to Subscriber Management)

-	UBS (measurements related to UMTS Bearer Service)

## 3.2	Abbreviations

For the purposes of the present document, the following abbreviations apply:

3G	3rd Generation

3GPP	3G Partnership Project

ASN.1	Abstract Syntax Notation 1

BER	Basic Encoding Rules

CN	Core Network

DTD	Document Type Definition

EGQM	Enhanced Goal, Question, Metric

EM	(Network) Element Manager

FTAM	File Transfer Access and Management

FTP	File Transfer Protocol

GQM		Goal, Question, Metric

IEEE	Institute of Electrical and Electronics Engineers, Inc.

Itf	Interface

ITU-T	International Telecommunication Union - Telecommunications Standardisation Sector

MSC	Mobile services Switching Centre

NE	Network Element

NM	Network Manager

OA&M	Operation, Administration and Maintenance

OS	Operations System (EM, NM)

OSI	Open Systems Interconnection

PM	Performance Management

QoS	Quality of Service

RNC	Radio Network Controller

TFTP	Trivial FTP

UMTS	Universal Mobile Telecommunications System

UTRAN	Universal Terrestrial Radio Access Network

You can find below a list of abbreviations used within the measurement types for field E of the measurement template (see subclause 3.3).

Assn	Assign(ment,ed)

Att	Attempt(s,ed)

Auth	Authentication

Bgrd	Background

Call	Call

Chg	Change

Conn	Connection

Combi	Combined

CS	Circuit switched

Ctrl	Controlled

Conv	Conversational

Del	Deletion

Drop	Drop(ped)

Estab	Establish (ed,ment)

Fail	Fail(ed, ure)

FDD	Frequency Division Duplex

HO	Handover

Inc	Incoming

Intact	Interactive

Inter	Inter

Intra	Intra

Invol	Involve(d)

Max	Maximum

MM	Mobility Management

Nat	National

Netw	Network

NodeB	NodeB

Oct	Octet(s)

Out	Outgoing

Pkt	Packet(s)

Prep	Preparation

Proc	Procedure

PS	Packet switched

RAT	Radio Access Technology

ReEstab	Re-establish (ed,ment)

Rel	Released

Reloc	Relocation

Req	Request(s,ed)

Res	Resource

RL	Radio Link

RNC	RNC

RRC	Radio Resource Control

RTWP	Received Total Wideband Power

Setup	Setup

SGSN	SGSN

Sig	Signalling

Strm	Streaming

Sub	Subscriber

Succ	Success(es,ful)

TCP	Transmitted Carrier Power

UE	User Equipment

UTRAN	UTRAN

# 4	Measurements related to the SGSN

## 4.1	Mobility Management

### 4.1.1	GPRS attach procedures

The four measurements defined in the clause 4.1.1 are subject to the "3 out of 4 approach".

NOTE:	Number of Attempted GPRS attach procedures = Number of Successful GPRS attach procedures + Total Number of Failed GPRS attach procedures + Aborted GPRS attach procedures.

#### 4.1.1.1	Attempted GPRS attach procedures

- This measurement provides the number of attempted GPRS attach procedures initiated within this SGSN area.
The three measurement types defined in e) are subject to the "2 out of 3 approach".

- CC.

- Receipt of "ATTACH REQUEST" message with "Attach type" information element indicating "GPRS attach" from the MS (TS 24.008 [15]).

- A single integer value per measurement type defined in e).

- MM.AttGprsAttach:

-	MM.AttGprsAttach	Combined (don't care);

-	MM.AttGprsAttach.G	GSM;

-	MM.AttGprsAttach.U	UMTS.

- RA, specified by a concatenation of the MCC, MNC, LAC and the RAC.

- Valid for packet switching.

- GSM/UMTS.

#### 4.1.1.2	Successful GPRS attach procedures

- This measurement provides the number of successfully performed GPRS attach procedures within this SGSN area.
The three measurement types defined in e) are subject to the "2 out of 3 approach".

- CC.

- Transmission of a "ATTACH ACCEPT" message to the MS, in response to a "ATTACH REQUEST" message with the "Attach type" information element indicating "GPRS attach". If the "ATTACH ACCEPT" message is caused by a retransmission, this will not cause the counter to be increased. (TS 24.008 [15]).

- A single integer value per measurement type defined in e).

- MM.SuccGprsAttach:

-	MM.SuccGprsAttach	Combined (don't care);

-	MM.SuccGprsAttach.G	GSM;

-	MM.SuccGprsAttach.U	UMTS.

- RA, specified by a concatenation of the MCC, MNC, LAC and the RAC.

- Valid for packet switching.

- GSM/UMTS.

#### 4.1.1.3	Failed GPRS attach procedures

- This measurement provides the number of failed GPRS attach procedures. The measurement is split into subcounters per the reject cause. The three measurements defined in e) are subject to the "2 out of 3 approach".

- CC

- Transmission by the SGSN of the ATTACH REJECT message to the MS, in response to a "ATTACH REQUEST" message with the "Attach type" information element indicating "GPRS attach", the relevant measurement is incremented according to the reject cause. Possible reject causes are defined within TS 24.008 [15].
The sum of all supported per cause measurements shall be equal to the total number of failed GPRS attach procedures. In case only a subset of per cause measurements is supported, a sum subcounter will be provided first.

- Each measurement (as defined in e) is an integer value. The number of measurements is equal to the number of causes supported plus a possible sum value identified by the .sum suffix.

- MM. FailedGprsAttach.Cause:

-	MM. FailedGprsAttach.Cause	Combined (don't care);

-	MM. FailedGprsAttach.Cause.G	GSM;

-	MM. FailedGprsAttach.Cause.U	UMTS;

where Cause identifies the reject cause.

- RA, specified by a concatenation of the MCC, MNC, LAC and the RAC.

- Valid for packet switching

- GSM/UMTS

#### 4.1.1.4	Aborted GPRS attach procedures

- This measurement provides the number of aborted GPRS attach procedures. The three measurements defined in e) are subject to the "2 out of 3 approach".

- CC

- Abortion of a GPRS attach procedure, or a "ATTACH REQUEST" message with "Attach type" information element indicating "GPRS attach" would not be treated (TS 24.008 [15]).

- A single integer value per measurement type defined in e).

- MM.AbortedGprsAttach:

-	MM. AbortedGprsAttach	Combined (don't care);

-	MM. AbortedGprsAttach.G	GSM;

-	MM. AbortedGprsAttach.U	UMTS.

- RA, specified by a concatenation of the MCC, MNC, LAC and the RAC.

- Valid for packet switching

- GSM/UMTS

### 4.1.2	Intra-SGSN Routing Area update procedures

#### 4.1.2.1	Attempted intra-SGSN Routing Area update procedures

- This measurement provides the number of attempted intra-SGSN Routing Area Update procedures initiated within this SGSN area.
The three measurement types defined in e) are subject to the "2 out of 3 approach".

- CC.

- Receipt of a "ROUTING AREA UPDATE REQUEST" message from the MS, where the old RA and the new RA are served by this SGSN (TS 24.008 [15]).

- A single integer value per measurement type defined in e).

- MM.AttIntraSgsnRaUpdate:

-	MM.AttIntraSgsnRaUpdate	Combined (don't care);

-	MM.AttIntraSgsnRaUpdate.G	GSM;

-	MM.AttIntraSgsnRaUpdate.U	UMTS.

- RA, specified by a concatenation of the MCC, MNC,  LAC and the RAC.

- Valid for packet switching.

- GSM/UMTS.

#### 4.1.2.2	Successful intra-SGSN Routing Area update procedures

- This measurement provides the number of successfully performed intra-SGSN Routing Area Update procedures initiated in this SGSN.
The three measurement types defined in e) are subject to the "2 out of 3 approach".

- CC.

- Transmission of "ROUTING AREA UPDATE ACCEPT" message to the MS (TS 24.008 [15]).

- A single integer value.

- MM.SuccIntraSgsnRaUpdate:

-	MM.SuccIntraSgsnRaUpdate	Combined (don't care);

-	MM.SuccIntraSgsnRaUpdate.G	GSM;

-	MM.SuccIntraSgsnRaUpdate.U	UMTS.

- RA, specified by a concatenation of the MCC, MNC, LAC and the RAC.

- Valid for packet switching.

- GSM/UMTS.

#### 4.1.2.3	Failed intra-SGSN Routing Area update procedures

- This measurement provides the number of failed GPRS intra-SGSN Routing Area Update procedures initiated in this SGSN. The measurement is split into subcounters per the reject cause. The three measurements defined in e) are subject to the "2 out of 3 approach".

- CC

- Transmission by the SGSN of the ROUTING AREA UPDATE REJECT message to the MS, in response to a " ROUTING AREA UPDATE REQUEST" (Intra-Sgsn) message, the relevant measurement is incremented according to the reject cause. Possible reject causes are defined within TS 24.008 [15].
The sum of all supported per cause measurements shall be equal to the total number of failed GPRS routing area update procedures. In case only a subset of per cause measurements is supported, a sum subcounter will be provided first.

- Each measurement (as defined in e) is an integer value. The number of measurements is equal to the number of causes supported plus a possible sum value identified by the .sum suffix.

- MM. FailIntraSgsnRaUpdate.Cause:

- -	MM. FailIntraSgsnRaUpdate.Cause	Combined (don't care);

- -	MM. FailIntraSgsnRaUpdate.Cause.G	GSM;

- -	MM. FailIntraSgsnRaUpdate.Cause.U	UMTS;

- where Cause identifies the reject cause.

- RA, specified by a concatenation of the MCC, MNC, LAC and the RAC.

- Valid for packet switching.

- GSM/UMTS.

### 4.1.3	GPRS detach procedures initiated by MS

#### 4.1.3.1	Attempted GPRS detach procedures initiated by MS

- This measurement provides the number of MS initiated GPRS detach procedures within this SGSN area.
The three measurement types defined in e) are subject to the "2 out of 3 approach".

- CC.

- Receipt of a "DETACH REQUEST" message from the MS indicating a GPRS detach (TS 24.008 [15]).

- A single integer value per measurement type defined in e).

- MM.AttGprsDetachMs:

-	MM.AttGprsDetachMs	Combined (don't care);

-	MM.AttGprsDetachMs.G	GSM;

-	MM.AttGprsDetachMs.U	UMTS.

- RA, specified by a concatenation of the MCC, MNC, LAC and the RAC.

- Valid for packet switching.

- GSM/UMTS.

### 4.1.4	GPRS detach procedures initiated by SGSN

#### 4.1.4.1	Attempted GPRS detach procedures initiated by SGSN

- This measurement provides the number of attempted GPRS detach procedures initiated by SGSN.
The three measurement types defined in e) are subject to the "2 out of 3 approach".

- CC.

- Transmission of a "DETACH REQUEST" message to the MS (TS 24.008 [15]).

- A single integer value per measurement type defined in e).

- MM.AttGprsDetachSgsn:

-	MM.AttGprsDetachSgsn	Combined (don't care);

-	MM.AttGprsDetachSgsn.G	GSM;

-	MM.AttGprsDetachSgsn.U	UMTS.

- RA, specified by a concatenation of the MCC, MNC, LAC and the RAC.

- Valid for packet switching.

- GSM/UMTS.

#### 4.1.4.2	Successful GPRS detach procedures initiated by SGSN

- This measurement provides the number of successfully completed GPRS detach procedures SGSN-initiated within this SGSN area.
The three measurement types defined in e) are subject to the "2 out of 3 approach".

- CC.

- Receipt of "DETACH ACCEPT" message from the MS (TS 24.008 [15]).

- A single integer value per measurement type defined in e).

- MM.SuccGprsDetachSgsn:

-	MM.SuccGprsDetachSgsn	Combined (don't care);

-	MM.SuccGprsDetachSgsn.G	GSM;

-	MM.SuccGprsDetachSgsn.U	UMTS.

- RA, specified by a concatenation of the MCC, MNC, LAC and the RAC.

- Valid for packet switching.

- GSM/UMTS

### 4.1.5	Inter-SGSN Routing Area update procedures

#### 4.1.5.1	Attempted inter-SGSN Routing Area update procedures

- This measurement provides the number of attempted inter-SGSN Routing Area Update procedures initiated in this SGSN.
The three measurement types defined in e) are subject to the "2 out of 3 approach".

- CC.

- Receipt of an "ROUTING AREA UPDATE REQUEST" message from the MS where the old RA is served by another SGSNs (TS 24.008 [15]).

- A single integer value per measurement type defined in e).

- MM.AttInterSgsnRaUpdate:

-	MM.AttInterSgsnRaUpdate	Combined (don't care);

-	MM.AttInterSgsnRaUpdate.G	GSM;

-	MM.AttInterSgsnRaUpdate.U	UMTS.

- RA, specified by a concatenation of the MCC, MNC, LAC and the RAC.

- Valid for packet switching.

- GSM/UMTS.

#### 4.1.5.2	Successful inter-SGSN Routing Area update procedures

- This measurement provides the number of successfully completed inter-SGSN Routing Area Update procedures in this SGSN.
The three measurement types defined in e) are subject to the "2 out of 3 approach".

- CC.

- Transmission of a "ROUTING AREA UPDATE ACCEPT" message to the MS (TS 24.008 [15]).

- A single integer value per measurement type defined in e).

- MM.SuccInterSgsnRaUpdate:

-	MM.SuccInterSgsnRaUpdate	Combined (don't care);

-	MM.SuccInterSgsnRaUpdate.G	GSM;

-	MM.SuccInterSgsnRaUpdate.U	UMTS.

- RA, specified by a concatenation of the MCC, MNC, LAC and the RAC.

- Valid for packet switching.

- GSM/UMTS.

#### 4.1.5.3	Failed inter-SGSN Routing Area update procedures

- This measurement provides the number of failed GPRS inter-SGSN Routing Area Update procedures initiated in this SGSN. The measurement is split into subcounters per the reject cause. The three measurements defined in e) are subject to the "2 out of 3 approach".

- CC

- Transmission by the SGSN of the ROUTING AREA UPDATE REJECT message to the MS, in response to a " ROUTING AREA UPDATE REQUEST" (Inter-Sgsn) message, the relevant measurement is incremented according to the reject cause. Possible reject causes are defined within TS 24.008 [15].
The sum of all supported per cause measurements shall be equal to the total number of failed GPRS routing area update procedures. In case only a subset of per cause measurements is supported, a sum subcounter will be provided first.

- Each measurement (as defined in e) is an integer value. The number of measurements is equal to the number of causes supported plus a possible sum value identified by the .sum suffix.

- MM. FailInterSgsnRaUpdate.Cause:

- -	MM. FailInterSgsnRaUpdate.Cause	Combined (don't care);

- -	MM. FailInterSgsnRaUpdate.Cause.G	GSM;

- -	MM. FailInterSgsnRaUpdate.Cause.U	UMTS;

- where Cause identifies the reject cause.

- RA, specified by a concatenation of the MCC, MNC, LAC and the RAC.

- Valid for packet switching

- GSM/UMTS

### 4.1.6	GPRS attach procedures with IMSI already attached

The four measurements defined in the clause 5.1.6 are subject to the "3 out of 4 approach".

NOTE:	Number of Attempted GPRS attach procedures with IMSI already attached = Number of Successful GPRS attach procedures with IMSI already attached + Total Number of Failed GPRS attach procedures with IMSI already attached + Aborted GPRS attach procedures with IMSI already attached.

#### 4.1.6.1	Attempted GPRS attach procedures with IMSI already attached

- This measurement provides the number of attempted GPRS attach procedures, while IMSI is already attached. 
The three measurement types defined in e) are subject to the "2 out of 3 approach".

- CC.

- Receipt of "ATTACH REQUEST" Message with "Attach type" information element indicating "GPRS attach while IMSI attached" from the MS (TS 24.008 [15]).

- A single integer value per measurement type defined in e).

- MM.AttGprsAttachImsiAttached:

-	MM.AttGprsAttachImsiAttached	Combined (don't care);

-	MM.AttGprsAttachImsiAttached.G	GSM;

-	MM.AttGprsAttachImsiAttached.U	UMTS.

- RA, specified by a concatenation of the MCC, MNC, LAC and the RAC.

- Valid for packet switching.

- GSM/UMTS.

#### 4.1.6.2	Successful GPRS attach procedures with IMSI already attached

- This measurement provides the number of successfully performed GPRS attach procedures with IMSI already attached within this SGSN area.
The three measurement types defined in e)  are subject to the "2 out of 3 approach".

- CC.

- Transmission of a "ATTACH ACCEPT" message to the MS, in response to a "ATTACH REQUEST" message with the "Attach type" information element indicating "GPRS attach while IMSI attached". If the "ATTACH ACCEPT" message is caused by a retransmission, this will not cause the counter to be increased. (TS 24.008 [15]).

- A single integer value per measurement type defined in e).

- MM.SuccGprsAttachImsiAttached:

-	MM.SuccGprsAttachImsiAttached	Combined (don't care);

-	MM.SuccGprsAttachImsiAttached.G	GSM;

-	MM.SuccGprsAttachImsiAttached.U	UMTS.

- RA, specified by a concatenation of the MCC, MNC, LAC and the RAC.

- Valid for packet switching.

- GSM/UMTS.

#### 4.1.6.3	Failed GPRS attach procedures with IMSI already attached

- This measurement provides the number of failed GPRS attach procedures with IMSI already attached. The measurement is split into subcounters per the reject cause. The three measurements defined in e) are subject to the "2 out of 3 approach".

- CC

- Transmission by the SGSN of the ATTACH REJECT message to the MS, in response to a "ATTACH REQUEST" message with the "Attach type" information element indicating "GPRS attach while IMSI attached", the relevant measurement is incremented according to the reject cause. Possible reject causes are defined within TS 24.008 [15]. 
The sum of all supported per cause measurements shall be equal to the total number of failed GPRS attach procedures with IMSI already attached. In case only a subset of per cause measurements is supported, a sum subcounter will be provided first.

- Each measurement (as defined in e) is an integer value. The number of measurements is equal to the number of causes supported plus a possible sum value identified by the .sum suffix.

- MM. FailedGprsAttachImsiAttached.Cause:

-	MM. FailedGprsAttachImsiAttached.Cause	Combined (don't care);

-	MM. FailedGprsAttachImsiAttached.Cause.G	GSM;

-	MM. FailedGprsAttachImsiAttached.Cause.U	UMTS;

where Cause identifies the reject cause.

- RA, specified by a concatenation of the MCC, MNC, LAC and the RAC.

- Valid for packet switching.

- GSM/UMTS.

#### 4.1.6.4	Aborted GPRS attach procedures with IMSI already attached

- This measurement provides the number of aborted GPRS attach procedures with IMSI already attached. The three measurements defined in e) are subject to the "2 out of 3 approach".

- CC.

- Abortion of a GPRS attach procedure with IMSI already attached, or a "ATTACH REQUEST" message with "Attach type" information element indicating "GPRS attach while IMSI attached" would not be treated (TS 24.008 [15]).

- A single integer value per measurement type defined in e).

- MM.AbortedGprsAttachImsiAttached:

-	MM. AbortedGprsAttachImsiAttached	Combined (don't care);

-	MM. AbortedGprsAttachImsiAttached.G	GSM;

-	MM. AbortedGprsAttachImsiAttached.U	UMTS.

- RA, specified by a concatenation of the MCC, MNC, LAC and the RAC.

- Valid for packet switching.

- GSM/UMTS.

### 4.1.7	IMSI detach procedures initiated by MS

#### 4.1.7.1	Attempted IMSI detach procedures initiated by MS

- This measurement provides the number of attempted IMSI detach procedures MS-initiated within this SGSN area.
The three measurement types defined in e) are subject to the "2 out of 3 approach".

- CC.

- Receipt of a "DETACH REQUEST" message from the MS, indicating a IMSI detach (TS 24.008 [15]).

- A single integer value per measurement type defined in e).

- MM.AttImsiDetachMs:

-	MM.AttImsiDetachMs	Combined (don't care);

-	MM.AttImsiDetachMs.G	GSM;

-	MM.AttImsiDetachMs.U	UMTS.

- RA, specified by a concatenation of the MCC, MNC, LAC and the RAC.

- Valid for packet switching.

- GSM/UMTS.

### 4.1.8	Combined GPRS/IMSI attach procedures

The four measurements defined in the clause 4.1.8 are subject to the "3 out of 4 approach".

NOTE:	Number of Attempted combined GPRS/IMSI attach procedures = Number of Successful combined GPRS/IMSI attach procedures + Total Number of Failed combined GPRS/IMSI attach procedures + Aborted combined GPRS/IMSI attach procedures.

#### 4.1.8.1	Attempted combined GPRS/IMSI attach procedures

- This measurement provides the number of attempts of combined GPRS/IMSI attach procedures initiated within this SGSN area.
The three measurement types defined in e)  are subject to the "2 out of 3 approach".

- CC.

- Receipt of a "ATTACH REQUEST" message with "Attach type" information element indicating "Combined GPRS/IMSI attach" from the MS (TS 24.008 [15]).

- A single integer value per measurement type defined in  e) .

- MM.AttCombiAttach:

-	MM.AttCombiAttach	Combined (don't care);

-	MM.AttCombiAttach.G	GSM;

-	MM.AttCombiAttach.U	UMTS.

- RA, specified by a concatenation of the MCC, MNC, LAC and the RAC.

- Valid for packet switching.

- GSM/UMTS.

#### 4.1.8.2	Successful combined GPRS/IMSI attach procedures

- This measurement provides the number of successfully performed combined GPRS/IMSI attach procedures within this SGSN area.
The three measurement types defined in e) are subject to the "2 out of 3 approach".

- CC.

- Transmission of a "ATTACH ACCEPT" message to the MS, in response to a "ATTACH REQUEST" message with the "Attach type" information element indicating "Combined GPRS/IMSI attach". If the "ATTACH ACCEPT" message is caused by a retransmission, this will not cause the counter to be increased. (TS 24.008 [15]).

- A single integer value per measurement type defined in e).

- MM.SuccCombiAttach:

-	MM.SuccCombiAttach	Combined (don't care);

-	MM.SuccCombiAttach.G	GSM;

-	MM.SuccCombiAttach.U	UMTS.

- RA, specified by a concatenation of the MCC, MNC, LAC and the RAC.

- Valid for packet switching.

- GSM/UMTS.

#### 4.1.8.3	Failed combined GPRS/IMSI attach procedures

- This measurement provides the number of failed combined GPRS/IMSI attach procedures. The measurement is split into subcounters per the reject cause. The three measurements defined in e) are subject to the "2 out of 3 approach".

- CC

- Transmission by the SGSN of the ATTACH REJECT message to the MS, in response to a "ATTACH REQUEST" message with the "Attach type" information element indicating "Combined GPRS/IMSI attach", the relevant measurement is incremented according to the reject cause. Possible reject causes are defined within TS 24.008 [15]. 
The sum of all supported per cause measurements shall be equal to the total number of failed combined GPRS/IMSI attach procedures. In case only a subset of per cause measurements is supported, a sum subcounter will be provided first.

- Each measurement (as defined in e) is an integer value. The number of measurements is equal to the number of causes supported plus a possible sum value identified by the .sum suffix.

- MM. FailedCombiAttach.Cause:

-	MM. FailedCombiAttach.Cause	Combined (don't care);

-	MM. FailedCombiAttach.Cause.G	GSM;

-	MM. FailedCombiAttach.Cause.U	UMTS;

where Cause identifies the reject cause.

- RA, specified by a concatenation of the MCC, MNC, LAC and the RAC.

- Valid for packet switching.

- GSM/UMTS.

#### 4.1.8.4	Aborted combined GPRS/IMSI attach procedures

- This measurement provides the number of aborted combined GPRS/IMSI attach procedures. The three measurements defined in e) are subject to the "2 out of 3 approach".

- CC.

- Abortion of a combined GPRS/IMSI attach procedure, or a "ATTACH REQUEST" message with "Attach type" information element indicating "Combined GPRS/IMSI attach" would not be treated (TS 24.008 [15]).

- A single integer value per measurement type defined in e).

- MM.AbortedCombiAttach:

-	MM. AbortedCombiAttach	Combined (don't care);

-	MM. AbortedCombiAttach.G	GSM;

-	MM. AbortedCombiAttach.U	UMTS.

- RA, specified by a concatenation of the MCC, MNC, LAC and the RAC.

- Valid for packet switching.

- GSM/UMTS.

### 4.1.9	Combined GPRS/IMSI detach procedures initiated by MS

#### 4.1.9.1	Attempted combined GPRS/IMSI detach procedures initiated by MS

- This measurement provides the number of attempted Combined GPRS/IMSI detach procedures MS-initiated within this SGSN area.
The three measurement types defined in e) are subject to the "2 out of 3 approach".

- CC.

- Receipt of "DETACH REQUEST" message from the MS, indicating a Combined GPRS/IMSI detach (TS 24.008 [15]).

- A single integer value per measurement type defined in e).

- MM.AttCombiDetachMs:

-	MM.AttCombiDetachMs	Combined (don't care);

-	MM.AttCombiDetachMs.G	GSM;

-	MM.AttCombiDetachMs.U	UMTS.

- RA, specified by a concatenation of the MCC, MNC, LAC and the RAC.

- Valid for packet switching.

- GSM/UMTS.

### 4.1.10	Combined RA/LA intra-SGSN Routing Area update procedures

#### 4.1.10.1	Attempted combined RA/LA intra-SGSN Routing Area update procedures

- This measurement provides the number of combined RA/LA updates (intra-SGSN) procedures initiated in this SGSN. These are counted as attempts.
The three measurement types defined in e) are subject to the "2 out of 3 approach".

- CC.

- Receipt of "Routing Area Update REQUEST" message from the MS, indicating a combined RA/LA update (TS 24.008 [15]).

- A single integer value per measurement type defined in e).

- MM.AttCombiIntraSgsnRaUpdate:

-	MM.AttCombiIntraSgsnRaUpdate	Combined (don't care);

-	MM.AttCombiIntraSgsnRaUpdate.G	GSM;

-	MM.AttCombiIntraSgsnRaUpdate.U	UMTS.

- RA, specified by a concatenation of the MCC, MNC, LAC and the RAC.

- Valid for packet switching.

- GSM/UMTS.

#### 4.1.10.2	Successful combined RA/LA intra-SGSN Routing Area update procedures

- This measurement provides the number of success-fully performed combined RA/LA updates (intra-SGSN) procedures initiated in this SGSN.
The three measurement types defined in e) are subject to the "2 out of 3 approach".

- CC.

- Transmission of "Routing Area Update ACCEPT" message to the MS (TS 24.008 [15]).

- A single integer value per measurement type defined in e).

- MM.SuccCombiIntraSgsnRaUpdate:

-	MM.SuccCombiIntraSgsnRaUpdate	Combined (don't care);

-	MM.SuccCombiIntraSgsnRaUpdate.G	GSM;

-	MM.SuccCombiIntraSgsnRaUpdate.U	UMTS.

- RA, specified by a concatenation of the MCC, MNC, LAC and the RAC.

- Valid for packet switching.

- GSM/UMTS.

#### 4.1.10.3	Failed combined RA/LA intra-SGSN Routing Area update procedures

- This measurement provides the number of failed GPRS combined RA/LA intra-SGSN Routing Area Update procedures initiated in this SGSN. The measurement is split into subcounters per the reject cause. The three measurements defined in e) are subject to the "2 out of 3 approach".

- CC

- Transmission by the SGSN of the ROUTING AREA UPDATE REJECT message to the MS, in response to a " ROUTING AREA UPDATE REQUEST" (combined RA/LA Intra-Sgsn) message, the relevant measurement is incremented according to the reject cause. Possible reject causes are defined within TS 24.008 [15].
The sum of all supported per cause measurements shall be equal to the total number of failed GPRS routing area update procedures. In case only a subset of per cause measurements is supported, a sum subcounter will be provided first.

- Each measurement (as defined in e) is an integer value. The number of measurements is equal to the number of causes supported plus a possible sum value identified by the .sum suffix.

- MM. FailCombiIntraSgsnRaUpdate.Cause:

- -	MM. FailedCombiIntraSgsnRaUpdate.Cause	Combined (don't care);

- -	MM. FailedCombiIntraSgsnRaUpdate.Cause.G	GSM;

- -	MM. FailedCombiIntraSgsnRaUpdate.Cause.U	UMTS;

- where Cause identifies the reject cause.

- RA, specified by a concatenation of the MCC, MNC, LAC and the RAC.

- Valid for packet switching

- GSM/UMTS

### 4.1.11	"Combined RA/LA with IMSI Attach" intra-SGSN Routing Area update procedures

#### 4.1.11.1	Attempted "combined RA/LA with IMSI Attach" intra-SGSN Routing Area update procedures

- This measurement provides the number of combined RA/LA updates with IMSI attach (intra-SGSN) procedures initiated in this SGSN. These are counted as attempts.
The three measurement types defined in e) are subject to the "2 out of 3 approach".

- CC.

- Receipt of "Routing Area Update REQUEST" message from the MS, indicating a combined RA/LA update with IMSI attach (TS 24.008 [15]).

- A single integer value per measurement type defined in e).

- MM.AttImsiCombiIntraSgsnRAUpdate:

-	MM.AttImsiCombiIntraSgsnRAUpdate	Combined (don't care);

-	MM.AttImsiCombiIntraSgsnRAUpdate.G	GSM;

-	MM.AttImsiCombiIntraSgsnRAUpdate.U	UMTS.

- RA, specified by a concatenation of the MCC, MNC, LAC and the RAC.

- Valid for packet switching.

- GSM/UMTS.

#### 4.1.11.2	Failed "combined RA/LA with IMSI Attach"  intra-SGSN Routing Area update procedures

- This measurement provides the number of failed GPRS "combined RA/LA with IMSI Attach" intra-SGSN Routing Area Update procedures initiated in this SGSN. The measurement is split into subcounters per the reject cause. The three measurements defined in e) are subject to the "2 out of 3 approach".

- CC

- Transmission by the SGSN of the ROUTING AREA UPDATE REJECT message to the MS, in response to a " ROUTING AREA UPDATE REQUEST" ("combined RA/LA with IMSI Attach"  Intra-Sgsn) message, the relevant measurement is incremented according to the reject cause. Possible reject causes are defined within TS 24.008 [15].
The sum of all supported per cause measurements shall be equal to the total number of failed GPRS routing area update procedures. In case only a subset of per cause measurements is supported, a sum subcounter will be provided first.

- Each measurement (as defined in e) is an integer value. The number of measurements is equal to the number of causes supported plus a possible sum value identified by the .sum suffix.

- MM. FailImsiCombiIntraSgsnRAUpdate.Cause:

- -	MM. FailImsiCombiIntraSgsnRAUpdate.Cause	Combined (don't care);

- -	MM. FailImsiCombiIntraSgsnRAUpdate.Cause.G	GSM;

- -	MM. FailImsiCombiIntraSgsnRAUpdate.Cause.U	UMTS;

- where Cause identifies the reject cause.

- RA, specified by a concatenation of the MCC, MNC, LAC and the RAC.

- Valid for packet switching

- GSM/UMTS

### 4.1.12	Combined RA/LA inter-SGSN Routing Area update procedures

#### 4.1.12.1	Attempted combined RA/LA inter-SGSN Routing Area update procedures

- This measurement provides the number of combined RA/LA updates (inter-SGSN) procedures initiated in this SGSN. These are counted as attempts.
The three measurement types defined in e) are subject to the "2 out of 3 approach".

- CC.

- Receipt of "Routing Area Update REQUEST" message from the MS, indicating a combined RA/LA update (TS 24.008 [15]).

- A single integer value per measurement type defined in e).

- MM.AttCombiInterSgsnRaUpdate:

-	MM.AttCombiInterSgsnRaUpdate	Combined (don't care);

-	MM.AttCombiInterSgsnRaUpdate.G	GSM;

-	MM.AttCombiInterSgsnRaUpdate.U	UMTS.

- RA, specified by a concatenation of the MCC, MNC, LAC and the RAC.

- Valid for packet switching.

- GSM/UMTS.

#### 4.1.12.2	Successful combined RA/LA inter-SGSN Routing Area update procedures

- This measurement provides the number of success-fully performed combined RA/LA updates (inter-SGSN) procedures initiated in this SGSN.
The three measurement types defined in e) are subject to the "2 out of 3 approach".

- CC.

- Transmission of "Routing Area Update ACCEPT" message to the MS (TS 24.008 [15]).

- A single integer value per measurement type defined in e).

- MM.SuccCombiInterSgsnRaUpdate:

-	MM.SuccCombiInterSgsnRaUpdate	Combined (don't care);

-	MM.SuccCombiInterSgsnRaUpdate.G	GSM;

-	MM.SuccCombiInterSgsnRaUpdate.U	UMTS.

- RA, specified by a concatenation of the MCC, MNC, LAC and the RAC.

- Valid for packet switching.

- GSM/UMTS.

#### 4.1.12.3	Failed combined RA/LA inter-SGSN Routing Area update procedures

- This measurement provides the number of failed GPRS combined RA/LA inter-SGSN Routing Area Update procedures initiated in this SGSN. The measurement is split into subcounters per the reject cause. The three measurements defined in e) are subject to the "2 out of 3 approach".

- CC

- Transmission by the SGSN of the ROUTING AREA UPDATE REJECT message to the MS, in response to a " ROUTING AREA UPDATE REQUEST" (combined RA/LA Inter-Sgsn) message, the relevant measurement is incremented according to the reject cause. Possible reject causes are defined within TS 24.008 [15].
The sum of all supported per cause measurements shall be equal to the total number of failed GPRS routing area update procedures. In case only a subset of per cause measurements is supported, a sum subcounter will be provided first.

- Each measurement (as defined in e) is an integer value. The number of measurements is equal to the number of causes supported plus a possible sum value identified by the .sum suffix.

- MM. FailCombiInterSgsnRaUpdate.Cause:

- -	MM. FailedCombiInterSgsnRaUpdate.Cause	Combined (don't care);

- -	MM. FailedCombiInterSgsnRaUpdate.Cause.G	GSM;

- -	MM. FailedCombiInterSgsnRaUpdate.Cause.U	UMTS;

- where Cause identifies the reject cause.

- RA, specified by a concatenation of the MCC, MNC, LAC and the RAC.

- Valid for packet switching

- GSM/UMTS

### 4.1.13	"Combined RA/LA with IMSI Attach" inter-SGSN Routing Area update procedures

#### 4.1.13.1	Attempted "combined RA/LA with IMSI Attach" inter-SGSN Routing Area update procedures

- This measurement provides the number of combined RA/LA updates with IMSI attach (inter-SGSN) procedures initiated in this SGSN. These are counted as attempts:
The three measurement types defined in e) are subject to the "2 out of 3 approach".

- CC.

- Receipt of "Routing Area Update REQUEST" message from the MS, indicating a combined RA/LA update with IMSI attach.E52.

- A single integer value per measurement type defined in e).

- MM.AttImsiCombiInterSgsnRAUpdate:

-	MM.AttImsiCombiInterSgsnRAUpdate	Combined (don't care);

-	MM.AttImsiCombiInterSgsnRAUpdate.G	GSM;

-	MM.AttImsiCombiInterSgsnRAUpdate.U	UMTS.

- RA, specified by a concatenation of the MCC, MNC, LAC and the RAC.

- Valid for packet switching.

- GSM/UMTS.

#### 4.1.13.2	Failed "combined RA/LA with IMSI Attach" inter-SGSN Routing Area update procedures

- This measurement provides the number of failed GPRS "combined RA/LA with IMSI Attach" inter-SGSN Routing Area Update procedures initiated in this SGSN. The measurement is split into subcounters per the reject cause. The three measurements defined in e) are subject to the "2 out of 3 approach".

- CC

- Transmission by the SGSN of the ROUTING AREA UPDATE REJECT message to the MS, in response to a " ROUTING AREA UPDATE REQUEST" ("combined RA/LA with IMSI Attach"  Intra-Sgsn) message, the relevant measurement is incremented according to the reject cause. Possible reject causes are defined within TS 24.008 [15].
The sum of all supported per cause measurements shall be equal to the total number of failed GPRS routing area update procedures. In case only a subset of per cause measurements is supported, a sum subcounter will be provided first.

- Each measurement (as defined in e) is an integer value. The number of measurements is equal to the number of causes supported plus a possible sum value identified by the .sum suffix.

- MM. FailImsiCombiInterSgsnRAUpdate.Cause:

-	MM. FailImsiCombiInterSgsnRAUpdate.Cause	Combined (don't care);

-	MM. FailImsiCombiInterSgsnRAUpdate.Cause.G	GSM;

-	MM. FailImsiCombiInterSgsnRAUpdate.Cause.U	UMTS;

where Cause identifies the reject cause.

- RA, specified by a concatenation of the MCC, MNC, LAC and the RAC.

- Valid for packet switching

- GSM/UMTS

### 4.1.14	Number of received invalid P-TMSI's during detach

- This measurement provides the number of received invalid P-TMSI's during detach. 
The three measurement types defined in e) are subject to the "2 out of 3 approach".

- CC.

- Receipt of an "DETACH_REQUEST" with invalid P-TMSI (TS 24.008 [15]).

- A single integer value per measurement type defined in e).

- MM.NbrPTMSIDetachFail:

-	MM.NbrPTMSIDetachFail	Combined (don't care);

-	MM.NbrPTMSIDetachFail.G	GSM;

-	MM.NbrPTMSIDetachFail.U	UMTS.

- RA, specified by a concatenation of the MCC, MNC, LAC and the RAC.

- Valid for packet switching.

- GSM/UMTS.

### 4.1.15	GSM PS paging procedures

The three measurement types defined in clauses 4.1.15.n are subject to the "2 out of 3 approach".

##### 4.1.15.1	Attempted GSM PS paging procedures

- This measurement provides the number of attempted PS paging procedures initiated at the SGSN over the Gb interface. The initial paging procedures as well as the repeated paging procedures are counted.

- CC.

- Incremented when a GSM PS paging procedure is started, i.e. at the transmission of the first BSSGP Paging Request (GSM 08.18 [16]) from the SGSN to the MS.

- A single integer value.

- MM.AttPsPagingProcGb.

- RA, specified by a concatenation of the MCC, MNC, LAC and the RAC.

- Valid for packet switching.

- GSM.

##### 4.1.15.2	Successful GSM PS paging procedures

- This measurement provides the number of successful PS paging procedures initiated at the SGSN over the Gb interface. The initial paging procedures as well as the repeated paging procedures are counted.

- CC.

- Incremented when an uplink_trigger (any LLC frame) is received by the SGSN from the MS (over the Gb interface) as response to a GSM PS paging procedure (TS 23.060 [2]) or during intersystem change UMTS -> GSM.

- A single integer value.

- MM.SuccPsPagingProcGb.

- RA, specified by a concatenation of the MCC, MNC, LAC and the RAC.

- Valid for packet switching.

- GSM.

##### 4.1.15.3	Failed GSM PS paging procedures

- This measurement provides the number of failed PS paging procedures initiated at the SGSN over the Gb interface, i.e. PS paging procedures that time out. The initial paging procedures as well as the repeated paging procedures are counted.

- CC.

- Incremented when a GSM PS paging procedure times out.

- A single integer value.

- MM.FailPsPagingProcGb.

- RA, specified by a concatenation of the MCC, MNC, LAC and the RAC.

- Valid for packet switching.

- GSM.

### 4.1.16	UMTS PS paging procedures

The three measurement types defined in clauses 4.1.16.n are subject to the "2 out of 3 approach".

##### 4.1.16.1	Attempted UMTS PS paging procedures

- This measurement provides the number of attempted PS paging procedures initiated at the SGSN over the Iu interface. The initial paging procedures as well as the repeated paging procedures are counted.

- CC.

- Incremented when a UMTS PS paging procedure is started i.e. at the transmission of the first "Paging" message (TS 25.413 [5]) from the SGSN to the MS.

- A single integer value.

- MM.AttPsPagingProcIu.

- RA, specified by a concatenation of the MCC, MNC, LAC and the RAC.

- Valid for packet switching.

- UMTS.

##### 4.1.16.2	Successful UMTS PS paging procedures

- This measurement provides the number of successful PS paging procedures initiated at the SGSN over the Iu interface. The initial paging procedures as well as the repeated paging procedures are counted.

- CC.

- Incremented when a paging_response is received by the SGSN from the MS (over the Iu interface) as response to a UMTS PS paging procedure (Receipt of "Service Request" message with Service Type = Paging Response from the MS (TS 24.008 [15])) or during intersystem change GSM -> UMTS.

- A single integer value.

- MM.SuccPsPagingProcIu.

- RA, specified by a concatenation of the MCC, MNC, LAC and the RAC.

- Valid for packet switching.

- UMTS.

##### 4.1.16.3	Failed UMTS PS paging procedures

- This measurement provides the number of failed PS paging procedures initiated at the SGSN over the Iu interface, i.e. PS paging procedures that time out. The initial paging procedures as well as the repeated paging procedures are counted.

- CC.

- Incremented when a UMTS PS paging procedure times out.

- A single integer value.

- MM.FailPsPagingProcIu.

- RA, specified by a concatenation of the MCC, MNC, LAC and the RAC.

- Valid for packet switching.

- UMTS.

### 4.1.17	PS paging procedures with unknown access type

#### 4.1.17.1	Attempted PS paging procedures with unknown access type

- This measurement provides the number of attempted PS paging procedures initiated at the SGSN with access type unknown. In this case the paging will be done both over the Gb and the Iu interface. The initial paging procedures as well as the repeated paging procedures are counted.

- CC.

- Incremented when a paging procedure is started for which MM doesn't know the access type i.e. at the transmission of the first BSSGP Paging Request (GSM 08.18 [16]) and/or "Paging" message (TS 25.413 [5]) from the SGSN to the MS.

- A single integer value.

- MM.AttPsPagingProcGbIu.

- RA, specified by a concatenation of the MCC, MNC, LAC and the RAC.

- Valid for packet switching.

- Combined.

### 4.1.18	Number of PS paging message sends from 2G-SGSN to the MS

- This measurement provides the Number of PS paging message sends from 2G-SGSN to the MS.

- CC.

- Transmission of "GMM-PAGING.req" (GSM 08.18 [16]) from the SGSN to the MS. Each paging message will be counted separately, addressed to all BSS in this certain RA.

- A single integer value.

- MM.NbrPsPagingMesGb.

- RA, specified by a concatenation of the MCC, MNC, LAC and the RAC.

- Valid for packet switching.

- GSM.

### 4.1.19	Number of PS paging message sends from 3G-SGSN to the MS

- This measurement provides the Number of PS paging message sends from 3G-SGSN to the MS.

- CC.

- Transmission of "Paging" message (CN Domain Indicator = PS Domain) from the SGSN to the MS (TS 25.413 [5]). Each paging message will be counted separately, addressed to all RNC in this certain RA.

- A single integer value.

- MM.NbrPsPagingMesIu.

- RA, specified by a concatenation of the MCC, MNC, LAC and the RAC.

- Valid for packet switching.

- UMTS.

### 4.1.20	GSM subscribers state

#### 4.1.20.1	Subscribers in STANDBY state

##### 4.1.20.1.1	Number of subscribers in STANDBY state

- This measurement provides the number of subscribers in STANDBY state.

- GAUGE.

- Incremented at transition of a subscriber registered in the SGSN into STANDBY state, decremented at transition of a subscriber registered in the SGSN out from STANDBY state.

- A single integer value.

- MM.NbrSubStandby.

- SgsnFunction.

- Valid for packet switching.

- GSM.

##### 4.1.20.1.2	Mean number of subscribers in STANDBY state

- This measurement provides the mean number of subscribers in STANDBY state.

- SI.

- This measurement is obtained by sampling at a pre-defined interval the number of subscribers in STANDBY state and then taking the arithmetic mean.

- A single integer value.

- MM.MeanNbrSubStandby.

- SgsnFunction.

- Valid for packet switching.

- GSM.

##### 4.1.20.1.3	Max number of subscribers in STANDBY state

- This measurement provides the maximum number of subscribers in STANDBY state.

- SI.

- This measurement is obtained by sampling at a pre-defined interval the number of subscribers in STANDBY state and then taking the maximum.

- A single integer value.

- MM.MaxNbrSubStandby.

- SgsnFunction.

- Valid for packet switching.

- GSM.

#### 4.1.20.2	Subscribers in READY state

##### 4.1.20.2.1	Number of subscribers in READY state

- This measurement provides the number of subscribers in READY state.

- GAUGE.

- Incremented at transition of a subscriber registered in the SGSN into READY state, decremented at transition of a subscriber registered in the SGSN out from READY state.

- A single integer value.

- MM.NbrSubReady

- SgsnFunction.

- Valid for packet switching.

- GSM.

##### 4.1.20.2.2	Mean number of subscribers in READY state

- This measurement provides the mean number of subscribers in READY state.

- SI.

- This measurement is obtained by sampling at a pre-defined interval the number of subscribers in READY state and then taking the arithmetic mean.

- A single integer value.

- MM.MeanNbrSubReady.

- SgsnFunction.

- Valid for packet switching.

- GSM.

##### 4.1.20.2.3	Max number of subscribers in READY state

- This measurement provides the maximum number of subscribers in READY state.

- SI.

- This measurement is obtained by sampling at a pre-defined interval the number of subscribers in READY state and then taking the maximum.

- A single integer value.

- MM.MaxNbrSubReady.

- SgsnFunction.

- Valid for packet switching.

- GSM.

### 4.1.21	UMTS subscribers state

#### 4.1.21.1	Subscribers in PMM-IDLE state

##### 4.1.21.1.1	Number of subscribers in PMM-IDLE state

- This measurement provides the number of subscribers in PMM-IDLE state.

- GAUGE.

- Incremented at PS Signalling Connection Release (Iu Release), decremented at PS Detach or PS Signalling Connection Establish (Service Request).

- A single integer value.

- MM.NbrSubPmmIdle.

- SgsnFunction.

- Valid for packet switching.

- UMTS.

##### 4.1.21.1.2	Mean number of subscribers in PMM-IDLE state

- This measurement provides the mean number of subscribers in PMM-IDLE state.

- SI.

- This measurement is obtained by sampling at a pre-defined interval the number of subscribers in PMM-IDLE state and then taking the arithmetic mean.

- A single integer value.

- MM.MeanNbrSubPmmIdle.

- SgsnFunction.

- Valid for packet switching.

- UMTS.

##### 4.1.21.1.3	Max number of subscribers in PMM-IDLE state

- This measurement provides the maximum number of subscribers in PMM-IDLE state.

- SI.

- This measurement is obtained by sampling at a pre-defined interval the number of subscribers in PMM-IDLE state and then taking the maximum.

- A single integer value.

- MM.MaxNbrSubPmmIdle.

- SgsnFunction.

- Valid for packet switching.

- UMTS.

#### 4.1.21.2	Subscribers in PMM-CONNECTED state

##### 4.1.21.2.1	Number of subscribers in PMM-CONNECTED state

- This measurement provides the number of subscribers in PMM-CONNECTED state.

- GAUGE.

- Decremented at PS Signalling Connection Release (Iu Release), Detach, PS Attach Reject or RAU Reject, incremented at PS Attach or PS Signalling Connection Establish (Service Request).

- A single integer value.

- MM.NbrSubPmmConnected

- SgsnFunction.

- Valid for packet switching.

- UMTS.

##### 4.1.21.2.2	Mean number of subscribers in PMM-CONNECTED state

- This measurement provides the mean number of subscribers in PMM-CONNECTED state.

- SI.

- This measurement is obtained by sampling at a pre-defined interval the number of subscribers in PMM-CONNECTED state and then taking the arithmetic mean.

- A single integer value.

- MM.MeanNbrSubPmmConnected.

- SgsnFunction.

- Valid for packet switching.

- UMTS.

##### 4.1.21.2.3	Max number of subscribers in PMM-CONNECTED state

- This measurement provides the maximum number of subscribers in PMM-CONNECTED state.

- SI.

- This measurement is obtained by sampling at a pre-defined interval the number of subscribers in PMM-CONNECTED state and then taking the maximum.

- A single integer value.

- MM.MaxNbrSubPmmConnected.

- SgsnFunction.

- Valid for packet switching.

- UMTS.

### 4.1.22	Attached subscribers

#### 4.1.22.1	Number of attached subscribers

- This measurement provides the number of attached subscribers within this SGSN area.
The three measurement types defined in e) are subject to the "2 out of 3 approach".

- GAUGE.

- Incremented when a subscriber enters the GMM_REGISTERED state in the SGSN Location Register, and decremented when a subscriber leaves the GMM_REGISTERED state.

NOTE:	The GMM state machine in the SGSN Location Register is described in TS 24.008 [15], clause 4.1.3.3 (Figure 4.1c/3GPP TS 24.008 [15]: GMM main states on the network side).

- A single integer value per measurement type defined in e).

- MM.NbrActAttachedSub:

-	MM.NbrActAttachedSub	Combined (don't care);

-	MM.NbrActAttachedSub.G	GSM;

-	MM.NbrActAttachedSub.U	UMTS.

- SgsnFunction.

- Valid for packet switching.

- GSM/UMTS.

#### 4.1.22.2	Mean number of attached subscribers

- This measurement provides the mean number of attached subscribers within this SGSN area.
The three measurement types defined in e) are subject to the "2 out of 3 approach".

- SI.

- This measurement is obtained by sampling at a pre-defined interval the number of attached subscribers and then taking the arithmetic mean.

- A single integer value per measurement type defined in e).

- MM.MeanNbrAttachedSub:

-	MM.MeanNbrAttachedSub	Combined (don't care);

-	MM.MeanNbrAttachedSub.G	GSM;

-	MM.MeanNbrAttachedSub.U	UMTS.

- SgsnFunction.

- Valid for packet switching.

- GSM/UMTS.

#### 4.1.22.3	Max number of attached subscribers

- This measurement provides the maximum number of attached subscribers within this SGSN area.
The three measurement types defined in e) are subject to the "2 out of 3 approach".

- SI.

- This measurement is obtained by sampling at a pre-defined interval the number of attached subscribers and then taking the maximum.

- A single integer value per measurement type defined in e).

- MM.MaxNbrAttachedSub:

-	MM.MaxNbrAttachedSub	Combined (don't care);

-	MM.MaxNbrAttachedSub.G	GSM;

-	MM.MaxNbrAttachedSub.U	UMTS.

- SgsnFunction.

- Valid for packet switching.

- GSM/UMTS.

### 4.1.23	Home subscribers

#### 4.1.23.1	Number of home subscribers

- This measurement provides the number of GPRS home subscribers located in the SGSN location register. The GPRS MM state of this subscriber is GMM_REGISTERED or GMM_DEREGISTERED. Only GPRS subscribers that are homed in the same GPRS network are considered.
The three measurement types defined in e) are subject to the "2 out of 3 approach".

- GAUGE.

- Incremented by one when GPRS home subscriber is successfully registered in the SGSN location registered and decremented by one when GPRS home subscriber is successfully deregistered out of the SGSN location register (TS 24.008 [15]).

- A single integer value per measurement type defined in e).

- MM.NbrHomeSub:

-	MM.NbrHomeSub	Combined (don't care);

-	MM.NbrHomeSub.G	GSM;

-	MM.NbrHomeSub.U	UMTS.

- SgsnFunction.

- Valid for packet switching.

- GSM/UMTS.

#### 4.1.23.2	Mean number of home subscribers

- This measurement provides the mean number of GPRS home subscribers located in the SGSN location register.
The three measurement types defined in e) are subject to the "2 out of 3 approach".

- SI.

- This measurement is obtained by sampling at a pre-defined interval the number of GPRS home subscribers located in the SGSN location register and then taking the arithmetic mean.

- A single integer value per measurement type defined in e).

- MM.MeanNbrHomeSub:

-	MM.MeanNbrHomeSub	Combined (don't care);

-	MM.MeanNbrHomeSub.G	GSM;

-	MM.MeanNbrHomeSub.U	UMTS.

- SgsnFunction.

- Valid for packet switching.

- GSM/UMTS.

#### 4.1.23.3	Max number of home subscribers

- This measurement provides the maximum number of GPRS home subscribers located in the SGSN location register.
The three measurement types defined in e) are subject to the "2 out of 3 approach".

- SI.

- This measurement is obtained by sampling at a pre-defined interval the number of GPRS home subscribers located in the SGSN location register and then taking the maximum.

- A single integer value per measurement type defined in e).

- MM.MaxNbrHomeSub:

-	MM.MaxNbrHomeSub	Combined (don't care);

-	MM.MaxNbrHomeSub.G	GSM;

-	MM.MaxNbrHomeSub.U	UMTS.

- SgsnFunction.

- Valid for packet switching.

- GSM/UMTS.

### 4.1.24	Visiting national subscribers

#### 4.1.24.1	Number of visiting national subscribers

- This measurement provides the number of visiting national GPRS subscribers located in the SGSN location register. The GPRS MM state of this subscriber is GMM_REGISTERED or GMM_DEREGISTERED. Only GPRS subscribers that are homed in a partner GPRS network of the same country are considered.
The three measurement types defined in e) are subject to the "2 out of 3 approach".

- GAUGE.

- This measurement provides the number of visiting national GPRS subscribers located in the SGSN location register. The GPRS MM state of this subscriber is GMM_REGISTERED or GMM_DEREGISTERED. Only GPRS subscribers that are homed in a partner GPRS network of the same country are considered.

- A single integer value per measurement type defined in e).

- MM.NbrVisitingNatSub:

-	MM.NbrVisitingNatSub	Combined (don't care);

-	MM.NbrVisitingNatSub.G	GSM;

-	MM.NbrVisitingNatSub.U	UMTS.

- SgsnFunction.

- Valid for packet switching.

- GSM/UMTS.

#### 4.1.24.2	Mean number of visiting national subscribers

- This measurement provides the mean number of visiting national GPRS subscribers located in the SGSN location register.
The three measurement types defined in e) are subject to the "2 out of 3 approach".

- SI.

- This measurement is obtained by sampling at a pre-defined interval the number of visiting national GPRS subscribers located in the SGSN location register and then taking the arithmetic mean.

- A single integer value per measurement type defined in e).

- MM.MeanNbrVisitingNatSub:

-	MM.MeanNbrVisitingNatSub	Combined (don't care);

-	MM.MeanNbrVisitingNatSub.G	GSM;

-	MM.MeanNbrVisitingNatSub.U	UMTS.

- SgsnFunction.

- Valid for packet switching.

- GSM/UMTS.

#### 4.1.24.3	Max number of visiting national subscribers

- This measurement provides the maximum number of visiting national GPRS subscribers located in the SGSN location register.
The three measurement types defined in e) are subject to the "2 out of 3 approach".

- SI.

- This measurement is obtained by sampling at a pre-defined interval the number of visiting national GPRS subscribers located in the SGSN location register and then taking the maximum.

- A single integer value per measurement type defined in e).

- MM.MaxNbrVisitingNatSub:

-	MM.MaxNbrVisitingNatSub	Combined (don't care);

-	MM.MaxNbrVisitingNatSub.G	GSM;

-	MM.MaxNbrVisitingNatSub.U	UMTS.

- SgsnFunction.

- Valid for packet switching.

- GSM/UMTS.

### 4.1.25	Visiting foreign subscribers

#### 4.1.25.1	Number of visiting foreign subscribers

- This measurement provides the number of visiting foreign GPRS located in the SGSN location register. The GPRS MM state of this subscriber is GMM_REGISTERED or MM_DEREGISTERED. Only GPRS subscribers that are homed in a GPRS network of a foreign country are considered.
The three measurement types defined in e) are subject to the "2 out of 3 approach".

- GAUGE.

- Incremented by one when GPRS subscriber is successfully registered in the SGSN location registered and decremented by one when GPRS subscriber is successfully deregistered out of the SGSN location register (TS 24.008 [15]).

- A single integer value per measurement type defined in e).

- MM.NbrVisitingForeign:

-	MM.NbrVisitingForeign	Combined (don't care);

-	MM.NbrVisitingForeign.G	GSM;

-	MM.NbrVisitingForeign.U	UMTS.

- SgsnFunction.

- Valid for packet switching.

- GSM/UMTS.

#### 4.1.25.2	Mean number of visiting foreign subscribers

- This measurement provides the mean number of visiting foreign GPRS subscribers located in the SGSN location register.
The three measurement types defined in e) are subject to the "2 out of 3 approach".

- SI.

- This measurement is obtained by sampling at a pre-defined interval the number of visiting foreign GPRS subscribers located in the SGSN location register and then taking the arithmetic mean.

- A single integer value per measurement type defined in e).

- MM.MeanNbrVisitingForeign:

-	MM.MeanNbrVisitingForeign	Combined (don't care);

-	MM.MeanNbrVisitingForeign.G	GSM;

-	MM.MeanNbrVisitingForeign.U	UMTS.

- SgsnFunction.

- Valid for packet switching.

- GSM/UMTS.

#### 4.1.25.3	Max number of visiting foreign subscribers

- This measurement provides the maximum number of visiting foreign GPRS subscribers located in the SGSN location register.
The three measurement types defined in e) are subject to the "2 out of 3 approach".

- SI.

- This measurement is obtained by sampling at a pre-defined interval the number of visiting foreign GPRS subscribers located in the SGSN location register and then taking the maximum.

- A single integer value per measurement type defined in e).

- MM.MaxNbrVisitingForeign:

-	MM.MaxNbrVisitingForeign	Combined (don't care);

-	MM.MaxNbrVisitingForeign.G	GSM;

-	MM.MaxNbrVisitingForeign.U	UMTS.

- SgsnFunction.

- Valid for packet switching.

- GSM/UMTS.

### 4.1.26	Void

### 4.1.27	Void

### 4.1.28	Void

### 4.1.29	Void

### 4.1.30	CAMEL subscribers

#### 4.1.30.1	Number of CAMEL subscribers

- This measurement provides the number of attached subscriber within this SGSN area with CAMEL service.
The three measurement types defined in e) are subject to the "2 out of 3 approach".

- GAUGE.

- Incremented when a CAMEL subscriber enters the GMM_REGISTERED state in the SGSN Location Register, and decremented when a subscriber leaves the GMM_REGISTERED state.
Note: the GMM state machine in the SGSN Location Register is described in 3GPP TS 24.008 [15], subclause 4.1.3.3 (Figure 4.1c/3GPP TS 24.008 [15]: GMM main states on the network side).

- A single integer value per measurement type defined in e).

- MM.NbrCamelSub:

-	MM.NbrCamelSub	Combined (don't care);

-	MM.NbrCamelSub.G	GSM;

-	MM.NbrCamelSub.U	UMTS.

- SgsnFunction.

- Valid for packet switching.

- GSM/UMTS.

#### 4.1.30.2	Mean Number of CAMEL subscribers

- This measurement provides the mean number of attached subscribers with CAMEL service.
The three measurement types defined in e) are subject to the "2 out of 3 approach".

- SI.

- This measurement is obtained by sampling at a pre-defined interval the number of attached subscribers with CAMEL service and then taking the arithmetic mean.

- A single integer value per measurement type defined in e).

- MM.MeanNbrCamelSub:

-	MM.MeanNbrCamelSub	Combined (don't care);

-	MM.MeanNbrCamelSub.G	GSM;

-	MM.MeanNbrCamelSub.U	UMTS.

- SgsnFunction.

- Valid for packet switching.

- GSM/UMTS.

#### 4.1.30.3	Max number of CAMEL subscribers

- This measurement provides the maximum number of attached subscribers with CAMEL service.
The three measurement types defined in e) are subject to the "2 out of 3 approach".

- SI.

- This measurement is obtained by sampling at a pre-defined interval the number of attached subscribers with CAMEL service and then taking the maximum.

- A single integer value per measurement type defined in e).

- MM.MaxNbrCamelSub:

-	MM.MaxNbrCamelSub	Combined (don't care);

-	MM.MaxNbrCamelSub.G	GSM;

-	MM.MaxNbrCamelSub.U	UMTS.

- SgsnFunction.

- Valid for packet switching.

- GSM/UMTS.

### 4.1.31	Void

### 4.1.32	InsertSubscriberData requests received from a HLR during GPRS Update Location procedure

#### 4.1.32.1	Attempted InsertSubscriberData requests received from a HLR during GPRS Update Location procedure

- This measurement provides the number of InsertSubscriberData requests received from a HLR during GPRS Update Location procedure.

- CC.

- Receipt of a "MAP-INSERT-SUBSCRIBER-DATA" service request (TS 29.002 [4]) during a GPRS Update Location procedure.

- A single integer value.

- MM.AttInsertSubscrDataHlrUpdLoc.

- SgsnFunction.

- Valid for packet switching.

- Combined.

### 4.1.33	GPRS Update Locations sent to the HLR

#### 4.1.33.1	Attempted GPRS Update Locations sent to the HLR

- This measurement provides the number of GPRS Update Locations sent to the HLR.

- CC.

- Transmission of a 'MAP_UPDATE_LOCATION' service request (TS 29.002 [4]).

- A single integer value.

- MM.AttUpdateGprsLocationHlr.

- SgsnFunction.

- Valid for packet switching.

- Combined.

#### 4.1.33.2	Successful GPRS Update Locations returned from the HLR

- This measurement provides the number of successful GPRS Update Locations returned from the HLR.

- CC.

- Receipt of a 'MAP_UPDATE_LOCATION ack' service response/confirm  indicating a successful GPRS Update location (TS 29.002 [4]).

- A single integer value.

- MM.SuccUpdateGprsLocationHlr.

- SgsnFunction.

- Valid for packet switching.

- Combined.

### 4.1.34	CancelLocation requests received from an HLR-operator, in case of a HLR-initiated Detach

#### 4.1.34.1	Attempted CancelLocation requests received from an HLR-operator, in case of a HLR-initiated Detach

- This measurement provides the number of CancelLocation requests received from an HLR-operator, in case of a HLR-initiated Detach.

- CC.

- Receipt of a 'MAP_CANCEL_LOCATION' service request (TS 29.002 [4]).

- A single integer value.

- MM.AttCancelLocHlrOp.

- SgsnFunction.

- Valid for packet switching.

- Combined.

### 4.1.35	CancelLocation requests received from a HLR due to a SGSN-change (previous SGSN)

#### 4.1.35.1	Attempted CancelLocation requests received from a HLR due to a SGSN-change (previous SGSN)

- This measurement provides the number of CancelLocation requests received from a HLR due to a SGSN-change(previous SGSN).

- CC.

- Receipt of a 'MAP_CANCEL_LOCATION' service request (TS 29.002 [4]) due to a SGSN-change (previous SGSN).

- A single integer value.

- MM.AttCancelLocHlrSgsnChg.

- SgsnFunction.

- Valid for packet switching.

- Combined.

### 4.1.36	Reset requests received from a HLR due to an HLR restart, indicating that a failure occurred

#### 4.1.36.1	Attempted Reset requests received from a HLR due to an HLR restart, indicating that a failure occurred

- This measurement provides the number of Reset requests received from a HLR due to an HLR restart, indicating that a failure occurred.

- CC.

- Receipt of a 'MAP_RESET' service request (TS 29.002 [4]) from a HLR.

- A single integer value.

- MM.AttResetHlr.

- SgsnFunction.

- Valid for packet switching.

- Combined.

### 4.1.37	Periodic Routing Area update procedures

#### 4.1.37.1	Attempted periodic Routing Area update procedures

- This measurement provides the number of periodic Routing Area updates  procedures initiated in this SGSN. These are counted as attempts.
The three measurement types defined in e) are subject to the "2 out of 3 approach".

- CC.

- Receipt of "Routing Area Update REQUEST" message from the MS with the value of the update type IE in the ROUTING AREA UPDATE REQUEST message indicating "periodic updating"(TS 24.008 [15]).

- A single integer value per measurement type defined in e).

- MM.AttPeriodRaUpdate:

-	MM.AttPeriodRaUpdate	Combined (don't care);

-	MM.AttPeriodRaUpdate.G	GSM;

-	MM.AttPeriodRaUpdate.U	UMTS.

- RA, specified by a concatenation of the MCC, MNC, LAC and the RAC.

- Valid for packet switching.

- GSM/UMTS.

#### 4.1.37.2	Successful periodic Routing Area update procedures

- This measurement provides the number of successfully performed Routing Area periodic updates procedures initiated in this SGSN.
The three measurement types defined in e) are subject to the "2 out of 3 approach".

- CC.

- Transmission of "Routing Area Update ACCEPT" message to the MS in response to a Routing Area Update REQUEST message with  the value of the update type IE in the ROUTING AREA UPDATE REQUEST message indicating "periodic updating"  (TS 24.008 [15]).

- A single integer value per measurement type defined in e).

- MM.SuccPeriodRaUpdate:

-	MM.SuccPeriodRaUpdate	Combined (don't care);

-	MM.SuccPeriodRaUpdate.G	GSM;

-	MM.SuccPeriodRaUpdate.U	UMTS.

- RA, specified by a concatenation of the MCC, MNC, LAC and the RAC.

- Valid for packet switching.

- GSM/UMTS.

#### 4.1.37.3	Failed Periodic Routing Area update procedures

- This measurement provides the number of failed GPRS Periodic Routing Area Update procedures initiated in this SGSN. The measurement is split into subcounters per the reject cause. The three measurements defined in e) are subject to the "2 out of 3 approach".

- CC

- Transmission by the SGSN of the ROUTING AREA UPDATE REJECT message to the MS, in response to a " ROUTING AREA UPDATE REQUEST" (Periodic) message, the relevant measurement is incremented according to the reject cause. Possible reject causes are defined within TS 24.008 [15].
The sum of all supported per cause measurements shall be equal to the total number of failed GPRS routing area update procedures. In case only a subset of per cause measurements is supported, a sum subcounter will be provided first.

- Each measurement (as defined in e) is an integer value. The number of measurements is equal to the number of causes supported plus a possible sum value identified by the .sum suffix.

- MM. FailPeriodRaUpdate.Cause:

-	MM.FailPeriodRaUpdate.Cause	Combined (don't care);

-	MM.FailPeriodRaUpdate.Cause.G	GSM;

-	MM.FailPeriodRaUpdate.Cause.U	UMTS;

where Cause identifies the reject cause.

- RA, specified by a concatenation of the MCC, MNC, LAC and the RAC.

- Valid for packet switching

- GSM/UMTS

## 4.2	Subscriber Management

### 4.2.1	Insert Subscriber Data requests received from a HLR due to an HLR-operator intervention

#### 4.2.1.1	Attempted Insert Subscriber Data requests received from a HLR due to an HLR-operator intervention

- This measurement provides the number of Insert Subscriber Data requests received from a HLR due to an HLR-operator intervention.
The three measurement types defined in e) are subject to the "2 out of 3 approach".

- CC.

- Receipt of a "MAP-INSERT-SUBSCRIBER-DATA" service request (TS 29.002 [4]).

- A single integer value per measurement type defined in e).

- SUB.AttInsertSubscrDataHlrOp:

-	SUB.AttInsertSubscrDataHlrOp	Combined (don't care);

-	SUB.AttInsertSubscrDataHlrOp.G	GSM;

-	SUB.AttInsertSubscrDataHlrOp.U	UMTS.

- SgsnFunction.

- Valid for packet switching.

- GSM/UMTS.

### 4.2.2	Delete Subscriber Data requests received from a HLR due to an HLR-operator intervention

#### 4.2.2.1	Attempted Delete Subscriber Data requests received from a HLR due to an HLR-operator intervention

- This measurement provides the number of Delete Subscriber Data requests received from a HLR due to an HLR-operator intervention.
The three measurement types defined in e) are subject to the "2 out of 3 approach".

- CC.

- Receipt of a "MAP_DELETE_SUBSCRIBER_DATA" service request (TS 29.002 [4]).

- A single integer value per measurement type defined in e).

- SUB.AttDeleteSubscrDataHlrOp:

-	SUB.AttDeleteSubscrDataHlrOp	Combined (don't care);

-	SUB.AttDeleteSubscrDataHlrOp.G	GSM;

-	SUB.AttDeleteSubscrDataHlrOp.U	UMTS.

- SgsnFunction.

- Valid for packet switching.

- GSM/UMTS.

## 4.3	SRNS Relocation

### 4.3.1	Intra 3G-SGSN SRNS Relocations

The four measurements defined in the clause 4.3.1 are subject to the "3 out of 4 approach".

NOTE:	Number of Attempted intra 3G-SGSN SRNS Relocations = Number of Successful intra 3G-SGSN SRNS Relocations + Number of Failed intra 3G-SGSN SRNS Relocations, due to internal reasons + Number of Failed intra 3G-SGSN SRNS Relocations, due to external reasons.

#### 4.3.1.1	Attempted intra 3G-SGSN SRNS Relocations

- This measurement provides the number of attempts intra 3G-SGSN SRNS Relocations.

- CC.

- Receipt of "Relocation Required" message (TS 25.413 [5]) from SRNC, where the Target ID indicates Intra SGSN SRNS Relocation.

- A single integer value.

- RELOC.AttIntraSGSN.

- SgsnFunction.

- Valid for packet switching.

- UMTS.

#### 4.3.1.2	Successful intra 3G-SGSN SRNS Relocations

- This measurement provides the number of successful intra 3G-SGSN SRNS Relocations.

- CC.

- Receipt of "Relocation Complete" message for intra 3G-SGSN SRNS Relocation (TS 25.413 [5]) from TRNC.

- A single integer value.

- RELOC.SuccIntraSGSN.

- SgsnFunction.

- Valid for packet switching.

- UMTS.

#### 4.3.1.3	Failed intra 3G-SGSN SRNS Relocations, due to internal reasons

- This measurement provides the number of failed intra 3G-SGSN SRNS Relocations, due to internal reasons.

- CC.

- Intra 3G-SGSN SRNS Relocation fails due to reasons located inside this SGSN:- internal resource problem- recovery- ...

- A single integer value.

- RELOC.FailIntraSGSNInt.

- SgsnFunction.

- Valid for packet switching.

- UMTS.

#### 4.3.1.4	Failed intra 3G-SGSN SRNS Relocations, due to external reasons

- This measurement provides the number of failed intra 3G-SGSN SRNS Relocations, due to external reasons.

- CC.

- Intra 3G-SGSN SRNS Relocation fails due to reasons located in NE outside this SGSN:- "Relocation Preparation Failure" (TS 25.413 [5]) is sent to the SRNC- "Relocation Failure" (TS 25.413 [5]) is received from the TRNC- "Relocation Cancel" (TS 25.413 [5]) is received from the SRNC- missing expected message from RNC (timer expiry)- ...

- A single integer value.

- RELOC.FailIntraSGSNExt.

- SgsnFunction.

- Valid for packet switching.

- UMTS.

### 4.3.2	Inter 3G-SGSN SRNS Relocations

The four measurements defined in the clause 4.3.2 are subject to the "3 out of 4 approach".

NOTE:	Number Attempted inter 3G-SGSN SRNS Relocations, counted in the old 3G-SGSN = Number of Successful inter 3G-SGSN SRNS Relocations, counted in the old 3G-SGSN + Number of Failed inter 3G-SGSN SRNS Relocations, due to external reasons, counted in the old 3G-SGSN + Number of Failed inter 3G-SGSN SRNS Relocations, due to external reasons, counted in the old 3G-SGSN.

#### 4.3.2.1	Attempted inter 3G-SGSN SRNS Relocations, counted in the old 3G-SGSN

- This measurement provides the number of attempted inter 3G-SGSN SRNS Relocations.

- CC.

- Receipt of "Relocation Required" message (TS 25.413 [5]) from SRNC, where the Target ID indicates Inter SGSN SRNS Relocation.

- A single integer value.

- RELOC.AttInterSGSN.

- SgsnFunction.

- Valid for packet switching.

- UMTS.

#### 4.3.2.2	Successful inter 3G-SGSN SRNS Relocations, counted in the old 3G-SGSN

- This measurement provides the number of successful Inter 3G-SGSN SRNS Relocations, counted in the old 3G-SGSN.

- CC.

- Receipt of "Forward Relocation Complete" message (TS 29.060 [6]) from the new SGSN.

- A single integer value.

- RELOC.SuccInterSGSN.

- SgsnFunction.

- Valid for packet switching.

- UMTS.

#### 4.3.2.3	Failed inter 3G-SGSN SRNS Relocations, due to internal reasons, counted in the old 3G-SGSN

- This measurement provides the number of failed Inter 3G-SGSN SRNS Relocations, due to internal reasons.

- CC.

- Inter 3G-SGSN SRNS Relocation fails due to reasons located inside this SGSN:- internal resource problem- recovery- ...

- A single integer value.

- RELOC.FailInterSGSNInt.

- SgsnFunction.

- Valid for packet switching.

- UMTS.

#### 4.3.2.4	Failed inter 3G-SGSN SRNS Relocations, due to external reasons, counted in the old 3G-SGSN

- This measurement provides the number of failed Inter 3G-SGSN SRNS Relocations, due to external reasons.

- CC.

- Inter 3G-SGSN SRNS Relocation fails due to reasons located in NE outside this SGSN:- the Cause in "Forward Relocation Response" (TS 29.060 [6]) from the new 3G-SGSN is not "Request Accepted"- "Relocation Preparation Failure" (TS 25.413 [5]) is sent to the SRNC- "Relocation Failure" (TS 25.413 [5]) is received from the TRNC- "Relocation Cancel" (TS 25.413 [5]) is received from the SRNC- missing expected message from RNC or new 3G-SGSN (timer expiry)- ...

- A single integer value.

- RELOC.FailInterSGSNExt.

- SgsnFunction.

- Valid for packet switching.

- UMTS.

### 4.3.3	Inter 3G-SGSN SRNS Relocations, counted in the new 3G-SGSN

#### 4.3.3.1	Attempted inter 3G-SGSN SRNS Relocations, counted in the new 3G-SGSN

- This measurement provides the number of attempted inter 3G-SGSN SRNS Relocations, counted in the new 3G-SGSN.

- CC.

- Receipt of "Forward Relocation Request" message (TS 29.060 [6]) from the old SGSN.

- A single integer value.

- RELOC.AttInterSGSNNew.

- SgsnFunction.

- Valid for packet switching.

- UMTS.

#### 4.3.3.2	Successful Inter 3G-SGSN SRNS Relocations, counted in the new 3G-SGSN

- This measurement provides the number of successful Inter 3G-SGSN SRNS Relocations, counted in the new 3G-SGSN.

- CC.

- Transmission of "Forward Relocation Complete" message (TS 29.060 [6]) to the old SGSN.

- A single integer value.

- RELOC.SuccInterSGSNNew.

- SgsnFunction.

- Valid for packet switching.

- UMTS.

## 4.4	Security

### 4.4.1	P-TMSI reallocation procedures

#### 4.4.1.1	Attempted P-TMSI reallocation procedures

- This measurement provides the number of attempted P-TMSI reallocation, or implicitly as part of the Location Updating, procedures in this SGSN.
The three measurement types defined in e) are subject to the "2 out of 3 approach".

- CC.

- Transmission of "P-TMSI REALLOCATION COMMAND" message by the SGSN (TS 24.008 [15]).

- A single integer value per measurement type defined in e).

- SEC.AttPTMSIRealloc:

-	SEC.AttPTMSIRealloc	Combined (don't care);

-	SEC.AttPTMSIRealloc.G	GSM;

-	SEC.AttPTMSIRealloc.U	UMTS.

- SgsnFunction.

- Valid for packet switching.

- GSM/UMTS.

#### 4.4.1.2	Successful P-TMSI reallocation procedures

- This measurement provides the number of successfully performed P-TMSI reallocation procedures in this SGSN.
The three measurement types defined in e) are subject to the "2 out of 3 approach".

- CC.

- Receipt of "P-TMSI REALLOCATION COMPLETE" message by the SGSN (TS 24.008 [15]).

- A single integer value per measurement type defined in e).

- SEC.SuccPTMSIRealloc:

-	SEC.SuccPTMSIRealloc	Combined (don't care);

-	SEC.SuccPTMSIRealloc.G	GSM;

-	SEC.SuccPTMSIRealloc.U	UMTS.

- SgsnFunction.

- Valid for packet switching.

- GSM/UMTS.

### 4.4.2	Identity Request procedures initiated by this SGSN

#### 4.4.2.1	Attempted Identity Request procedures initiated by this SGSN

- This measurement provides the number of attempted Identity Request procedures initiated by this SGSN.
The three measurement types defined in e) are subject to the "2 out of 3 approach".

- CC.

- Transmission of an "IDENTITY REQUEST" (with Identity Type = IMSI) message to the MS (TS 24.008 [15]).

- A single integer value per measurement type defined in e).

- SEC.AttIdentityReqImsi:

-	SEC.AttIdentityReqImsi	Combined (don't care);

-	SEC.AttIdentityReqImsi.G	GSM;

-	SEC.AttIdentityReqImsi.U	UMTS.

- SgsnFunction.

- Valid for packet switching.

- GSM/UMTS.

#### 4.4.2.2	Successful completed Identity Request procedures initiated by this SGSN

- This measurement provides the number of successfully completed Identity Request procedures initiated by this SGSN.
The three measurement types defined in e) are subject to the "2 out of 3 approach".

- CC.

- Receipt of an "IDENTITY RESPONSE" message with IMSI by the SGSN from the MS (TS 24.008 [15]).

- A single integer value per measurement type defined in e).

- SEC.SuccIdentityReqImsi:

-	SEC.SuccIdentityReqImsi	Combined (don't care);

-	SEC.SuccIdentityReqImsi.G	GSM;

-	SEC.SuccIdentityReqImsi.U	UMTS.

- SgsnFunction.

- Valid for packet switching.

- GSM/UMTS.

### 4.4.3	Identification information requests sent to a partner (previous) SGSN for subscribers registering afresh in this SGSN

#### 4.4.3.1	Attempted identification information requests sent to a partner (previous) SGSN for subscribers registering afresh in this SGSN

- This measurement provides the number of identification information requests sent to a partner (previous) SGSN for subscribers registering afresh in this SGSN.
The three measurement types defined in e) are subject to the "2 out of 3 approach".

- CC.

- Transmission of the "Identification Request" message to the old SGSN (TS 29.060 [6]).

- A single integer value per measurement type defined in e).

- SEC.AttIdentityReqToPsgsn:

-	SEC.AttIdentityReqToPsgsn	Combined (don't care);

-	SEC.AttIdentityReqToPsgsn.G	GSM;

-	SEC.AttIdentityReqToPsgsn.U	UMTS.

- SgsnFunction.

- Valid for packet switching.

- GSM/UMTS.

#### 4.4.3.2	Successful replied identification information requests that were sent to a partner (previous) SGSN

- This measurement provides the number of successfully replied identification information requests that were sent to a partner (previous) SGSN. 
The three measurement types defined in e) are subject to the "2 out of 3 approach".

- CC.

- Receipt of the "Identification Response" message from the old SGSN (TS 29.060 [6]).

- A single integer value per measurement type defined in e).

- SEC.SuccIdentityReqToPsgsn:

-	SEC.SuccIdentityReqToPsgsn	Combined (don't care);

-	SEC.SuccIdentityReqToPsgsn.G	GSM;

-	SEC.SuccIdentityReqToPsgsn.U	UMTS.

- SgsnFunction.

- Valid for packet switching.

- GSM/UMTS.

### 4.4.4	Attempted Identity Requests sent to the MS

#### 4.4.4.1	Attempted Identity Requests sent to the MS

- This measurement provides the number of Identity Requests sent to the MS.
The three measurement types defined in e) are subject to the "2 out of 3 approach".

- CC.

- Transmission of an "IDENTITY REQUEST" message (TS 24.008 [15]).

- A single integer value per measurement type defined in e).

- SEC.AttIdentityRequest:

-	SEC.AttIdentityRequest	Combined (don't care);

-	SEC.AttIdentityRequest.G	GSM;

-	SEC.AttIdentityRequest.U	UMTS.

- SgsnFunction.

- Valid for packet switching.

- GSM/UMTS.

#### 4.4.4.2	Successful replied Identity Requests from the MS

- This measurement provides the number of successfully replied Identity Requests from the MS.
The three measurement types defined in e) are subject to the "2 out of 3 approach".

- CC.

- Receipt of an "IDENTITY RESPONSE" message (TS 24.008 [15]) from the MS.

- A single integer value per measurement type defined in e).

- SEC.SuccIdentityRequest:

-	SEC.SuccIdentityRequest	Combined (don't care);

-	SEC.SuccIdentityRequest.G	GSM;

-	SEC.SuccIdentityRequest.U	UMTS.

- SgsnFunction.

- Valid for packet switching.

- GSM/UMTS.

### 4.4.5	Authentication procedures that are started within this SGSN area for a subscriber using a SIM

#### 4.4.5.1	Attempted authentication procedures that are started within this SGSN area for a subscriber using a SIM

- This measurement provides the number of authentication procedures that are started within this SGSN area for a subscriber using a SIM.
The three measurement types defined in e) are subject to the "2 out of 3 approach".

- CC.

- Transmission of an "AUTHENTICATION AND CIPHERING REQUEST" message to a MS using a SIM (TS 24.008 [15]).

- A single integer value per measurement type defined in e).

- SEC.AttAuthProcsSgsnSim:

-	SEC.AttAuthProcsSgsnSim	Combined (don't care);

-	SEC.AttAuthProcsSgsnSim.G	GSM;

-	SEC.AttAuthProcsSgsnSim.U	UMTS.

- SgsnFunction.

- Valid for packet switching.

- GSM/UMTS.

#### 4.4.5.2	Successful authentication procedures within this SGSN area, for a subscriber using a SIM

- This measurement provides the number of successful authentication procedures within this SGSN area, for a subscriber using a SIM.
The three measurement types defined in e) are subject to the "2 out of 3 approach".

- CC.

- Receipt of an "AUTHENTICATION AND CIPHERING RESPONSE" message from the MS, using a SIM, where the receipt SRES parameter value matches the value stored in the SGSN (TS 24.008 [15]).

- A single integer value per measurement type defined in e).

- SEC.SuccAuthProcsSgsnSim:

-	SEC.SuccAuthProcsSgsnSim	Combined (don't care);

-	SEC.SuccAuthProcsSgsnSim.G	GSM;

-	SEC.SuccAuthProcsSgsnSim.U	UMTS.

- SgsnFunction.

- Valid for packet switching.

- GSM/UMTS.

### 4.4.6	Authentication procedures that are started within this SGSN area for a subscriber using a USIM

#### 4.4.6.1	Attempted authentication procedures that are started within this SGSN area for a subscriber using a USIM

- This measurement provides the number of authentication procedures that are started within this SGSN area for a subscriber using a USIM.
The three measurement types defined in e) are subject to the "2 out of 3 approach".

- CC.

- Transmission of an "AUTHENTICATION AND CIPHERING REQUEST" message to a MS using a USIM (TS 24.008 [15]).

- A single integer value per measurement type defined in e).

- SEC.AttAuthProcsSgsnUsim:

-	SEC.AttAuthProcsSgsnUsim	Combined (don't care);

-	SEC.AttAuthProcsSgsnUsim.G	GSM;

-	SEC.AttAuthProcsSgsnUsim.U	UMTS.

- SgsnFunction.

- Valid for packet switching.

- GSM/UMTS.

#### 4.4.6.2	Successful authentication procedures within this SGSN area, for a subscriber using a USIM

- This measurement provides the number of successful authentication procedures within this SGSN area, for a subscriber using a USIM.
The three measurement types defined in e) are subject to the "2 out of 3 approach".

- CC.

- Receipt of an "AUTHENTICATION AND CIPHERING RESPONSE" message from the MS , using a USIM, where the receipt RES parameter value matches the value stored in the SGSN (TS 24.008 [15]).

- A single integer value per measurement type defined in e).

- SEC.SuccAuthProcsSgsnUsim:

-	SEC.SuccAuthProcsSgsnUsim	Combined (don't care);

-	SEC.SuccAuthProcsSgsnUsim.G	GSM;

-	SEC.SuccAuthProcsSgsnUsim.U	UMTS.

- SgsnFunction.

- Valid for packet switching.

- GSM/UMTS.

#### 4.4.6.3	Received ciphering and Authentication failures within this SGSN area

- This measurement provides the number of ciphering and Authentication failures within this SGSN area.
The three measurement types defined in e) are subject to the "2 out of 3 approach".

- CC.

- Receipt of an "AUTHENTICATION AND CIPHERING FAILURE" message from the MS (TS 24.008 [15]).

- A single integer value per measurement type defined in e).

- SEC.RecPOAuthFailSgsn:

-	SEC.RecPOAuthFailSgsn	Combined (don't care);

-	SEC.RecPOAuthFailSgsn.G	GSM;

-	SEC.RecPOAuthFailSgsn.U	UMTS.

- SgsnFunction.

- Valid for packet switching.

- GSM/UMTS.

### 4.4.7	Identification information requests that were received from a partner (new) SGSN for subscribers de-registering from this SGSN

#### 4.4.7.1	Attempted identification information requests that were received from a partner (new) SGSN for subscribers de-registering from this SGSN

- This measurement provides the number of identification information requests that were received from a partner (new) SGSN for subscribers de-registering from this SGSN.
The three measurement types defined in e) are subject to the "2 out of 3 approach".

- CC.

- Receipt of the "Identification Request" message from a partner (new) SGSN (TS 29.060 [6]).

- A single integer value per measurement type defined in e).

- SEC.AttIdentityReqFromPsgsn:

-	SEC.AttIdentityReqFromPsgsn	Combined (don't care);

-	SEC.AttIdentityReqFromPsgsn.G	GSM;

-	SEC.AttIdentityReqFromPsgsn.U	UMTS.

- SgsnFunction.

- Valid for packet switching.

- GSM/UMTS.

#### 4.4.7.2	Successfully replied identification information requests that were received from a partner (new) SGSN

- This measurement provides the number of successfully replied identification information requests that were received from a partner (new) SGSN.
The three measurement types defined in e) are subject to the "2 out of 3 approach".

- CC.

- Transmission of the "Identification Response" message to the new SGSN (TS 29.060 [6]).

- A single integer value per measurement type defined in e).

- SEC.SuccIdentityReqFromPsgsn:

-	SEC.SuccIdentityReqFromPsgsn	Combined (don't care);

-	SEC.SuccIdentityReqFromPsgsn.G	GSM;

-	SEC.SuccIdentityReqFromPsgsn.U	UMTS.

- SgsnFunction.

- Valid for packet switching.

- GSM/UMTS.

### 4.4.8	SGSN context requests sent to a partner (previous) SGSN for subscribers registering afresh in this SGSN

#### 4.4.8.1	Attempted SGSN context requests sent to a partner (previous) SGSN for subscribers registering afresh in this SGSN

- This measurement provides the number of SGSN context requests sent to a partner (previous) SGSN for subscribers registering afresh in this SGSN.
The three measurement types defined in e) are subject to the "2 out of 3 approach".

- CC.

- Transmission of the "Context Request" message to the previous SGSN (TS 29.060 [6]).

- A single integer value per measurement type defined in e).

- SEC.AttContextRequestToPsgsn:

-	SEC.AttContextRequestToPsgsn	Combined (don't care);

-	SEC.AttContextRequestToPsgsn.G	GSM;

-	SEC.AttContextRequestToPsgsn.U	UMTS.

- SgsnFunction.

- Valid for packet switching.

- GSM/UMTS.

#### 4.4.8.2	Successfully replied SGSN context requests that were sent to a partner (previous) SGSN

- This measurement provides the number of successfully replied SGSN context requests that were sent to a partner (previous) SGSN.
The three measurement types defined in e) are subject to the "2 out of 3 approach".

- CC.

- Receipt of the "Context Response" message from the previous SGSN (TS 29.060 [6]).

- A single integer value per measurement type defined in e).

- SEC.SuccContextRequestToPsgsn:

-	SEC.SuccContextRequestToPsgsn	Combined (don't care);

-	SEC.SuccContextRequestToPsgsn.G	GSM;

-	SEC.SuccContextRequestToPsgsn.U	UMTS.

- SgsnFunction.

- Valid for packet switching.

- GSM/UMTS.

### 4.4.9	SGSN context requests received from a partner (new) SGSN for a subscriber de-registering from this SGSN

#### 4.4.9.1	Attempted SGSN context requests received from a partner (new) SGSN for a subscriber de-registering from this SGSN

- This measurement provides the number of SGSN context requests received from a partner (new) SGSN for a subscriber de-registering from this SGSN.
The three measurement types defined in e) are subject to the "2 out of 3 approach".

- CC.

- Receipt of the "Context Request" message from the new SGSN (TS 29.060 [6]).

- A single integer value per measurement type defined in e).

- SEC.AttContextRequestFromPsgsn:

-	SEC.AttContextRequestFromPsgsn	Combined (don't care);

-	SEC.AttContextRequestFromPsgsn.G	GSM;

-	SEC.AttContextRequestFromPsgsn.U	UMTS.

- SgsnFunction.

- Valid for packet switching.

- GSM/UMTS.

#### 4.4.9.2	Successfully replied SGSN context requests received from a partner (new) SGSN

- This measurement provides the number of successfully replied SGSN context requests received from a partner (new) SGSN.
The three measurement types defined in e) are subject to the "2 out of 3 approach".

- CC.

- Transmission of the "Context Response" message to the new SGSN (TS 29.060 [6]).

- A single integer value per measurement type defined in e).

- SEC.SuccContextRequestFromPsgsn:

-	SEC.SuccContextRequestFromPsgsn	Combined (don't care);

-	SEC.SuccContextRequestFromPsgsn.G	GSM;

-	SEC.SuccContextRequestFromPsgsn.U	UMTS.

- SgsnFunction.

- Valid for packet switching.

- GSM/UMTS.

### 4.4.10	Number of P-TMSI - IMSI correlation failures (User Identity Confidentiality (TS 23.060 [2]))

- This measurement provides the number of P-TMSI - IMSI correlation failures (User Identity Confidentiality (TS 23.060 [2])).

- CC.

- This counter is triggered before the handling of the "Security Functions" (TS 23.060 [2]), in case of "Attach Request", "Routing Area Update Request", or "Service Request": if the correlation between the received P-TMSI and the stored IMSI is not valid then this counter is incremented.

- A single integer value.

- SEC.NbrPTMSICorrFailRnc.

- SgsnFunction.

- Valid for packet switching.

- UMTS.

### 4.4.11	Security mode control procedures started by the SGSN

#### 4.4.11.1	Attempted security mode control procedures started by the SGSN

- This measurement provides the number of security mode control procedures started by the SGSN.

- CC.

- Transmission of a "SECURITY MODE COMMAND" message to the MS (TS 25.413 [5]).

- A single integer value.

- SEC.AttSecMode.

- SgsnFunction.

- Valid for packet switching.

- UMTS.

#### 4.4.11.2	Successful security mode procedures

- This measurement provides the number of successful security mode procedures. The Security mode command response from MS starts the uplink integrity protection (and possible ciphering), i.e. also all following messages sent from the MS are integrity protected (and possibly ciphered).

- CC.

- Receipt of a "SECURITY MODE COMPLETE" message from the MS (TS 25.413 [5]).

- A single integer value.

- SEC.SuccSecMode.

- SgsnFunction.

- Valid for packet switching.

- UMTS.

### 4.4.12	Ciphering procedures started by the SGSN

#### 4.4.12.1	Attempted GSM ciphering procedures started by the SGSN

- This measurement provides the number of GSM ciphering procedures started by the SGSN.

- CC.

- Transmission of an "AUTHENTICATION AND CIPHERING REQUEST" message with ciphering activated ("Ciphering algorithm" is not "ciphering not used (0)") to the MS (TS 24.008 [15]).

- A single integer value.

- SEC.AttGsmCiphering.

- SgsnFunction.

- Valid for packet switching.

- GSM.

#### 4.4.12.2	Successful GSM ciphering procedures started by the SGSN

- This measurement provides the number of successful GSM ciphering procedures started by the SGSN.

- CC.

- Receipt of an "AUTHENTICATION AND CIPHERING RESPONSE" message with ciphering activated from the MS (TS 24.008 [15]).

- A single integer value.

- SEC.SuccGsmCiphering.

- SgsnFunction.

- Valid for packet switching.

- GSM.

#### 4.4.12.3	Attempted UMTS ciphering procedures started by the SGSN

- This measurement provides the number of UMTS ciphering procedures started by the SGSN.

- CC.

- Transmission of a "SECURITY MODE COMMAND" message with ciphering activated ("Encryption Algorithm" is not "no encryption (0)") to the MS (TS 25.413 [5]).

- A single integer value.

- SEC.AttUmtsCiphering.

- SgsnFunction.

- Valid for packet switching.

- UMTS.

#### 4.4.12.4	Successful UMTS ciphering procedures started by the SGSN

- This measurement provides the number of successful UMTS ciphering procedures started by the SGSN.

- CC.

- Receipt of a "SECURITY MODE COMPLETE" message with ciphering activated from the MS (TS 25.413 [5]).

- A single integer value.

- SEC.SuccUmtsCiphering.

- SgsnFunction.

- Valid for packet switching.

- UMTS.

### 4.4.13	MAP V1 requests for authentication sets

#### 4.4.13.1	Attempted MAP V1 requests for authentication sets, sent to the HLR by SGSN

- This measurement provides the number of attempted MAP V1 requests for authentication sets, sent to the HLR by SGSN.

- CC.

- Transmission of a "MAP V1 SEND_AUTHENTICATION_INFO" service request, requesting authentication sets present (TS 29.002 [4]).

- A single integer value.

- SEC.AttReqAuthSetsHlrV1.

- SgsnFunction.

- Valid for packet switching.

- Combined.

#### 4.4.13.2	Successful MAP V1 requests for authentication sets that were sent to the HLR

- This measurement provides the number of successful MAP V1 requests for authentication sets that were sent to the HLR.

- CC.

- Receipt of a "MAP V1 SEND_AUTHENTICATION_INFO" service confirmation, containing requested authentication sets (parameter "AuthenticationSetList" present TS 29.002 [4]).

- A single integer value.

- SEC.SuccReqAuthSetsHlrV1.

- SgsnFunction.

- Valid for packet switching.

- Combined.

#### 4.4.13.3	Number of empty responses to the MAP V1 request for authentication sets that were sent to the HLR

- This measurement provides the number of empty responses to the MAP V1 request for authentication sets that were sent to the HLR.

- CC.

- Receipt of a MAP V1 "SEND_AUTHENTICATION_INFO_ACK" service confirmation, no Authentication sets present (TS 29.002 [4]).

- A single integer value.

- SEC.NbrEmptyRespAuthSetsHlrV1.

- SgsnFunction.

- Valid for packet switching.

- Combined.

### 4.4.14	MAP V3 requests for Authentication sets

#### 4.4.14.1	Attempted MAP V3 requests for Authentication sets sent to the HLR by SGSN

- This measurement provides the number of attempted MAP V3 requests for Authentication sets sent to the HLR by SGSN.

- CC.

- Transmission of a MAP V3 "SEND_AUTHENTICATION_INFO" service request, requesting authentication sets present (TS 29.002 [4]).

- A single integer value.

- SEC.AttReqAuthSetsHlrV3.

- SgsnFunction.

- Valid for packet switching.

- Combined.

#### 4.4.14.2	Successful MAP V3 requests for authentication sets that were sent to the HLR

- This measurement provides the number of successful MAP V3 requests for authentication sets that were sent to the HLR.

- CC.

- Receipt of a MAP V3 "SEND_AUTHENTICATION_INFO" service confirmation, containing requested authentication sets (parameter "AuthenticationSetList" present TS 29.002 [4]).

- A single integer value.

- SEC.SuccReqAuthSetsHlrV3.

- SgsnFunction.

- Valid for packet switching.

- Combined.

#### 4.4.14.3	Number of empty responses to the MAP V3 request for authentication sets that were sent to the HLR

- This measurement provides the number of empty responses to the MAP V3 request for authentication sets that were sent to the HLR.

- CC.

- Receipt of a MAP V3 "SEND_AUTHENTICATION_INFO_ACK" service confirmation, no Authentication sets present (TS 29.002 [4]).

- A single integer value.

- SEC.NbrEmptyRespAuthSetsHlrV3.

- SgsnFunction.

- Valid for packet switching.

- Combined.

## 4.5	Void

## 4.6	Session Management

### 4.6.1	PDP context activation procedures initiated by MS

#### 4.6.1.1	Attempted PDP context activation procedures initiated by MS

- This measurement provides the number of attempted PDP context activation procedures. These include the static as well as the dynamic PDP addresses.
The three measurement types defined in e) are subject to the "2 out of 3 approach".

- CC.

- Receipt of a "Activate PDP Context Request" message from the MS (TS 24.008 [15]).

- A single integer value.

- SM.AttActPdpContext:

-	SM.AttActPdpContext	Combined (don't care);

-	SM.AttActPdpContext.G	GSM;

-	SM.AttActPdpContext.U	UMTS.

- SgsnFunction.

- Valid for packet switching.

- GSM/UMTS.

#### 4.6.1.2	Successful PDP context activation procedures initiated by MS

- This measurement provides the number of successfully completed PDP context activations. For these context activations, the GGSN is updated successfully.
The three measurement types defined in e) are subject to the "2 out of 3 approach".

- CC.

- Transmission of a "Activate PDP Context Accept" message to the MS (TS 24.008 [15]).

- A single integer value per measurement type defined in e).

- SM.SuccActPdpContext:

-	SM.SuccActPdpContext	Combined (don't care);

-	SM.SuccActPdpContext.G	GSM;

-	SM.SuccActPdpContext.U	UMTS.

- SgsnFunction.

- Valid for packet switching.

- GSM/UMTS.

#### 4.6.1.3	Failed PDP context activation procedures initiated by MS

- This measurement provides the number of Failed PDP context activation procedures. These include the static as well as the dynamic PDP addresses. This measurement is split into subcounters per failure cause. The three measurement types defined in e) are subject to the "2 out of 3 approach".

- CC.

- Transmission by the SGSN of an ACTIVATE PDP CONTEXT REJECT message indicating a PDP context activation failure, the relevant measurement is incremented according to the failure cause. Possible causes are included in TS 24.008 [15] Annex G. The sum of all supported per cause measurements should equal the total number of PDP context activation failures. In case only a subset of per cause measurements is supported, a sum subcounter will be provided first.

- A single integer value per measurement type as defined in e). The number of measurements is equal to the number of implemented per cause measurements plus a possible sum value identified by the .sum suffix.

- The measurement name has the form SM.FailActPdpCtxtMs.Cause where Cause identifies the failure cause.

-	SM. FailActPdpCtxtMs.Cause	Combined (don't care);

-	SM. FailActPdpCtxtMs.Cause.G	GSM;

-	SM. FailActPdpCtxtMs.Cause.U	UMTS;

where Cause identifies the failure cause.

- SgsnFunction.

- Valid for packet switching.

- GSM/UMTS.

### 4.6.2	dynamic PDP context activation procedures initiated by MS

#### 4.6.2.1	Attempted dynamic PDP context activation procedures initiated by MS

- This measurement provides the number of attempted PDP context activation requests where a dynamic PDP address is required to be used.
The three measurement types defined in e) are subject to the "2 out of 3 approach".

- CC.

- Receipt of a "Activate PDP Context Request" message from the MS with an empty PDP address (TS 24.008 [15]).

- A single integer value per measurement type defined in e).

- SM.AttActPdpContextDyn:

-	SM.AttActPdpContextDyn	Combined (don't care);

-	SM.AttActPdpContextDyn.G	GSM;

-	SM.AttActPdpContextDyn.U	UMTS.

- SgsnFunction.

- Valid for packet switching.

- GSM/UMTS.

#### 4.6.2.2	Successful dynamic PDP context activation procedures initiated by MS

- This measurement provides the number of successfully completed PDP context activations where a dynamic PDP address is used.
The three measurement types defined in e) are subject to the "2 out of 3 approach".

- CC.

- Transmission of a "Activate PDP Context Accept" message to the MS (TS 24.008 [15]), the PDP address has been dynamically assigned.

- A single integer value per measurement type defined in e).

- SM.SuccActPdpContextDyn:

-	SM.SuccActPdpContextDyn	Combined (don't care);

-	SM.SuccActPdpContextDyn.G	GSM;

-	SM.SuccActPdpContextDyn.U	UMTS.

- SgsnFunction.

- Valid for packet switching.

- GSM/UMTS.

### 4.6.3	Mean number of activated PDP contexts

- Mean number of activated PDP contexts.
The three measurement types defined in e) are subject to the "2 out of 3 approach".

- SI.

- This measurement is obtained by sampling at a pre-defined interval, the number activated PDP contexts, and then taking the arithmetic mean.

- A single integer value per measurement type defined in e).

- SM.MeanActPDPContext:

-	SM.MeanActPDPContext	Combined (don't care);

-	SM.MeanActPDPContext.G	GSM;

-	SM.MeanActPDPContext.U	UMTS.

- SgsnFunction.

- Valid for packet switching.

- GSM/UMTS.

### 4.6.4	PDP context deactivation procedures initiated by the MS

#### 4.6.4.1	Attempted PDP context deactivation procedures initiated by the MS

- This measurement provides the number of PDP context deactivation procedures initiated by the MS.
The three measurement types defined in e) are subject to the "2 out of 3 approach".

- CC.

- Receipt of a "Deactivate PDP Context Request" message from the MS (TS 24.008 [15]).

- A single integer value per measurement type defined in e).

- SM.AttDeactPdpContextMs:

-	SM.AttDeactPdpContextMs	Combined (don't care);

-	SM.AttDeactPdpContextMs.G	GSM;

-	SM.AttDeactPdpContextMs.U	UMTS.

- SgsnFunction.

- Valid for packet switching.

- GSM/UMTS.

#### 4.6.4.2	Successful PDP context deactivation procedures initiated by the MS

- This measurement provides the number of successfully completed PDP context deactivations. For these context deactivations, the GGSN is updated successfully (i.e. deletion of the PDP context).
The three measurement types defined in e) are subject to the "2 out of 3 approach".

- CC.

- Transmission of a "Deactivate PDP Context Accept" message to the MS (TS 24.008 [15]).

- A single integer value per measurement type defined in e).

- SM.SuccDeactPdpContextMs:

-	SM.SuccDeactPdpContextMs	Combined (don't care);

-	SM.SuccDeactPdpContextMs.G	GSM;

-	SM.SuccDeactPdpContextMs.U	UMTS.

- SgsnFunction.

- Valid for packet switching.

- GSM/UMTS.

### 4.6.5	Number of active PDP context

- This measurement provides the number of active PDP context. 
The three measurement types defined in e) are subject to the "2 out of 3 approach".

- GAUGE.

- The gauge will be incremented when a PDP context is created and will be decremented when a PDP context is deleted.

- A single integer value per measurement type defined in e).

- SM.NbrActPdpContext:

-	SM.NbrActPdpContext	Combined (don't care);

-	SM.NbrActPdpContext.G	GSM;

-	SM.NbrActPdpContext.U	UMTS.

- SgsnFunction.

- Valid for packet switching.

- GSM/UMTS.

### 4.6.6	Subscribers with activated PDP context

#### 4.6.6.1	Number of subscribers with activated PDP context

- This measurement provides the number of subscribers having an activated PDP context (i.e. subscribers that can send/receive GPRS packet data).
The three measurement types defined in e) are subject to the "2 out of 3 approach".

- GAUGE.

- Addition of first PDP context or removal of last PDP context in SGSN location register for a particular subscriber.

- A single integer value per measurement type defined in e).

- SM.NbrActivePdpPerSgsn:

-	SM.NbrActivePdpPerSgsn	Combined (don't care);

-	SM.NbrActivePdpPerSgsn.G	GSM;

-	SM.NbrActivePdpPerSgsn.U	UMTS.

- SgsnFunction.

- Valid for packet switching.

- GSM/UMTS.

#### 4.6.6.2	Mean number of subscribers with activated PDP context

- This measurement provides the mean number of subscribers having an activated PDP context (i.e. subscribers that can send/receive GPRS packet data).
The three measurement types defined in e) are subject to the "2 out of 3 approach".

- SI.

- This measurement is obtained by sampling at a pre-defined interval the number of subscribers having an activated PDP context in SGSN and then taking the arithmetic mean.

- A single integer value per measurement type defined in e).

- SM.MeanActivePdpPerSgsn:

-	SM.MeanActivePdpPerSgsn	Combined (don't care);

-	SM.MeanActivePdpPerSgsn.G	GSM;

-	SM.MeanActivePdpPerSgsn.U	UMTS.

- SgsnFunction.

- Valid for packet switching.

- GSM/UMTS.

#### 4.6.6.3	Max number of subscribers with activated PDP context

- This measurement provides the maximum number of subscribers having an activated PDP context (i.e. subscribers that can send/receive GPRS packet data). 
The three measurement types defined in e) are subject to the "2 out of 3 approach".

- SI.

- This measurement is obtained by sampling at a pre-defined interval the number of subscribers having an activated PDP context in SGSN and then taking the maximum.

- A single integer value per measurement type defined in e).

- SM.MaxActivePdpPerSgsn:

-	SM.MaxActivePdpPerSgsn	Combined (don't care);

-	SM.MaxActivePdpPerSgsn.G	GSM;

-	SM.MaxActivePdpPerSgsn.U	UMTS.

- SgsnFunction.

- Valid for packet switching.

- GSM/UMTS.

### 4.6.7	Void

### 4.6.8	PDP context deactivation procedures initiated by the GGSN

#### 4.6.8.1	Attempted PDP context deactivation procedures initiated by the GGSN

- This measurement provides the number of PDP context deactivation procedures initiated by the GGSN.
The three measurement types defined in e) are subject to the "2 out of 3 approach".

- CC.

- Receipt of a "Delete PDP Context Request" message from the GGSN (TS 29.060 [6]).

- A single integer value per measurement type defined in e).

- SM.AttDeactPdpContextGgsn:

-	SM.AttDeactPdpContextGgsn	Combined (don't care);

-	SM.AttDeactPdpContextGgsn.G	GSM;

-	SM.AttDeactPdpContextGgsn.U	UMTS.

- SgsnFunction.

- Valid for packet switching.

- GSM/UMTS.

#### 4.6.8.2	Successful PDP context deactivation procedures initiated by the GGSN

- This measurement provides the number of successfully handled PDP context deactivations initiated by the GGSN. For these context deactivations, the MS has accepted the PDP context deactivation.
The three measurement types defined in e) are subject to the "2 out of 3 approach".

- CC.

- Transmission of a "Delete PDP Context Response" message to the GGSN (TS 29.060 [6]).

- A single integer value per measurement type defined in e).

- SM.SuccDeactPdpContextGgsn:

-	SM.SsuccDeactPdpContextGgsn	Combined (don't care);

-	SM.SsuccDeactPdpContextGgsn.G	GSM;

-	SM.SsuccDeactPdpContextGgsn.U	UMTS.

- SgsnFunction.

- Valid for packet switching.

- GSM/UMTS.

### 4.6.9	PDP context deactivation procedures initiated by the SGSN

#### 4.6.9.1	Attempted PDP context deactivation procedures initiated by the SGSN

- This measurement provides the number of PDP context deactivation procedures initiated by the SGSN.
The three measurement types defined in e) are subject to the "2 out of 3 approach".

- CC.

- Transmission of a "Delete PDP Context Request" message to the GGSN (TS 29.060 [6]).

- A single integer value per measurement type defined in e).

- SM.AttDeactPdpContextSgsn:

-	SM.AttDeactPdpContextSgsn	Combined (don't care);

-	SM.AttDeactPdpContextSgsn.G	GSM;

-	SM.AttDeactPdpContextSgsn.U	UMTS.

- SgsnFunction.

- Valid for packet switching.

- GSM/UMTS.

#### 4.6.9.2	Successful PDP context deactivations initiated by the SGSN

- This measurement provides the number of successfully handled PDP context deactivations initiated by the SGSN.
The three measurement types defined in e) are subject to the "2 out of 3 approach".

- CC.

- Receipt of a "deactivate PDP Context Accept" message from the MS (TS 24.008 [15]).

- A single integer value per measurement type defined in e).

- SM.SuccDeactPdpContextSgsn:

-	SM.SuccDeactPdpContextSgsn	Combined (don't care);

-	SM.SuccDeactPdpContextSgsn.G	GSM;

-	SM.SuccDeactPdpContextSgsn.U	UMTS.

- SgsnFunction.

- Valid for packet switching.

- GSM/UMTS.

#### 4.6.9.3	Abnormal PDP context Deactivation procedures

- This measurement provides the number of PDP context deactivation procedures initiated by the SGSN. This measurement is split into subcounters per cause.

- CC.

- Transmission of a "Delete PDP Context Request" message to the GGSN (TS 29.060 [6]). the measurement is incremented according to the deletion cause. Possible causes are included in TS 24.008 [15]. The sum of all supported per cause measurements should equal the total number of PDP context activation failures.

- A single integer value.

- SM.AttDeactPdpContextSgsn.cause

- SgsnFunction.

- Valid for packet switching.

- GSM/UMTS.

### 4.6.10	SGSN-Initiated PDP context update procedures

#### 4.6.10.1	Attempted SGSN-Initiated PDP context update procedures

- This measurement provides the number of attempted SGSN-Initiated PDP context update procedures. An Update PDP Context Request message shall be sent from a SGSN to a GGSN as part of the GPRS Inter SGSN Routeing Update procedure or the PDP Context Modification procedure or to redistribute contexts due to load sharing. It shall be used to change the QoS and the path. The message shall be sent by the new SGSN at the Inter SGSN Routeing Update procedure.
The three measurement types defined in e) are subject to the "2 out of 3 approach".

- CC.

- Transmission of an "Update PDP Context Request" message to the GGSN (TS 29.060 [6]).

- A single integer value per measurement type defined in e).

- SM.AttUpdPdpContextSgsn:

-	SM.AttUpdPdpContextSgsn	Combined (don't care);

-	SM.AttUpdPdpContextSgsn.G	GSM;

-	SM.AttUpdPdpContextSgsn.U	UMTS.

- SgsnFunction.

- Valid for packet switching.

- GSM/UMTS.

#### 4.6.10.2	Successful SGSN-Initiated PDP context update procedures

- This measurement provides the number of successfully handled SGSN-Initiated PDP context update procedures. These updates are performed successfully when a positive update PDP context response is received from the GGSN.
The three measurement types defined in e) are subject to the "2 out of 3 approach".

- CC.

- Receipt of an "Update PDP Context Response" message from the GGSN (TS 29.060 [6]).

- A single integer value per measurement type defined in e).

- SM.SsuccUpdPdpContextSgsn:

-	SM.SsuccUpdPdpContextSgsn	Combined (don't care);

-	SM.SsuccUpdPdpContextSgsn.G	GSM;

-	SM.SsuccUpdPdpContextSgsn.U	UMTS.

- SgsnFunction.

- Valid for packet switching.

- GSM/UMTS.

### 4.6.11	GGSN-Initiated PDP context update procedures

#### 4.6.11.1	Attempted GGSN-Initiated PDP context update procedures

- This measurement provides the number of attempted GGSN-Initiated PDP context update procedures. An Update PDP Context Request may also be sent from a GGSN to a SGSN to re-negotiate the QoS of a PDP context. This GGSN-initiated Update PDP Context Request can also be used to provide a PDP address to the SGSN (and MS). The latter shall be used by GGSN when it acts as a DHCP Relay Agent or Mobil IP Foreign Agent.
The three measurement types defined in e) are subject to the "2 out of 3 approach".

- CC.

- Receipt of an "Update PDP Context Request" message from the GGSN (TS 29.060 [6]).

- A single integer value per measurement type defined in e).

- SM.AttUpdPdpContextGgsn:

-	SM.AttUpdPdpContextGgsn	Combined (don't care);

-	SM.AttUpdPdpContextGgsn.G	GSM;

-	SM.AttUpdPdpContextGgsn.U	UMTS.

- SgsnFunction.

- Valid for packet switching.

- GSM/UMTS.

#### 4.6.11.2	Successful GGSN-Initiated PDP context update procedures

- This measurement provides the number of successfully handled GGSN-Initiated PDP context update procedures. These updates are performed successfully when a positive update PDP context response is received from the SGSN.
The three measurement types defined in e) are subject to the "2 out of 3 approach".

- CC.

- Transmission of an "Update PDP Context Response" message to the GGSN (TS 29.060 [6]).

- A single integer value per measurement type defined in e).

- SM.SuccUpdPdpContextGgsn:

-	SM.SuccUpdPdpContextGgsn	Combined (don't care);

-	SM.SuccUpdPdpContextGgsn.G	GSM;

-	SM.SuccUpdPdpContextGgsn.U	UMTS.

- SgsnFunction.

- Valid for packet switching.

- GSM/UMTS.

### 4.6.12	SGSN-Initiated PDP context modifications procedures

#### 4.6.12.1	Attempted SGSN-Initiated PDP context modifications procedures

- This measurement provides the number of attempted SGSN-Initiated PDP context modifications procedures.
The three measurement types defined in e) are subject to the "2 out of 3 approach".

- CC.

- Transmission of an "Modify PDP Context Request" message to the MS (TS 24.008 [15]).

- A single integer value per measurement type defined in e).

- SM.AttModPdpContextSgsn:

-	SM.AttModPdpContextSgsn	Combined (don't care);

-	SM.AttModPdpContextSgsn.G	GSM;

-	SM.AttModPdpContextSgsn.U	UMTS.

- SgsnFunction.

- Valid for packet switching.

- GSM/UMTS.

#### 4.6.12.2	Successfully SGSN-Initiated PDP context modifications procedures

- This measurement provides the number of successfully handled SGSN-Initiated PDP context modifications procedures. These modifications are performed successfully when a positive Modify PDP Context Accept is received from the MS.
The three measurement types defined in e) are subject to the "2 out of 3 approach".

- CC.

- Receipt of an "Modify PDP Context Accept" message from the MS (TS 24.008 [15]).

- A single integer value per measurement type defined in e).

- SM.SuccModPdpContextSgsn:

-	SM.SuccModPdpContextSgsn	Combined (don't care);

-	SM.SuccModPdpContextSgsn.G	GSM;

-	SM.SuccModPdpContextSgsn.U	UMTS.

- SgsnFunction.

- Valid for packet switching.

- GSM/UMTS.

### 4.6.13	MS-Initiated PDP context modifications procedures

#### 4.6.13.1	Attempted MS-Initiated PDP context modifications procedures

- This measurement provides the number of attempted MS-Initiated PDP context modifications procedures.
The three measurement types defined in e) are subject to the "2 out of 3 approach".

- CC.

- Receipt of an "Modify PDP Context Request" message from the MS (TS 24.008 [15]).

- A single integer value per measurement type defined in e).

- SM.AttModPdpContextMs:

-	SM.AttModPdpContextMs	Combined (don't care);

-	SM.AttModPdpContextMs.G	GSM;

-	SM.AttModPdpContextMs.U	UMTS.

- SgsnFunction.

- Valid for packet switching.

- GSM/UMTS.

#### 4.6.13.2	Successfully MS-Initiated PDP context modifications procedures

- This measurement provides the number of successfully handled MS-Initiated PDP context modifications procedures. These modifications are performed successfully when a positive Modify PDP Context Accept is received from the MS.
The three measurement types defined in e) are subject to the "2 out of 3 approach".

- CC.

- Transmission of an "Modify PDP Context Accept" message to the MS (TS 24.008 [15]).

- A single integer value per measurement type defined in e).

- SM.SsuccModPdpContextMs:

-	SM.SsuccModPdpContextMs	Combined (don't care);

-	SM.SsuccModPdpContextMs.G	GSM;

-	SM.SsuccModPdpContextMs.U	UMTS.

- SgsnFunction.

- Valid for packet switching.

- GSM/UMTS.

### 4.6.14	Secondary PDP context activation procedures

#### 4.6.14.1	Attempted Secondary PDP context activation procedures

- This measurement provides the number of attempted Secondary PDP context activation procedures.
The three measurement types defined in e) are subject to the "2 out of 3 approach".

- CC.

- Receipt of a "Activate Secondary PDP Context Request" message from the MS (TS 24.008 [15]).

- A single integer value per measurement type defined in e).

- SM.AttActSecondPdpContext:

-	SM.AttActSecondPdpContext	Combined (don't care);

-	SM.AttActSecondPdpContext.G	GSM;

-	SM.AttActSecondPdpContext.U	UMTS.

- SgsnFunction.

- Valid for packet switching.

- GSM/UMTS.

#### 4.6.14.2	Successful Secondary PDP context activations

- This measurement provides the number of successfully completed Secondary PDP context activations.
The three measurement types defined in e) are subject to the "2 out of 3 approach".

- CC.

- Transmission of a "Activate Secondary PDP Context Accept" message to the MS (TS 24.008 [15]).

- A single integer value per measurement type defined in e).

- SM.SuccActSecondPdpContext:

-	SM.SuccActSecondPdpContext	Combined (don't care);

-	SM.SuccActSecondPdpContext.G	GSM;

-	SM.SuccActSecondPdpContext.U	UMTS.

- SgsnFunction.

- Valid for packet switching.

- GSM/UMTS.

### 4.6.15	PDP context activation procedures initiated by Network

#### 4.6.15.1	Attempted PDP context activation procedures initiated by Network

- This measurement provides the number of attempted PDP context activation procedures initiated by Network. These include the static as well as the dynamic PDP addresses.
The three measurement types defined in e)  are subject to the "2 out of 3 approach".

- CC.

- Receipt of a "PDU Notification Request" message from the MS (TS29.060 [6]) message from GGSN.

- A single integer value.

- SM.AttActPdpCtxtNtwk:

-	SM.AttActPdpCtxtNtwk	Combined (don't care);

-	SM.AttActPdpCtxtNtwk.G	GSM;

-	SM.AttActPdpCtxtNtwk.U	UMTS.

- SgsnFunction.

- Valid for packet switching.

- GSM/UMTS.

#### 4.6.15.2	Successful PDP context activation procedures initiated by Network

- This measurement provides the number of successful PDP context activation procedures. These include the static as well as the dynamic PDP addresses.
The three measurement types defined in e)  are subject to the "2 out of 3 approach".

- CC.

- Transmission by the SGSN of a "Activate PDP Context Accept" message (TS 24.008 [15]) to MS for the PDP context activation initiated by Network.

- A single integer value.

- SM.SuccActPdpCtxtNtwk:

-	SM.SuccActPdpCtxtNtwk	Combined (don't care);

-	SM.SuccActPdpCtxtNtwk.G	GSM;

-	SM.SuccActPdpCtxtNtwk.U	UMTS.

- SgsnFunction.

- Valid for packet switching.

- GSM/UMTS.

#### 4.6.15.3	Failed PDP context activation procedures initiated by Network

- This measurement provides the number of Failed PDP context activation procedures. These include the static as well as the dynamic PDP addresses. This measurement is split into subcounters per failure cause.

- CC.

- Receipt of a " REQUEST PDP CONTEXT ACTIVATION REJECT " message from the MS (TS 24.008 [15]) message indicating a PDP context activation failure, the measurement is incremented according to the failure cause. Possible causes are included in TS 24.008 [15]. The sum of all supported per cause measurements should equal the total number of PDP context activation failures.

- A single integer value.

- The measurement name has the form SM.FailActPdpCtxtNtwk.Cause 
where Cause identifies the failure cause.

- SgsnFunction.

- Valid for packet switching.

- GSM/UMTS.

### 4.6.16	PDP Context set-up time, initiated by MS (Mean)

- This measurement provides the mean time it takes for the SGSN to establish a PDP context during each collection interval. The measurement is split into subcounters per traffic class per APN (see TS 23.003 [9] for APN definition), these measurements will only be provided for a subset of all APNs.  The way the list of monitored APNs is configured is outside the scope of this TS.

- DER (n=1).

- This measurement is obtained by accumulating the time intervals for each successful mobile originated PDP context activation between the receipt by the SGSN of an " ACTIVATE PDP CONTEXT REQUEST" from the MS and the corresponding transmission by the SGSN to the MS of an "ACTIVATE PDP CONTEXT" message over a granularity period using DER, see TS 29.060 [6], TS 24.008 [15] and TS 23.107 [8] for service class definitions. This end value of the time will then be divided by the number of successful mobile originated PDP context activations observed in the granularity period to give the arithmetic mean, the accumulator shall be reinitialised at the beginning of each granularity period.

- Each measurement is an integer value.(in milliseconds).

- SM. SuccActPdpContextAPNTimeMOMean.Conv
SM. SuccActPdpContextAPNTimeMOMean.Strm
SM. SuccActPdpContextAPNTimeMOMean.Intact
SM. SuccActPdpContextAPNTimeMOMean.Bgrd.

- SgsnFunction, per APN.

- Valid for packet switched traffic.

- GSM/UMTS.

### 4.6.17	PDP Context set-up time, initiated by MS (Max)

- This measurement provides the maximum time it takes for the SGSN to establish a PDP context during each collection interval. The measurement is split into subcounters per traffic class per APN (see TS 23.003 [9] for APN definition), these measurements will only be provided for a subset of all APNs.  The way the list of monitored APNs is configured is outside the scope of this TS.

- GAUGE.

- This measurement is obtained by monitoring the time intervals for each successful mobile originated PDP context activation between the receipt by the SGSN of an " ACTIVATE PDP CONTEXT REQUEST" from the MS and the corresponding transmission by the SGSN to the MS of an "ACTIVATE PDP CONTEXT" message over a granularity period using DER, see TS 29.060 [6], TS 24.008 [15] and TS 23.107 [8] for service class definitions. The high tide mark of this time will be stored in a gauge, the gauge shall be reinitialised at the beginning of each granularity period.

- Each measurement is an integer value.(in milliseconds).

- SM. SuccActPdpContextAPNTimeMOMax.Conv
SM. SuccActPdpContextAPNTimeMOMax.Strm
SM. SuccActPdpContextAPNTimeMOMax.Intact
SM. SuccActPdpContextAPNTimeMOMax.Bgrd

- SgsnFunction, per APN.

- Valid for packet switched traffic.

- GSM/UMTS.

### 4.6.18	PDP Context set-up time, initiated by Network (Mean)

- This measurement provides the mean time it takes for the SGSN to establish a PDP context initiated by the network during each collection interval. The measurement is split into subcounters per traffic class per APN (see TS 23.003 [9] for APN definition), these measurements will only be provided for a subset of all APNs. The way the list of monitored APNs is configured is outside the scope of this TS.

- DER (n=1).

- This measurement is obtained by accumulating the time intervals for each successful mobile terminated PDP context activation between the transmission by the SGSN of a "REQUEST PDP CONTEXT ACTIVATION" for the MS and the corresponding transmission by the SGSN to the MS of an "ACTIVATE PDP CONTEXT" message over a granularity period using DER, see TS 29.060 [6], TS 24.008 [15] and TS 23.107 [8] for service class definitions. This end value of the time will then be divided by the number of successful mobile originated PDP context activations observed in the granularity period to give the arithmetic mean, the accumulator shall be reinitialised at the beginning of each granularity period.

- Each measurement is an integer value.(in milliseconds).

- SM. SuccActPdpContextAPNTimeMTMean.Conv
SM. SuccActPdpContextAPNTimeMTMean.Strm
SM. SuccActPdpContextAPNTimeMTMean.Intact
SM. SuccActPdpContextAPNTimeMTMean.Bgrd.

- SgsnFunction, per APN.

- Valid for packet switched traffic.

- GSM/UMTS.

### 4.6.19	PDP Context set-up time, initiated by Network (Max)

- This measurement provides the maximum time it takes for the SGSN to establish a PDP context initiated by the network during each collection interval. The measurement is split into subcounters per traffic class per APN (see TS 23.003 [9] for APN definition), these measurements will only be provided for a subset of all APNs.  The way the list of monitored APNs is configured is outside the scope of this TS.

- GAUGE

- This measurement is obtained by monitoring the time intervals for each successful mobile terminated PDP context activation between the transmission by the SGSN of a "REQUEST PDP CONTEXT ACTIVATION" for the MS and the corresponding transmission by the SGSN to the MS of an "ACTIVATE PDP CONTEXT" message over a granularity period using DER, see TS 29.060 [6], TS 24.008 [15] and TS 23.107 [8] for service class definitions. The high tide mark of this time will be stored in a gauge, the gauge shall be reinitialised at the beginning of each granularity period.

- Each measurement is an integer value.(in milliseconds).

- SM. SuccActPdpContextAPNTimeMTMax.Conv
SM. SuccActPdpContextAPNTimeMTMax.Strm
SM. SuccActPdpContextAPNTimeMTMax.Intact
SM. SuccActPdpContextAPNTimeMTMax.Bgrd.

- SgsnFunction, per APN.

- Valid for packet switched traffic.

- GSM/UMTS.

## 4.7	CAMEL Measurements

### 4.7.1	CAMEL dialogues

#### 4.7.1.1	Attempted CAMEL dialogues

- Total number of CAMEL dialogue attempts.
The three measurement types defined in e) are subject to the "2 out of 3 approach".

- CC.

- Incremented when a TDP (Trigger Detection Point) is reached and CAP is informed.

- A single integer value per measurement type defined in e).

- CAM.AttCamelDialogues:

-	CAM.AttCamelDialogues	Combined (don't care);

-	CAM.AttCamelDialogues.G	GSM;

-	CAM.AttCamelDialogues.U	UMTS.

- SgsnFunction.

- Valid for packet switching.

- GSM/UMTS.

#### 4.7.1.2	Failed CAMEL dialogues, aborted locally by gprsSSF

- Number of failed CAMEL dialogues, aborted locally by gprsSSF.
The three measurement types defined in e) are subject to the "2 out of 3 approach".

- CC.

- Incremented when a CAMEL dialogue is aborted locally by SSF.

- A single integer value per measurement type defined in e).

- CAM.FailDialoguesSsf:

-	CAM.FailDialoguesSsf	Combined (don't care);

-	CAM.FailDialoguesSsf.G	GSM;

-	CAM.FailDialoguesSsf.U	UMTS.

- SgsnFunction.

- Valid for packet switching.

- GSM/UMTS.

#### 4.7.1.3	Failed CAMEL dialogues, error or reject from gsmSCF

- Number of failed CAMEL dialogues, error or reject from gsmSCF.
The three measurement types defined in e) are subject to the "2 out of 3 approach".

- CC.

- Incremented when a CAMEL dialogue is aborted by SCF.

- A single integer value per measurement type defined in e).

- CAM.FailDialoguesScf:

-	CAM.FailDialoguesScf	Combined (don't care);

-	CAM.FailDialoguesScf.G	GSM;

-	CAM.FailDialoguesScf.U	UMTS.

- SgsnFunction.

- Valid for packet switching.

- GSM/UMTS.

## 4.8	UMTS-GSM Intersystem Change

### 4.8.1	Intra SGSN inter system changes from UMTS to GSM

#### 4.8.1.1	Attempted intra SGSN inter system changes from UMTS to GSM

- Number of attempted intra SGSN inter system changes from UMTS to GSM.

- CC.

- Receipt of "Routing Area Update REQUEST" message from the MS, where the SGSN determines that it concerns a intra SGSN inter system changes from UMTS to GSM. (TS 24.008 [15]).

- A single integer value.

- ISYSC.AttIntraSgsnUmtsGsmRau.

- SgsnFunction.

- Valid for packet switching.

- Combined.

#### 4.8.1.2	Successful intra SGSN inter system changes from UMTS to GSM

- Successful intra SGSN inter system changes from UMTS to GSM.

- CC.

- Transmission of "ROUTING AREA UPDATE ACCEPT" message to the MS (TS 24.008 [15]). Only the cases where this message is sent for "UMTS to GSM Intra SGSN Change" are counted.

- A single integer value.

- ISYSC.SuccIntraSgsnUmtsGsmRau.

- SgsnFunction.

- Valid for packet switching.

- Combined.

#### 4.8.1.3	Failed intra SGSN inter system changes UMTS to GSM RAU, due to internal reasons

- Number of failed intra SGSN inter system UMTS to GSM RAU, due to internal reasons.

- CC.

- "UMTS to GSM Intra SGSN Change" fails due to reasons located inside this 2G+3G-SGSN:- internal resource problem- recovery- ...

- A single integer value.

- ISYSC.FailIntraSgsnUmtsGsmRauInt.

- SgsnFunction.

- Valid for packet switching.

- Combined.

#### 4.8.1.4	Failed intra SGSN inter system changes UMTS to GSM RAU, due to external reasons

- Number of failed intra SGSN inter system UMTS to GSM RAU, due to external reasons.

- CC.

- "UMTS to GSM Intra SGSN Change" fails due to reasons located in NE outside this 2G+3G-SGSN, such as abnormal (reject, failure,..)/missing responses from SRNS, MSC/VLR, HLR, ...

- A single integer value.

- ISYSC.FailIntraSgsnUmtsGsmRauExt.

- SgsnFunction.

- Valid for packet switching.

- Combined.

### 4.8.2	Intra SGSN inter system changes from GSM to UMTS

#### 4.8.2.1	Attempted intra SGSN inter system changes from GSM to UMTS

- Number of attempted intra SGSN inter system changes from GSM to UMTS.

- CC.

- Receipt of "Routing Area Update REQUEST" message from the MS, where the SGSN determines that it concerns a intra SGSN inter system changes from GSM to UMTS (TS 24.008 [15]).

- A single integer value.

- ISYSC.AttIntraSgsnGsmUmtsRau.

- SgsnFunction.

- Valid for packet switching.

- Combined.

#### 4.8.2.2	Successful intra SGSN inter system changes from GSM to UMTS

- Successful intra SGSN inter system changes from GSM to UMTS.

- CC.

- Transmission of "ROUTING AREA UPDATE ACCEPT" message to the MS (TS 24.008 [15]). Only the cases where this message is sent for "GSM to UMTS Intra SGSN Change" are counted.

- A single integer value.

- ISYSC.SuccIntraSgsnGsmUmtsRau.

- SgsnFunction.

- Valid for packet switching.

- Combined.

#### 4.8.2.3	Failed intra SGSN inter system changes GSM to UMTS RAU, due to internal reasons

- Number of failed intra SGSN inter system GSM to UMTS RAU, due to internal reasons.

- CC.

- "GSM to UMTS Intra SGSN Change" fails due to reasons located inside this 2G+3G-SGSN:- internal resource problem- recovery- ...

- A single integer value.

- ISYSC.FailIntraSgsnGsmUmtsRauInt.

- SgsnFunction.

- Valid for packet switching.

- Combined.

#### 4.8.2.4	Failed intra SGSN inter system changes GSM to UMTS RAU, due to external reasons

- Number of failed intra SGSN inter system GSM to UMTS RAU, due to external reasons.

- CC.

- "GSM to UMTS Intra SGSN Change" fails due to reasons located in NE outside this 2G+3G-SGSN, such as abnormal (reject, failure,..)/missing responses from SRNS, MSC/VLR, HLR, ...

- A single integer value.

- ISYSC.FailIntraSgsnGsmUmtsRauExt.

- SgsnFunction.

- Valid for packet switching.

- Combined.

## 4.9	UMTS GTP Measurements

### 4.9.1	GTP-U Iu

#### 4.9.1.1	Number of outgoing GTP data packets on the Iu interface

- This measurement provides the number of GTP data PDUs which have been generated by the GTP-U protocol entity on the Iu interface.

- CC.

- Transmission by the SGSN of a GTP data PDU on the Iu interface to the MS.

- A single integer value.

- GTP.GtpuOutDataPktIu.

- SgsnFunction.

- Valid for packet switching.

- UMTS.

#### 4.9.1.2	Number of incoming GTP data packets on the Iu interface

- This measurement provides the number of GTP data PDUs which have been accepted and processed by the GTP-U protocol entity on the Iu interface.

- CC.

- Reception by the SGSN of a GTP data PDU on the Iu interface from the MS.

- A single integer value.

- GTP.GtpuInDataPktIu.

- SgsnFunction.

- Valid for packet switching.

- UMTS.

#### 4.9.1.3	Number of octets of outgoing GTP data packets on the Iu interface

- This measurement provides the byte number of outgoing data packets on the Iu interface without the GTP-U header.

- CC.

- Transmission by the SGSN of an GTP-Data-PDU (T-PDU) on the Iu interface to the MS.

- A single integer value.

- GTP.GtpuOutDataOctIu.

- SgsnFunction.

- Valid for packet switching.

- UMTS.

#### 4.9.1.4	Number of octets of incoming GTP data packets on the Iu interface

- This measurement provides the byte number of incoming data packets on the Iu interface without the GTP-U header.

- CC.

- Reception by the SGSN of an GTP-Data-PDU (T-PDU) on the Iu interface from the MS.

- A single integer value.

- GTP.GtpuInDataOctIu.

- SgsnFunction.

- Valid for packet switching.

- UMTS.

### 4.9.2	GTP Gn

#### 4.9.2.1	Number of outgoing GTP data packets on the Gn interface, from SGSN to GGSN

- This measurement provides the number of GTP data PDUs which have been generated by the GTP protocol entity on the Gn interface.
The three measurement types defined in e) are subject to the "2 out of 3 approach".

- CC.

- Transmission by the SGSN of a GTP data PDU on the Gn interface to the GGSN.

- A single integer value per measurement type defined in e).

- GTP.OutDataPktGnGGSN:

-	GTP.OutDataPktGnGGSN	the total regardless of the GTP version used;

-	GTP.OutDataPktGnGGSN.v0	only the GTPv0 part;

-	GTP.OutDataPktGnGGSN.v1	only the GTPv1 part.

- SgsnFunction.

- Valid for packet switching.

- Combined.

#### 4.9.2.2	Number of incoming GTP data packets on the Gn interface, from GGSN to SGSN

- This measurement provides the number of GTP Data PDUs which have been accepted and processed by the GTP protocol entity on the Gn interface.
The three measurement types defined in e) are subject to the "2 out of 3 approach".

- CC.

- Reception by the SGSN of a GTP data PDU on the Gn interface from the GGSN.

- A single integer value per measurement type defined in e).

- GTP.InDataPktGnGGSN:

-	GTP.InDataPktGnGGSN	the total regardless of the GTP version used;

-	GTP.InDataPktGnGGSN.v0	only the GTPv0 part;

-	GTP.InDataPktGnGGSN.v1	only the GTPv1 part.

- SgsnFunction.

- Valid for packet switching.

- Combined.

#### 4.9.2.3	Number of octets of outgoing GTP data packets on the Gn interface, from SGSN to GGSN

- This measurement provides the number of octets of outgoing GTP data packets on the Gn interface.
The three measurement types defined in e) are subject to the "2 out of 3 approach".

- CC.

- Transmission by the SGSN of an GTP-Data-PDU (T-PDU) on the Gn interface to the GGSN.

- A single integer value per measurement type defined in e).

- GTP.OutDataOctGnGGSN:

-	GTP.OutDataOctGnGGSN	the total regardless of the GTP version used;

-	GTP.OutDataOctGnGGSN.v0	only the GTPv0 part;

-	GTP.OutDataOctGnGGSN.v1	only the GTPv1 part.

- SgsnFunction.

- Valid for packet switching.

- Combined.

#### 4.9.2.4	Number of octets of incoming GTP data packets on the Gn interface, from GGSN to SGSN

- This measurement provides the number of octets of incoming GTP data packets on the Gn interface.
The three measurement types defined in e) are subject to the "2 out of 3 approach".

- CC.

- Reception by the SGSN of an GTP-Data-PDU (T-PDU) on the Gn interface from the GGSN.

- A single integer value per measurement type defined in e).

- GTP.InDataOctGnGGSN:

-	GTP.InDataOctGnGGSN	the total regardless of the GTP version used;

-	GTP.InDataOctGnGGSN.v0	only the GTPv0 part;

-	GTP.InDataOctGnGGSN.v1	only the GTPv1 part.

- SgsnFunction.

- Valid for packet switching.

- Combined.

#### 4.9.2.5	Number of outgoing GTP signalling packets on the Gn interface, from SGSN to GGSN

- This measurement provides the number of GTP signalling PDUs which have been generated by the GTP protocol entity on the Gn interface.
The three measurement types defined in e) are subject to the "2 out of 3 approach".

- CC.

- Transmission by the SGSN of a GTP signalling PDU on the Gn interface to the GGSN.

- A single integer value per measurement type defined in e).

- GTP.OutSigPktGnGGSN:

-	GTP.OutSigPktGnGGSN	the total regardless of the GTP version used;

-	GTP.OutSigPktGnGGSN.v0	only the GTPv0 part;

-	GTP.OutSigPktGnGGSN.v1	only the GTPv1 part.

- SgsnFunction.

- Valid for packet switching.

- Combined.

#### 4.9.2.6	Number of incoming GTP signalling packets on the Gn interface, from GGSN to SGSN

- This measurement provides the number of GTP signalling PDUs which have been accepted and processed by the GTP protocol entity on the Gn interface.
The three measurement types defined in e) are subject to the "2 out of 3 approach".

- CC.

- Reception by the SGSN of a GTP signalling PDU on the Gn interface from the GGSN.

- A single integer value per measurement type defined in e).

- GTP.InSigPktGnGGSN:

-	GTP.InSigPktGnGGSN	the total regardless of the GTP version used;

-	GTP.InSigPktGnGGSN.v0	only the GTPv0 part;

-	GTP.InSigPktGnGGSN.v1	only the GTPv1 part.

- SgsnFunction.

- Valid for packet switching.

- Combined.

#### 4.9.2.7	Number of octets of outgoing GTP signalling packets on the Gn interface, from SGSN to GGSN

- This measurement provides the number of octets of outgoing GTP signalling packets on the Gn interface.
The three measurement types defined in e) are subject to the "2 out of 3 approach".

- CC.

- Transmission by the SGSN of an GTP-Signalling-PDU on the Gn interface to the GGSN.

- A single integer value per measurement type defined in e).

- GTP.OutSigOctGnGGSN:

-	GTP.OutSigOctGnGGSN	the total regardless of the GTP version used;

-	GTP.OutSigOctGnGGSN.v0	only the GTPv0 part;

-	GTP.OutSigOctGnGGSN.v1	only the GTPv1 part.

- SgsnFunction.

- Valid for packet switching.

- Combined.

#### 4.9.2.8	Number of octets of incoming GTP signalling packets on the Gn interface, from GGSN to SGSN

- This measurement provides the number of octets of incoming GTP signalling packets on the Gn interface.
The three measurement types defined in e) are subject to the "2 out of 3 approach".

- CC.

- Reception by the SGSN of an GTP-Signalling-PDU on the Gn interface from the GGSN.

- A single integer value per measurement type defined in e).

- GTP.InSigOctGnGGSN:

-	GTP.InSigOctGnGGSN	the total regardless of the GTP version used;

-	GTP.InSigOctGnGGSN.v0	only the GTPv0 part;

-	GTP.InSigOctGnGGSN.v1	only the GTPv1 part.

- SgsnFunction.

- Valid for packet switching.

- Combined.

#### 4.9.2.9	Number of outgoing GTP data packets on the Gn interface, from SGSN to SGSN

- This measurement provides the number of GTP data PDUs which have been generated by the GTP protocol entity on the Gn interface.
The three measurement types defined in e) are subject to the "2 out of 3 approach".

- CC.

- Transmission by the SGSN of a GTP data PDU on the Gn interface to the SGSN.

- A single integer value per measurement type defined in e).

- GTP.OutDataPktGnSGSN:

-	GTP.OutDataPktGnSGSN	the total regardless of the GTP version used;

-	GTP.OutDataPktGnSGSN.v0	only the GTPv0 part;

-	GTP.OutDataPktGnSGSN.v1	only the GTPv1 part.

- SgsnFunction.

- Valid for packet switching.

- Combined.

#### 4.9.2.10	Number of incoming GTP data packets on the Gn interface, from SGSN to SGSN

- This measurement provides the number of GTP Data PDUs which have been accepted and processed by the GTP protocol entity on the Gn interface.
The three measurement types defined in e) are subject to the "2 out of 3 approach".

- CC.

- Reception by the SGSN of a GTP data PDU on the Gn interface from the SGSN.

- A single integer value per measurement type defined in e).

- GTP.InDataPktGnSGSN:

-	GTP.InDataPktGnSGSN	the total regardless of the GTP version used;

-	GTP.InDataPktGnSGSN.v0	only the GTPv0 part;

-	GTP.InDataPktGnSGSN.v1	only the GTPv1 part.

- SgsnFunction.

- Valid for packet switching.

- Combined.

#### 4.9.2.11	Number of octets of outgoing GTP data packets on the Gn interface, from SGSN to SGSN

- This measurement provides the number of octets of outgoing GTP data packets on the Gn interface.
The three measurement types defined in e) are subject to the "2 out of 3 approach".

- CC.

- Transmission by the SGSN of an GTP-Data-PDU (T-PDU) on the Gn interface to the SGSN.

- A single integer value per measurement type defined in e).

- GTP.OutDataOctGnSGSN:

-	GTP.OutDataOctGnSGSN	the total regardless of the GTP version used;

-	GTP.OutDataOctGnSGSN.v0	only the GTPv0 part;

-	GTP.OutDataOctGnSGSN.v1	only the GTPv1 part.

- SgsnFunction.

- Valid for packet switching.

- Combined.

#### 4.9.2.12	Number of octets of incoming GTP data packets on the Gn interface, from SGSN to SGSN

- This measurement provides the number of octets of incoming GTP data packets on the Gn interface.
The three measurement types defined in e) are subject to the "2 out of 3 approach".

- CC.

- Reception by the SGSN of an GTP-Data-PDU (T-PDU) on the Gn interface from the SGSN.

- A single integer value per measurement type defined in e).

- GTP.InDataOctGnSGSN:

-	GTP.InDataOctGnSGSN	the total regardless of the GTP version used;

-	GTP.InDataOctGnSGSN.v0	only the GTPv0 part;

-	GTP.InDataOctGnSGSN.v1	only the GTPv1 part.

- SgsnFunction.

- Valid for packet switching.

- Combined.

#### 4.9.2.13	Number of outgoing GTP signalling packets on the Gn interface, from SGSN to SGSN

- This measurement provides the number of GTP signalling PDUs which have been generated by the GTP protocol entity on the Gn interface.
The three measurement types defined in e) are subject to the "2 out of 3 approach".

- CC.

- Transmission by the SGSN of a GTP signalling PDU on the Gn interface to the SGSN.

- A single integer value per measurement type defined in e).

- GTP.OutSigPktGnSGSN:

-	GTP.OutSigPktGnSGSN	the total regardless of the GTP version used;

-	GTP.OutSigPktGnSGSN.v0	only the GTPv0 part;

-	GTP.OutSigPktGnSGSN.v1	only the GTPv1 part.

- SgsnFunction.

- Valid for packet switching.

- Combined.

#### 4.9.2.14	Number of incoming GTP signalling packets on the Gn interface, from SGSN to SGSN

- This measurement provides the number of GTP signalling PDUs which have been accepted and processed by the GTP protocol entity on the Gn interface.
The three measurement types defined in e) are subject to the "2 out of 3 approach".

- CC.

- Reception by the SGSN of a GTP signalling PDU on the Gn interface from the SGSN.

- A single integer value per measurement type defined in e).

- GTP.InSigPktGnSGSN:

-	GTP.InSigPktGnSGSN	the total regardless of the GTP version used;

-	GTP.InSigPktGnSGSN.v0	only the GTPv0 part;

-	GTP.InSigPktGnSGSN.v1	only the GTPv1 part.

- SgsnFunction.

- Valid for packet switching.

- Combined.

#### 4.9.2.15	Number of octets of outgoing GTP signalling packets on the Gn interface, from SGSN to SGSN

- This measurement provides the number of octets of outgoing GTP signalling packets on the Gn interface.
The three measurement types defined in e) are subject to the "2 out of 3 approach".

- CC.

- Transmission by the SGSN of an GTP-Signalling-PDU on the Gn interface to the SGSN.

- A single integer value per measurement type defined in e).

- GTP.OutSigOctGnSGSN:

-	GTP.OutSigOctGnSGSN	the total regardless of the GTP version used;

-	GTP.OutSigOctGnSGSN.v0	only the GTPv0 part;

-	GTP.OutSigOctGnSGSN.v1	only the GTPv1 part.

- SgsnFunction.

- Valid for packet switching.

- Combined.

#### 4.9.2.16	Number of octets of incoming GTP signalling packets on the Gn interface, from SGSN to SGSN

- This measurement provides the number of octets of incoming GTP signalling packets on the Gn interface.
The three measurement types defined in e) are subject to the "2 out of 3 approach".

- CC.

- Reception by the SGSN of an GTP-Signalling-PDU on the Gn interface from the SGSN.

- A single integer value per measurement type defined in e).

- GTP.InSigOctGnSGSN:

-	GTP.InSigOctGnSGSN	the total regardless of the GTP version used;

-	GTP.InSigOctGnSGSN.v0	only the GTPv0 part;

-	GTP.InSigOctGnSGSN.v1	only the GTPv1 part.

- SgsnFunction.

- Valid for packet switching.

- Combined.

## 4.10	UMTS Bearer Service

### 4.10.1	UMTS Bearer Service CS time to register (Mean)

- This measurement provides the mean time it takes for the subscribers to register with the network for circuit switched (CS) services during each granularity period.

- DER (n=1)

- This measurement is obtained by accumulating the time intervals for each successful attach between the receipt by the VLR of an "ATTACH REQUEST" from the MS and the corresponding receipt by the VLR of an "ATTACH COMPLETE" message over a granularity period using DER, see TS 24.008 [15]. This end value of the time will then be divided by the number of successful attach requests for CS domain observed in the granularity period to give the arithmetic mean, the accumulator shall be reinitialised at the beginning of each granularity period.

- Each measurement is an integer value.(in milliseconds).

- UBS.TimeToRegisterCSMean

- VlrFunction

- Valid for circuit switched traffic.

- UMTS.

### 4.10.2	UMTS Bearer Service CS time to register (Max)

- This measurement provides the maximum time it takes for the subscribers to register with the network for circuit switched (CS) services during each granularity period.

- GAUGE.

- This measurement is obtained by monitoring the time intervals for each successful attach between the receipt by the VLR of an "ATTACH REQUEST" from the MS and the corresponding receipt by the VLR of an "ATTACH COMPLETE" message over a granularity period, see TS 24.008 [15]. The high tide mark of this time will be stored in a gauge, the gauge shall be reinitialised at the beginning of each granularity period.

- Each measurement is an integer value.(in milliseconds).

- UBS.TimeToRegisterCSMax.

- VlrFunction.

- Valid for circuit switched traffic.

- UMTS.

### 4.10.3	UMTS Bearer Service PS time to register (Mean)

- This measurement provides the mean time it takes for the subscribers to register with the network for packet switched (PS) services during each granularity period.

- DER (n=1).

- This measurement is obtained by accumulating the time intervals for each successful attach between the receipt by the SGSN of an "ATTACH REQUEST" from the MS and the corresponding receipt by the SGSN of an "ATTACH COMPLETE" message over a granularity period using DER, see TS 24.008 [15]. This end value of the time will then be divided by the number of successful attach requests for PS domain observed in the granularity period to give the arithmetic mean.

- Each measurement is an integer value.(in milliseconds).

- UBS.TimeToRegisterPSMean.

- SgsnFunction.

- Valid for packet switched traffic.

- UMTS.

### 4.10.4	UMTS Bearer Service PS time to register (Max)

- This measurement provides the maximum time it takes for the subscribers to register with the network for packet switched (PS) services during each granularity period.

- GAUGE.

- This measurement is obtained by monitoring the time intervals for each successful attach between the receipt by the SGSN of an "ATTACH REQUEST" from the MS and the corresponding receipt by the SGSN of an "ATTACH COMPLETE" message over a granularity period using DER, see TS 24.008 [15]. The high tide mark of this time will be stored in a gauge, the gauge shall be reinitialised at the beginning of each granularity period.

- Each measurement is an integer value.(in milliseconds).

- UBS.TimeToRegisterPSMax.

- SgsnFunction.

- Valid for packet switched traffic.

- UMTS.

### 4.10.5	UMTS Bearer Service time to establish Communications Management (CM) radio access connectivity (Mean)

- This measurement provides the mean time it takes for the radio access network to establish a service connection (for circuit switched connection establishment, supplementary services activation, short message transfer, location services) during each granularity period.

- DER (n=1).

- This measurement is obtained by accumulating the time intervals for each successful service request between the receipt by the MSC of a "CM SERVICE REQUEST" from the MS and the corresponding receipt by the MSC of an "CM SERVICE ACCEPT" message over a granularity period using DER, see TS 24.008 [15]. This end value of the time will then be divided by the number of successful service requests observed in the granularity period to give the arithmetic mean, the accumulator shall be reinitialised at the beginning of each granularity period.

- Each measurement is an integer value.(in milliseconds).

- UBS.TimeForCMConnectRANMean.

- MscFunction.

- Valid for circuit switched traffic.

- UMTS.

### 4.10.6	UMTS Bearer Service time to establish Communications Management (CM) radio access connectivity (Max)

- This measurement provides the maximum time it takes for the radio access network to establish a service connection (for circuit switched connection establishment, supplementary services activation, short message transfer, location services) during each granularity period.

- GAUGE.

- This measurement is obtained by monitoring the time intervals for each successful service request between the receipt by the MSC of a "CM SERVICE REQUEST" from the MS and the corresponding receipt by the MSC of an "CM SERVICE ACCEPT" message over a granularity period, see TS 24.008 [15]. The high tide mark of this time will be stored in a gauge, the gauge shall be reinitialised at the beginning of each granularity period.

- Each measurement is an integer value.(in milliseconds).

- UBS.TimeForCMConnectRANMax

- MscFunction.

- Valid for circuit switched traffic.

- UMTS.

## 4.11	LLC frames

### 4.11.1	Number of LLC frames sent

- This measurement provides the number of LLC frames sent by the SGSN.

- CC.

- Transmission of a LLC frame to a peer entity (TS 44.064 [11]).

- A single integer value

- LLC.NbrFramesSent.

- SgsnFunction.

- Valid for packet switching.

- GSM.

### 4.11.2	Number of LLC frames received

- This measurement provides the number of LLC frames received by the SGSN.

- CC.

- Receipt of a LLC frame from a peer entity before any error checking (TS 44.064 [11]).

- A single integer value

- LLC.NbrFramesReceived.

- SgsnFunction.

- Valid for packet switching.

- GSM.

### 4.11.3	Number of erroneous LLC frames received

- This measurement provides the number of erroneous LLC frames received by the SGSN.

- CC.

- Discard of an erroneous LLC frame received from a peer entity (TS 44.064 [11]).

- A single integer value

- LLC.NbrErroneousFrames.

- SgsnFunction.

- Valid for packet switching.

- GSM.

### 4.11.4	Number of LLC frames retransmitted

- This measurement provides the number of LLC frames retransmitted in LLC acknowledge mode by the SGSN.

- CC.

- Receipt of a NACK or SACK frame from a peer entity (TS 44.064 [11]).

- A single integer value

- LLC.NbrFramesRetransmitted.

- SgsnFunction.

- Valid for packet switching.

- GSM.

## 4.12	SNDCP N-PDUs

### 4.12.1	Number of SNDCP N-PDUs sent

- This measurement provides the number of SNDCP N-PDUs sent by the SGSN.

- CC.

- Transmission of a SN-DATA or SN-UNITDATA request (TS 44.065 [12]).

- A single integer value

- SNDCP.NbrNpdusSent.

- SgsnFunction.

- Valid for packet switching.

- GSM.

### 4.12.2	Number of SNDCP N-PDU octets sent

- This measurement provides the number of octets in SNDCP N-PDUs sent by the SGSN.

- CC.

- Transmission of a SN-DATA or SN-UNITDATA request (TS 44.065 [12]).

- A single integer value

- SNDCP.NbrNpduOctetsSent.

- SgsnFunction.

- Valid for packet switching.

- GSM.

### 4.12.3	Number of SNDCP N-PDUs received

- This measurement provides the number of SNDCP N-PDUs received by the SGSN.

- CC.

- Receipt of a SN-DATA or SN-UNITDATA indication (TS 44.065 [12]).

- A single integer value

- SNDCP.NbrNpdusReceived.

- SgsnFunction.

- Valid for packet switching.

- GSM.

### 4.12.4	Number of SNDCP N-PDU octets received

- This measurement provides the number of octets in SNDCP N-PDUs received by the SGSN.

- CC.

- Receipt of a SN-DATA or SN-UNITDATA indication (TS 44.065 [12]).

- A single integer value

- SNDCP.NbrNpduOctetsReceived.

- SgsnFunction.

- Valid for packet switching.

- GSM.

## 4.13	IMEI checking procedure

### 4.13.1	Number of check IMEI requests

- This measurement provides the number of check IMEI requests sent to the EIR.
The three measurement types defined in e) are subject to the "2 out of 3 approach".

- CC.

- Transmission of a "MAP_CHECK_IMEI" request to the EIR (TS 29.002 [4]).

- A single integer value.

- IMEI.CheckImeiReq.

- SgsnFunction.

- Valid for packet switching.

- Combined.

### 4.13.2	Number of check IMEI white list responses

- This measurement provides the number of check IMEI white list responses received from the EIR.
The three measurement types defined in e) are subject to the "2 out of 3 approach".

- CC.

- Receipt of a "MAP_CHECK_IMEI" response from the EIR with parameter "equipment status" indicating a white listed equipment (TS 29.002 [4]).

- A single integer value.

- IMEI.CheckImeiRspWhite.

- SgsnFunction.

- Valid for packet switching.

- Combined.

### 4.13.3	Number of check IMEI grey list responses

- This measurement provides the number of check IMEI grey list responses received from the EIR.
The three measurement types defined in e) are subject to the "2 out of 3 approach".

- CC.

- Receipt of a "MAP_CHECK_IMEI" response from the EIR with parameter "equipment status" indicating a grey listed equipment (TS 29.002 [4]).

- A single integer value.

- IMEI.CheckImeiRspGrey.

- SgsnFunction.

- Valid for packet switching.

- Combined.

### 4.13.4	Number of check IMEI black list responses

- This measurement provides the number of check IMEI black list responses received from the EIR.
The three measurement types defined in e) are subject to the "2 out of 3 approach".

- CC.

- Receipt of a "MAP_CHECK_IMEI" response from the EIR with parameter "equipment status" indicating a black listed equipment (TS 29.002 [4]).

- A single integer value.

- IMEI.CheckImeiRspBlack.

- SgsnFunction.

- Valid for packet switching.

- Combined.

### 4.13.5	Number of check IMEI unknown equipment responses

- This measurement provides the number of check IMEI unknown equipment responses received from the EIR.
The three measurement types defined in e) are subject to the "2 out of 3 approach".

- CC.

- Receipt of a "MAP_CHECK_IMEI" response from the EIR with parameter "user error" indicating an unknown equipment (TS 29.002 [4]).

- A single integer value.

- IMEI.CheckImeiRspUnknown.

- SgsnFunction.

- Valid for packet switching.

- Combined.

## 4.14	DNS

The performance counters presented in this subclause are mainly intended to:

- monitor the communication between the SGSN and the DNS server;

These counters are associated to the DNS protocol, which is playing the role to translate a domain address into an IP address, and is defined in RFC 1034 [13].

The figure below presents the message sequence during a DNS query procedure initiated by the SGSN.

- Figure: DNS Query procedure initiated by SGSN

### 4.14.1	Attempted DNS Query Procedures

- This measurement provides the number of attempted DNS query procedures SGSN-initiated within this SGSN area.
The three measurement types defined in e) are subject to the "2 out of 3 approach".

- CC.

- Transmission of a DNS query request from the SGSN to the DNS server (RFC 1035 [14]).

- A single integer value.

- DNS.AttDnsQuery:

- SgsnFunction.

- Valid for packet switching.

- Combined.

### 4.14.2	Successful DNS Query Procedures

- This measurement provides the number of successful DNS query procedures SGSN-initiated within this SGSN area.
The three measurement types defined in e) are subject to the "2 out of 3 approach".

- CC.

- Receipt of a DNS query response by the SGSN from the DNS server (RFC 1035 [14]).

- A single integer value.

- DNS.SuccDNSResp:

- SgsnFunction.

- Valid for packet switching.

- Combined.

# 5	Measurements related to the GGSN

## 5.1	Session Management

### 5.1.1	Session establishments

The performance counters presented in this subclause are mainly intended to:

- monitor the session establishment success at the GGSN level;

- identify the main causes for GGSN originating session establishment failures;

- and study the repartition of the different traffic classes within session establishment attempts and successes.

These counters are associated to GPRS Tunnelling Protocol signalling (GTP-C for the control plane), between the SGSN and the GGSN, and defined in TS 23.060 [2] and TS 29.060 [6].

The figure below, from TS 23.060 [2], recalls the sequence of messages exchanged for a primary PDP context activation and a subsequent secondary PDP context activation and details the events triggering the update of the counters values.

- Figure from TS 23.060 [2]

The three measurement types defined in the clause 5.1.1 are subject to the "2 out of 3 approach".

#### 5.1.1.1	Attempted session establishments

- This measurement provides the number of attempted session establishments. This measurement is split into subcounters per traffic class and allocation/retention priority (or precedence class) indicated in the QoS profile.

- CC.

- On receipt of a CREATE PDP CONTEXT REQUEST message by the GGSN, the relevant measurement is incremented according to the traffic class and allocation/retention priority (or precedence class) indicated in the message. In case of a PDP context activated with R97/98 QoS attributes, the fields traffic class and allocation/retention priority used for screening are derived from delay class and precedence class respectively, as ruled in TS 23.107 [8]. See also TS 24.008 [15] and TS 29.060 [6].

- A single integer value per measurement type defined in e).

- SM.AttActPdpCtxt.Bgrd.Low
SM.AttActPdpCtxt.Conv.Low
SM.AttActPdpCtxt.Intact.Low
SM.AttActPdpCtxt.Strm.Low
SM.AttActPdpCtxt.Bgrd.High
SM.AttActPdpCtxt.Conv.High
SM.AttActPdpCtxt.Intact.High
SM.AttActPdpCtxt.Strm.High
SM.AttActPdpCtxt.Bgrd.Medium
SM.AttActPdpCtxt.Conv.Medium
SM.AttActPdpCtxt.Intact.Medium
SM.AttActPdpCtxt.Strm.Medium.

- GgsnFunction.

- Valid for packet switched traffic.

- COMB.

- This measurement is mainly dedicated to Operator Business and Vendor Performance Modelling communities.

#### 5.1.1.2	Successful session establishments

- This measurement provides the number of sessions successfully established. This measurement is split into subcounters per traffic class and allocation/retention priority (or precedence class) given in the QoS profile of the related PDP context.

- CC.

- The relevant measurement is incremented on transmission by the GGSN of a CREATE PDP CONTEXT RESPONSE message sent with cause "Request Accepted", according to the traffic class and allocation/retention priority of the related PDP context. In case of a PDP context activated with R97/98 QoS attributes, the fields traffic class and allocation/retention priority used for screening are derived from delay class and precedence class respectively, as ruled in TS 23.107 [8]. See also TS  24.008 [15] and TS 29.060 [6].

- A single integer value per measurement type defined in e).

- SM.SuccActPdpCtxt.Bgrd.Low
SM.SuccActPdpCtxt.Conv.Low
SM.SuccActPdpCtxt.Intact.Low
SM.SuccActPdpCtxt.Strm.Low
SM.SuccActPdpCtxt.Bgrd.High
SM.SuccActPdpCtxt.Conv.High
SM.SuccActPdpCtxt.Intact.High
SM.SuccActPdpCtxt.Strm.High
SM.SuccActPdpCtxt.Bgrd.Medium
SM.SuccActPdpCtxt.Conv.Medium
SM.SuccActPdpCtxt.Intact.Medium
SM.SuccActPdpCtxt.Strm.Medium.

- GgsnFunction.

- Valid for packet switched traffic.

- COMB.

- This measurement is mainly dedicated to Operator Business and Vendor Performance Modelling communities.

#### 5.1.1.3	Failed session establishments

- This measurement provides the number of session establishment failures. This measurement is split into subcounters per failure cause.

- CC.

- On transmission by the GGSN of a CREATE PDP CONTEXT RESPONSE message indicating a PDP context activation failure, the measurement is incremented according to the failure cause. Possible causes are included in TS 29.060 [6]. The sum of all supported per cause measurements should equal the total number of PDP context activation failures.

- Each measurement is an integer value. The number of measurements is equal to the number of causes supported.

- The measurement name has the form SM.FailActPdpCtxt.Cause
where Cause identifies the failure cause.

- GgsnFunction.

- Valid for packet switched traffic.

- COMB.

- This measurement is mainly dedicated to Operator Maintenance and Vendor Performance Modelling communities.

### 5.1.2	Network-initiated session establishments

The performance counters presented in this subclause focus on network initiated PDP context activation procedure, that allows the GGSN to initiate the activation of a PDP context on receipt of a PDP PDU on the Gi interface. The counters proposed are mainly intended to

- monitor the signalling exchanged between the HLR and the GGSN during this procedure; and

- monitor the success rate for network-initiated session establishments. It has to be noted that measurements proposed enable to distinguish between the establishment failures occurring before and after the SGSN has sent the context activation request to the MS.

These counters are associated to the Mobile Application Part (MAP) protocol layer (defined in TS 29.002 [4]) and to GPRS Tunnelling Protocol signalling (GTP-C for the control plane), between the SGSN and the GGSN (defined in TS 29.060 [6]).

The figure below, from TS 23.060 [2], recalls the sequence of messages exchanged for a network initiated PDP context activation and details the events triggering the update of the counters values.

- Figure from TS 23.06

#### 5.1.2.1	Number of routing information requests for network-initiated session establishment attempts

- This measurement provides the number of «Send Routing Info for GPRS » requests sent to the HLR.

- CC.

- The measurement is incremented on transmission by the GGSN of a MAP SEND ROUTING INFO FOR GPRS message to the HLR. See TS 23.060 [2] and TS 29.002 [4].

- Integer.

- SM.SendRoutInfoGprsHlrReq.

- GgsnFunction.

- Valid for packet switched traffic.

- COMB.

- This measurement is mainly dedicated to Vendor Performance Modelling community.

#### 5.1.2.2	Number of routing information successful responses for network-initiated session establishment attempts

- This measurement provides the number of "Send Routing Info for GPRS" response messages received from HLR indicating a positive outcome.

- CC.

- The measurement is incremented on receipt by the GGSN of a MAP SEND ROUTING INFO FOR GPRS response message containing an SGSN address, which indicates a successful outcome. See TS 23.060 [2] and TS 29.002 [4].

- Integer.

- SM.SuccSendRoutInfoGprsHlrRsp.

- GgsnFunction.

- Valid for packet switched traffic.

- COMB.

- This measurement is mainly dedicated to Vendor Performance Modelling and Operator Maintenance communities.

#### 5.1.2.3	Attempted Network-initiated session establishments

##### 5.1.2.3.1	Attempted Network-initiated session establishments

- This measurement provides the number of network-initiated session establishments attempted. Only the session establishment attempts for which a successful routing response from the HLR has been received are counted (i.e. for which a response including an SGSN address).

- CC.

- The measurement is incremented on transmission by the GGSN of a PDU NOTIFICATION REQUEST message to the SGSN. See TS 23.060 [2] and TS 29.060 [6].

- Integer.

- SM.AttActPdpCtxtNetw.

- GgsnFunction.

- Valid for packet switched traffic.

- COMB.

- This measurement is mainly dedicated to Vendor Performance Modelling community.

##### 5.1.2.3.2	Failed Network-initiated session establishments - failures occurred before sending PDP context activation request to the MS

- This measurement provides the number of network initiated session establishment failures. This measurement is split into subcounters per failure cause.

- CC.

- On receipt by the GGSN of a PDU NOTIFICATION RESPONSE message with cause different from "Request Accepted", indicating a PDP context activation failure, the relevant measurement is incremented according to the failure cause. Possible causes are included in TS 29.060 [6]. The sum of all supported per cause measurements should equal the total number of PDP context activation failures occurred before sending REQUEST PDP CONTEXT ACTIVATION message to the MS.

- Each measurement is an integer value. The number of measurements is equal to the number of causes supported.

- The measurement name has the form SM.FailActPdpCtxtNetw.NetwCause 
where NetwCause identifies the failure cause.

- GgsnFunction.

- Valid for packet switched traffic.

- COMB.

- This measurement is mainly dedicated to Vendor Performance Modelling and Operator Maintenance communities.

##### 5.1.2.3.3	Failed Network-initiated session establishments - failures occurred after sending PDP context activation request to the MS

- This measurement provides the number of network initiated session establishment failures. This measurement is split into subcounters per failure cause.

- CC.

- On receipt by the GGSN of a PDU NOTIFICATION REJECT REQUEST, the relevant measurement is incremented according to the failure cause. Possible causes are included in TS 29.060 [6]. The sum of all supported per cause measurements should equal the total number of PDP context activation failures occurred after sending REQUEST PDP CONTEXT ACTIVATION message to the MS.

- Each measurement is an integer value. The number of measurements is equal to the number of causes supported.

- The measurement name has the form SM.FailActPdpCtxtNetw.MsCause 
where MsCause identifies the failure cause.

- GgsnFunction.

- Valid for packet switched traffic.

- COMB.

- This measurement is mainly dedicated to Vendor Performance Modelling and Operator Maintenance communities.

### 5.1.3	Subscribers with activated PDP context

The performance counters presented in this subclause are mainly intended to establish a subscriber profile. Such a profile details the number of elementary procedures per active subscriber (PDP context activations, modifications, updates, …), usually during a busy hour. This profile may be used for 2 main purposes:

- to estimate the current load of the equipment, with details on the respective weight of each procedure in the overall load;

- to estimate the impact on the equipment of a modification of a factor in this subscriber profile (e.g. increase of the number of simultaneous active PDP contexts per subscriber, increase of the number of subscribers, …).

#### 5.1.3.1	Number of subscribers with activated PDP context

- This measurement provides the number of subscribers having an activated PDP context.

- GAUGE.

- The measurement is incremented on transmission by the GGSN of a CREATE PDP CONTEXT RESPONSE message with cause "Request Accepted" for an MSISDN that had no PDP context already activated. 
The measurement is decremented on transmission by the GGSN of a DELETE PDP CONTEXT RESPONSE message with cause "Request Accepted" related to the last PDP context for an MSISDN. 
See TS 29.060 [6] and TS 23.060 [2].

- A single integer value.

- SM.NbrActSubs.

- GgsnFunction.

- Valid for packet switched traffic.

- Combined.

- This measurement is mainly dedicated to Operator Business and Vendor Performance Modelling communities.

#### 5.1.3.2	Mean number of subscribers with activated PDP context

- This measurement provides the mean number of subscribers having an activated PDP context.

- SI.

- This measurement is obtained by sampling at a pre-defined interval the number of subscribers having an activated PDP context in the GGSN and then taking the arithmetic mean.

- A single integer value.

- SM.MeanActSubs.

- GgsnFunction.

- Valid for packet switched traffic.

- Combined.

- This measurement is mainly dedicated to Operator Business and Vendor Performance Modelling communities.

#### 5.1.3.3	Max number of subscribers with activated PDP context

- This measurement provides the maximum number of subscribers having an activated PDP context.

- SI.

- This measurement is obtained by sampling at a pre-defined interval the number of subscribers having an activated PDP context in the GGSN and then taking the maximum.

- A single integer value.

- SM.MaxActSubs.

- GgsnFunction.

- Valid for packet switched traffic.

- Combined.

- This measurement is mainly dedicated to Operator Business and Vendor Performance Modelling communities.

### 5.1.4	Session conclusions

The performance counters presented in this subclause are related to PDP context deactivation procedure. The counters proposed are mainly intended to evaluate the ratio of GGSN-initiated PDP context deactivations in overall PDP context deactivations, estimate the PDP context deactivation success rate, and may also be used in the subscriber or session profile.

The figures below, from TS 23.060 [2], recall the sequence of messages exchanged for MS, SGSN or GGSN initiated PDP context deactivations and detail the events triggering the update of the counters values.

- Figure: MS initiated PDP context deactivation

- Figure:  GSN initiated PDP context deactivation

- Figure: GGSN initiated PDP context deactivation

#### 5.1.4.1	MS & SGSN-initiated session conclusions

##### 5.1.4.1.1	Attempted MS & SGSN-initiated session conclusions

- This measurement provides the number of attempted PDP context deactivations initiated by MS & SGSN.

- CC.

- The measurement is incremented on receipt by the GGSN of a DELETE PDP CONTEXT REQUEST message. See TS 29.060 [6].

- Integer.

- SM.AttDeactPdpCtxtMsAndSgsn.

- GgsnFunction.

- Valid for packet switched traffic.

- COMB.

- This measurement is mainly dedicated to Vendor Performance Modelling community.

##### 5.1.4.1.2	Successful MS & SGSN-initiated session conclusions

- This measurement provides the number of successful PDP context deactivations initiated by MS & SGSN.

- CC.

- The measurement is incremented on transmission by the GGSN of a DELETE PDP CONTEXT RESPONSE message with cause "Request Accepted". See TS 29.060 [6].

- Integer.

- SM. SuccDeactPdpCtxtMsAndSgsn.

- GgsnFunction.

- Valid for packet switched traffic.

- COMB.

- This measurement is mainly dedicated to Vendor Performance Modelling community.

#### 5.1.4.2	GGSN-initiated session conclusions

##### 5.1.4.2.1	Attempted GGSN-initiated session conclusions

- This measurement provides the number of attempted PDP context deactivations initiated by GGSN.

- CC.

- The measurement is incremented on transmission by the GGSN of a DELETE PDP CONTEXT REQUEST message. See TS 29.060 [6].

- Integer.

- SM.AttDeactPdpCtxtGgsn.

- GgsnFunction.

- Valid for packet switched traffic.

- COMB.

- This measurement is mainly dedicated to Vendor Performance Modelling community.

##### 5.1.4.2.2	Successful GGSN-initiated session conclusions

- This measurement provides the number of successful PDP context deactivations initiated by GGSN.

- CC.

- The measurement is incremented on receipt by the GGSN of a DELETE PDP CONTEXT RESPONSE message with cause "Request Accepted". See TS 29.060 [6].

- Integer.

- SM.SuccDeactPdpCtxtGgsn.

- GgsnFunction.

- Valid for packet switched traffic.

- COMB.

- This measurement is mainly dedicated to Vendor Performance Modelling community.

## 5.2	Per APN measurements

These measurements will only be provided for a subset of all the APNs of the GGSN (see TS 23.003 [9] for APN definition). The way the list of monitored APNs is configured is outside the scope of the present document.

### 5.2.1	Session establishments

The performance counters presented in this subclause are intended to bring a more detailed view on session activations compared to counters defined in subclause 1.1. Especially, they enable to monitor the session establishment success rate when user authentication is required and when a dynamic PDP address is to be allocated by the GGSN.

Furthermore, the definition of "per APN" measurements allows to let performance monitoring focus on a "specific service" handled by a GGSN: TS 23.003 [9] indicates that an APN Network Identifier may be used to access a service associated with a GGSN and that this may be achieved by defining:

- an APN that corresponds to a DNS name of a GGSN and is locally interpreted by the GGSN as a request for a specific service; or

- an APN Network Identifier consisting of 3 or more labels and starting with a Reserved Service Label, or an APN Network Identifier consisting of a Reserved Service Label alone, that indicates a GGSN by the nature of the requested service.

The figure below, from TS 29.061 [10] details the message sequence during a PDP context activation for the non-transparent IP case, where a dynamic PDP address is to be allocated and user authentication is required.

- Figure

#### 5.2.1.1	Session establishments, per APN

##### 5.2.1.1.1	Attempted session establishments, per APN

- This measurement provides the number of PDP context activation procedures on a per APN of the GGSN basis.

- CC.

- The measurement is incremented on receipt by the GGSN of a CREATE PDP CONTEXT REQUEST message from the SGSN. See TS 29.060 [6].

- Integer.

- SM.AttActPdpCtxt.Apn.

- GgsnFunction, per APN.

- Valid for packet switched traffic.

- COMB.

- This measurement is mainly dedicated to Operator Business and Vendor Performance Modelling communities.

#### 5.2.1.1.2	Successfully established sessions, per APN

- This measurement provides the number of successfully completed activation PDP context procedures on a per APN of the GGSN basis.

- CC.

- The measurement is incremented on transmission of a CREATE PDP CONTEXT RESPONSE message with cause "Request Accepted " from GGSN. See TS 29.060 [6].

- Integer.

- SM.SuccActPdpCtxt.Apn.

- GgsnFunction, per APN.

- Valid for packet switched traffic.

- COMB.

- This measurement is mainly dedicated to Operator Business and Vendor Performance Modelling communities.

#### 5.2.1.2	Session establishments with dynamic PDP address allocation required, per APN

##### 5.2.1.2.1	Attempted session establishments with dynamic PDP address allocation required, per APN

- This measurement provides the number of dynamic PDP context activation procedures where a dynamic PDP address is requested on a per APN of the GGSN basis.

- CC.

- The measurement is incremented on receipt by the GGSN of a CREATE PDP CONTEXT REQUEST message with an empty PDP address, which indicates that the MS requires a dynamic PDP address. See TS 29.060 [6].

- Integer.

- SM.AttDynActPdpCtxt.Apn.

- GgsnFunction, per APN.

- Valid for packet switched traffic.

- COMB.

- This measurement is mainly dedicated to Vendor Performance Modelling and Operator Maintenance communities.

##### 5.2.1.2.2	Successfully established sessions with dynamic PDP address allocation required, per APN

- This measurement provides the number of successfully attempted dynamic PDP context activation procedures where a dynamic PDP address is requested on a per APN of the GGSN basis.

- CC.

- The measurement is incremented on transmission by the GGSN of a CREATE PDP CONTEXT RESPONSE message with cause "Request Accepted" where the PDP address has been dynamically assigned. See TS 23.060 [2] and TS 29.060 [6].

- Integer.

- SM.SuccDynActPdpCtxt.Apn.

- GgsnFunction, per APN.

- Valid for packet switched traffic.

- COMB.

- This measurement is mainly dedicated to Vendor Performance Modelling and Operator Maintenance communities.

#### 5.2.1.3	Session establishments with user authentication required, per APN

##### 5.2.1.3.1	Attempted session establishments with user authentication required, per APN

- This measurement provides the number of PDP context activation procedures for which user authentication is required.

- CC.

- The measurement is incremented when a CREATE PDP CONTEXT REQUEST message is received by the GGSN, for which protocol configuration options indicates that user authentication is required to access the external PDN. See TS 29.060 [6] and TS 24.008 [15].

- Integer.

- SM.AttActPdpCtxtAutReq.Apn.

- GgsnFunction, per APN.

- Valid for packet switched traffic.

- COMB.

- This measurement is mainly dedicated to Vendor Performance Modelling and Operator Maintenance communities.

##### 5.2.1.3.2	Failed session establishments due to user authentication failure, per APN

- This measurement provides the number of PDP context activation procedures failed due to user authentication failure.

- CC.

- The measurement is incremented when a CREATE PDP CONTEXT RESPONSE message with cause "User Authentication Failed" is received by the GGSN. See TS 29.060 [6] and TS 24.008 [15].

- Integer.

- SM.FailActPdpCtxtAutReq.Apn.

- GgsnFunction, per APN.

- Valid for packet switched traffic.

- COMB.

- This measurement is mainly dedicated to Vendor Performance Modelling and Operator Maintenance communities.

### 5.2.2	Active sessions

The performance counters presented in this subclause are defined on a per APN basis and are mainly intended:

- to monitor the repartition of QoS attributes defined for current active sessions; and

- to establish a session profile. A session profile details the number of elementary procedures per active session (PDP context modifications, updates, …), usually during a busy hour.

#### 5.2.2.1	Number of simultaneous active sessions, per APN

- This measurement provides the current number of simultaneous active sessions per APN. This measurement is split into subcounters per traffic class and allocation/retention priority (or precedence class) indicated in the QoS profile.

- GAUGE.

- The relevant measurement is incremented on transmission by the GGSN of a CREATE PDP CONTEXT RESPONSE message with cause "Request Accepted" according to the traffic class or allocation/retention priority indicated in the QoS profile.
The relevant measurement is decremented on transmission or receipt of DELETE PDP CONTEXT RESPONSE with cause "Request Accepted" according to the traffic class or the allocation/retention priority of the PDP context. 
In case of a PDP context activated with R97/98 QoS attributes, the fields traffic class and allocation/retention priority used for screening are derived from delay class and precedence class respectively, as ruled in TS 23.107 [8].
See also TS 24.008 [15] and TS 29.060 [6].

- A single integer value per measurement type defined in e).

- SM.NbrActPdpCtxt.Apn.Low
SM.NbrActPdpCtxt.Apn.Medium
SM.NbrActPdpCtxt.Apn.High
SM.NbrActPdpCtxt.Apn.Conv
SM.NbrActPdpCtxt.Apn.Strm
SM.NbrActPdpCtxt.Apn.Intact
SM.NbrActPdpCtxt.Apn.Bgrd.

- GgsnFunction, per APN.

- Valid for packet switched traffic.

- COMB.

- This measurement is mainly dedicated to Operator Business and Vendor Performance Modelling communities.

#### 5.2.2.2	Peak number of simultaneous active sessions, per APN

- This measurement provides the peak number of active PDP contexts in GGSN per APN. This measurement is obtained by comparing following an update of the actual number of active PDP context in GGSN per APN, this value with the currently maximal value within the actual granularity period.

- GAUGE.

- The measurement is incremented on transmission by the GGSN of a CREATE PDP CONTEXT RESPONSE message with cause "Request Accepted" and decremented on transmission or receipt by the GGSN of a DELETE PDP CONTEXT RESPONSE message with cause "Request Accepted". The measurement value keeps track of the highest value experienced in the collection interval. See TS 29.060 [6].

- Integer.

- SM.MaxNbrActPdpCtxt.Apn.

- GgsnFunction, per APN.

- Valid for packet switched traffic.

- COMB.

- This measurement is mainly dedicated to Operator Business and Vendor Performance Modelling communities.

#### 5.2.2.3	MS & SGSN-initiated session modifications, per APN

##### 5.2.2.3.1	Attempted MS & SGSN-initiated session modifications, per APN

- This measurement provides the number of PDP context updates attempted, either by MS or SGSN.

- CC.

- The measurement is incremented on receipt by the GGSN of an UPDATE PDP CONTEXT REQUEST message. See TS 29.060 [6].

- Integer.

- SM.AttUpdPdpCtxtMsAndSgsn.Apn.

- GgsnFunction, per APN.

- Valid for packet switched traffic.

- COMB.

- This measurement is mainly dedicated to Vendor Performance Modelling community.

##### 5.2.2.3.2	Successfully performed MS & SGSN-initiated session modifications, per APN

- This measurement provides the number of successfully performed PDP context updates initiated either by MS or SGSN.

- CC.

- The measurement is incremented on transmission by the GGSN of an UPDATE PDP CONTEXT RESPONSE message with cause "Request Accepted". See TS 29.060 [6].

- Integer.

- SM.SuccUpdPdpCtxtMsAndSgsn.Apn.

- GgsnFunction, per APN.

- Valid for packet switched traffic.

- COMB.

- This measurement is mainly dedicated to Vendor Performance Modelling community.

### 5.2.3	Session conclusions

#### 5.2.3.1	MS-initiated session conclusions, per APN

##### 5.2.3.1.1	Attempted MS-initiated session conclusions, per APN

- This measurement provides the number of PDP context deactivation procedures initiated by the MS on a per APN of the GGSN basis.

- CC.

- The measurement is incremented on receipt by the GGSN of a DELETE PDP CONTEXT REQUEST message from the SGSN. See TS 23.060 [2] and TS 29.060 [6].

- Integer.

- SM.AttDeactPdpCtxtMs.Apn.

- GgsnFunction, per APN.

- Valid for packet switched traffic.

- COMB.

- This measurement is mainly dedicated to Vendor Performance Modelling community.

##### 5.2.3.1.2	Successful MS-initiated session conclusions, per APN

- This measurement provides the number of successfully completed PDP context deactivation procedures initiated by the MS on a per APN of the GGSN basis.

- CC.

- The measurement is incremented on transmission by the GGSN of a DELETE PDP CONTEXT RESPONSE message with cause "Request Accepted" to the SGSN. See TS 29.060 [6].

- Integer.

- SM.SuccDeactPdpCtxtMs.Apn.

- GgsnFunction, per APN.

- Valid for packet switched traffic.

- COMB.

- This measurement is mainly dedicated to Vendor Performance Modelling community.

#### 5.2.3.2	GGSN-initiated session conclusions, per APN

##### 5.2.3.2.1	Attempted GGSN-initiated session conclusions, per APN

- This measurement provides the number of PDP context deactivation procedures initiated by the GGSN, on a per APN of the GGSN basis.

- CC.

- The measurement is incremented on transmission by the GGSN of a DELETE PDP CONTEXT REQUEST message to the SGSN. See TS 29.60.

- Integer.

- SM.AttDeactPdpCtxtGgsn.Apn.

- GgsnFunction, per APN.

- Valid for packet switched traffic.

- COMB.

- This measurement is mainly dedicated to Vendor Performance Modelling community.

##### 5.2.3.2.2	Successful GGSN-initiated session conclusions, per APN

- This measurement provides the number of successfully completed PDP context deactivation procedures initiated by the GGSN, on a per APN of the GGSN basis.

- CC.

- The measurement is incremented on receipt of DELETE PDP CONTEXT RESPONSE message with cause "Request Accepted" from the SGSN.

- Integer.

- SM.SuccDeactPdpCtxtGgsn.Apn.

- GgsnFunction, per APN.

- Valid for packet switched traffic.

- COMB.

- This measurement is mainly dedicated to Vendor Performance Modelling community.

## 5.3	GTP measurements

The performance counters presented in this subclause are mainly intended to:

- monitor the signalling and bearer traffic exchanged between the GGSN and peer GSNs;

- establish the session profile (including GTP average packet size, signalling overhead, uplink and downlink GTP traffic per session, …); and

- monitor the GGSN load (through measurements such as the total bit rate handled by the node, the number of GTP tunnels handled or the ratio of packets discarded at GGSN level).

These counters are associated to GPRS Tunnelling Protocol (GTP-C and GTP-U), between the SGSN and the GGSN, and defined in TS 23.060 [2] and TS 29.060 [6]. The breakdown per traffic class allows to monitor the way traffic is handled by the GGSN according to QoS attributes attached to the relevant PDP context.

### 5.3.1	Number of incoming GTP data packets on the Gn interface

- This measurement provides the number of GTP Data Packets received on the Gn interface. This measurement is split into subcounters per traffic class.

- CC.

- The relevant measurement is incremented on receipt of a GTP data packet on the Gn interface, according to the traffic class of the related PDP context. In case of a PDP context activated with R97/98 QoS attributes, the field traffic class used for screening is derived from delay class, as ruled in TS 23.107 [8]. See also TS 29.060 [6].

- A single integer value per measurement type defined in e).

- GTP.IncDataPkt.Bgrd 
GTP.IncDataPkt.Conv 
GTP.IncDataPkt.Intact 
GTP.IncDataPkt.Strm.

- GgsnFunction.

- Valid for packet switched traffic.

- COMB.

- This measurement is mainly dedicated to Vendor Performance Modelling, Operator Traffic Engineering and Vendor Development Engineering communities.

### 5.3.2	Number of outgoing GTP data packets on the Gn interface

- This measurement provides the number of GTP Data Packets sent onto the Gn interface. This measurement is split into subcounters per traffic class.

- CC.

- The relevant measurement is incremented on transmission of a GTP data packet on the Gn interface, according to the traffic class indicated in the QoS profile of the related PDP context. In case of a PDP context activated with R97/98 QoS attributes, the field traffic class used for screening is derived from delay class, as ruled in TS 23.107 [8]. See also  TS 29.060 [6].

- A single integer value per measurement type defined in e).

- GTP.OutDataPkt.Bgrd 
GTP.OutDataPkt.Conv 
GTP.OutDataPkt.Intact 
GTP.OutDataPkt.Strm.

- GgsnFunction.

- Valid for packet switched traffic.

- COMB.

- This measurement is mainly dedicated to Vendor Performance Modelling, Operator Traffic Engineering and Vendor Development Engineering communities.

### 5.3.3	Number of discarded GTP data packets

- This measurement provides the number of GTP Data Packets discarded. This measurement is split into subcounters per traffic class.

- CC.

- The relevant measurement is incremented when a GTP data packet is discarded, according to the traffic class indicated in the QoS profile of the related PDP context. In case of a PDP context activated with R97/98 QoS attributes, the field traffic class used for screening is derived from delay class, as ruled in TS 23.107 [8]. See also TS 29.060 [6].

- A single integer value per measurement type defined in e).

- GTP.DiscDataPkt.Bgrd 
GTP.DiscDataPkt.Conv 
GTP.DiscDataPkt.Intact 
GTP.DiscDataPkt.Strm.

- GgsnFunction.

- Valid for packet switched traffic.

- COMB.

- This measurement is mainly dedicated to Vendor Performance Modelling, Operator Traffic Engineering and Vendor Development Engineering communities.

### 5.3.4	Number of octets of incoming GTP data packets on the Gn interface

- This measurement provides the number of GTP payload octets received. This measurement is split into subcounters per traffic class.

- CC.

- The relevant measurement is incremented on receipt of a GTP data packet on the Gn interface, according to the traffic class indicated in the QoS profile of the related PDP context. In case of a PDP context activated with R97/98 QoS attributes, the field traffic class used for screening is derived from delay class, as ruled in TS 23.107 [8]. The data packet size is extracted from the GTP header and added on to the measurement value. See TS 29.060 [6].

- A single integer value per measurement type defined in e).

- GTP.IncDataOct.Bgrd 
GTP.IncDataOct.Conv 
GTP.IncDataOct.Intact 
GTP.IncDataOct.Strm.

- GgsnFunction.

- Valid for packet switched traffic.

- COMB.

- This measurement is mainly dedicated to Vendor Performance Modelling and Operator Traffic Engineering communities.

### 5.3.5	Number of octets of outgoing GTP data packets on the Gn interface

- This measurement provides the number of GTP payload octets sent. This measurement is split into subcounters per traffic class.

- CC.

- The relevant measurement is incremented on transmission of a GTP data packet on the Gn interface, according to the traffic class indicated in the QoS profile of the related PDP context. In case of a PDP context activated with R97/98 QoS attributes, the field traffic class used for screening is derived from delay class, as ruled in TS 23.107 [8]. The data packet size is extracted from the GTP header and added on to the measurement value. See  TS 29.060 [6].

- A single integer value per measurement type defined in e).

- GTP.OutDataOct.Bgrd 
GTP.OutDataOct.Conv 
GTP.OutDataOct.Intact 
GTP.OutDataOct.Strm.

- GgsnFunction.

- Valid for packet switched traffic.

- COMB.

- This measurement is mainly dedicated to Vendor Performance Modelling and Operator Traffic Engineering communities.

### 5.3.6	Number of incoming GTP signalling packets on the Gn interface

- This measurement provides the number of GTP signalling packets received on the Gn interface. This measurement is split into subcounters per traffic class.

- CC.

- The relevant measurement is incremented on receipt of a GTP signalling packet on the Gn interface, according to the traffic class of the related PDP context. In case of a PDP context activated with R97/98 QoS attributes, the field traffic class used for screening is derived from delay class, as ruled in TS 23.107 [8]. See also TS 29.060 [6].

- A single integer value per measurement type defined in e).

- GTP.IncSigPkt.Bgrd 
GTP.IncSigPkt.Conv 
GTP.IncSigPkt.Intact 
GTP.IncSigPkt.Strm.

- GgsnFunction.

- Valid for packet switched traffic.

- COMB.

- This measurement is mainly dedicated to Vendor Performance Modelling, Operator Traffic Engineering and Vendor Development Engineering communities.

### 5.3.7	Number of outgoing GTP signalling packets on the Gn interface

- This measurement provides the number of GTP signalling packets sent onto the Gn interface. This measurement is split into subcounters per traffic class.

- CC.

- The relevant measurement is incremented on transmission of a GTP signalling packet on the Gn interface, according to the traffic class indicated in the QoS profile of the related PDP context. In case of a PDP context activated with R97/98 QoS attributes, the field traffic class used for screening is derived from delay class, as ruled in TS 23.107 [8]. See also TS 29.060 [6].

- A single integer value per measurement type defined in e).

- GTP.OutSigPkt.Bgrd 
GTP.OutSigPkt.Conv 
GTP.OutSigPkt.Intact 
GTP.OutSigPkt.Strm.

- GgsnFunction.

- Valid for packet switched traffic.

- COMB.

- This measurement is mainly dedicated to Vendor Performance Modelling, Operator Traffic Engineering and Vendor Development Engineering communities.

### 5.3.8	Number of discarded GTP signalling packets

- This measurement provides the number of GTP signalling packets discarded. This measurement is split into subcounters per traffic class.

- CC.

- The relevant measurement is incremented when a GTP signalling packet is discarded, according to the traffic class indicated in the QoS profile of the related PDP context. In case of a PDP context activated with R97/98 QoS attributes, the field traffic class used for screening is derived from delay class, as ruled in TS 23.107 [8]. See also TS 29.060 [6].

- A single integer value per measurement type defined in e).

- GTP.DiscSigPkt.Bgrd 
GTP.DiscSigPkt.Conv 
GTP.DiscSigPkt.Intact 
GTP.DiscSigPkt.Strm.

- GgsnFunction.

- Valid for packet switched traffic.

- COMB.

- This measurement is mainly dedicated to Vendor Performance Modelling, Operator Traffic Engineering and Vendor Development Engineering communities.

### 5.3.9	Number of octets of incoming GTP signalling packets on the Gn interface

- This measurement provides the number of octets of received GTP signalling packets. This measurement is split into subcounters per traffic class.

- CC.

- The relevant measurement is incremented on receipt of a GTP signalling packet on the Gn interface, according to the traffic class indicated in the QoS profile of the related PDP context. In case of a PDP context activated with R97/98 QoS attributes, the field traffic class used for screening is derived from delay class, as ruled in TS 23.107 [8]. The signalling packet size is extracted from the GTP header and added on to the measurement value. See TS 29.060 [6].

- A single integer value per measurement type defined in e).

- GTP.IncSigOct.Bgrd
GTP.IncSigOct.Conv
GTP.IncSigOct.Intact
GTP.IncSigOct.Strm.

- GgsnFunction.

- Valid for packet switched traffic.

- COMB.

- This measurement is mainly dedicated to Vendor Performance Modelling and Operator Traffic Engineering communities.

### 5.3.10	Number of octets of outgoing GTP signalling packets on the Gn interface

- This measurement provides the number of octets of sent GTP signalling packets. This measurement is split into subcounters per traffic class.

- CC.

- The relevant measurement is incremented on transmission of a GTP signalling packet on the Gn interface, according to the traffic class indicated in the QoS profile of the related PDP context. In case of a PDP context activated with R97/98 QoS attributes, the field traffic class used for screening is derived from delay class, as ruled in TS 23.107 [8]. The signalling packet size is extracted from the GTP header and added on to the measurement value. See TS 29.060 [6].

- A single integer value per measurement type defined in e).

- GTP.OutSigOct.Bgrd
GTP.OutSigOct.Conv
GTP.OutSigOct.Intact
GTP.OutSigOct.Strm.

- GgsnFunction.

- Valid for packet switched traffic.

- COMB.

- This measurement is mainly dedicated to Vendor Performance Modelling and Operator Traffic Engineering communities.

### 5.3.11	Number of GTP tunnels on the Gn interface

- This measurement provides the current number of simultaneous GTP tunnels on Gn interface handled by the GGSN.

- GAUGE.

- The measurement is incremented on transmission by the GGSN of a CREATE PDP CONTEXT RESPONSE message with cause "Request Accepted".
It is decremented on transmission by the GGSN of a DELETE PDP CONTEXT RESPONSE message with cause "Request Accepted".
The measurement includes GTP tunnels for data (user plane) as well as GTP tunnels for signalling (control plane). See TS 29.060 [6].

- Integer.

- GTP.NbrTunnels.

- GgsnFunction.

- Valid for packet switched traffic.

- COMB.

- This measurement is mainly dedicated to Vendor Performance Modelling and Operator Traffic Engineering communities.

### 5.3.12	Number of GTP tunnels created on the Gn interface

- This measurement provides the number of GTP Tunnels created on Gn interface.

- CC.

- The measurement is incremented on transmission by the GGSN of a CREATE PDP CONTEXT RESPONSE message with cause "Request Accepted".
The measurement includes GTP tunnels for data (user plane) as well as GTP tunnels for signalling (control plane). See TS 29.060 [6].

- Integer.

- GTP.NbrCreatTunnels.

- GgsnFunction.

- Valid for packet switched traffic.

- COMB.

- This measurement is mainly dedicated to Vendor Performance Modelling and Operator Traffic Engineering communities.

## 5.4	GTP' measurements

The performance counters presented in this subclause are intended to monitor the transfer of G-CDRs to the CGF; in particular

- The number of CDR transfer attempts, together with the cause triggering the transfer enables to dimension both the CGF / Billing System and the Ga interface. The breakdown of causes for transfer attempts may also help in tuning the parameters associated to partial CDR creation.

- The breakdown of causes for transfer failure is provided to track and investigate any problem that could be detected thanks to the CDR transfer success rate.

These counters are associated to the GTP' protocol between the GGSN and the CGF, as defined in TS 29.060 [6] and TS 32.015.

The figure below from TS 32.015 shows a normal CDR transfer between a GSN and a CGF and details the events triggering the update of the counters values.

- Figure from TS 32.015

The three measurement types defined in the clause 5.4 are subject to the "2 out of 3 approach".

### 5.4.1	Attempted CDR information transfers

- This measurement provides the number of CDR information transfers attempted. This measurement is split into subcounters per transfer triggering cause.

- CC.

- The relevant measurement is incremented when a DATA RECORD TRANSFER REQUEST message used to transmit CDR information is sent to the CGF, according to the cause that triggered the transfer. Possible causes are included in TS 32.015.

- Each measurement is an integer value.

- The measurement name has the form GTPP.CdrTransfReq.Cause 
where Cause indicates the cause that triggered the transfer.

- GgsnFunction.

- Valid for packet switched traffic.

- COMB.

- This measurement is mainly dedicated to Vendor Performance Modelling and Operator Traffic Engineering communities.

### 5.4.2	Successful CDR information transfers

- This measurement provides the number of CDR information successfully transmitted to CGF.

- CC.

- The measurement is incremented on receipt by the GGSN of a DATA RECORD TRANSFER RESPONSE message with cause code "Request Accepted".

- Integer.

- GTPP.SuccCdrTransf.

- GgsnFunction.

- Valid for packet switched traffic.

- COMB.

- This measurement is mainly dedicated to Vendor Performance Modelling and Operator Traffic Engineering communities.

### 5.4.3	Failed CDR information transfers

- This measurement provides the number of CDR information failed to be transferred to CGF. This measurement is split into subcounters per failure cause. Possible causes are included in TS 32.015.

- CC.

- The relevant measurement is incremented on receipt by the GGSN of a DATA RECORD TRANSFER RESPONSE message according to the failure cause.

- Each measurement is an integer value.

- The measurement name has the form GTPP.FailCdrTransf.Cause
where Cause indicates the failure cause.

- GgsnFunction.

- Valid for packet switched traffic.

- COMB.

- This measurement is mainly dedicated to Vendor Performance Modelling, Operator Maintenance and Operator Traffic Engineering communities.

## 5.5	IP measurements

The performance counters presented in this subclause are mainly intended to:

- monitor the bearer traffic exchanged between the GGSN and the external PDN on the Gi interface;

- establish the session profile (including IP average packet size, uplink and downlink IP traffic per session, …), possibly per traffic class; and

- monitor the GGSN load (through measurements such as the total bit rate handled by the node, the ratio of packets discarded at GGSN level, …).

These counters are associated to IP protocol on the Gi interface.

These counters are proposed to be screened with regards to the protocol configuration options, as defined in TS 24.008 [15] and TS 29.061 [10], i.e. a set of the counters is associated to any valid combination of the different options below:

- transparent or non-transparent access to the external PDN;

- user data encryption (IPSec, …);

- tunnelling of packets onto the Gi interface.

Any valid combination of these options fully defines a "Gi reference point". The figure below gives an overview of some Gi reference points.

- Figure: Overview of some Gi reference points

### 5.5.1	Number of incoming IP data packets on the Gi interface

This measurement provides the number of IP data packets received on the Gi interface. This measurement is split into subcounters per traffic class of the related PDP context.

- CC.

- The relevant measurement is incremented on receipt of an IP data packet on the Gi interface, according to the traffic class of the related PDP context. In case of a PDP context activated with R97/98 QoS attributes, the field traffic class used for screening is derived from delay class, as ruled in TS 23.107 [8]. See also 07 and TS 29.061 [10].

- A single integer value per measurement type defined in e).

- IP.IncDataPkt.Bgrd
IP.IncDataPkt.Conv
IP.IncDataPkt.Intact
IP.IncDataPkt.Strm.

- GgsnFunction, per Gi reference point.

- Valid for packet switched traffic.

- COMB.

- This measurement is mainly dedicated to Vendor Performance Modelling, Operator Traffic Engineering and Vendor Development Engineering communities.

### 5.5.2	Number of outgoing IP data packets on the Gi interface

- This measurement provides the number of IP data packets sent onto the Gi interface. This measurement is split into subcounters per traffic class of the related PDP context.

- CC.

- The relevant measurement is incremented on transmission of an IP data packet on the Gi interface, according to the traffic class of the related PDP context. In case of a PDP context activated with R97/98 QoS attributes, the field traffic class used for screening is derived from delay class, as ruled in TS 23.107 [8]. See also TS 29.061 [10].

- A single integer value per measurement type defined in e).

- IP.OutDataPkt.Bgrd 
IP.OutDataPkt.Conv
IP.OutDataPkt.Intact
IP.OutDataPkt.Strm.

- GgsnFunction, per Gi reference point.

- Valid for packet switched traffic.

- COMB.

- This measurement is mainly dedicated to Vendor Performance Modelling, Operator Traffic Engineering and Vendor Development Engineering communities.

### 5.5.3	Number of IP data packets discarded due to node congestion

- This measurement provides the number of IP data packets discarded. This measurement is split into subcounters per traffic class of the related PDP context.

- CC.

- The relevant measurement is incremented when a received IP data packet is discarded due to node congestion, according to the traffic class of the related PDP context. In case of a PDP context activated with R97/98 QoS attributes, the field traffic class used for screening is derived from delay class, as ruled in TS 23.107 [8]. See also TS 29.061 [10].

- A single integer value per measurement type defined in e).

- IP.DiscDataPkt.Bgrd
IP.DiscDataPkt.Conv
IP.DiscDataPkt.Intact
IP.DiscDataPkt.Strm.

- GgsnFunction, per Gi reference point.

- Valid for packet switched traffic.

- COMB.

- This measurement is mainly dedicated to Vendor Performance Modelling, Operator Traffic Engineering and Vendor Development Engineering communities.

### 5.5.4	Number of octets of incoming IP data packets on the Gi interface

- This measurement provides the number of IP payload octets received on the Gi interface. This measurement is split into subcounters per traffic class of the related PDP context.

- CC.

- The relevant measurement is incremented on receipt of an IP data packet on the Gi interface, according to the traffic class of the related PDP context. In case of a PDP context activated with R97/98 QoS attributes, the field traffic class used for screening is derived from delay class, as ruled in TS 23.107 [8]. The data packet size is extracted from the IP header and added on to the measurement value. See TS 29.061 [10].

- A single integer value per measurement type defined in e).

- IP.IncDataOct.Bgrd 
IP.IncDataOct.Conv 
IP.IncDataOct.Intact 
IP.IncDataOct.Strm.

- GgsnFunction, per Gi reference point.

- Valid for packet switched traffic.

- COMB.

- This measurement is mainly dedicated to Vendor Performance Modelling and Operator Traffic Engineering communities.

### 5.5.5	Number of octets of outgoing IP data packets on the Gi interface

- This measurement provides the number of IP payload octets sent onto the Gi interface. This measurement is split into subcounters per traffic class of the related PDP context.

- CC.

- The relevant measurement is incremented on transmission of an IP data packet on the Gi interface, according to the traffic class of the related PDP context. In case of a PDP context activated with R97/98 QoS attributes, the field traffic class used for screening is derived from delay class, as ruled in TS 23.107 [8]. The data packet size is extracted from the IP header and added on to the measurement value. See TS 29.061 [10].

- A single integer value per measurement type defined in e).

- IP.OutDataOct.Bgrd 
IP.OutDataOct.Conv 
IP.OutDataOct.Intact 
IP.OutDataOct.Strm.

- GgsnFunction, per Gi reference point.

- Valid for packet switched traffic.

- COMB.

- This measurement is mainly dedicated to Vendor Performance Modelling and Operator Traffic Engineering communities.

## 5.6	AAA Authentication measurements

The performance counters presented in this subclause are mainly intended to:

- monitor the AAA server performance;

- monitor the network connectivity.

These counters are associated to the AAA server, which is playing a more and more important role in a 3G environment, and is defined in RFC 2903 [17].

The figure below presents the message sequence during an AAA authentication procedure initiated by the GGSN.

- Figure: AAA authentication procedure initiated by GGSN

NOTE:	In Radius, the AAA Authentication Request message is Access-Request and AAA Authentication Response is Access-Accept/ Access-Reject/ Access-Challenge (RFC 2138 [18]), where the AAA successful authentication response message is Access-Accept.

### 5.6.1	Attempted Authentication Procedures, per APN

- This measurement provides the number of attempted authentication procedures per APN.

- CC.

- Transmission by the GGSN of an AAA authentication request message to the AAA server.

- A single integer value.

- AAA.AttAuthQuery.Apn.

- GgsnFunction, per APN.

- Valid for packet switched traffic.

- Combined

### 5.6.2	Successful Authentication Procedures, per APN

  - This measurement provides the number of successful authentication procedures per APN.

  - CC.

  - Receipt by the GGSN of an AAA successful authentication response message from the AAA server.

  - A single integer value.

  - AAA.SuccAuthResp.Apn.

  - GgsnFunction, per APN.

  - Valid for packet switched traffic.

  - Combined.

Annex A (informative):
Change history

| Change history |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Date | TSG # | TSG Doc. | CR | Rev | Subject/Comment | Cat | Old | New |
| Mar 2006 | SA_31 | SP-060109 | -- | -- | Split of 32.403-710 into four new TSs 32.404, 32.405, 32.406, 32.408. Submitted to TSG SA #31 for Approval. | -- | 1.0.0 | 7.0.0 |
| Jun 2006 | SA_32 | SP-060260 | 0001 | -- | Move SMS related Measurements from 32.406 to 32.408 | F | 7.0.0 | 7.1.0 |
| Dec 2008 | SA_42 | -- | -- | -- | Upgrade to Release 8 | -- | 7.1.0 | 8.0.0 |
| Dec 2009 | -- | -- | -- | -- | Upgrade to Release 9 | -- | 8.0.0 | 9.0.0 |
| Sep 2014 | SA_65 | SP-140557 | 0011 | 1 | Correction of scope | A | 9.0.0 | 9.1.0 |