3GPP TSG-SA WG6 Meeting #39-bis-e	S6-201707

e-meeting, 12th – 20th October 2020	(revision of S6-xxxxxx)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.282 | CR | 0241 | rev | - | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Enhancement to IP connectivity point-to-point MCData service to allow optionally a direct connection between the involved clients |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Kontron Transportation France |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | eMONASTERY2 |  |  |  |  |  | Date: |  |  | 2020-10-05 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | This CR provides an enhancement to the IP connectivity point-to-point MCData service to allow optionally a direct connection between the involved clients, which reduces latency and load of the MCData server. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Change for functional model to introduce the new option. Change in the information flows to support the new option. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Use cases with low high data rate or latency cannot be fully meet and cause unnecessary high load on the MCData server. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.8.1, 7.14.2.1.1, 7.14.2.1.2, 7.14.2.1.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

### 6.8.1	On-network functional model

Figure 6.5.1-1 shows the application plane functional model for User-IP connectivity.

Figure 6.8.1-1: Application plane functional model for IP connectivity

In the model shown in figure 6.8.1-1, the following apply:

-	MCData-IPcon-1 reference point is used for MCData application signalling for establishing a session in support of MCData IP connectivity.

-	MCData-IPcon-2 reference point carries bidirectional IP Data for point-to-point MCData IP connectivity over the media plane between the U-IPcon distribution function of the MCData server and the IPcon function of the MCData client(s).

NOTE:	For MCData IP connectivity point-to-point this reference point optionally carries the media plane directly between the involved MCData IP connectivity clients.

-	MCData-IPcon-3 reference point is used by the IP-con distribution function of the MCData server to send unidirectional downlink IP Data to the IP-con function of the MCData clients.

-	IPcon-host reference point is used for a data host, e.g. server, to use IPconectivity service capabilities. This reference point is outside the scope of the present document.

* * * End of Change * * * *

* * * Next Change * * * *

##### 7.14.2.1.1	MCData IPcon point-to-point request

Table 7.14.2.1.1-1 describes the information flow of the MCData IPcon point-to-point request sent from the MCData client to the MCData server.

Table 7.14.2.1.1-1: MCData IPcon point-to-point request (MCData client to MCData server)

| Information element | Status | Description |
| --- | --- | --- |
| MCData ID | M | The MCData identity of the originator MCData user; |
| Functional alias | O | The associated functional alias of the originator MCData user; |
| MCData ID | O (NOTE 2) | The MCData identity of the target MCData client IP connectivity is requested. |
| Functional alias | O (NOTE 2) | The functional alias of the target MCData client. |
| Requested Priority (NOTE 3) | O | Application priority level requested for this communication. |
| Location Information | O (NOTE 1) | Actual location information of the originating MCData user; |
| Time Limit | O | Proposed time limit of the requested IP connectivity (1min- infinite); |
| Establishment reason | O | IP connectivity establishment reason |
| IP originating endpoint | M | Endpoint of the media plane |
| Connection mode | O (NOTE 4) | Direct or relay (via server) |
| Tunnel type list | O (NOTE 5 | GRE, Open VPN, Wireguard, no tunnel, … |
| NOTE 1: This information contains the latest available location information of the requesting MCData user that may be different to the latest available location information in the MC system. NOTE 2: Either the MCData ID or the functional alias of the target MCData user must be present. NOTE 3: The predefined priority of the MC service user is applied by the MCData server if the requested priority is not present or not accepted by the MCData server. NOTE 4: If the IE Connection mode is not present, the default connection mode is via server. NOTE 5: If the IE Tunnel type list is not present,the default tunnel type is GRE. |  |  |

Table 7.14.2.1.1-2: MCData IPcon point-to-point request (MCData server to MCData client)

| Information element | Status | Description |
| --- | --- | --- |
| MCData ID | M | The MCData identity of the originator MCData user; |
| MCData ID | M | The MCData identity of the target MCData client IP connectivity is requested. |
| Location Information | O (NOTE 1) | Actual location information of the originating MCData user; |
| Time Limit | O | Proposed time limit of the requested IP connectivity (1min- infinite); |
| Establishment reason | O | IP connectivity establishment reason |
| IP originating endpoint | M | Endpoint of the media plane |
| Connection mode | O (NOTE 2) | Direct or relay (via server) |
| Tunnel type list | O (NOTE 3 | GRE, Open VPN, Wireguard, no tunnel, … |
| NOTE 1: This information contains the latest available location information of the requesting MCData user. NOTE 2: If the IE Connection mode is not present, the default connection mode is via server. NOTE 3: If the IE Tunnel type list is not present,the default tunnel type is GRE. |  |  |

##### 7.14.2.1.2	MCData IPcon point-to-point response

Table 7.14.2.1.2-1 describes the information content of the MCData IPcon point-to-point response as answer to MCData IPcon point-to-point request.

Table 7.14.2.1.2-1: MCData IPcon point-to-point response

| Information element | Status | Description |
| --- | --- | --- |
| MCData ID | M | The MCData identity of the targeted MCData user. |
| MCData ID | M | The MCData identity of the requesting MCData user. |
| Time Limit | O | Negotiated time (1 min – infinite) |
| IP connectivity status | M | IP connectivity establishment result |
| IP originating endpoint | M | Endpoint of the media plane |
| Connection mode | O (NOTE 1) | Direct or relay (via server) |
| Tunnel type list | O (NOTE 2 | GRE, Open VPN, Wireguard, no tunnel, … |
| NOTE 1: If the IE Connection mode is not present, the default connection mode is via server. NOTE 2: If the IE Tunnel type list is not present,the default tunnel type is GRE. |  |  |

##### 7.14.2.1.3	MCData remote IPcon point-to-point request

Table 7.14.2.1.3-1 describes the information flow of the MCData remote IPcon point-to-point request sent from the remote MCData client to the MCData server and from the MCData server to the asked MCData client.

Table 7.14.2.1.3-1: MCData remote IPcon point-to-point request

| Information element | Status | Description |
| --- | --- | --- |
| MCData ID remote | M | The MCData identity of the remote MCData client that requests another MCData user to establish an IP connectivity point-to-point session. |
| Functional alias remote | O | The associated functional alias of the remote MCData user. |
| MCData ID asked | M | The MCData identity of the MCData client that is required to establish an IP connectivity point-to-point session. |
| Functional alias asked | O | The functional alias associated with the MCData identity of the MCData client that is required to establish an IP connectivity point-to-point session. |
| MCData ID targeted (NOTE 1) | O | The MCData identity of the MCData client that is the target of the requested IP connectivity point-to-point session. |
| Functional alias targeted (NOTE 1) | O | The functional alias associated with the MC MCData identity of the MCData client that is the target of the requested IP connectivity point-to-point session. |
| Requested Priority (NOTE 2) | O | Application priority level requested for this call. |
| Time Limit | O | Proposed time limit of the requested IP connectivity (1min- infinite). |
| Establishment reason | O | IP connectivity establishment reason |
| Connection mode | O (NOTE 3 | Direct or relay (via server) |
| Tunnel type list | O (NOTE 4 | GRE, Open VPN, Wireguard, no tunnel, … |
| NOTE 1: Either the MCData ID or the functional alias of the targeted MCData user must be present. NOTE 2: The predefined priority of the MC service user is applied by the MCData server if the requested priority is not present or not accepted by the MCData server. NOTE 3: If the IE Connection mode not present, the default connection mode is via server. NOTE 4: If the IE Tunnel type list is not present,the default tunnel type is GRE. |  |  |

* * * End of Change * * * *