3GPP TSG-SA WG6 Meeting #41-e	S6-210073

e-meeting, 18th – 26th January 2021	(revision of S6-21xxxx)

Source:	Lenovo, Motorola Mobility

Title:	Update of Solution #16 on TSN policy negotiation

Spec:	3GPP TR 23.745 v1.2.0

Agenda item:	8.2

Document for:	Approval

Contact:	Emmanouil (Manos) Pateromichelakis <epateromiche@lenovo.com>

1. Introduction

This contribution provides an update of solution #16 on TSN policy negotiation via FAE layer.

2. Reason for Change

This contribution provides an update of the solution to:

- align with the updated FFAPP architecture

- align with the SEAL services as provided in Solutions #10 (NRx server) and Solution #22 (NRz server) for TSC services

- add an additional optional step for acquiring UE monitoring event, as input to FAE-S to trigger a TSN policy update

- address the Editor’s Notes in clause 7.16.1.2 and 7.16.2.

In particular, for the EN related to the need for TSN policy negotiation, involving the enabler layer, some further elaboration is provided:

- The proposed TSN policy update capability at FAE-S takes into account inputs from the network side (QoS monitoring) as well as the device side (UE-related monitoring from the application clients). Some of this information may not be always exposed to the FF application specific server (e.g. CNC); however, this can be translated to an alert indicating a service/QoS parameter which needs to be modified.

- Some TSC service parameters (as defined in TS 22.104) may not be decided at the FF application specific server (e.g. CNC). For example, the Survival Time (which is defined as an application characteristic is the maximum number of consecutive frames whose loss can be tolerated by a given application) is not used in IEEE 802.1 TSN standards. So, the setting/update of the survival time may need to be translated at the enabler layer to a trigger for an application driven action (e.g. change of the tolerance to loss value, change of automation level for the mobile robots).

- FAE-S outputs a trigger for a proposed change of the application behavior to the FF application specific server. FF application specific server may then decide on the modification of the service /QoS requirement for one or more devices running the TSC services. So, the decision point is the FF application specific server. Without this solution, the FF application specific server needs to collect and understand all the QoS/UE monitoring events and process them so as to derive a TSN policy modification per TSN flow.

3. Proposal

It is proposed to agree the following changes to 3GPP TR 23.745 v1.2.0.

* * * Change * * * *

## 7.16	Solution #16: TSN policy negotiation via FAE layer

### 7.16.1	Solution description

This solution function addresses the Key issue #4 for enabling the negotation via the FAE layer of the TSN application QoS requirements (e.g. survival time) and their mapping to network policies / QoS parameters.

#### 7.16.1.1	General

This solution takes as input a trigger event based on the monitored QoS parameters (by SEAL / NRx or by the network) and based on this, it generates a trigger which may be in the form of a proposed action, which can be a policy related to the 1) adaptation of service requirements (e.g. survival time, TSC service area, mobility change) or 2) adaptation of port management policies (DS-TT, NW-TT policies). TSN system (FF application specific server which is equivalent to CNC) may then provide a request for adaptation of the service requirements or port management policies based on the recommendation, and FAE server will send these policies to the corresponding Devices (FAE clients) and TSN AF/NRz server.

An example for TSN policy negotiation corresponding to port management policies and service requirements is when high jitter is monitored for an ongoing session, the hold & forward buffering parameters may be adapted to de-jitter the TSC flows. However, this will have impact on the survival time, since the delayed reception of the traffic, may lead to service termination (due to reaching the survival time threshold). In this case, FAE server interacts with the TSN system (CNC) for triggering the adaptation of the survival time, in combination with the hold & forward buffering parameters. After negotiating, FAE server provides the new survival time to the FAE client, and the new hold & forward buffering parameters to NW-TT.

#### 7.16.1.2	Procedure

Pre-condition 1: UE is registered to 5GS and FAE client has established a connection with FAE server.

Pre-condition 2: FF application specific server has subscribed to FAE server to receive TSN related triggers.

Pre-condition 3: FAE server has subscribed to the SEAL server / NRx (based on Solution #10 of TR 23.745) or to the PLMN (e.g. QoS monitoring for URLLC as specified in clause 5.33.3 of TS 23.501) for receiving QoS events.

Figure 7.16.1.2-1: TSN Policy update negotiation

1a.	A QoS monitoring event is send from either SEAL/NRx (based on Solution #10 of TR 23.745) or PLMN/NEF to the FAE server based on the subscription (e.g. event related to high jitter for TSC flow).

1b. Alternatively to 1a, a UE related monitoring event, which may relate to a UE related event (e.g. change of UE or group mobility/speed, inter-UE distance change, location reporting) or a application-related event (notification that maximum survival time is reached for the TSC service, high jitter, etc).

2.	FAE server upon receiving the QoS and/or UE-related monitoring event for one or more TSC flows, determines a trigger, which can be a proposed TSN policy update for all or subsets of the TSC flows within the FF application. The trigger may take the form of a new service requirement, QoS requirement or network requirement for the TSC flows, taking also into account the location of the UEs (which can be provided by LM server).

3.	Then, the FAE server sends an abstracted report, enclosing a notification on an experienced/expected change of service /QoS metric based on the received event to FF application specific server / CNC.

4. The FAE server receives a TSN policy update, including the new policy parameters based on the proposed policy parameters. The new policy update may take the form of a new service requirement, QoS requirement or network requirement for the TSC flows. The FAE server may also store the new TSN policies. Here to mention that FAE-S supports the IEEE 802.1Qcc management protocol for receiving the new TSN policies.

5.	The FAE server sends a TSN flow configuration update to the NRz server (acting as TSN AF), if the TSN policy update relates to the update of the QoS policies for the TSN flows. If the TSN policy update relates to NW-TT/DS-TT port management policy update, NRz interacts with NW-TT and DS-TT to apply the new policy (TSN AF to NW-TT/ DW-TT transfer of port or bridge management information, as defined in TS23.501, clause 5.28.3.2). NRz server has been defined in Solution #22 of TR23.745.

6.   The FAE server sends a service requirement update message to the involved FAE clients. This message includes the agreed policy update type and parameters. Then, further interaction with FF application specific client is needed for applying the new parameters (e.g. survival time increase).

### 7.16.2	Solution evaluation

This solution provides a mechanism, involving the FAE, for transforming a monitoring event from the network or UE side, to an abstracted report which can be used by FF application specific server to adapt the TSN policies for one or more devices running TSC services. This solution assumes that FAE-S supports the IEEE 802.1Qcc management protocol. This solution provides an enhancement for pro-actively alerting the CNC on expected performance downgrade, while minimizing the complexity/overhead at the FF application specific server (for processing all related network/UE triggered monitoring events).

Editor’s Note: The compatibility of Steps 3 and 4 with respect to IEEE defined capabilities is FFS.

* * * End of Change * * * *