3GPP TSG-SA WG6 Meeting #43	S6-211147

e-meeting, 24th May – 2nd June 2021	(revision of S6-21xxxx)

Source:	Tencent

Title:	UAV and UAV-C Pairing and C2 QoS Provisioning using Group ID

Spec:	3GPP TS 23.255 v0.4.0

Agenda item:	7.10

Document for:	Approval

Contact:	Shuai Zhao <shuaiizhao@tencent.com>

1. Introduction

This contribution adds the information flows for Procedure 6.3

2. Reason for Change

The support of Information flow is necessary for precedure 6.3

3. Proposal

It is proposed to agree the following changes to 3GPP TS 23.255 v0.4.0

* * * First Change * * * *

### 6.3.3	Information flows

Editor's Note:	This clause will describe the information flow tables

#### 6.3.3.1	Group creation request

Table 6.3.3.1-1 describes the information flow of group creation request from UAE client (one pair of UAV and UAV-C) to UAE server.

Table 6.3.3.1-1: Group creation request

| Information element | Status | Description |
| --- | --- | --- |
| UAE client ID | M | The identifier of the UAE client |
| UAV UE ID | M (NOTE) | The Identifier of UAV UE ID |
| UAV-C UE ID | M (NOTE) | The Identifier of UAV-C UE ID |
| CAA-level UAV ID | O | The CAA-Level UAV ID |
| Timeout | O | The timeout period when group creation response is not received. A request may be sent again after timeout. |
| NOTE: Either of the IEs should be present |  |  |

#### 6.3.3.2	Group creation response

Table 6.3.3.2-1 describes the information flow of group creation response from UAE server to UAE client.

Table 6.3.3.2-1: Group creation response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | The identification of successfulness of UAS (one pair of UAV and UAV-C) group creation. When successful, a UAS group ID shall be provided in the result message. |

#### 6.3.3.3	Group-based approach for C2 QoS provisioning request

Table 6.3.3.3-1 describes the information flow of group-based approach for C2 QoS provisioning from UAE client to UAE server.

Table 6.3.3.3-1: Group-based approach for C2 QoS provisioning request

| Information element | Status | Description |
| --- | --- | --- |
| UAS group ID | M | The identifier of the UAS group (one pair of UAV and UAV-C). |
| C2 Resource requirement | M | The required C2 communication resources (e.g., bandwidth, latency) |

#### 6.3.3.4	Group-based approach for C2 QoS provisioning response

Table 6.3.3.4-1 describes the information flow of group creation response from UAE server to UAE client.

Table 6.3.3.4-1: Group-based approach for C2 QoS provisioning response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | The identification of successfulness of QoS provisioning per a UAS group ID. |

* * * End of Change * * * *