3GPP TSG-WG SA6 Meeting #45 bis	S6-212240

# Online, October 11 – 19, 2021	(revision of S6-210xxxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.289 | CR | XXXX | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | 5G MBS session configuration and service announcement |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CBN,Huawei, Hisilicon, Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | MCOver5MBS |  |  |  |  |  | Date: |  |  | 2021-10-11 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In TR 23.783, the solutions #10, #12-#15 are for session configuration and service announcement. Sol#10 and Sol#12 ~ Sol#15 address how MC service server configure the MBS session to be established and provide the service announcement to the MC service UE. They are technical aligned with each other, but with different documentation fashions. It is concluded in TR 23.783 that Sol#10 will be used as the baseline for the TS. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce Sol#10 of TS 23.783 in TS 23.289. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | There is no description of MBS session configuration and service announcement in current technical specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 7.X (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.228: "IP Multimedia Subsystem (IMS); Stage 2".

[3]	3GPP TS 23.280: "Common functional architecture to support mission critical services; Stage 2".

[4]	3GPP TS 23.281: "Functional architecture and information flows to support Mission Critical Video (MCVideo); Stage 2".

[5]	3GPP TS 23.282: "Functional architecture and information flows to support Mission Critical Data (MCData); Stage 2".

[6]	3GPP TS 23.379: "Functional architecture and information flows to support Mission Critical Push To Talk (MCPTT); Stage 2".

[7]	3GPP TS 23.501: "System architecture for the 5G System (5GS)".

[8]	3GPP TS 23.002: "Network Architecture".

[9]	3GPP TS 23.503: "Policy and Charging Control Framework for the 5G System (5GS); Stage 2".

[10]	3GPP TS 23.502: "Procedures for the 5G System (5GS)".

[11]	3GPP TS 22.179: "Mission Critical Push to Talk (MCPTT); Stage 1".

[12]	3GPP TS 22.280: "Mission Critical Services Common Requirements (MCCoRe); Stage 1".

[13]	3GPP TS 22.281: "Mission Critical (MC) Video".

[14]	3GPP TS 22.282: "Mission Critical (MC) Data".

[xx]	3GPP TS 23.247: "Architectural enhancements for 5G multicast-broadcast services; Stage 2".

# * * * * Second change * * * *

7.X	MC service over 5G MBS

7.X.Y	MBS session configuration and service announcement

7.X.Y.A1	General

When the MC service server decides to use MBS sessions for MC service group communications, the MC service server is first required to configure with the 5GC one or more MBS sessions to deliver the data associated to the MC service group communications.

Service Announcement provides the MC users by MC service server with descriptions specifying the multicast or broadcast services to be delivered as part of MBS Session.

7.X.Y.A2	MBS session configuration

Editor's note:	The term "configuration" can be revisited if SA2 determines to use other names.

As being specified in 3GPP TS 23.247 [xx], the MBS session configuration request includes the MBS session type and required service requirements associated to the MBS session. Among other aspects, the MBS session configuration request may also include MBS service area information, which can be Cell ID list, TAI list or geographical area information.

NOTE 1:	It is implementation specific whether the MC service server decides to use multicast or broadcast MBS sessions for MC service group communications.

When using N29mb, MC service server use the mechanism as defined in being specified TS 23.247 [xx] for MB-SMF discovery and selection.

Moreover, the MC service can decide to configure one MBS session per MC service group. In principle, an MBS session should be sufficient to handle all MC service group communications related to the corresponding MC service group, where different service requirements can be addressed by different MBS QoS flows within the MBS session. For instance, if different required QoS requirements are considered for application level control messages, e.g. floor control messages, and media for an MCPTT group call, a QoS flow can be configured for the application level control messages and another QoS flow for the media.

However, for the case of multicast MBS sessions, the MC service server could also decide to configure more than one MBS session per MC service group to deliver the same MC service data. This can avoid creating a one-to-one mapping between the members of an MC service group and the dedicated set of UEs associated to a multicast MBS session. Hence, the MC service server can hide any sensitive MC service group configuration from the 5GC.

NOTE 2:	It is implementation specific whether the MC service server decides to configure one or multiple MBS sessions to transmit the MC service data associated to an MC service group.

As in eMBMS, the MC service server could also decide to not permanently associated an MBS session to a specific MC service group or MC service group communication. This could particularly be applicable for broadcast MBS sessions where all MC service UEs within a broadcast service area may be able to receive the same broadcasted data. However, for the case of multicast MBS sessions where the same data is only distributed to a specific set of users, the MC service server should, therefore, configure an MBS session per MC service group.

The MC service server can decide to configure MBS sessions based on different conditions. For instance, the MC service server can configure one or multiple MBS sessions based on MC service group status, e.g. when a specific group is active with affiliated users. This can be considered regardless if any MC service group communication has been initiated. The MBS session(s) can later be used by the MC service server for the different MC group communications associated to the specific MC service group. For this case and based on the MC service group configuration, the MC service server can in advance provide expected required service requirements to be associated to the MBS session. Also, the MC service server may dynamically update MBS sessions, e.g., to modify service areas or provide new required service requirements on demand. For this purpose, the MC service server initiates an MBS session update towards the 5GS. As described in 3GPP TS 23.247 [xx], this can either be an MBS session update to the MB-SMF when PCC is not used, or an MBS session configuration update to the PCF when PCC is used.

Likewise, the MC service server can also decide to dynamically configure MBS sessions on demand based on, e.g., the establishment of specific MC service group communications, as defined in 3GPP TS 23.280 [3] for MBMS transmissions.

NOTE 3:	It is implementation specific whether the MC service server decides to configure MBS sessions in a pre-established mode or dynamic mode.

For multicast MBS sessions, as described in 3GPP TS 23.247 [xx], three states have been defined: configured state, active state and inactive state. Based on these states, the MC service server can request the configuration of multicast MBS sessions either in active or inactive state. For instance, the MC service server can request the configuration of an MBS session in inactive state while there are no active group communications associated to a specific MC service group. The MC service server will then need to activate the MBS session. This can occur when MC service data becomes available to be transmitted over the MBS sessions, e.g. when a group communication is established for the corresponding MC service group. Hence, network resources can be handled in a more efficient way.

As part of the MBS session configuration, specific MBS information like MBS session ID and how the data can be provided to be distributed over the MBS session is defined and available at the MC service server.

7.X.Y.A3	MBS service announcement

When an MBS session is configured, the MC service server shall announce the corresponding MBS session(s) to the target MC users via an MBS service announcement. As being defined in 3GPP TS 23.247 [xx], the MBS service announcement shall include, among others, the MBS session ID, session type, SDP information, and any other information required to enable that MC service UEs can receive MC service data over a specific MBS session.

Based on the MBS service announcement the MC service UE can become able to receive MC service data over the MBS session.

Table 7.X.Y.A2-1 specifically describes the MBS service announcement to be sent from the MC service server to MC service clients to enable that MC service UEs can receive MC service group communication data over MBS sessions.

Table 7.X.Y.A2-1: MBS service announcement for the use of MBS session for MC service group communications

| Information element | Status | Description |
| --- | --- | --- |
| MBS session ID | M | The identity of the MBS session used to deliver MC service group communication data. It is either TMGI for broadcast MBS and multicast MBS sessions, or source specific IP multicast address for multicast MBS session |
| MBS session mode | M | Indicate the service type of the MBS session, either a multicast MBS session or a broadcast MBS session |
| MC service group ID | O | Indicate the MC service group ID associated to the MBS session |
| SDP information | M | SDP related to application level control signalling or media to be transmitted over the MBS session (e.g. codec, protocol ID, FEC information, IP address and ports) |
| List of MBS Service Area information | O | For the case of local MBS services, it indicates either multicast service area identifier(s) for multicast MBS session, or broadcast service area identifier(s) for broadcast MBS session |
| Service announcement acknowledgement | O | Indicate if the MC service server requires an acknowledgement to the MBS service announcement |
| Multicast MBS session related information (NOTE 1) | O | Additional information to be used by the MC service client to join the multicast MBS session |
| MBS session UE join notification (NOTE 1) | O | Indicate if the MC service server requires a notification from the MC service client once it has joined the multicast MBS session |
| Monitoring state (NOTE 2) | O | Indicate if the MC service client is required to actively monitor the broadcast MBS session quality and report it to the MC service server |
| NOTE 1: It is applicable for multicast MBS session NOTE 2: It is applicable for broadcast MBS session |  |  |

Editor's note: It is FFS which multicast MBS session related information is required to be included in the MBS service announcement to enable that the MC service client can trigger the MBS session join request towards the 5GS. This depends on the corresponding further specification in 3GPP TS 23.247 [xx].

7.X.Y.A4	Procedure

The procedure in figure 7.X.Y.A3-1 describes how MBS session configuration and MBS service announcement can be used for the transmission of MC service group communication data over either multicast or broadcast MBS sessions.

Pre-conditions:

-	MC service clients 1 to n are attached to the 5GS, registered and affiliated to the same MC service group X.

-	The MC service server has decided to use an MBS session for MC service group communications associated to MC service group X.

-	The MC service server has directly performed (or via NEF/MBSF) an MB-SMF discovery and selection, unless the corresponding information is locally configured.

Figure 7.X.Y.A3-1: MBS service announcement for multicast and broadcast MBS sessions

1.	The MC service server decides to configure a multicast or broadcast MBS session targeting the MC service group X, e.g. based on the affiliation status of the group members. As being specified in 3GPP TS 23.247 [xx], the MBS session configuration procedure is initiated by the MC service server by requesting an MBS session to the 5GC, where the session can either be configured with or without PCC. The MC service server provides, among others, the MBS session type and service requirements, e.g. required QoS, for the MBS session. The MC service server may also indicate how data can be distributed to the 5GC to be transmitted over the MBS session. For the case of a multicast MBS session, the MC service server can indicate the state for the MBS session to be configured. As a response, the MC service server receives the MBS session ID and other parameters associated to the configured MBS session.

2.	The MC service server provides the MC service clients affiliated to MC service group X with the information related to the configured MBS session via an MBS service announcement. As described in table 7.X.Y.A2-1, the service announcement includes information such as the MBS session ID, MBS session mode (broadcast or multicast service type), and SDP information.

For multicast MBS sessions, further information related to the MBS session may be sent to enable that MC service clients join the announced multicast MBS session.

For broadcast MBS sessions, the session announcement may indicate if the MC service server requires the MC service client to actively monitor and report the quality of the announced broadcast MBS session.

3.	MC service clients store and process the received MBS session information.

4.	MC service clients may provide an MBS service announcement acknowledgment to the MC service server to indicate the reception of the corresponding MBS service announcement.

5.	For broadcast MBS sessions, the MC service clients start monitoring the reception quality of the broadcast MBS session. If indicated in the MBS service announcement information, MC service clients report the monitoring state back to the MC service server.

NOTE 1:		It is implementation specific whether the broadcast reception quality level is determined per MBS session, per media stream or per MBS QoS flow level via e.g., measurements of radio level signalling such as the reference signals from the NG-RAN node(s), packet loss.

6.	For multicast MBS sessions, MC service clients initiate a multicast MBS Session UE Join towards the 5GS using the information provided by the MBS service announcement.

7.	The MC service clients provide a notification related to the announced MBS session. For the case of multicast MBS sessions, the clients send a multicast MBS session UE join notification towards the MC service server to indicate that the announced MBS session has been joined successfully. For the case of broadcast MBS sessions, the clients send a notification to the MC service server to indicate the monitoring status of the announced MBS session, e.g., similar to the listening status report used in MBMS.

# * * * * End of changes * * * *