3GPP TSG-WG SA6 Meeting #45-bis-e	S6-212xxx

# e-meeting, October 11 – 19, 2021	(revision of S6-212327)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.289 | CR | 0004 | rev | 1 | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | MBS session release |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | MCOver5MBS |  |  |  |  |  | Date: |  |  | 2021-10-05 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In TR 23.783, the Sol#24 and Sol#28 address the MBS session release and de-announcement aspect when the MBS session is no longer required. The MBS session release is supported by the following clauses/procedures in TS 23.247. Clause in 23.247 (v 17.0.0); Analysis. 7.1.1.4 Removal of MBS session configuration without PCC; No open issues nor editor’s note in this clause.. 7.1.1.5 Removal of MBS session configuration with PCC; No open issues nor editor’s note in this clause.. 7.2.2.3 SMF removing joined UEs from MBS session; No open issues nor editor’s note in this clause.. 7.2.2.4 Release of shared delivery toward RAN node; No open issues nor editor’s note in this clause.. 7.3.2 MBS Session Release for Broadcast; No open issues nor editor’s note in this clause.. As shown above, the MBS session release feature and procedures are stable without any opening issue and editor’s note in latest 23.247 v17.0.0, which allows the SA6 to make conclusion for the corresponding aspect in TR 23.783. As a result, it is proposed to also add the proposal in the conclusion to TS 23.289. |
| Summary of change: |  | Introduce Sol#24 and Sol#28 of TS 23.783 in TS 23.289. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | There is no description of MBS session release in current technical specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.x (new), 7.x.y (new), 7.x.y.1 (new), 7.x.y.1 (new), 7.x.y.2 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev1: The de-announcement procedure is used for service trigger UE leave, therefore in rev1 this procedure is moved from this contribution to the revision of S6-212328. |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

## 7.X	MC service over 5G MBS

### 7.X.Y	MBS session release

##### 7.X.Y.1	General

The MC service server can decide to release a certain MBS session, once it is no longer further utilized for the associated MC service group communication, e.g., the MC service group is no longer active, the MC media transmission is over and no further MC data to be delivered. The MBS session release procedure leads to releasing the network resources associated to that MBS session.

To release the MBS session, the MC service server sends an MBS session release request to the 5GS providing the corresponding MBS session ID. The MBS session release request is sent to the MB-SMF (directly or via NEF/MBSF) when PCC is not used. Further details are provided in 3GPP TS 23.247 [xx]. MC service server further informs the MC service client with the MBS session de-announcement, so that the MC client UE stops monitoring the broadcast MBS session or leave the multicast MBS session.

This procedure is applied for both broadcast MBS session and multicast MBS session.

##### 7.X.Y.2	Procedure

The procedure in figure 7.X.Y.2-1 describes the MBS session release aspects for group communication.

Pre-conditions:

-	MC service clients 1 to n are attached to the 5GS, registered and affiliated to the same active MC service group.

-	An MBS session is configured to address the corresponding MC service group with certain service requirements and optionally with a certain broadcast/multicast service area. The session is announced and established for group communication purposes for the MC service group.

Figure 7.X.Y.2-1: MBS session release for MC group communication.

1.	The MC service server decides to stop using the MBS session for the associated MC group communication, either multicast or broadcast session.

2.	The MC service server initiates the MBS session release procedure with the 5GC (either directly or through NEF/MBSF) in order to stop using the configured MBS session. The MC service server indicates within the MBS session release request the corresponding MBS session ID. Hence, the associated network resources are released. The MBS session release procedures are performed according to 3GPP TS 23.247 [xx] clause 7.1.1.4 and clause 7.1.1.5.

3.	For multicast MBS session, if the 5GS has identified any MC service UE(s), which have a joined session status towards the mentioned MBS session, the 5GC may remove the MC service UE(s). The UE removing procedure by the 5GC is described in 3GPP TS 23.247 [19].

# * * * * End of changes * * * *