3GPP TSG-WG SA6 Meeting #45-bis-e	S6-212xxx

# e-meeting, October 11 – 19, 2021	(revision of S6-212330)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.289 | CR | 0007 | rev | 1 | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | MC service media distribution over 5G MBS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | MCOver5MBS |  |  |  |  |  | Date: |  |  | 2021-10-05 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | MC service media distribution focuses on how media related to MC service group communications with different service requirements can be transmitted over multicast or broadcast MBS sessions. And such feature is considered as the basic feature for MC service. This CR is to add the related procedure for MC service media distribution over 5G MBS for a group communication, based on TR conclusion on this topic. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Adding general description about Selection of multicast and broadcast service for a group communication. - Adding general call flow about MC service media distribution over 5G MBS. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No related content about supporting MC service media distribution over 5G MBS. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.X (new), 7.x.y (new), 7.x.y.1 (new), 7.x.y.2 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

## 7.X	MC service over 5G MBS

### 7.X.Y	MC service group media transmissions over 5G MBS sessions

#### 7.X.Y.1	General

The MC service server can decide to configure an MBS session per MC service group to transmit the media related to the corresponding MC service group communications. Such group communications can comprise different service requirements. For that, multicast and broadcast MBS sessions need to be configured with multiple MBS QoS flows to address different service requirements, e.g. different required QoS, provided by the MC service server. For instance, application-level control messages or media associated to a group communication can comprise different QoS requirement. Also, different type of group communications can comprise different QoS requirements, e.g. emergency group calls should be handled with a higher priority than normal group calls.

Editor's Note:	The terms "Configured" and "Configuration" will be align with SA WG2.

The configuration of multiple MBS QoS flows to address different service requirements is associated to the assignment of different streams (e.g. different ports) within an MBS session.

Based on the functionalities being specified in 3GPP TS 23.247 [xx], the MC service server can request the configuration of an MBS session providing in advance required service requirements. However, dynamically, the MC service server should also be able to provide additional service requirements for a configured MBS session, e.g., based on the establishment of an MC service group communication with a specific required QoS.

Furthermore, the MBS session establishment procedure depends on the MBS session mode, i.e., whether the MBS session is of a broadcast or a multicast type. In case of broadcast MBS sessions, the session is established as part of the session configuration procedure where the NEF/MBSF initiates an MBS session start request towards the MB-SMF in order to reserve the network resources.

NOTE 1: The MC service server allocates an MBS session ID once (via TMGI allocation request towards NEF/MBSF as part of the MBS session configuration procedure), but the MBS session of type broadcast can start multiple times as indicated in 3GPP TS 23.247 [xx] upon request to start MC media transmission.

On the other hand, a multicast MBS session is established when the first UE session join request being sent from an MC service client towards the 5GS is being accepted. The UE session join request is sent by the MC service UE as a NAS message in the form of a PDU session modification request towards the 5GS. The message indicates the MBS session ID(s) which the MC service UE is requesting to join, as discussed in 3GPP TS 23.247 [xx]. The established multicast MBS session can either have an active or inactive state, where the former indicates the activation of radio resources hence transmitting the MC media to the associated MC service group, and the latter indicates their deactivation as no MC media is being transmitted. The MC service server may trigger the activation of multicast MBS sessions once the MC service group is established and active, as well as once the MC media is available for transmission. For this purpose, the MC service server sends a multicast MBS session activation request towards the 5GC indicating the MBS session ID to be activated.

Similar to the use of eMBMS, the MC service server shall provide the associated information between a specific group communication and the stream to be used within an MBS session. This information could be sent in advance over the MBS service announcement or should be provided on demand over an additional signalling message over the MBS session, e.g., like the MapGroupToBearer in eMBMS. For that, table 7.X.Y.1-1 defines the MapGroupToSessionStream to be sent from the MC service server to MC service clients to provide specific required information to receive the media related to a group communication within an MBS session.

Table 7.X.Y.1-1: MapGroupToSessionStream

| Information element | Status | Description |
| --- | --- | --- |
| MC service group ID | M | This element identifies the MC service group related to a group communication to be delivered over the MBS session |
| Media stream identifier | M | This element identifies the media stream of the SDP used for the group communication within the MBS session |
| MBS session ID | O | The MBS session identifier if the MapGroupToSessionStream message is not sent on the same session as the MC media |

#### 7.X.Y.2	Procedure

The procedure in figure 7.X.Y.2-1 describes how media related to a specific group communication can be distributed over a configured MBS session which consist of multiple QoS flows, i.e. addressing different service requirements.

Pre-conditions:

-	MC service clients 1 to n are attached to the 5GS, registered and affiliated to the same MC service group X.

-	The MC service server has decided to use an MBS session for MC service group communications associated to MC service group X.

-	MC service clients 2 to n are within the MBS service area where the MBS session is configured.

Figure 7.X.Y.2-1: MC service group media transmission over MBS sessions

1.	The MC service server configures a multicast or broadcast MBS session targeting group communications associated to MC service group X, as being specified in 3GPP TS 23.247 [xx]. Therefore, the MC service server can provide default service requirements to be addressed by the MBS session, e.g. associated to MC 5QIs and specific allocation and retention priority (ARP) to transmit the media associated to MC service group communications.

The MBS session is announced and received by MC service client 2 to n, which are within the MBS service area. The MC service server has identified that MC service clients 2 to n can receive media over the MBS sessions, e.g. based on a notification from the MC service clients indicating the successful join of the multicast MBS session or a monitoring report of the broadcast MBS session (similar to the listening status report used for MBMS).

2.	A new MC service group communication is established for the MC service group X consisting of a specific required QoS, e.g. an MC service emergency group communication. The group communication setup can be done over unicast.

2a. For broadcast MBS sessions, the session is established upon sending a session start request as part of the MBS session configuration procedure, which is described in 3GPP TS 23.247 [xx].

2b. For multicast MBS session, the session is established upon the acceptance of the first UE session join request initiated from the MC service UE towards the 5GS, as described in 3GPP TS 23.247 [xx]. The multicast session can then have either active or inactive state.

3. 	The MC service server may send a multicast MBS session activation request towards the 5GC in order to activate the multicast MBS session in case the session has an inactive state. For this purpose, the MC service server indicates the MBS session ID to be activated.

4.	Considering that the established group communication requires a specific QoS, e.g. an MC service emergency group communication which requires higher priority (i.e. better ARP), the MC service server requests an MBS session update to the 5GS to provide the new required QoS, if not done during the MBS session configuration in step 1. The MBS session configuration should then be updated and an additional QoS flow may be configured.

5.	The MC service server sends a MapGroupToSessionStream to MC service clients 2 to n over the configured MBS session providing the required stream information to receive the media related to the specific established MC service group communication within the MBS session.

6.	MC service clients process the MapGroupToSessionStream information to receive the related media over the specific MBS session stream.

7.	MC service client 1 sends media to the MC service server over unicast to be distributed for the established group communication.

8.	The MC service server distributes the media to MC service clients 2 to n over the indicated stream within the established MBS session.

9.	The MC service server may send a multicast MBS session deactivation request towards the 5GC in order to deactivate the multicast MBS session. For this purpose, the MC service server indicates the MBS session ID to be deactivated.

10.	The MC service server may further trigger the UE to leave multicast MBS session, see clause 7.x.y.

# * * * * End of changes * * * *