3GPP TSG-SA WG6 Meeting #46-e	S6-212694

e-meeting, 15th – 23rd November 2021	(revision of S6-21xxxx)

Title:	LS on ad hoc group communication

Response to:

Release:	Release 18

Work Item:

Source:	3GPP TSG SA WG6

To:	3GPP TSG SA WG1

Cc:

Contact person:	Ling Zhang

zhangling@catt.cn

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	S6-212xxx

1	Overall description

A new study has been proposed in SA6 on Ad hoc Group Communication Support for Mission Critical Services based on requirements for Ad hoc group communication in 3GPP TS 22.280.

SA6 identifies that the following aspects need to be clarified by SA1 requirements such that the gaps between ad hoc group and the existing MC service group can be captured:

Q1: Is the Group ID required for MCX Service or an authorized MCX user to establish Ad hoc group communication? Whether the MCX Service is required to enable the MCX users participating in the Ad hoc group communication to receive the Ad hoc group related information (e.g. Group ID) prior to or during the Ad hoc group communication?

Q2: 3GPP TS 22.280 sub clause 6.15.5.2 and 6.15.5.3 capture the below requirements:

[R-6.15.5.2-004] The MCX Service shall provide a mechanism for an MCX Service Administrator to configure additional conditions under which MCX Service Ad hoc Group Communication shall be terminated (e.g., last Participant leaving, second last Participant leaving, initiator leaving).

[R-6.15.5.3-001] The MCX Service shall provide a mechanism for an MCX Service Administrator to configure which MCX Users, within their authority, are authorized to initiate a MCX Service Ad hoc Group Communication.

[R-6.15.5.3-002] The MCX Service shall provide a mechanism for an MCX Service Administrator to configure the maximum number of MCX Users who can participate in a MCX Service Ad hoc Group Communication.

[R-6.15.5.3-003] The MCX Service shall provide a mechanism for an MCX Service Administrator to configure which MCX Users are authorized to participate in a MCX Service Ad hoc Group Communication. [TS 22.280 R-6.7.2-003]

[R-6.15.5.3-004] The MCX Service shall provide a mechanism for an MCX Service Administrator to define the default parameters for MCX Service Ad hoc Group communication (e.g., priority, hang time, broadcast mode).

[R-6.15.5.3-005] The MCX Service shall provide a mechanism for an MCX Service Administrator to configure whether MCX Service Ad hoc Group Communication is allowed on the MCX system regardless of individual MCX User authorizations.

Clarify whether the MCX Service is required to enable the MCX users participating the Ad hoc group communication to receive the above configurations or default parameters prior to or during the ad hoc group communication, and whether the MCX Service is required to provide configurations for Ad hoc group communication  differently than providing the normal group configurations for other type of MC service group communication.

SA6 kindly ask SA1 to clarify the above questions raised by the proposed study.

2	Actions

To SA1 group

ACTION: 	SA6 kindly asks SA1 to reply to this LS.

3	Dates of next TSG SA WG 6 meetings

SA6#47-e          14th – 22nd Feb 2021 	e-meeting

SA6#48-e          4th – 8th April 2021 	TBC