3GPP TSG-SA WG6 Meeting #49-bis-e	S6-221931

e-meeting, 22nd June – 1st July 2022	(revision of S6-221671)

Source:	Qualcomm

Title:	Pseudo-CR on Concluding architecture enhancement option#4

Spec:	3GPP TR 23.700-98 v1.0.1

Agenda item:	9.8

Document for:	Approval

Contact:	Nishant Gupta <gnishant@qti.qualcomm.com>

1. Introduction

This contribution concludes architecture enhancement option 4.

2. Reason for Change

The architecture enhancement captured as option#4 introduces support for TEs in the EdgeApp architecture. With this enhancements ACs running on a TE will be able to communicate with the EEC running on a UE to avail services of the EEC over EDGE-5 interface. No specific enhancement over EDGE-5 is identified during study to support this functionality.

How to depict the architecture enhancement needs to be discussed during normative work, and therefore the related EN is removed.

3. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-98 v1.0.1.

* * * First Change * * * *

## 6.4	Option #4: Constrained devices with limited capabilities

### 6.4.1	Architecture enhancements

#### 6.4.1.1	General

This architecture option adds support for certain constrained devices which either don't have enough capabilities to execute its own EEC (e.g., the constrained device may not have a Mobile Termination entity) or do not execute its own EEC to save essential resources such as processing power and battery. Such constrained devices (e.g., terminal equipments as defined in TR 22.944 [21]) will benefit by being able to utilize services of an EEC running on a different UE, using EDGE-5.

#### 6.4.1.2	Architecture

Figure 6.4.1.2-1: Architecture supporting constrained devices

NOTE 1:	The architecture option should utilize existing functionalities specified by SA2 and avoid creating new requirements for system architecture.

NOTE 2:	How the TE communicates with the UE (e.g., Bluetooth, Wi-Fi, Cellular, ProSe etc.) is out of scope of SA6.

* * * Next Change * * * *

## 10.1	Architecture enhancements

### 10.1.x	Support for Terminal Equipment (TE)

The architecture enhancement captured as option#4 introduces support for TEs in the EdgeApp architecture. The interface between the TEs and the UEs running the EEC is out of scope of SA6.

With this enhancements ACs running on a TE will be able to communicate with the EEC running on a UE to avail services of the EEC over EDGE-5 interface. No specific enhancement over EDGE-5 is identified during the study to support this functionality.

* * * Next Change * * * *

### 11.2.2	Architecture enhancement conclusions

The study concludes with following architectural enhancements considerations for the normative work:

1.	Architecture enhancements from clause 6 corresponding to the concluded solutions will be used for updating baseline edge application architecture specified in 3GPP TS 23.558:

a.	architecture corresponding to solution #x as specified in clause 6.x

x.	architecture enhancement as specified in clause 6.4

* * * Next Change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TR 23.558: "Architecture for enabling Edge Applications".

[3]	ETSI ISG MEC ETSI GS MEC 003, "Multi-access Edge Computing (MEC); Framework and Reference Architecture".

[4]	GSMA OPG.02: "Operator Platform Telco Edge Requirements", https://www.gsma.com/futurenetworks/wp-content/uploads/2021/07/GSMA-OPG-Telco-Edge-Requirements-2021.pdf

[5]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[6]	3GPP TR 23.758: "Study on application architecture for enabling Edge Applications".

[7]	3GPP TR 23.721: "Study on Sponsored Data Connectivity Improvements".

[8]	3GPP TS 23.502: "Procedures for the 5G System; Stage 2".

[9]	3GPP TS 23.203: "Policies and Charging control architecture; Stage 2".

[10]	3GPP TS 23.682: "Architecture enhancements to facilitate communications with packet data networks and applications".

[11]	3GPP TS 23.503: "Policies and Charging control architecture; Stage 2".

[12]	3GPP TS 22.261: "Service requirements for the 5G system; Stage 1".

[13]	ETSI GS MEC 010-2: "Multi-access Edge Computing (MEC); MEC Management; Part 2: Application lifecycle, rules and requirements management".

[14]	ETSI GS MEC 011 v3.0.5: " Multi-access Edge Computing (MEC); Edge Platform Application Enablement".

[15]	ETSI GS MEC 001: " Multi-access Edge Computing (MEC); Terminology ".

[16]	3GPP TS 23.222: "Common API Framework for 3GPP Northbound APIs; Stage 2".

[17]	3GPP TS 29.522: "Network Exposure Function Northbound APIs; Stage 3".

[18]	3GPP TS 29.122: "T8 reference point for Northbound APIs".

[19]	3GPP TS 23.548: "5G System Enhancements for Edge Computing; Stage 2".

[20]	3GPP TS 28.538: "Management and orchestration; Edge Computing Management".

[21]	3GPP TR 22.944: “Report on service requirements for UE functionality split”.

[x]	<doctype> <#>[ ([up to and including]{yyyy[-mm]|V<a[.b[.c]]>}[onwards])]: "<Title>".