3GPP TSG-SA WG6 Meeting #50-e	S6-222449

E-meeting, 22th – 31th August 2022	(revision of S6-222133)

Source:	CATT

Title:	Pseudo-CR on Solution#6 update

Spec:	3GPP TR 23.700-96 v0.6.0

Agenda item:	9.7

Document for:	Approval

Contact:	Liping Wu, liping.wu@cictmobile.com

1. Introduction

This contribution updates the solution#6: Location service configuration.

2. Reason for Change

According to the new architecture of KI#1, the fused location client has been removed and replaced by location management client, and the LM-Uu for non-3GPP access interface is defined to support interaction between the target UE and fused location function which is part of location management server, so the location service configuration between them needs to be updated accordingly.

3. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-96 v0.6.0.

* * * First Change * * * *

7.6	Solution #6: Location service configuration

7.6.1	Solution description

This solution addresses key issue #5: Initialization and configuration for fused location service.

7.6.1.1	Service flow for fused location service configuration

The high-level service flow for fused location service configuration is illustrated in figure 7.6.1.1-1. This service flow is based on the fused location architecture of solution#1.

Figure 7.6.1.1-1: Service flow for fused location service configuration

1.	The location service is initiated at the Fused Location Function(FLF) within Location Management Server(LMS). This can be triggered by an application through e.g. a service request or triggered by an event, by which the location service requirements for a target UE are identified.

2.	The FLF may query the location capability of the target UE, e.g. the location system or location service, or location methods supported by the UE. The FLF may need to decide the location source from which to receive location information based on the location service requirements as well as the target UE's location capability.

3a.	The FLF may invoke the LCS service e.g. 5GC-MT-LR Procedure (as defined in 3GPP TS 23.273 [4]) by acting as AF or LCS client.

3b.	The FLF may invoke the SUPL service from the 3rd party location server ,e.g. network initiated flows (as defined in OMA AD SUPL [10]) by acting as the SUPL agent.

3c	. If the FLF decides to receive location from the 3rd party location system, the FLF configures the location service with the 3rd party location server to establish the secured data connection and session for location reports, and to support the location system configuration tailored for the location service requirements.

3d. The LMS interacts with the location management client of the target UE to provide the location service configurations including information about the fused location configuration.

7.6.2	Solution evaluation

This solution is based on the architecture proposed in the new solution of KI#1. The Fused Location Function (FLF) which is part of Location Management Server (LMS) can acquire different location information from multiple resources. For example, from 3GPP access (LCS location retrieved from either NEF or GMLC, etc) , non-3GPP access(target UE location retrieved via LM-UU), or the 3rd party location server (SUPL location retrieved from SLP, etc) to provide an accurate UE location. Besides, the FLF used as an additional source of the LMS can configure the location service with the 3rd party location server and communicate with the location management client of the target UE to inform the location service configurations.