3GPP TSG-SA WG6 Meeting #50-e	S6-222552

e-meeting, 22nd – 31st Aug 2022	(revision of S6-222098)

Source:	Ericsson

Title:	Solve ENs in sol#30

Spec:	3GPP TR 23.700-98 v1.1.1

Agenda item:	9.8

Document for:	Approval

Contact:	Wenliang Xu; wenliang.xu@ericsson.com

1. Introduction

This contribution solves ENs in solution #30.

2. Reason for Change

This solution addresses the EN related to ACR for solution #30. When ACR happens the same discovery mechanism in the same solution can be re-used.

1. For solving issue about fairness of all UE group members, it is a best effort approach.

Noting that latency sensitive application may have strict requirement (RTT < 200ms) for latency to ensure the relative fairness to all group members so that high latency UE (with RTT = 300ms) may be rejected to join the group. Or high latency UE can join the group if the server has lag compensation function. Of course, this compensation has certain threshold limitation (e.g. 100ms). If the high latency UE joins with group with 400ms RTT, user using the application may have bad experience (e.g. application server can emulate AC’s behavior and probably decide events that seem strange to the high latency UE).

The EAS can measure the RTT for each connected AC with consideration of the user plane routing optimization in CN and DN so that the EAS can estimate its service area during deployment stage to ensure relative fairness for all ACs distributed in the EDN (e.g. > 95% ACs having RTT < 200ms).

With proper dimensioning of service area, there is no impact in EEL so that the 1st EN can be solved.

2.

For binding update/removal, it may be part of the ACR procedure or termination of app session.

The solution evaluation is provided, too.

3. Conclusions

<Conclusion part (optional)>

4. Proposal

It is proposed to agree the following changes in TR 23.700-98.

* * * First Change * * * *

### 7.30.2	Solution description

This solution addresses KI#17.

The UEs can be grouped together to consume EAS services on the same EAS endpoint.

The group information (e.g. group id) can be used as part of the binding information to support anchor those UEs to the same EAS. The binding information are maintained on a Binding Server (BS). When an EES is aware of the selected EAS (e.g. via EEC sent EAS selection declaration), the BS is contacted by the EES and the BS can decide whether to proceed the currently selected EAS or instruct to use another EAS (which already has other established session(s) for the group).

The EDN id is used to identify EDN and is also part of the binding information. Figure 7.30.2-1 show the detailed procedure with a Central Binding Server (CBS) for common EAS pre-selection in EES.

In the following figures, ACs with same AC ID on different UEs is shown as example, it is also possible that ACs with different AC IDs supplied by the same AC provider (e.g. AC developed for iphone and AC developed for Android phone) need to communicate with each other via EAS. It is assumed that the CBS can be accessed by all EESs deployed in different EDNs.

Pre-conditions:

1.	The EEC is aware of the group info from AC via EDGE-5 reference point.

Figure 7.30.2-1: select a common EAS for UEs, common EAS pre-selection in EES

In EDN1:

1.	During EAS discovery, EEC 1 sends EAS discovery request to EES 1 including EAS ID, UE ID (of UE1), AC ID (of AC1) and group info (e.g. group id x).

2.	The EES 1 pre-selects an EAS and sends group binding request message to CBS with the received information from EEC 1 and in addition the EDN id (of EDN 1).

3.	There is no existing binding information found so that the CBS creates a new binding information for the UE group.

4.	The CBS responds group binding request with a result indicating OK to proceed with the requested EAS.

5-7.	With the received EAS information, the EES 1 responds the EAS discovery request with the selected EAS to the EEC 1 and consequently AC1 connects to the selected EAS after being informed by EEC 1.

In the same EDN (i.e. EDN1):

8.	During EAS discovery, EEC 2 sends EAS discovery request to EES 2 with the selected EAS information including EAS ID, UE ID (of UE2), AC ID (of AC1) and group info (e.g. group id x).

9.	The EES 2 pre-selects an EAS and sends group binding request message to CBS with the received information from EEC 2 and in addition the EDN id (of EDN 1).

10.	The CBS finds existing binding information for the UE group based on group info.

11.	The CBS responds with a result indicating recommendation to use common EAS (including the associated EES info) which is EES 1 selected EAS.

12-14.	With the received EAS information, the EES 2 responds the EAS selection to the EEC 2 and consequently AC1 connects to the common EAS. EEC 2 may contact EES 1 for subsequent procedures (e.g. EEC registration, ACR subscription).

NOTE:	It is assumed that within an EDN (with proper dimensioning for the service area), the EAS service experience is almost the same for all ACs distributed in the EDN. For instance, the EAS can measure the RTT for connected AC with consideration of the user plane routing optimization in CN and DN so that the EAS can estimate its service area during deployment stage to ensure relative fairness for all ACs distributed in the EDN (e.g. > 95% ACs having RTT < 200ms).

The binding update may be done for a common EAS when:

1)	EAS service continuity (e.g. a new common EAS is selected by the current EAS due to load re-balancing reason in the same EDN) happens. In this case, the EES receives the T-EAS discovery from EEC or S-EAS or triggers T-EAS discovery by itself, then the EES pre-selects T-EAS and update the binding information in the CBS.

2)	AC in a group terminates the application session. In this case, the EES is informed by EAS/EEC about the session termination and updates the binding information in the CBS for the terminated AC. The group binding for the common EAS still remains the same.

The binding information in the CBS is created upon the 1st AC’s need to discover an EAS, therefore, when the last AC in a group terminates the application session, the binding information for the group is removed in the CBS.

For common EAS pre-selection in the UE, the procedure is similar as Figure 7.30.2-1 with the difference that the (T-)EAS pre-selection is done by the EEC.

7.30.3	Solution evaluation

This is a viable solution addressing the KI#17. This solution proposes a new entity CBS to maintain the selected common EAS which is determined by the 1st UE's activity in a group in an EDN and has impact to the (T-)EAS discovery procedure. Both initial EAS discovery and subsequent T-EAS discovery in ACR are covered by this solution.

This solution relies on group information (e.g. group id) conveyed from the AC to the EEC. As a pre-condition, the group is formed before EEC triggers EAS discovery. A pre-configured group can take advantage of this solution to enable common EAS selection.

Editor's note:	Whether the solution can also be used for dynamic group is FFS.

For fairness of all involved ACs in a group to communicate with common EAS, this solution has assumption that the service area is properly dimensioned during EAS deployment so the EAS can ensure good and fair service experience for the ACs within its service area.

* * * End of Changes * * * *