3GPP TSG-SA WG6 Meeting #51-e	S6-222903

e-meeting, 10th – 19th October 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.434 | CR | 0127 | rev | 1 | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Enhance the APIs of the network slice adaptation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | eSEAL2 |  |  |  |  |  | Date: |  |  | 2022-09-28 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Enhanced network slice adaptation were discussed and captured in solution #10 and solution #15, TR 23.700-99. The enhancement of information flows of network slice adaptaion request triggered by VAL Sever and network slice adptation trigger triggered by NSCE Client are required. Besides the step3 in figure 16.3.2.4-1 is missing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Enhance the information flows of network slice adaption. Add the step3 in figure 16.3.2.4-1 and APIs of network slice trigger. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Network slice adpation based on NSCE determination or UE policies is not supported. Step3 in figure 16.3.2.4-1 is missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 16.3.2.2, 16.3.2.3, 16.3.2.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

16.3.2.2	Information flows

16.3.2.2.1	Network slice adaptation request

Table 16.3.2.2.1-1 describes the information flow network slice adaptation request from the VAL server to the NSCE server.

Table 16.3.2.2.1-1: Network slice adaptation request

| Information element | Status | Description |
| --- | --- | --- |
| VAL service ID | M | The VAL service ID of the VAL application for which the network slice adaptation may corresponds to. |
| List of VAL UE IDs | M | List of the VAL UE IDs within the VAL service for which the slice adaptation request corresponds |
| Requested slice information | O | Indication of the new slice which is requested. The slice information can be either the S-NSSAI or the ENSI (as specified in TS 33.501, for AF outside 3GPP MNO domain). |
| Requested DNN | O | Indication of the new DNN which is requested. |
| Requested Adaptation threshold | O (NOTE 1) | The threshold of network slice adaptation |
| >Requested adaptation threshold of the delay of network slice | O | The network slice delay defined clause 5.1.1.1, 5.1.1.2, 5.1.1.3, in TS 28.552[y] and key performance indicators in clause 6.3, in TS 28.554 [z], and network slice related analytics report in clause 7.2.2.2, clause 7.2.2, in TS 28.104[n]. |
| >Requested adaptation threshold of network resources | O | The network slice resource usage status defined in clause 5.1.1.2.5, 5.1.1.2.7, 5.1.1.2.9, 5.1.1.2.10, in TS 28.552[y]. |
| NOTE 1: The UE is requested to adapt to the objective network slice when the current network slice status crossed the requested threshold, also the UE is not to access to the objective network slice when the objective network slice status crossed the requested threshold. |  |  |

16.3.2.2.2	Network slice adaptation response

Table 16.3.2.2.2-1 describes the information flow network slice adaptation response from the NSCE server to the VAL server.

Table 16.3.2.2.2-1: Network slice adaptation response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Result includes success or failure of the network slice adaptation with the underlying network. |

16.3.2.2.3	Network slice adaptation trigger

Table 16.3.2.2.3-1 describes the information flow Network slice adaptation trigger from the NSCE client to the NSCE server.

Table 16.3.2.2.3-1: Network slice adaptation trigger

| Information element | Status | Description |
| --- | --- | --- |
| VAL UE ID(s) | M | The VAL UE ID(s) within the VAL service, for which the network slice adaptation trigger applies |
| VAL service ID | M | The VAL service ID of the VAL application for which the network slice configuration may corresponds to. |
| Requested S-NSSAI | M | Indication of the new S-NSSAI which is requested. |
| Requested DNN | O | Indication of the new DNN which is requested. |
| Request application requirements | O | The application-related request parameters |
| >Requested time window | O | Indication of the new scheduled time window that is requested |
| >Requested location criteria | O | Indication of the new location criteria that is requested |
| >Requested access type reference | O | Indication of the new access type (3GPP, non-3GPP or multi-access) preference that is requested. |
| >Requested associated group id | O | Indication of the group id that is requested |
| >Requested UE IP address preservation indicator | O | Indication that UE IP address preservation is requested |

16.3.2.2.4	Network slice adaptation notification

Table 16.3.2.2.4-1 describes the information flow network slice adaptation notification from the NSCE server to the NSCE client and optionally to the VAL client.

Table 16.3.2.2.4-1: Network slice adaptation notification

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Result includes success or failure of the network slice adaptation. |

* * * Second Change * * * *

16.3.2.3	Procedure for VAL server-triggered and network-based network slice adaptation for VAL application

Figure 16.3.2.3-1 illustrates the VAL server-triggered and network-based procedure where the NSCE server supports the network slice adaptation with the underlying 3GPP system for the VAL UEs of the VAL application.

Figure 16.3.2.3-1: Network slice adaptation for VAL application

1.	The VAL server sends a network slice adaptation request to the NSCE server for the VAL application (and the VAL UEs within the VAL application). This request may be in the form of exact requested network slice (and optionally DNN) for all the VAL UEs of the VAL application; or indication that the VAL application needs to be remapped to a different network slice (and optionally DNN). The request optionally includes the adaptation threshold of network slice adaptation as defined in Table 16.3.2.2.1-1.

- NSCE server collects the network slice status information, including network slice performance measurements in clause 5.1.1.1, clause 5.1.1.2, clause 5.1.1.3, in TS 28.552[y] and key performance indicators in clause 6.3, in TS 28.554 [z], and network slice related analytics report in clause 7.2.2.2, clause 7.2.2, in TS 28.104[n] from network slice management functions by utilizing MnS of create MOI operation defined in clause 11.1 and MnS of streaming data reporting service or file data reporting service defined in clause 11.5 and 11.6, TS 28.532[x].

3.	The NSCE server processes the request and analyses the network slice status information collected in step2. If the threshold is crossed for the current network slice of adaptation and the objective network slice satisfy the requests, NSCE server triggers the network slice configuration per VAL UE within the VAL Application.

4.	The NSCE server acting as AF provides the updated S-NSSAI and DNN per VAL UE. In particular, NSCE server sends this information to the PCF via NEF as part of the AF-driven guidance for URSP determination to 5G system (as specified in TS23.502 clause 4.15.6.10, TS 23.503 clause 6.6.2.2, TS 23.548 clause 6.2.4). This guidance may update the route selection parameters to indicate different sets of PDU Session information (DNN, S-NSSAI) that can be associated with applications matching the application traffic.

NOTE:	NSCE server provides the updated S-NSSAI/DNN as a suggestion/guidance to PCF; however it is up to PCF to decide whether to perform the slice/DNN re-mapping

5.	Upon successful adaptation of the route selection parameters, the NSCE server provides a network slice adaptation response to the VAL server, providing information on the fulfilment of the network slice adaptation request per VAL application.

* * * Third Change * * * *

16.3.2.4	Procedure for VAL UE-triggered and network-based network slice adaptation for VAL application

Figure 16.3.2.4-1 illustrates the VAL UE-triggered and network-based procedure where the NSCE server supports the network slice adaptation with the underlying 3GPP system for the VAL UEs of the VAL application.

Pre-condition:

-	The NSCE client has connected to the NSCE server;

Figure 16.3.2.4-1: Network slice adaptation for VAL application

1.	The VAL client provides a new application requirement to the NSCE client, indicating a new service profile for the VAL application. This may be in the form of a change at the application requirements, including application QoS requirements, location requirements, time window requirement, access type preference(e.g., 3GPP, non-3GPP or multi access) service operation change, or other application-related parameters.

2.	The NSCE client sends a network slice adaptation trigger to the NSCE server for the VAL application. This trigger may be in the form of exact requested network slice (and optionally DNN) for the VAL UE of the VAL application; or indication that the VAL application needs to be remapped to a different network slice (and optionally DNN). The trigger may also include additional application requirements based on step1, e.g., the requested location criteria, time window.

NOTE 1：How the requested network slice is known by the NSCE client is out of scope of this release.

3.	The NSCE server processes the request and triggers the network slice configuration per VAL UE within the VAL Application.

NOTE 2：Whether and how the NSCE server triggers the network slice adaptation for all the VAL UEs within the VAL Application is out of scope of this release.

4.	The NSCE server acting as AF provides the updated S-NSSAI, application requirements and DNN per VAL UE. In particular, NSCE server sends this information to the PCF via NEF as part of the AF-driven guidance for URSP determination to 5G system (as specified in TS23.502 clause 4.15.6.10, TS 23.503 clause 6.6.2.2, TS 23.548 clause 6.2.4). This guidance may update the route selection parameters to indicate different sets of PDU Session information (DNN, S-NSSAI, application requirements) that can be associated with applications matching the application traffic. 5GC uses this information to update the URSP to the affected UE(s).

NOTE 3:	NSCE server provides the updated S-NSSAI/DNN as a suggestion/guidance to PCF; however it is up to PCF to decide whether to perform the slice/DNN re-mapping

5. The NSCE server may send a notification to the NSCE client indicating success or failure.