3GPP TSG-SA WG6 Meeting #51-e	S6-222980

e-meeting, 10th – 19th October 2022	(revision of S6-222674 rev3)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.558 | CR | 0124 | rev | 4 | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Implementation of TR 23.700-98 solution for KI#8 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | SA 6 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | <Source_if_TSG> |  |  |  |  |  |  |  |  |  |
| Work item code: | EDGEAPP_Ph2 |  |  |  |  |  | Date: |  |  | 2022-10-17 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - To enable AC/ EEC to provide a “Selected EAS Endpoint” IE to the EES via the AC Profile, to indicate EAS selection e.g. based on pre-provisioning. - To enable EEC to provide a UE type IE (e.g. constrained device) with the EEC registration request. - To enable EES to select EAS and provide information as a response to EEC registration under certain conditions, i.e. based on a received selection request indicator (e.g., for constrained device) and on EES local policies. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The following changes correspond to the solution captured in 3GPP TR 23.700-98 V.18.2.0, clause 7.39: - In Table 8.2.2-1 (AC Profile) added “Selected EAS Endpoint” IE - In Table 8.4.2.3.2-1(EEC registration request) added “EAS selection request indicator” and “UE type” IEs - In Table 8.4.2.3.3-1 (EEC registration response) added “Discovered EAS list” and “EAS profile” IEs - In clauses 8.4.2.2.2 (EEC registration) and 8.4.2.2.3 (EEC registration update) specified handling of the new IEs. - Included several NOTEs and ENs for clarification and follow up. In addition, two editorial corrections were made in the affected clauses. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No support is provided for EAS selection synchronization |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.2; 8.4.2.2.2; 8.4.2.2.3; 8.4.2.3.2; 8.4.2.3.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | N | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | n | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | N | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

### 8.2.2	AC Profile

An AC Profile includes information about AC used to determine services and service characteristics required.

Table 8.2.2-1: AC Profile

| Information element | Status | Description |
| --- | --- | --- |
| ACID | M | Identity of the AC. |
| AC Type | O | The category or type of AC (e.g. V2X). This is an implementation specific value. |
| Preferred ECSP list | O | When used in a service provisioning request, this IE indicates to the ECS which ECSPs are preferred for the AC. The ECS may use this information in the selection of EESs. |
| AC Schedule | O | The expected operation schedule of the AC (e.g. time windows) |
| Expected AC Geographical Service Area | O | The expected location(s) (e.g. route) of the hosting UE during the AC's operation schedule. This geographic information can express a geographic point, polygon, route, signalling map, or waypoint set. |
| AC Service Continuity Support | O | Indicates if service continuity support is required or not for the application. The IE also indicates which ACR scenarios are supported by the AC and which of these are preferred by the AC. |
| List of EASs | O | List of EAS that serve the AC along with the service KPIs required by the AC |
| > EASID | M | Identifier of the EAS |
| > Pre-provisioned EAS Endpoint (NOTE) | O | Endpoint information (e.g., URI, FQDN, IP 3-tuple) of a pre-provisioned EAS. |
| > Expected AC Service KPIs | O | KPIs expected in order for ACs to receive currently required services from the EAS, as described in Table 8.2.3-1 |
| > Minimum required AC Service KPIs | O | Minimum KPIs required in order for ACs to receive meaningful services from the EAS, as described in Table 8.2.3-1 |

* * * Next Change * * * *

##### 8.4.2.2.2	EEC registration

Figure 8.4.2.2.2-1 illustrates EEC registration procedure.

Pre-conditions:

1.	The EEC is authorized to access the EES for the purpose of performing registration and has received relevant security credentials as specified in clause 8.11; and

2.	The EEC has received service provisioning information from the ECS, including information for accessing the EES.

Figure 8.4.2.2.2-1: EEC registration procedure

1.	The EEC sends EEC registration request to the EES. The request from the client includes the security credentials received after successful authorization for edge computing services and may include a proposed expiration time. The request also optionally includes information indicating to the EES how the EEC expects to use the services of the EES.

If the EEC is moving to this EES from the purview of another EES, called S-EES, the request from the EEC may include the identity and endpoint of the S-EES and an EEC context ID that was provided by the S-EES to maintain continuity of the EEC context and to authorize EEC context relocation. If the EEC registration is being performed as part of ACR, the EEC shall not include the S-EES endpoint and the EEC context ID.

2.	Upon receiving the request from the EEC, the EES validates the registration request and verifies the security credentials. The EES further determines whether the requirements that were indicated in the AC Profile(s) can be fulfilled e.g., without failing to meet the requirements of the already registered EECs, and reserves corresponding resources (e.g., for EASs).

NOTE 1:	Resource reservation at EEC registration ensures that the EES provides services for the duration of the registration.

If the EEC provides in the registration request the EAS selection request indicator and based on EES local policies, the EES selects EASs providing the capabilities required by the AC Profile(s), reserves the corresponding resources, and provides the information to the EEC in the registration response. If an AC Profile includes the “Pre-provisioned EAS Endpoint” IE (along with the EAS selection request indicator), no EAS selection is performed based on any other capability requirements of that AC Profile.

If the EEC provides in the registration request a UE type, the EES may use this information to apply UE-type-specific local policies.

Editor’s Note: This functionality is to be aligned with how a “constrained device” is identified by the EES.

3.	Upon successful validation of the request, if the received EEC registration request contains an EEC context ID and a S-EES Endpoint, the EES performs an EEC Context Pull relocation (clause 8.9.2.2) from the S-EES. The source and target EES perform EEC Context handling as detailed in clause 8.9.1.

NOTE 2:	Only a single EEC Context ID may be provided in the EEC registration request.

NOTE 3:	In this version of specification, each registration procedure relocates a single EEC context.

NOTE 4:	Step 3 is executed when EEC determines to change its connection from S-EES to T-EES and ACR is not required.

If the EEC registration request fails after the EEC Context Pull relocation, e.g., the EES cannot reserve the necessary resources while meeting the capability requirements of the existing registered EECs, the EES shall determine the EEC Context information stale and send a failure response with a corresponding cause.

4.	The EES sends a successful EEC registration response, which includes the registration ID and may include a newly assigned EEC context ID. If the EEC registration request contains AC Profile(s), and the EES determines that the requirements indicated in the AC profile(s) cannot be fulfilled for some of the AC profile(s), the EES shall include the list of ACIDs of such AC Profile(s) and may include appropriate reasons. If step 3 was executed, the EEC registration response also includes EEC context retrieval result. The EEC stores the new EEC context ID and uses it if and when it registers with another EES. The EES may also provide an expiration time to indicate to the EEC when the registration will automatically expire. To maintain the registration, the EEC shall send a registration update request prior to the expiration. If a registration update request is not received prior to the expiration time, the EES shall treat the EEC as implicitly de-registered.

If the EEC context relocation status indicates that the EEC context relocation was not successful, then the EEC performs the required EDGE-1 subscriptions at the T-EES.

##### 8.4.2.2.3	EEC registration update

Figure 8.4.2.2.3-1 illustrates EEC registration update procedure.

Pre-conditions:

1.	EEC has already registered with the EES.

Figure 8.4.2.2.3-1: EEC registration update procedure

1.	The EEC sends EEC registration update request to the EES. The request from the client includes the security credentials received after successful authorization for edge computing services and may include a proposed expiration time and AC profile(s) parameters, including new or updated selected EAS information.

2.	Upon receiving the request from the EEC, the EES validates the registration update request and verifies the security credentials.

3.	Upon successful validation of the request, the EES sends a successful registration update response, which may include updated expiration time to indicate to the EEC when the updated registration will automatically expire. If the EEC registration update request contains AC Profile(s), and the EES determines that the requirements indicated in the AC profile(s) cannot be fulfilled for some of the AC profile(s), the EES shall include the list of ACIDs of such AC Profile(s) and may include appropriate reasons. To maintain the registration, the EEC shall send a registration update request prior to the expiration time. If a registration update request is not received prior to the expiration time, the EES shall treat the EEC as implicitly de-registered.

* * * NextChange * * * *

##### 8.4.2.3.2	EEC registration request

Table 8.4.2.3.2-1 describes information elements in the EEC registration request from the EEC to the EES.

Table 8.4.2.3.2-1: EEC registration request

| Information element | Status | Description |
| --- | --- | --- |
| EECID | M | Unique identifier of the EEC. |
| UE Identifier | O | The identifier of the hosting UE (i.e. GPSI or identity token) |
| Security credentials | M | Security credentials resulting from a successful authorization for the edge computing service. |
| AC Profile(s) | O | Profiles of ACs for which the EEC provides edge enabling services. AC Profiles are further described in Table 8.2.2-1. |
| EEC Service Continuity Support | O | Indicates if the EEC supports service continuity or not. The IE also indicates which ACR scenarios are supported by the EEC. |
| List of EEC-Selected EAS Endpoints (NOTE 1) | O | List with endpoint information (e.g., URI, FQDN, IP 3-tuple) of EEC-selected EASs. |
| EAS selection request indicator (NOTE 1) | O | Indicates the request for EAS selection support from the EES (e.g., for constrained device). |
| UE type | O | Indicates UE or device type (e.g. constrained device) |
| Proposed expiration time | O | Proposed expiration time for the registration. |
| EEC context ID (NOTE 2) | O | Identifier of the EEC context obtained from a previous registration. |
| Source EESID (NOTE 2) | O | Identifier of the EES that provided EEC context ID. |
| Source EES Endpoint (NOTE 2) | O | The endpoint address (e.g. URI, IP address) of the EES that provided EEC context ID. |
| NOTE 1: Only one of these IEs shall be present. NOTE 2: This IE shall not be present when EEC registration is performed as part of ACR. |  |  |

##### 8.4.2.3.3	EEC registration response

Table 8.4.2.3.3-1 describes information elements in the EEC registration response from the EES to the EEC.

Table 8.4.2.3.3-1: EEC registration response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response | O | Indicates that the registration request was successful. |
| > Registration ID | M | Identifier of the EEC registration. |
| > Expiration time | M | Indicates the expiration time of the registration. To maintain an active registration status, a registration update is required before the expiration time. |
| > EEC context ID | O | Identifier of the EEC Context information available at the EES that performed the registration. |
| > EEC Context Relocation status | O | Indicates whether the EEC context retrieval from the S-EES was successful or not. |
| > Discovered EAS list | O | List of EASs discovered to provide the capabilities required by the AC Profiles. If the request includes the EAS selection request indicator, then Discovered EAS list shall contain only one selected EAS. If the EES selects no EASs, the list may be empty. |
| >> EAS profile | M | Profile of the EAS. Each element is described in clause 8.2.4. |
| > list of unfulfilled AC information | O | List of ACIDs of the AC Profile(s) for which the requirements indicated in the AC profile(s) cannot be fulfilled |
| >> ACID | M | Application Identifier |
| >> reason | O | Reason indicating the cause (e.g. EAS not available, requirements cannot be fulfilled) |
| Failure response | O | Indicates that the registration request failed. |
| > Cause | M | Provides the cause for registration request failure. |