3GPP TSG-SA6 Meeting #58	S6-233475

# Chicago, United States, 13th Nov 2023 - 17th Nov 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.222 | CR | 0137 | rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | API invoker authorization corrections |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Vodafone |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | SNAAPP |  |  |  |  |  | Date: |  |  | 2023-11-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | from SA6#57, Xiamen, it was agreed that (CR 0133 in Tdoc S6-233296) that the role of resource owner client is out of scope of Rel-18. The title of clause 8.16 is misleading as it indicates that 8.16 describes API invoker authorization but 8.16 describes API invoker invocation of a service API. Also, 8.16 mandates a secure communication channel but does not describe what secure communication channel is. Clause 8.32 "Reducing authorization information inquiry in a nested API invocation" refers to clause 8.16 for step 1. API invoker requests authorization information but the correct reference is 8.11. (8.16 describes the AEF requesting authorization information, not the API invoker). |  |  |  |  |  |  |  |  |
| Summary of change: |  | one instance of "resource owner client" corrected 8.16 title changed and secure communication channel description added reference corrected from 8.16 to 8.11 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | incorrect reference to authorization procedure "secure communication channel" undefined out of scope in Rel-18 resource owner client role left in spec. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.7, 8.16.1, 8.32.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*** First Change ***

### 6.3.7	Authorization function

The authorization function consists of the following capabilities:

-	Receiving authorization from the resourse owner; and

-	Providing the API invoker with the authorization information which is needed to access the resource owner’s resources.

NOTE:	In the current release, the authorization function is an internal entity of the CAPIF core function.

*** Next Change ***

## 8.16	API invoker invocation of service APIs

### 8.16.1	General

The procedure in this subclause corresponds to the architectural requirements for invocation of service APIs by the API invoker.

Invocation of service APIs shall use a secure communication channel. The procedure for providing a secure communication channel is out of scope of this specification.

NOTE 1: Examples of secure API communication are described in ETSI GS MEC 009 (V1.1.1) [8] and OMA-ER_Autho4API-V1_0-20141209-A [9].

To reduce latency during API invocation, the API invoker associated authorization information can be made available at the AEF after authentication between the API invoker and the CAPIF core function.

NOTE 2:	The security aspects of making API invoker authorization information available at the AEF are out of scope of the present document.

*** Next Change ***

## 8.32	Reducing authorization information inquiry in a nested API invocation

### 8.32.1	General

The nested API invocation scenario is a scenario where the first API invocation towards the API exposing function 1 triggers this API exposing function to request another API invocation towards the API exposing function 2, which is in the same API provider domain that the API exposing function 1. Some service APIs may require invoking another service APIs. For example, if the API invoker invokes SEAL locationInfoRetrieval API, the location management server (acting as an API exposing server for the API invoker and as an API invoker for the NEF) may invoke NEF API to retrieve UE location information from 5GC. The CAPIF may reduce the authorization information inquiries for a nested API invocation scenario using procedure described in clause 8.32.3.

### 8.32.2	Information flows

NOTE:	The security aspects of this procedure will be specified in TS 33.122 [12].

### 8.32.3		Procedure

Figure 8.32.3-1 illustrates the procedure to obtain authorization information in a nested API invocation, in which an API exposing function receiving the service API invocation request interacts with another API exposing function to provide the service.

Pre-conditions:

1.	The resource owner can communicate with the API invoker.

2.	The API exposing functions 1 and 2 are in the same trust domain.

Figure 8.32.3-1: Procedure for obtaining authorization information in a nested API invocation

1.	The API invoker requests authorization information to invoke the service API exposed by the API exposing function 1.

NOTE:	This step may use either the existing procedure to obtain authorization to access service API specified in clause 8.11 or the procedure that involves the resource owner client to get authorization information. For the latter case, the detailed procedure will be specified in SA3.

2.	The API invoker sends service API invocation request to the API exposing function 1 with the authorization information received in step 1.

3.	Based on the service API invocation request, the API exposing function 1 decides to invoke another service API exposed by the API exposing function 2.

4.	The API exposing function 1, acting as an API invoker, obtains the authorization information to access the service API exposed by the API exposing function 2.

5.	The API invoker sends service API invocation request to the API exposing function with the authorization information received in step 4.

*** End of Changes ***