3GPP TSG-SA WG6 Meeting #58	S6-233535

Chicago, USA, 13th – 17th November 2023	(revision of S6-23xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.434 | CR | 0253 | rev | - | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections to the Information flows and APIs for Group Management |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | SA6 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | Samsung |  |  |  |  |  |  |  |  |  |
| Work item code: | SEAL |  |  |  |  |  | Date: |  |  | 2023-11-07 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Some of the descriptions related to the information flows and information elements are in-correct and it may lead to confusion for the readers especially for stage-3. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This CR corrects the information flows and information elements |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect specification |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.3.2.1, 10.3.2.2, 10.3.2.4, 10.3.2.5, 10.3.2.6, 10.3.2.7,10.3.2.18, 10.3.2.19, 10.3.2.20, 10.3.2.21 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

#### 10.3.2.1	Group creation request

Table 10.3.2.1-1 describes the information flow group creation request from the group management client or VAL server to the group management server.

Table 10.3.2.1-1: Group creation request

| Information element | Status | Description |
| --- | --- | --- |
| Requester Identity | M | The identity of the group management client or VAL server performing the request. |
| Identity list | M | List of VAL user IDs or VAL UE IDs that are part of the group to be created corresponding to the list of the configured services |
| VAL service ID list (see NOTE 1) | O | List of VAL services whose service communications are to be enabled on the group. |
| VAL service specific information (see NOTE 2) | O | Placeholder for VAL service specific information |
| NOTE 1: This information element shall be included in the message for creating a group configured for multiple VAL services. NOTE 2: The details of this information element are specified in VAL service specific specification and are out of scope of the present document. |  |  |

#### 10.3.2.2	Group creation response

Table 10.3.2.2-1 describes the information flow group creation response from the group management server to the group management client or VAL server.

Table 10.3.2.2-1: Group creation response

| Information element | Status | Description |
| --- | --- | --- |
| VAL group ID | M (see NOTE) | VAL group ID of the group |
| Result | M | Indicates the success or failure for the operation |
| NOTE: If the Result information element indicates failure then the value of VAL group ID information element has no meaning. |  |  |

* * * Next Change * * * *

#### 10.3.2.4	Group information query request

Table 10.3.2.4-1 describes the information group information query request from group management client or VAL server to group management server.

Table 10.3.2.4-1: Group information query request

| Information element | Status | Description |
| --- | --- | --- |
| Identity | M | The identity of the VAL server, VAL user or VAL UE performing the query. |
| VAL group ID | M | The identity of the VAL group to be queried. |
| Query type | M | It indicates the query type, i.e., membership information. |

#### 10.3.2.5	Group information query response

Table 10.3.2.5-1 describes the information flow group information query response from group management server to group management client or VAL server.

Table 10.3.2.5-1: Group information query response

| Information element | Status | Description |
| --- | --- | --- |
| VAL group ID | M (see NOTE) | The identity of the VAL group to be queried. |
| Query type | M (see NOTE) | It indicates the query type, e.g. membership information. |
| Query result | M (see NOTE) | The group information retrieved from the group management server based on the query type, i.e., a list of group members. |
| Result | M | Indicates the success or failure for the operation |
| NOTE: If the Result information element indicates failure then the values of other information elements have no meaning.. |

* * * Next Change * * * *

#### 10.3.2.6	Group membership update request

Table 10.3.2.6-1 describes the information flow group membership update request from the group management client or VAL server to the group management server.

Table 10.3.2.6-1: Group membership update request

| Information element | Status | Description |
| --- | --- | --- |
| Requester Identity | M | The identity of the group management client or VAL server performing the request. |
| VAL group ID | M | Identity of the VAL group |
| Identity | M | List of identities of the VAL users and VAL UEs affected by this operation |
| Operations | M | Add to or delete from the group |
| VAL service specific information (see NOTE) | O | Placeholder for VAL service specific information |
| NOTE: The details of this information element are specified in VAL service specific specification and are out of scope of the present document. |  |  |

#### 10.3.2.7	Group membership update response

Table 10.3.2.7-1 describes the information flow group membership update response from the group management server to the group management client or VAL server.

Table 10.3.2.7-1: Group membership update response

| Information element | Status | Description |
| --- | --- | --- |
| VAL group ID | M | Identity of the VAL group |
| Result | M | Indicates the success or failure for the operation |

* * * Next Change * * * *

#### 10.3.2.18	Store group configuration request

Table 10.3.2.18-1 describes the information flow store group configuration request from the group management client or VAL server to the group management server.

Table 10.3.2.18-1: Store group configuration request

| Information element | Status | Description |
| --- | --- | --- |
| Requester Identity | M | The identity of the group management client or VAL server performing the request. |
| VAL group ID | M | VAL group ID of the group |
| VAL group configuration data | M | VAL group configuration data |

#### 10.3.2.19	Store group configuration response

Table 10.3.2.19-1 describes the information flow store group configuration response from the group management server to the group management client or VAL server.

Table 10.3.2.19-1: Store group configuration response

| Information element | Status | Description |
| --- | --- | --- |
| VAL group ID | M | VAL group ID of the group |
| Result | M | Indicates the success or failure for the result |

* * * Next Change * * * *

#### 10.3.2.20	Get group configuration request

Table 10.3.2.20-1 describes the information flow get group configuration request from the group management client or VAL server to the group management server.

Table 10.3.2.20-1: Get group configuration request

| Information element | Status | Description |
| --- | --- | --- |
| Requester Identity | M | The identity of the group management client or VAL server performing the request. |
| VAL group ID | M | VAL group ID of the group |
| VAL group information reference | M | Reference to configuration data for the VAL group |
| VAL services requested (see NOTE) | O | Service(s) for which group configuration is requested |
| NOTE: If 'VAL services requested' is not present, group configuration is requested for all services defined for the VAL group |  |  |

#### 10.3.2.21	Get group configuration response

Table 10.3.2.21-1 describes the information flow get configuration response from the group management server to the group management client or VAL server.

Table 10.3.2.21: Get group configuration response

| Information element | Status | Description |
| --- | --- | --- |
| VAL group ID | M | VAL group ID of the group |
| VAL group configuration data | M (see NOTE) | VAL group configuration data |
| Result | M | Indicates the success or failure for the operation |
| NOTE: If the Result information element indicates failure then the value of the VAL group configuration data information element has no meaning. |  |  |

* * * End Changes * * * *