3GPP TSG-SA WG6 Meeting #58	S6-233550

Chicago, USA, 13th – 17th November 2023	(revision of S6-23xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.280 | CR | 0473 | rev | - | Current version: | 19.0.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | MC service user authentication to use MC Gateway UE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | MCGWUE |  |  |  |  |  | Date: |  |  | 2023-11-07 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The MC service user at non-3gpp device to use any service/functionalities from MC Gateway UE needs to authenticate with MC Gateway UE. However in the existing specification it is not clear whether MC service user authentication is required or not. |  |  |  |  |  |  |  |  |
| Summary of change: |  | New information flow Connection authentication request and Connection authentication response are defined. A new procedure for MC service user authentication with MC Gateway UE is defined and is achieved by using the Connection authentication procedures. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Any user from non-3gpp device can start using the functionalities from the MC Gateway UE and incomplete information leads to implementation issues by stage 3. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.5.0, 11.5.0.1, 11.5.0.2, 11.5.0.2.1, 11.5.0.2.2, 11.5.0.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

### 11.5.0	Connection authentication mechanism

#### 11.5.0.1	General

The MC service user at non-3gpp device can access the MC service via MC Gateway UE. This requires MC service user authentication with MC Gateway UE to access the MC Gateway UE services followed by authorization with MC service server to receive the MC services via the MC Gateway UE. The connection authentication procedure is used to support the MC service user authentication with MC Gateway UE. The required data for authentication is provisioned at both non-3gpp device and MC Gateway UE (How these authentication related information is obtained at non-3gpp device and MC Gateway UE is outside of the present document).

#### 11.5.0.2	Information flows

##### 11.5.0.2.1	Connection authentication request

Table 11.5.0.2.1-1 describes the information flow connection authentication request sent from the MC gateway client, which resides on a non-3GPP device, to the MC gateway UE.

Table 11.5.0.2.1-1: Connection authentication request

| Information element | Status | Description |
| --- | --- | --- |
| MC ID (See NOTE 1) | M | The MC ID of the requesting MC service user. |
| Authentication Related Parameters (See NOTE 2) | M | The authentication related parameters |
| NOTE 1: The MC ID indicates identity of the MC user requesting for authentication. NOTE 2: The authentication related parameters are defined by SA3 and is outside the scope of present document. |  |  |

##### 11.5.0.2.2	Connection authentication response

Table 11.5.0.2.2-1 describes the information flow connection authentication response sent from the MC gateway UE to the MC gateway client, which resides on a non-3GPP device.

Table 11.5.0.2.2-1: Connection authentication response

| Information element | Status | Description |
| --- | --- | --- |
| MC ID (See NOTE 1) | M | The MC ID of the requesting MC service user. |
| Response | M | Result of the connection authentication request |

#### 11.5.0.3	Connection authentication procedure

The clause is applicable to both non-3GPP devices which can host an MC client and non-3GPP devices which cannot host an MC client. The MC gateway UE performs MC service user authentication using connection authentication procedure and the MC gateway UE will creates the binding between MC user identified by MC ID and MC service ID.

The procedure for connection authentication towards an MC gateway UE is shown in figure 11.5.0.3-1.

Pre-conditions

-	The MC service user wishes to have access to MC services using a non-3GPP device.

-	The MC gateway client has been configured with the necessary parameters needed for connectivity with the MC gateway UE.

-	The MC gateway client hosted at the non-3GPP device has been provided with an appropriate MC service ID, MC ID and authentication related information of MC service user at non-3GPP device.

-	The MC gateway UE has performed service authorization for one or more MC services with the MC system as described in 3GPP TS 23.379 [16], 3GPP TS 23.281 [12], and 3GPP TS 23.282 [13].

-	The MC gateway client has selected an MC gateway UE or alternatively, the MC gateway client has performed a selection by internal criteria.

NOTE:	The internal criteria are outside the scope of the present document.

Figure 11.5.0.3-1: Connection authentication with an MC gateway UE

1.	The MC gateway client requests connection authentication with the MC gateway UE for the MC service user authentication. The MC gateway client provides the MC ID of the MC service user and authentication related information.

2.	The MC gateway UE performs the requested MC service user authentication, as indicated by the MC ID and authentication related information and with any other required information. Once the MC user is authenticated to use the MC Gateway UE, the binding of MC service user identity with MC service ID of the user is created. The MC gateway UE uses the MC service ID for identifying the user while providing the MC Gateway UE services to the MC service user.

3.	The MC gateway UE sends the connection authentication response to the MC gateway client. If the connection authentication is not successful then the MC Gateway UE shall not allow the MC service user at non-3GPP device to use the services at MC gateway UE or MC gateway UE for MC service. It is implementation specific how the MC Gateway UE restrict its use by MC service user at non-3GPP device.

After successful connection authentication with the MC gateway UE, the MC gateway client may continue with MC service user authorisation towards MC service server via MC gateway UE for MC service and followed by MC clients to have MC service access from the MC server may continue with user authentication and service authorization.

* * * * End of changes * * * *