3GPP TSG-SA WG6 Meeting #58	S6-233562

Chicago, USA, 13th – 17th November 2023	(revision of S6-23xxxx)

Source:	Lenovo

Title:	Solution on support for FL member registration

Spec:	3GPP TR 23.700-82 v0.1.0

Agenda item:	8.3

Document for:	Approval

Contact:	Emmanouil Pateromichelakis <epateromiche@lenovo.com>

1. Introduction

This paper provides a solution for FL member registration.

2. Reason for Change

This paper provides a solution to KI#3 for supporting at an AI/ML model registry the FL member registration.

3. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-82 v0.1.0.

* * * First Change * * * *

## 8.x	Solution #x: Support for FL member registration

### 8.x.1	Solution description

This solution addresses Key Issue #3.

This solution provides the procedures for the registration and registration update of candidate FL members in an AI/ML Service Registry. Candidate FL members can be application layer entities at the server or VAL UE side, which can potentially be selected as FL clients or FL server, FL server/aggregator for a particular ASP /vertical requirement.

NOTE: AI/ML service registry can be deployed as logical functionality in AI/ML enabler server or as enhanced capability of A-ADRF or at an edge / cloud platform repository or gateway.

#### 8.x.1.1	Procedure for FL member registration

Figure 8.x.1.1-1 illustrates the procedure where the registration of a candidate FL member happens via the AI/ML Service Registry.

This procedure covers the registration of the candidate FL member at the AI/ML Service Registry. Such candidate member can be a server functionality or a VAL UE functionality or an enabler layer functionality which is registering to the application layer AI/ML service registry to act as FL member for a given application event (analytics event or event triggered by a VAL layer application server or client).

The following figure illustrates the procedure for registering FL member at the AI/ML Service Registry.

Figure 8.x.1.1-1 Procedure for registration on AI/ML model registry

1.	The candidate FL member onboards to the platform (edge or cloud), hosting the AI/ML enabler service producer. Such entity is used as a middle layer between the FL members and the registry and facilitates the registration process.

NOTE: the FL member onboarding procedure can be done via CAPIF operations for the API provider domain functionality registration (as provided in TS 23.222).

2.	For registration of FL members on the AI/ML service registry, the AI/ML enabler service producer sends a registration request to the AI/ML service registry. The registration request contains a list of information about the FL member, which require registration on the AI/ML service registry.  Such information includes the FL member type and capabilities as well as other information such as time availability scheduling information, area of interest or location the member resides, interoperability information, use case availability/limitations.

3.	The AI/ML service registry validates the received request and generates the identity and other security related information for all the FL members listed in the registration request.

4.	The AI/ML service registry sends the generated information in the registration response message to the AI/ML enabler service producer.

5.	The AI/ML enabler service producer configures the received information to the individual candidate FL members.

#### 8.x.1.2	Procedure for FL member registration update

Figure 8.x.1.2-1 illustrates the procedure where the registration update of a candidate FL member happens via the AI/ML Service Registry.

Figure 8.x.1.2-1: Procedure for registration update

1.	The candidate FL member (acting as AI/ML enabler service consumer) either decides to offboard or optionally change the capabilities or a trigger is initiated based on the condition changes (e.g. VAL UE moves to an area with limited coverage) and provides this information to the AI/ML enabler service producer. Such update can be also update of the location of the FL member in different DN/EDN, or the temporary unavailability of the FL member which requires to update the registration.

NOTE: the FL member offboarding procedure can be done via CAPIF operations for the API provider domain functionality registration (as provided in TS 23.222).

2.	For registration update of FL members on the AI/ML service registry, the AI/ML enabler service producer sends a registration update request to the AI/ML service registry. The registration update request contains a list of information about the FL member (what is changed or the change of availability status), which require registration update on the AI/ML service registry.

3.	AI/ML service registry validates the received update request and generates the identity and other security related information for all the FL members listed in the registration update request.

4.	The AI/ML service registry sends the generated information in the registration update response message to the AI/ML enabler service producer or to the candidate FL member.

### 8.x.2	Architecture Impacts

The application enabler layer architecture impacts are the following:

- An AI/ML Service Registry is introduced for enabling the registration of candidate FL members. Based on whether this registry is part of the AI/ML Enabler Services or A-ADRF or CCF or another repository, this may have further architecture implications.

### 8.x.3	Corresponding APIs

Editor's note:	This clause provides the corresponding APIs for supporting the solution.

### 8.x.4	Solution evaluation

Editor's note:	This clause provides an evaluation of the solution. The evaluation should include the descriptions of the impacts to existing architectures.

* * * Next Change * * * *

## 8.0	Mapping of solutions to key issues

Table 8.1-1 Mapping of solutions to key issues

|  | KI #1 | KI #2 | KI #3 | KI #4 |
| --- | --- | --- | --- | --- |
| Sol #1 |  |  |  |  |
| Sol #2 |  |  |  |  |
| Sol #x |  |  | X |  |