3GPP TSG-SA WG6 Meeting #58	S6-233582

Chicago, IL, USA, 13th – 17th November 2023	(revision of S6-23xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.281 | CR | 0207 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Change list of participants when criteria was used to setup the ad hoc group call (MCVideo) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | FRMCS_Ph5 |  |  |  |  |  | Date: |  |  | 2023-11-13 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Ad hoc can be formed by a list of participants or by criteria provided by an authorized user. The list of participants created by the criteria may not be complete or some necessary users may not be in the group. An authorized user should be able to override the list of participants if needed by providing a distinct list of participants also when criteria was used to setup the ad hoc group. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Specifies that the critieria is not longer in use when an authorised user overrides the criteria by modifying the list of participants using a list of MCVideo IDs. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Missing functionality. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.19.3.1.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * 1st change * * * *

##### 7.19.3.1.4	Modification of ad hoc group call participants by the initiator

Figure 7.19.3.1.4-1 below illustrates the modification of ad hoc group call participants procedure by the initiator of the ad hoc group call.

Pre-conditions:

1.	The MCVideo client 1 is the initiator of the ad hoc group call.

2.	The MCVideo users on MCVideo client 1, MCVideo client 3 to MCVideo client n are on an ongoing ad hoc group call.

3.	The MCVideo user 1 requests to remove the user of MCVideo client 3 from the ad hoc group call and add user of MCVideo client 2 into the on-going ad hoc group call.

Figure 7.19.3.1.4-1: Modification of ad hoc group call participants by the initiator

1.	The MCVideo client 1 sends the modify ad hoc group call participants request to the MCVideo server in order to remove MCVideo client 3 from the ongoing ad hoc group call and add MCVideo client 2 into it.

2.	The MCVideo server verifies whether the MCVideo client 1 is authorized to add or remove (modify) the participants of the on-going ad hoc group call.

3.	The MCVideo server sends modify ad hoc group call participants response to the MCVideo client 1. If the ad hoc group was formed by using criteria, this request overrides the criteria and the MCVideo server stops checking whether MCVideo clients meet of no longer meet the criteria.

4.	The MCVideo server sends the ad hoc group call leave request to the MCVideo client 3 in order to remove it from the on-going ad hoc group call.

5.	The MCVideo client 3 notifies the user of the ad hoc group call leave request.

6.	The MCVideo client 3 sends the ad hoc group call leave response to the MCVideo server.

7.	The MCVideo server removes the MCVideo client 3 from the ongoing ad hoc group call.

8.	The MCVideo server sends the ad hoc group call request towards MCVideo client 2.

NOTE:	Steps 8 to 10 can occur at any time following step 3.

9.	The receiving MCVideo client 2 notifies the user about the incoming ad hoc group call.

10.	The MCVideo client 2 accepts the ad hoc group call request and send ad hoc group call responses to the MCVideo server. The response may also contain a functional alias of the responding MCVideo user, which is verified (valid and activated for the user) by the MCVideo server.

11.	The MCVideo server may notify the initiating MCVideo user of all the users who are added to the on-going ad hoc group call. This notification may be sent to the initiating MCVideo user by the MCVideo server more than once during the call when MCVideo users join or leave the ad hoc group call.

12.	The MCVideo server may notify the participants about the change in the participants list of on-going ad hoc group call.

# * * * * End of changes * * * *