3GPP TSG-SA WG6 Meeting #58	S6-233658

Chicago, USA, 13th - 17th November 2023	(revision of S6-23xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.289 | CR | 0112 | rev |  | Current version: | 18.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Enhancement to MCVideo with 5G network capabilities |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Hisilicom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | enhMC |  |  |  |  |  | Date: |  |  | 2023-11-07 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In Rel-18, several network capabilities are introduced to enhance the XR service as specified in clause 5.37 of 3GPP TS 23.501. Those capabilities also be used by the MCVideo service to improve the user experience in wirelesss. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce a new clause to define the procedure of enhancing the MCVideo with the 5G network capabilities: L4S marking and QoS monitoring. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The MCVideo cannot benefit from the new 5G network capabilities to improve the user experience. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | New {x, x.1, x.2, x.2.1, x.3, x.3.1} |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# /******************** First Changes ********************/

x	MCVideo enhancement with 5G network capabilities

x.1	General

These clauses specify the procedure for the MCVideo enhancement with 5GS functionalities defined in clause 5.37 of 3GPP TS 23.501 [7].

The following MCVideo features are enhanced:

-	One-from-server video pull

x.2	Information flows

x.2.1	ECN feedback report

Table x.2.1-1 describes the information flow ECN feedback report, from the MCVideo client to the MCVideo server.

Table x.2.1-1: ECN feedback report

| Information element | Status | Description |
| --- | --- | --- |
| MCVideo ID | M | Requester identity |
| Functional alias | O | Functional alias of the requester |
| Source identifier | M | Identifies the communication, e.g. by identifying the media flow within a media multiplex, present only if media multiplexing |
| ECN information | M | The ECN information to be report, e.g., ECT(0) counter, ECT(1) counter, ECN-CE counter. |

x.3	Procedures

x.3.1	One-from-server video pull enhancement with 5GS capabilities

Procedure in figure x.3.1-1 is the basic signalling control plane procedures for the one-from-server video pull enhancement with 5GS capabilities e.g., L4S and QoS monitoring.

Pre-conditions:

- The same pre-condition from clause 7.3.2.4.2 of 3GPP TS 23.281 [4] is applied.

MCVideo

server

5GS

MCVideo client 1

1. MCVideo pull from server request

4. Media plane and transmission control established

2. Authorization check

6b. ECN marking for L4S in the 5GS

3. MCVideo pull from server response

5. Video stream from MCVideo server

6a. Request for L4S marking

6c. ECN feedback report

7a. Request for QoS monitoring

7b. QoS monitoring in the 5GS

7c. QoS monitoring notification

8. DL stream control, e.g., adjust the bitrate

Figure x.3.1-1: one-from-server video pull enhancement with 5GS capabilities

1-5.	Same as step 1 to step 5 in clause 7.3.2.4.2 of 3GPP TS 23.281 [4].

6a.	During the media transmission, the MCVideo server may determine to utilize the 5GS L4S capabilities. The MCVideo server interacts with the 5GS to request the ECN marking for L4S as specified in 3GPP TS 23.502 [10].

6b.	The 5GS starts to performs the ECN marking as specified in 3GPP TS 23.501 [7].

6c.	The MCVideo client, based on the DL ECN marking, sends the ECN feedback report as described in IETF RFC 6679 [xx] towards the MCVideo server.

7a.	During the media transmission, the MCVideo server may determine to utilize the 5GS QoS monitoring mechanism to obtain the congestion information, data rate information, round trip delay information in UL and/or DL directions. The MCVideo server requests the 5GS to perform QoS monitoring as specified in 3GPP TS 23.501 [7], in 3GPP TS 23.502 [10].

7b.	The 5GS performs the QoS monitoring as requested.

7c.	The 5GS provides the QoS monitoring notification as specified in 3GPP TS 23.501 [7], in 3GPP TS 23.502 [10].

8.	The MCVideo server, based on the step 6c and/or step 7c to perform the DL media stream control, e.g., adjust the bitrate.