3GPP TSG-SA WG6 Meeting #58	S6-233718

Chicago, USA, 13th – 17th November 2023	(revision of S6-23xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.434 | CR | 0275 | rev | - | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to add missing update operation for BDT configuration |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | SEAL_Ph3 |  |  |  |  |  | Date: |  |  | 2023-11-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The BDT configuration feature in NRM server supports the initial BDT configuration creation. The update operation is missing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The update operation is added. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The stage 3 implementation will be jncorrect. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 14.3.13.y (new), 14.3.2.a (new), 14.3.2.b (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

#### 14.3.13.y	BDT configuration update

Figure 14.3.13.y-1 illustrates a procedure for updating the BDT configuration on the NRM server by the VAL server.

Pre-conditions:

1.	The VAL server has performed the BDT configuration request as specified in clause 14.3.13.2.

Figure 14.3.13.y-1: Update configuration of Background Data Transfer

1.	A VAL Server requests the NRM Server to update its background data transfer policy configuration using the BDT_Configuration_Update_request (see clause 14.4.x.4).

The request includes identifier for the BDT policy configuration stored at the NRM server and one or more updated information like expected data volume, expected number of UEs, expected time window for the background data transfer, geographic information for the UEs, a request expiration time, guidance for policy selection.

2.	The NRM Server may perform the BDT policy negotiation update as specified in 3GPP TS 23.502 [11] clause 4.16.7.2. The request for update of the BDT policy configuration by the VAL server may provide several updates to the requirements of the VAL server to perform BDT to the VAL UE or group of VAL UEs. This may impact the selected BDT policy.

3.	The NRM Server provides a response to the VAL server indicating success or failure of the operation.

* * * Next Change * * * *

#### 14.3.2.a	BDT configuration update request

Table 14.3.2.a-1 describes the information flow for the BDT configuration update request from the VAL server to the NRM Server.

Table 14.3.2.a-1: BDT configuration update request

| Information element | Status | Description |
| --- | --- | --- |
| VAL service ID | M | Identity of the VAL service for which the BDT configuration is requested to be updated. |
| BDT Configuration identifier | M | Identifier of the BDT configuration stored in the NRM server. |
| Volume per UE | O | Updated expected data volume for the background data transfer. |
| Desired time window | O | Updated desired time window for the background data transfer. |
| Desired area information | O | Updated desired geographical area for the background data transfer |
| Policy Selection Guidance | O | Updated list that includes guidance to the NRM in selecting from multiple transfer policies provided by underlying network. Possible values include: ”lowest cost”, “highest throughput given maximum cost of X”, etc. If value is empty, the NRM may choose from among multiple transfer policies, depending on local and ASP-provided policies. |

#### 14.3.2.b	BDT configuration update response

Table 14.3.2.b-1 describes the information flow for BDT configuration update response from the NRM server to the VAL server.

Table 14.3.2.b-1: BDT configuration update response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | The result indicates success or failure of BDT configuration update operation. |
| BDT Configuration identifier | M | Identifier of the BDT configuration stored in the NRM server. |