3GPP TSG-SA WG6 Meeting #58	S6-233728

Chicago, USA 13th – 17th November 2023	(revision of S6-23xxxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.280 | CR | 0488 | rev | - | Current version: | 18.7.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | MCGWUE functional model |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | MCGWUE |  |  |  |  |  | Date: |  |  | 2023-11-07 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | This CR provides corrections to the existing role of the MC gateway UE server such that the communication and the connection authorisation procedures done by the MC gateway client towards the MC gateway UE is out of scope of 3GPP as it depends on the type of the non-3GPP access. Furthermore, it indicates that the GW-Core reference point is based on existing MCx-n reference points. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - adding a note indicating that the communication and connection (authentication and) authorisation done by the MC gateway client is out of scope of 3GPP. - indicating the functionality of the MC gateway UE server is available upon successful MC service authentication and user authorisation. - indicating that the GW-Core reference point is based on existing MCx-n reference points. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The existing functional model is not aligned with the existing MCx-n reference points, and confusion may occur since the communication and connection authorisation procedure depends on the non-3GPP access between the MC gateway UE and the non-3GPP device. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.2.1, 11.2.2.2, and 11.2.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

### 11.2.1	Functional model

For the application-level signalling necessary for the association between the MC client on the non-3GPP device and the MC gateway UE reference point Gateway-local (GW-local) is used. The MC gateway UE uses the Gateway-Core (GW-Core) reference point towards the MC service server to announce the association between the non-3GPP device and MC gateway UE server. Once the use of the MC gateway UE has been authenticated and authorised by the corresponding MC service server, it can support the non-3GPP devices behind it.

The MC gateway UE sever enables the following functions:

-	Authentication and authorisation connection of non-3GPP devices, in specific, the MC gateway clients including the MC gateway UE function of the corresponding MC service server

NOTE (option A):	The communication and connection procedures associated to authenticating and authorising the non-3GPP device towards the MC gateway UE depend on the type of the non-3GPP access and are out of scope of this document.

-	(Option B) MC service and resources check done by the MC gateway UE upon receiving a new connection authorisation request from an MC gateway client

-	Supporting the MC client in performing MC service authentication and authorisation towards the corresponding MC server to ensure the binding, i.e., MC gateway client to MC gateway UE association at the MC service server

-	The following functions are available when the MC gateway client is successfully authenticated and authorised by the corresponding MC server:

-	Relay of signalling between MC client and MC service server/CSC server

-	Relay of signaling between the signaling user agent residing on the non-3GPP device and the SIP core

-	Relay of signaling between the HTTP client residing on the non-3GPP device and the HTTP proxy

-	Media plane forwarding between the MC service server and the MC clients

The MC gateway UE function as part of the corresponding MC service server enables the use of an MC gateway UE with a corresponding MC gateway UE function that authorises the association between the MC client and the corresponding MC gateway UE to enable forwarding of the media plane to MC clients via the MC gateway UE.

Figure 11.2.1-1: Functional model of MC gateway UE signalling plane

# * * * * Second change * * * *

#### 11.2.2.2	Reference point GW-local (between the MC gateway client and the MC gateway UE server)

The GW-local reference point between the MC gateway client and the MC gateway UE server in the MC gateway UE is used:

-	To authenticate and authorise the non-3GPP device towards the MC gateway UE,

NOTE (option A):	The connection authentication and authorisation procedures between the non-3GPP device and the MC gateway UE depend on the type of the non-3GPP access and are out of scope of this document,

-	(Option B) To support the MC gateway UE in performing MC service and resources check upon receiving a new connection request from an MC gateway client,

-	To manage (request/release) forwarding of the signalling, media from the MC gateway UE to the MC gateway client and its associated MC clients for unicast, multicast/broadcast communication using corresponding identifiers, e.g., TMGI,

-	To disconnect from the use of the MC gateway UE.

-	To notify about the connection status.

The GW-local reference point is based on HTTP-1 reference point.

# * * * * Third change * * * *

#### 11.2.2.3	Reference point GW-Core (between the MC gateway UE server and the MC service server)

The GW-Core reference point between the MC gateway UE server in the MC gateway UE and the MC service server shall be used:

-	To manage (request/release) transport resources between the MC gateway UE and the MC service server signalling purposes are associated with the MC clients residing on a non-3GPP device,

-	To disconnect from the use of the MC gateway UE.

NOTE 1:	Signalling of the MC clients utilizes the allocated resources (default bearer for EPS or corresponding QoS flow in 5GS) and if necessary, MC gateway UE may request additional resources.

NOTE 2:	The authorized use of necessary connectivity between the non-3GPP device and the MC gateway UE is outside 3GPP specification.

The GW-Core reference point is based on the existing MCx-n reference points as specified in 3GPP TS 23.379 [16], 3GPP TS 23.281 [12] and 3GPP TS 23.282 [13]..

# * * * * End of changes * * * *