3GPP TSG-SA6 Meeting #58	S6-233748

# Chicago, United States, 13th Nov 2023 - 17th Nov 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.542 | CR | 0046 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | PINE Communication via 5GS+PEGC |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | KPN N.V. |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | PINAPP |  |  |  |  |  | Date: |  |  | 2023-11-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Since PDU session granularity is not by PIN element, to enable service continuity in changing access to 5GS and ensure the correct forwarding of packets by the PEGC, each PINE should include the PIN ID of the destination PINE in the communication packets. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updating step 4 in clause 8.9.2.2 to specify that the PINE ID of the destination PIN is specified in the communications between the PIN elements. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The specification is unclear and ambiguous. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.9.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

#### 8.9.2.2	Service continuity in changing access to 5GS

This procedure solves the situation, that PINE A has application layer communication with PINE B via PEGC. But when the communication via PEGC is not viable, for example, the PINB moves out of the coverage of PEGC 1, only newly establish the 5GS communication to connect PINE B is the potential way.

Figure 8.9.2.2-1 illustrates the service continuity to change application layer communication to communication via PEGC + 5GS based on request/response model.

Pre-conditions:

1.	The PEMC in a PIN has been pre-configured or has discovered the address (e.g. IP address, FQDN, URI) of the PIN server;

2.	The PIN has already been created and a PIN ID is distributed by PIN server;

3.	The PINE A has the subscription that can communicate with PINE B via 5GS;

Figure 8.9.2.2-1: Change application layer communication to communication via 5GS

1.	The PINE A has application layer communication with PINE B via PEGC 1.

2.	The PEGC 1 receives the related parameter (PIN ID, MAC address/IP address, Traffic descriptors, Packet filters, requested QoS) as indicated in section 8.6.2.2 from PINE 1 or PEMC.

3.	According to the Packet filters, the PEGC may initiate PDU Session Modification as indicated in section 4.3.3.2 of TS 23.502[x] with the Packet filters and requested QoS towards 5G system in order to make 5GC configure the N4 rules for UPF(s), referred to section 8.6.2.2.

4.	The PINE A communicates with PINE 2 via 5GS. To enable this communication via PEGC + 5GS, each PINE includes the PINE ID of the destination PINE in its communication.

* * * END of Change * * * *