3GPP TSG-SA WG6 Meeting #58	S6-233916

Chicago, USA, 13th – 17th November 2023	(revision of S6-233915 was 233617)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.286 | CR | 0081 | rev | 4 | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | UE-triggered VRU high-risk zone configuration |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Convida Wireless LLC, Lenovo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI19 |  |  |  |  |  | Date: |  |  | 2023-11-07 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Added Procedure on VRU zone configuration based on VRU triggers, previously captured in TR 23.700-64 . The procedure allows VAE client to trigger the creation of VRU zones, if allowed by policies. Implementing this functionality requires VAE involvement because it involves signaling affecting VAE performance. In addition, it enables detection of vulnerable users in relationship to the initiating UE. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The procedure on UE-triggered dynamic VRU zone configuration from TR 23.700-64 is proposed for addition to TS 23.286. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | TS 23.286 does not provide support for UE-triggered dynamic VRU zone configuration |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.21.1; New: 9.21.2.x1; 9.21.2.x2; 9.21.2.x3; 9.21.x4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | N | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | N | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | N | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

# 3	Definitions, symbols and abbreviations

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

The terms and definitions as specified in 3GPP TS 23.286 [6] apply.

VRU high-risk zone: Refers to a geographic area with Vulnerable Road Users (VRUs) recognition mechanisms.

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. 
An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

BSM	Basic Safety Message

CAM	Cooperative Awareness Message

CPM	Collective Perception Message

DENM	Decentralized Environmental Notification Message

VHRZ	VRU high-risk zone.

V2P	Vehicle-to-Pedestrian

V2X	Vehicle-to-Everything

VAE	V2X Application Enabler

VAM	VRU awareness message

VASS	V2X Application Specific Server

VRU	Vulnerable Road Users

VRUP	Vulnerable Road User Protection

* * * Next Change * * * *

### 9.21.1	General

This capability supports the configuration and provisioning of the VRU high risk zone at the VAE layer based on a request from the V2X application specific server (VASS). ). UE-initiated triggering of configuration and provisioning of VRU high-risk zones (VHRZ) is also supported at the VAE layer.

NOTE: The configuration of VHRZ based on VRU triggers and via the VAE layer is limited via policies and subject to authorization.

* * * Next Change * * * *

#### 9.21.2.x1	VRU trigger request

Table 9.21.2.x1-1 describes information elements for the VRU trigger  request from the VAE client to the VAE server.

Table 9.21.2.x1-1: VRU trigger request

| Information element | Status | Description |
| --- | --- | --- |
| VAL UE ID | M | The identifier of the requestor . |
| VRUP information | M | This information identifies the VRUP scenarios to be supported. |
| VRU service parameters | M | Parameters for the VRU-specific services to be supported, including: - list of VAE services (e.g. location tracking, V2X message bundling), - list of transmission modes (unicast, groupcast, broadcast) - interface selection indication (Uu, PC5), - indication of use of application relaying or not - QoS requirements for VRU services |
| Area of Interest | O | The geographical or service area for which the subscription request applies |
| Adaptation assistance information | O | This IE provides information for the VAE layer to be able to adapt the configuration, e.g. desired adaptation triggers, which may in turn be based on location/route. |
| Time validity | O | The time validity of the request |

#### 9.21.2.x2	VRU trigger response

Table 9.21.2.x2-1 describes information elements for the VRU trigger response from the VAE server  to the VAE client.

Table 9.21.2.x2-1: VRU trigger response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | The result of the request (positive or negative acknowledgement) |

#### 9.21.2.x3	VHRZ condition notification

Table 9.21.2.x3-1 describes information elements for the VHRZ management notification from the VAE server to the VASS server.

Table 9.21.2.x3-1: VRU condition notification

| Information element | Status | Description |
| --- | --- | --- |
| VRU information | M | This information identifies, based on the VRUP scenario, the VRU trigger requestor and includes the type of UEs (V2X UEs, pedestrian UEs) are to be considered. The information is determined by the VAE server based on the VRU trigger request parameters and local policies. |
| VRU service parameters | M | Parameters for the VRU services to be supported, including: - list of VAE services (e.g. location tracking, V2X message bundling), - list of transmission modes (unicast, groupcast, broadcast) - interface selection indication (Uu, PC5), - indication of use of application relaying or not - QoS requirements for VRU services |
| Area of Interest | O | The geographical or service area for which the subscription request applies |
| Time validity | O | The time validity of the request |

* * * Next Change * * * *

### 9.21.x4	 Procedure on UE/VRU-triggered VRU high-risk zone configuration

The procedure in figure 9.21.x4-1 illustrates a UE/VRU-initiated request which complements the VASS Server-triggered VRU high-risk zone configuration procedure in clause 9.21.3.

NOTE 1: The configuration of VHRZ based on VRU triggers and via the VAE layer is limited via policies and subject to authorization.

Pre-conditions:

- The VAE server and the VAE client are preconfigured with policies allowing support of VRU high-risk zone configuration via UE/VRU-initiated requests.

- The V2X application specific server has subscribed to the VAE server to receive VHRZ condition notifications.

NOTE 2: All clause 9.21.3 pre-conditions also apply.

Figure 9.21.x4-1: UE-triggered VRU high-risk zone configuration

1. The VAE client (e.g. upon detecting V2X/VRU app #1 activation) sends a VRU trigger request to a VAE server  indicating the condition which may result in configuration of a VRU high-risk zone (VHRZ). The message includes the information elements specified in Table 9.21.2.x1-1.

2.	The VAE server authorizes the trigger request and sends a VHRZ condition notification to the VASS for triggering the creation of the VRU high-risk zone around the UE. The VAE server includes the information in Table 9.21.2.x3-1.

3.	The VASS authorizes the request and determines whether a VRU high-risk zone based on information received from the VAE server is to be created. The VASS determines one or more dynamic VRU high-risk zone configurations corresponding to the request.

4.   Steps 1-8 of clause 9.21.3 are executed for each of the dynamic VRU high-risk zones determined.

As part of this procedure, the VAE server configures CN and SEAL LMS subscriptions to obtain UE location from the 5G system for the protected UE and for vehicular UEs around the protected zone. Sources of UE mobility information may come from SEAL LMS servers, EESs, analytics functions (e.g. from ADAES or NWDAF), and the 5G core network. For V2X UE location obtained from the location management server, the procedures specified in 3GPP TS 23.434 (clause 9.3.12) apply.

- The VAE Server sends a VRU trigger response (see Table 9.21.2.x2-1) to the VAE client indicating that a VRU high-risk zone has been created based on the VRU trigger request.

NOTE 3: This step may be initiated by the VAE server before the completion of the clause 9.21.3 steps.

* * * End of Changes * * * *