3GPP TSG-SA WG6 Meeting #58	S6-233948

Chicago, USA 13th – 17th November 2023	(revision of S6-233530)

Source:	TNO, Novamint, Sateliot

Title:	Pseudo-CR on new KI on Enabling re-discovery and re-allocation of EASs on-board satellites

Spec:	3GPP TR 23.xxx v0.0.0

Agenda item:	8.6

Document for:	Approval

Contact:	Relja Djapic, (relja.djapic@tno.nl)

1. Introduction

This contribution proposes new KI on satellite access impacts on Enabling re-discovery and re-allocation of EASs on-board satellites.

2. Reason for Change

Study and normative work on 5G Satellite Backhaul (5GSATB) is carried out in 3GPP SA2 Rel-18 (TR 23.700-27). Architecture enhancements for support of UPF deployed on GEO satellite with gNB on the ground is listed as part of the study scope. Here, the satellite link acts as the backhaul between the 5GC and 5G RAN. This topic explored how to enable satellite edge computing services via UPF on-boad GEO satellites to reduce the latency for data transmission, and minimize the backhaul resources consumption in the feeder link between the GEO satellite and 5GC. This architecture also allows for the deployment of Edge on board GEO satellite.

As part of the 5GSATB study Key Issue #2: Support of Satellite Edge Computing via UPF on board has been specified. The corresponding solutions exploit EAS (re-)discovery functionalities specified in TS 23.548. However, these          (re-)discovery procedures are originally developed for terrestrial networks. Besides, TS 23.558 ‘Architecture for enabling Edge Applications’ provides specifications on Edge Application enablers covering, among the others, aspects such as general architecture, service provisioning, EAS discovery, Support for service continuity,  Dynamic EAS instantiation triggering,… The listed   procedures are originally developed for terrestrial networks. Although the operation of EDGEAPP entities and interfaces is developed to be access network agnostic, the usage of GEO satellites may significantly increase latency in the exchange of application (control) information and consequently impact the applications responsiveness.

In cases when Edge is deployed on board GEO satellite it is  required to study possible improvements/optimization of EAS (re-)discovery procedures in case of satellite networks. The motivation is to investigate whether exchange of information (control, context, etc) within the EDGEAPP enabler domain could be optimized (e.g. reduce latency, data traffic in the feeder link) for cases when e.g. EASs are deployed on board different GEO satellites. In particular, to investigate whether the EDGEAPP enhancements would allow for the deployment of a local resolver that would be used in cases when an EAS on board GEO satellite would be switched to another EAS onboard another GEO satellite.

The following example describes a possible use-case to be studied:in case an EAS onboard GEO becomes unavailable due to a failure or a need for a planned maintenance it is needed that another EAS e.g., located on another GEO (temporarily) takes over its functionality. Similarly, such a switch from one to another EAS could be triggered by the mobility of UEs. In such cases it might be beneficial that a local resolver deployed on-board GEO satellite is activated and that it identifies alternative EAS on neighbouring GEO satellites and coordinate the transfere of context.

3. Proposal

It is proposed to agree the following changes to 3GPP TR 23.xxx v0.0.0.

* * * First Change * * * *

## 4.x	Key issue #x: Enabling re-discovery and re-allocation of EASs on board satellites

### 4.x.1	Description

TR 23.700-27 has studied the architectureal aspects of placing UPF and edge on board GEO satellite that is used for backhauling purposes. Placement of UPF and Edge in the backhaul part of a 5GS (i.e. on board a GEO satellite) facilitates reduction of latency and faster service provisioning to end users. 3GPP TS 23.548 (SA2) specify 5G system enhancements for Edge computing while TS 23.558 (SA6) define Architectures for enabling Edge applications inclufing the procedures for EAS (re)discovery. Although TS 23.558 specification is created to cover the most generic EDGEAPP cases– it would be beneficial to study whether the placement of Edge on board satellite requires some enhancements to support application enablers. The motivation for this is to explore whether EDGEAPP enhancements could reduce latency in cases when Edge has been placed on board GEO satellite. Besides, the interest is to investigate whether EDGEAPP enhancements could reduce data load in the feeder link (between 5GC and GEO satellites).

### 4.x.2	Open issues

Deployment of Edge computing on board GEO satellites might impact the latency of application control and data information exchange. To assess the impacts of using Edge on board GEO it would be benefitial to study the following aspects::

- When/how could  exchange of control data used in the EDGEAPP context be optimzied (reduce latency and data exchange over satellite backhaul feederlink) in cases when EDGE is deployed on board GEO satellite?

- Can deployment of a local resolver on board GEO satellite enhance EDGEAPP procedures:

  - EAS (re-)discovery process when EASs are located on distinct GEO satellites

  - Context transfer between distinct EASs on board different GEO satellites

  - Support of Service continuity in case of EAS changes

* * * End of Change * * * *