3GPP TSG-SA WG6 Meeting #58	S6-233987

Chicago, US, 13th – 17th November 2023	(was S6-233669)

Source:	Ericsson

Title:	Pseudo-CR on The AI/ML member participation policies provisioning and management

Spec:	3GPP TS 23.700-82 V0.1.0

Agenda item:	8.3

Document for:	Approval

1. Introduction

The Application layer AI/ML entities have heterogeneous nature, e.g., different models, OS, capabilities, that require creation of AIML policies specific for application layer AIML enabler service. To cover the application layer specific requirements, which are missing in the existing mechanisms, the AI/ML member participation policies are studied in this solution. AI/ML member participation policies (i.e., the conditions when an AI/ML member is available or not available to participate in the AI/ML operations) are essential for AI/ML Services for member selection in regular, distributed, and federated learning scenarios. These policies can include:

-	Energy policies, e.g., the AI/ML member is able to participate in the AI/LM operations if the battery level is greater than 80%;

-	Time-schedule policies, e.g., the AI/ML member is not able to participate in the AI/LM operations during working hours (i.e., from 9:00 to 17:00 on Mon-Fri); and

-	Location-based policies, e.g., the AI/ML member is able to participate in the AI/LM operations at home only represented by civic address.

2. Reason for Change

The provisioning and management of the AI/ML member participation policies are essential for the AI/ML Services in regular, distributed, and federated learning scenarios.

3. Conclusions

This pCR proposes the provisioning and management of the AI/ML member participation policies in the AIMLAPP layer.

4. Proposal

It is proposed to agree the following changes to 3GPP 23.700-82 V0.1.0.

# *** First Change ***

## 8.A	Solution #A: AI/ML member participation policies provisioning and management

### 8.A.1	General

The following clauses specify procedures, information flows and APIs for Key issue# A to provision the AI/ML member participation policies for the AI/ML Enabler services.

Assumptions:

-	AI/ML member (e.g. AI/ML Enabler Client) is registered on the AI/ML Enabler Server to participate in the AI/ML operations (e.g., ML model training).

NOTE:	The AI/ML member policies provisioning service operation can be a part of the ML client config provisioning procedure defined in clause 8.3.2.

### 8.A.2	Procedures

#### 8.A.2.1	AI/ML member participation policies provisioning and management

Figure 8.3.2-1: AI/ML member participation policies provisioning and management fprocedure

1.	The AI/ML member (e.g. AI/ML Client) sends an AI/ML member participation policies provisioning and management request as defined in clause 8.A.3.1.

2.	Upon receiving the request, the AI/ML Enabler server performs an authorization check of the AI/ML member.

3.	If the AI/ML member is authorised, the AI/ML Enabler Server stores the AI/ML member polices and applies the provisioned policies to ongoing and further the AI/ML operations (e.g., ML model training).

4.	AI/ML Enabler Server provides the response to the AI/ML member with a status via AI/ML member participation policies provisioning and management response defined in clause 8.A.3.2.

NOTE:	The service operation defined in clause 8.A.2.1 can be utilized for AI/ML member participation policies management (i.e., update/delete) functionality.

### 8.A.3	Information flows

#### 8.A.3.1	AI/ML member participation policies provisioning and management request

Table 8.A.3.1-1 describes the information flow from the AI/ML member to the AI/ML Enabler server as a request for the AI/ML member participation policies provisioning and management.

Table 8.A.3.1-1: AI/ML member participation policies provisioning and management request

| Information element | Status | Description |
| --- | --- | --- |
| Requester Identity | M (NOTE) | The identity of the AI/ML member performing the request. |
| List of policies | M | Identify the list of policies. |
| > List of VAL service ID(s) | M | Identify the list of VAL service IDs for which the policies are applicable. |
| > List of energy-related policies | O | Provides the list of energy-related polices as defined in table 8.A.3.1-2. |
| > List of time-schedule policies | O | Provides the list of time-schedule policies, e.g., the AI/ML member is (not) available to participate in the AI/ML operations in the given time slot(s) and/or day(s) of the week. |
| > List of location-based policies | O | Provides the list of location-based policies, e.g., the AI/ML member is (not) available to participate in the AI/ML operations in the given locations represented by coordinates, civic addresses, network areas, or VAL service area ID. |
| > List of AI/ML traffic connectivity policies | O | Provides the list of the AI/ML traffic connectivity policies, e.g., NSWO usage for the AI/ML traffic. |
| > List of compute-utilization based policy | O | Provides the AI/ML member participation availability based on list of compute (GPU, CPU, AIPU, NPU) utilization based policy, e.g. the AI/ML member may participate the AI/ML operation if the GPU load is less than the given percent. |
| >Mode of member participation | O | Indicates the AI/ML member availability for the participation based on the modes like training (e.g., FL, DML, ML, and/or Split Learning), aggregator, coordinator and/or inference. |
| NOTE: This information element shall not be updated in the AI/ML member participation policies management request. |  |  |

Table 8.A.3.1-2: Energy-related policy

| Information element | Status | Description |
| --- | --- | --- |
| Policy ID | M (NOTE) | The policy identifier. |
| List of policy conditions | M | Represents the list of policy conditions. |
| > Availability indication | M | Indicates the AI/ML availability status (e.g., available or not available). |
| > Threshold value | M | Represents the threshold value (e.g., battery level or energy consumption). |
| > Threshold matching direction | M | Indicates the threshold matching direction (e.g., above, below, cross). |
| NOTE: This information element shall not be updated in the AI/ML member participation policies management request. |  |  |

NOTE:	In table 8.A.3.1-1, the policies identify the conditions and criteria, when satisfied the AI/ML member can participate in the AI/ML operations initiated by VAL service(s).

#### 8.A.3.2	AI/ML member participation policies provisioning and management response

Table 8.A.3.2-1 describes the information flow from the AI/ML Enabler server to the AI/ML member as a response for the AI/ML member participation policies provisioning and management.

Table 12.3.2.2-1: AI/ML member participation policies provisioning and management response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Indicates success or failure of the request. |

# *** End of Changes ***