3GPP TSG-SA WG6 Meeting #58	S6-234110

Chicago, US, 13th – 17th November 2023	(was S6-234085)

Source:	Ericsson

Title:	Pseudo-CR on Introducing information flow for FL client selection assistance procedure

Spec:	3GPP TR 23.700-82 v0.1.0

Agenda item:	8.3

Document for:	Approval

1. Introduction

This pCR proposes additional information to the existing solution described in clause 8.2.2 TR 23.700-82 v0.1.0 FS_AIMLAPP.

Comment to rapporter:

The reference to table X in Table 8.2.3.1-1 (this pCR) is impemented in pCR S6-234054.

2. Reason for Change

The name of the procedure is changed to introudce the readability and reflect the intention of the procedure defined in clause 8.2.2. The current solution was limited to the procedure description and does not provide information flow. The current procedure was model as Request-Response and this may lead to timeout problems because the AI/ML service producer can take time to provide the selection of FL clients and it may worsen with the large number of FL clients. It is very likely to have large number of FL clients due to the distributed nature of FL. Therefore, it is proposed for asynchronous design and providing response in the notification.

3. Conclusions

<Conclusion part (optional)>

4. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-82 v0.1.0.

* * * First Change * * * *

## 8.2	Solution #2:FL client discovery or selection assistance procedure

### 8.2.1	General

The following clauses specify procedures, information flows and APIs for Key issue 3 to provide the assistance to selection of FL clients.

Assumptions:

1.	The proposed solution is based on the AIML Enabler service producer/consumer architecture. The service producer can be a standalone new entity like an AIML Enabler server or collocated with a SEAL entity like an ADAE server or part of the ADAE server. The AIML Enabler service consumer could be VAL servers, VAL UE, or any other entities that consume the AIML Enabler services.

2.	The proposed solution assumes distributed clients in ML. E.g., Federated Learning.

Pre-conditions:

1. The FL client information is available at the AI/ML enabler service producer.

Editor's Note: This solution may need to be updated for FL case based on KI updates.

### 8.2.2	Procedures

Figure 8.2.2-1: FL client discovery or selection assistance procedure

1.	The consumer (e.g., VAL server) sends the FL client discovery or selection assistance request to the AIML enabler service producer. The request contains VAL service ID to identify the FL service, FL client selection policies  for the selection of FL client(s).

2.	The AI/ML service producer acknowledges the request and provides the FL client selection assistance response message.

3.	The AIML enabler service producer processes the request as per the available selection policies to select the FL clients and provides the FL client selection assistance notification response containing list of FL client identities to the AI/ML service consumer..

Editor's Note 1: The solution is subject to change based on the architecture and is FFS.

Editor’ Note: It is FFS how an FL client repository function is used in this procedure.

* * * Next Change * * * *

### 8.2.3	Information flows

#### 8.2.3.1	FL client discovery or selection assistance request

Table 8.2.3.1-1 describes the information flow from the AI/ML service consumer to the AIML service producer for FL client discovery or selection assistance request.

Table 8.2.3.1-1 FL client discovery or selection assistance request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor ID | M | Identity of the Requestor, |
| VAL service ID | M | Identity of the VAL service |
| FL client identities | O | Identifier of FL client, e.g. UE ID, UE address |
| FL client selection criteria | M | Indicates the criteria for AI/ML member selection. |

NOTE:	In table 8.2.3.1-1, the policies identify the conditions and criteria, when satisfied the AI/ML member can be selected for the AI/ML operations via the AI/ML service consumer.

Editor’s note: The details in the FL client selection criteria is FFS.

#### 8.2.3.2	FL client discovery or selection assistance response

Table 8.2.3.2-1 describes the information flow from the AIML service producer to the AI/ML service consumer to send FL client discovery or selection assistance response.

Table 8.2.3.2-1 FL client discovery or selection assistance response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Indicates success of failure |

#### 8.2.3.3	FL client discovery or selection assistance notification

Table 8.2.3.3-1 describes the information flow from the AIML service producer to the AI/ML service consumer to notify the FL client selection.

Table 8.2.3.3-1 FL client discovery or selection assistance response

| Information element | Status | Description |
| --- | --- | --- |
| >Selected FL client identities | M | Identifier of FL client, e.g. UE ID, UE address that satisfies the FL client selection policies |

* * * End of Changes * * * *