3GPP TSG-SA WG6 Meeting #60	S6-241619

Changsha, China 15th – 19th April 2024	(revision of S6-241493, 1152)

Source:	Huawei, Hisilicon

Title:	Key Issue on deployment enhancement for XR application

Spec:	3GPP TR 23.700-23 v0.2.0

Agenda item:	8.5

Document for:	Approval

Contact:	Yajie Hu (Huyajie1@huawei.com)

1. Introduction

This paper proposes a new key issue deployment enhancement for XR application.

2. Reason for Change

Due to one DNAI may be mapped to multiple UPFs, thus the issue may exist that the selected UPF and selected application server may still not in same data center and the N6 delay still contains the delay of UPF – APP server involving the data transmission via router (s) which the latency is longer than the data transmission via network adapter. In this case the E2E latency may not be able to satisfy the XR service requirement.

Therefore, further study the deployment enhancement for XR application is required.

The detailed clarification is in the discussion paper S6-241153

3. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-23 v0.2.0.

* * * First Change * * * *

4.x	Key issue #x: XR application server deployment enhancement

As for XR service, current XR application server deployment may not be able to satisify the strict latency requirement. And considering that XR application server may involve multi-model communication flow, to support multi-model communication flow synchronization, it is better to shorten E2E delay to provide ensure the operation on the multi-model synchrnization.

Considering the XR service has strict requirement on the E2E delay, thus the application server and UPF deployment should be further optimized to shorten the N6 latency. The application server and the UPF can be co-deployed in one data center, in doing so, the N6 latency can only contain the delay of RAN-UPF which is shorter than current N6 delay. In this case, the data is transmitted between UPF and APP server via network adapter.

However, due to one DNAI may be mapped to multiple UPFs in different EDNs, thus only using DNAI to and application server may not be enough consider the case:

The UE is in the overlapping area of EDN#1 and EDN#2, when performing service provisioning, the UE may anchor to UPF#2 however the UE may connect to the EES#1 for EAS information. Then the UE may connect to EAS#1 discovered from EES#1.

The issue may happen that the user plane path is not optimal, which may not be able to satisify the XR application requirement.

This key issue includes the following aspects:

- Whether and how enablement layer can support XR application server deployment to further reduce E2E latency?

- Whether and how enablement layer can support XR application server selection for the application server and UPF co-deployment case?

NOTE: For this Key issue, the coordinate with SA2, SA5 is required.