3GPP TSG-SA WG6 Meeting #65	S6-250453

Athens, Greece 17th – 21st February 2025	(revision of S6-25005)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.482 | CR | 0006 | rev | 1 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections related to ML model data types |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Apple |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_App |  |  |  |  |  | Date: |  |  | 2025-02-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Change 1: At least four model phases are considered: in training, trained, re-training & deployed, so not just trained. Change 2: ML model ID is a mandatory attributed of ML model information, therefore it shouldn’t be duplicated. Change 3: Throughout the GS a ML model with associated ID is referred to, rather than an AI/ML model. Change 4: The model information is provided storage request, rather than the profile Change 5: ML model information is a mandatory attribute of ML model profile, therefore it should not be respecified as Change 6 & 7: Model attributes should be listed under an identifier and ML Model already includes the ML Model ID. Change 8: ML Model already includes the ML Model ID, where ML model should only be included in a successful response. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Resolution is provided for the 8 changes list above. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Erroneous data type specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.10, 8.5.5.2, 8.10.3.1, 8.11.2.1, 8.16.4.2, 8.16.4.4 & 8.21.3.1. |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | In rev1, removed change 5 (renumbering accordingly). In change 8 (now 7) reverted the ML model information attribute to optional Table 8.21.3.2-1. |  |  |  |  |  |  |  |  |

* * * First Change * * * *

## 6.10	Support for ML model management

This functionality enables the AIMLE server to manage ML models through interaction with the ML repository. The AIMLE server provides the storage and discovery functionality for ML model information. The storage functionality allows the AIMLE server to store ML model information in the repository. The discovery functionality enables the AIMLE server to retrieve information about available ML models via an API following a request/response model.

* * * Second Change * * * *

#### 8.5.5.2	FL-related event subscription request

Table 8.5.5.2-1 describes information elements for FL-related event subscription request from the subscriber (AIMLE server) to the ML repository.

Table 8.5.5.2-1: FL-related event subscription request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor Identifier | M | The identifier to determine the identity of the requestor entity. This can be the VAL server ID. |
| Security Credentials | M | The security credentials of the requestor. |
| Time of validity | M | The time validity for the subscription. |
| ML model information | M | Information of the ML model, specified in Table 8.11.4.1-2, for which the subscription is needed. |
| FL member information | M | Information on the FL member (candidate or selected). |
| >FL member type | O (NOTE) | The type of FL member which can be an FL server or FL client. |
| >FL member ID | O (NOTE) | The ID of the entity serving as FL member. This can be used for events where the FL member is known and possibly selected, and the availability needs to be checked. |
| Event Information | M | Information on the FL-related event. |
| > Event ID | O | The identity of the FL-related event (if known by the requestor). |
| > Event type | M | The event type requested, based on the Definition of FL related events (shown in Table 8.5.4-1). |
| Notification endpoint | M | The information of the endpoint for receiving the notifications for the event. |
| NOTE: At least one of these shall be present. |  |  |

* * * Third Change * * * *

#### 8.10.3.1	AIMLE client participation request

Table 8.10.3.1-1 shows the request sent by the AIMLE server to each AIMLE client selected for AIMLE client participation procedure.

Table 8.10.3.1-1: AIMLE client participation request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identity | M | The identifier of the requestor. |
| AIMLE client set identifier | M | An identifier for the AIMLE client set. |
| Add/remove indicator | M | Indicator for adding/removing the AIMLE client to/from the AIMLE client set. |
| ML model ID | M | The ML model ID to use for the AI/ML operation. |
| AI/ML operations | M | A list of AI/ML operations (e.g., training) required to be performed. |
| Operational schedule | O | Schedule for the AI/ML operations. |
| Dataset requirement | M | Dataset requirements for the AI/ML operations. Requirements includes dataset identifier, dataset size and age, and/or dataset features. |
| Service requirement | M | Information about the required Service ID and its service permission level (e.g., premium resource usage standard resource usage, limited resource usage) |

* * * Fourth Change * * * *

#### 8.11.2.1	AIMLE server-initiated ML model information storage

Figure 8.11.2.1-1 illustrates the procedure of the AIMLE server-initiated ML model information storage.

Pre-condition:

1.	The AIMLE server has either received application specific model details from AIML consumer or produced analytics model.

Figure 8.11.2.1-1: AIMLE server-initiated ML model information storage

1.	The AIMLE server sends a ML model information storage request to the ML repository to store an ML model. The ML model included in the request can be one trained by the ML repository consumer or its information can be provisioned to the ML repository consumer. The request contains information elements as described in Table 8.11.4.1-1. The request message may contain Indication of continuous training and continuous training model parameter.

NOTE 1:	The security requirements within the ML model storage requirements are out of scope.

2.	Upon receiving the ML model information storage request, the ML repository verifies if the AIMLE server is authorized to store the ML model identified by Analytics ID and/or list of the allowed vendors provided within the ML model information attribute. If the AIMLE server is authorized, the ML repository processes the request and records information of the ML model (e.g., by creating a ML model profile). The ML repository sends an ML model information storage response to the AIMLE server with an identifier of the created ML model profile.

* * * Fifth Change * * * *

#### 8.16.4.2	Transfer learning model selection assistance response

Table 8.16.4.2-1 shows the response sent by the AIMLE server to the VAL server for the server-triggered transfer learning support procedure.

Table 8.16.4.2-1: Transfer learning model selection assistance response

| Information element | Status | Description |
| --- | --- | --- |
| Success response | O (NOTE) | Indicates that the transfer learning model selection assistance request was successful. |
| > List of ML models | O | List of ML models selected by AIMLE server for training as candidate pre-trained model. |
| >> ML repository identifier and address | O | Provides the ID and address of the ML repository which stores the pre-trained ML model selected by AIMLE server for training as pre-trained model. |
| >> ML model information | O | Information on the selected model, specified in Table 8.11.4.1-2. |
| >> ML model rating | O | If requested, a rating parameter for the ML model to serve as pre-trained. Such rating can be based on the ML task similarity score e.g. based on the feature. |
| Failure response | O (NOTE) | Indicates that the transfer learning model selection assistance request was failure. |
| > Cause | M | Reason for the failure. |
| NOTE: Only one of these information elements shall be present |  |  |

* * * Sixth Change * * * *

#### 8.16.4.4	UE transfer learning model selection assistance response

Table 8.16.4.4-1 shows the response sent by the AIMLE server to the AIMLE client for the client-triggered transfer learning support procedure.

Table 8.16.4.4-1: UE transfer learning model selection assistance response

| Information element | Status | Description |
| --- | --- | --- |
| Success response | O (NOTE) | Indicates that the UE transfer learning model selection assistance request was successful. |
| > List of ML models | O | List of ML models selected by AIMLE server for training as pre-trained model. |
| >> ML model information | O | Information on the selected model, specified in Table 8.11.4.1-2. |
| >> ML model rating | O | If requested, a rating parameter for the ML model to serve as pre-trained. |
| Failure response | O (NOTE) | Indicates that the UE transfer learning model selection assistance request was failure. |
| > Cause | M | Reason for the failure. |
| NOTE: Only one of these information elements shall be present |  |  |

* * * Seventh Change * * * *

#### 8.21.3.1	ML model update response

Table 8.21.3.2-1 details the ML model update response IEs.

Table 8.21.3.2-1: ML model update response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response | O (NOTE 1) | Indicates that the model has been updated. |
| > ML model | O (NOTE 2) | Provides the updated ML model. |
| > ML model retrieval endpoint | O (NOTE 2) | The endpoint (e.g., URL, URI, IP address) where the ML model file can be retrieved. |
| > ML model information | O | Provides information of the ML model, specified in Table 8.11.4.1-2. |
| Failure response | O (NOTE 1) | Indicates that the request has failed. |
| > Cause | O | Indicates the failure cause. |
| NOTE 1: At least one of these information elements shall be provided. |  |  |
| NOTE 2: At least one of these information elements shall be provided. |  |  |

* * * End of Changes * * * *