3GPP TSG-SA WG6 Meeting #65	S6-250553

Athens, Greece 17th – 21st February 2025	(revision of S6-250481, S6-250244)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.434 | CR | 0370 | rev | 2 | Current version: | 19.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | UE unavailability information |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GSAT_Ph3_App |  |  |  |  |  | Date: |  |  | 2025-02-10 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Study in TR 23.700-01 v19.0 has KI#3 for Supporting Satellite access with discontinuous coverage and has been concluded as follows: Solution #AE3 resolves open issue about application enabler (SEAL Configuration management server) obtaining the satellite coverage availability information and to retrieve unavailability period for UE (either from UE or from the 5GS), for supporting satellite discontinuous coverage. Further the solution also resolves open issue about leveraging UE unavailability period information for minimizing the service impact or adjusting the application behaviour by the SEALDD server. There is dependency with SA2 for obtaining exposure of loss of coverage event information. Solution #AE3 can be considered for normative work. So the proposal is to address the KI#3 according to Sol#AE3 in the TS for the normative work. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding procedure for getting and notifying UE unavailability information. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Service may be impacted when delivering via satellite access. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | New{21.x, 21.x.1, 21.x.2, 21.y, 21.y.1} |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

## 21.x	UE unavailability information configuration

### 21.x.1	General

The following clauses describe the procedure for configuration of UE unavailability information to the consumer entities e.g. VAL server, SEAL client, other SEAL servers.

### 21.x.2	Procedures for UE unavailability information configuration

The procedure for consumer entities like VAL server, SEAL client, SEAL servers obtaining the UE unavailability information for enabling optimal service delivery during a discontinuous coverage pattern is illustrated in figure 21.x.2-1.

Pre-conditions:

-	When the VAL UE is the consumer, the VAL UE has registered to the 3GPP network via satellite access as defined in clause 4.2.2.2.2 of 3GPP TS 23.502 [11].

-	5GC is aware of Satellite coverage availability information (SCAI) described in Annex Q of 3GPP TS 23.501 [10] and VAL UE location.

-	Consumer entities like VAL server, SEAL client, SEAL servers requested configuration server to provide UE unavailability information.

Figure 21.x.2-1: Procedures for UE unavailability information configuration

1.	The configuration server determines the UE unavailability information by subscribing with 5GC to loss of coverage event subscription to get notified about the unavailability period or unavailability duration when certain events occur e.g. start of unavailability period or before certain duration for an area location, considering the information received from the procedure described in clause 5.4.1.4 of 3GPP TS 23.501 [5].

NOTE:	Configuration management server can request UE unavailability information for a list of UEs.

2.	Configuration management server sends the UE unavailability period information to the requesting consumer entity (e.g. VAL server, SEAL client, SEAL servers).

3.	Consumer entity e.g. VAL server, SEAL client, SEAL servers utilizes the UE unavailability information for adjusting the application behaviour e.g. to trigger the downlink data delivery, bulky data exchanges can be deferred and planned during longer connectivity intervals, giving precedence to data suiting short connectivity intervals.

* * * Next Change * * * *

## 21.y	Information flows

#### 21.y.1	Notify UE unavailability information

Table 21.y.1-1 describes the information flow for the Configuration management server notifying UE unavailability information to the requesting consumer entity which can be VAL server or SEAL client or SEAL server.

Table 21.y.1-1: Notify UE unavailability information

| Information element | Status | Description |
| --- | --- | --- |
| UE ID(s) | M | Indicates the UE ID for which the unavailability information is being provided. |
| > UE unavailability information | M | Indicates the UE unavailability information for a service, duration and UE area location. |

* * * End of Changes * * * *