3GPP TSG-SA WG6 Meeting #65	S6-250568

Athens, Greece, 17th – 21st February 2025	(revision of S6-250229, S6-250469)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.482 | CR | 0026 | rev | 2 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updates to FL Member Grouping |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_App |  |  |  |  |  | Date: |  |  | 2025-02-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The step description for FL grouping indication and response, and the corresponding API clauses are missing. This paper proposes to complete the step description in FL grouping indication and add API clause for FL grouping indication and response. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The changes include: - Added step description for FL grouping indication and response. - Added API clause for FL grouping indication request and response. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The specification is incompleted, step discription and API clause for FL grouping indication request and response are missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.17.2, 8.17.3.x(new), 9.4.y(new), 9.4.y.1(new), 9.4.y.2(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

### 8.17.2	Procedure

In this procedure, the AIMLE support capability is described for grouping the FL members, where the grouping is tailored to a specific ML task (VAL triggered task or analytics event/ID).

Figure 8.17.2-1 illustrates the procedure for supporting the FL member grouping.

Pre-conditions:

1.	VAL server is connected to AIMLE Server.

2.	The candidate/selected FL member has registered to the FL member registry based on the capability in clause 8.4.

Figure 8.17.2-1 Procedure for FL member grouping

1.	The VAL server sends an FL member grouping support request to the AIMLE server for supporting an FL process.

2.	The AIMLE Server based on the request, determines the need for creating a group consisting of the needed FL members for a given ML task (i.e., an ML model training/inference job ID). The FL aggregator can be either the VAL server or the AIMLE server himself, based on the request. The need for creating a FL member group may be based on an ML task for a given AIMLE service area or for a given AIMLE service area where one or more ML tasks are expected to run, based on step 1 request.

3.	The AIMLE Server fetches the available FL members for the given ML task (i.e., an ML model training/inference job ID) from the ML repository. Based on this information, AIMLE server may select one or more FL members for the group for the ML task.

NOTE: If the FL member is an AIMLE client, this step re-uses procedure 8.9.2 on AIMLE client selection.

4.	The AIMLE Server configures the FL member group based on the available or selected FL members by the FL server/aggregator, generate group ID and store mapping to group member IDs. The criteria for determining the group members can be the capabilities of the FL participants, or whether the candidate participants are fixed or mobile nodes and their availabilities, the proximity of the participants among them.

5.	The AIMLE Server interacts with the each candidate FL member from the configured FL member group as shown in step 5a.

5a.	The AIMLE Server sends indication to the candidate FL member (the AIMLE client which is deployed on UE) about the group ID and the group member identities for the ML model ID / analytics ID (based on the request in step 1.

5b.	The candidate FL member sends to the AIMLE Server a FL grouping indication acknowledge.

6.	The AIMLE server sends a FL member grouping support response to the VAL request indicating the group creation and the group information.

* * * Next Change * * * *

#### 8.17.3.x	FL grouping indication acknowledge

Table 8.17.3.x-1 describes the information flow FL grouping indication acknowledge from the FL members (AIMLE clients which are deployed on UEs) to the AIMLE server.

Table 8.17.3.x-1: FL grouping indication acknowledge

| Information element | Status | Description |
| --- | --- | --- |
| Success response | O (NOTE) | Acknowledgement of FL grouping indication. |
| Failure response | O (NOTE) | Indicates that the FL grouping indication was failure. |
| > Cause | O | Reason for the failure. |
| NOTE: Only one of these information elements shall be present |  |  |

* * * Next Change * * * *

### 9.4.y	FL grouping indication API

#### 9.4.y.1	General

Table 9.4.y.1-1 illustrates the APIs for FL grouping indication. This API enables the communication between the AIMLE server and the FL member for indicating the FL grouping procedure.

Table 9.4.y.1-1: Aimlec_FLGroupIndication API

| API Name | API Operations | Operation Semantics | Consumer(s) |
| --- | --- | --- | --- |
| Aimlec_FLGroupIndication | Request | Request/Response | AIMLE Server |

#### 9.4.y.2	Aimlec_FLGroupIndication operation

API operation name: Aimlec_FLGroupIndication

Description: The consumer indicates to FL member (AIMLE clients which are deployed on UEs) for FL grouping procedure.

Inputs: See clause 8.17.3.3.

Outputs: See clause 8.17.3.x.

See clause 8.17.2 for details of usage of this operation.

* * * END of Changes * * * *