3GPP TSG-SA WG6 Meeting #66	S6-251302

Gothenburg, Sweden, 7th – 11st April 2025	(revision of S6-251102)

# Source:	CATT

# Title:	New SID on application enablement for Ambient IoT services

# Document for:	Approval

# Agenda Item:	11

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

Title: 	Application enablement for Ambient IoT services

Acronym: 	FS_AmbientIoT_APP

Unique identifier:

Potential target Release:	Rel-20

1	Impacts

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  | X |  | X |  |
| No | X |  | X |  |  |
| Don't know |  |  |  |  | X |

2	Classification of the Work Item and linked work items

2.1	Primary classification

This work item is a …

| X | Study |
| --- | --- |
|  | Normative – Stage 1 |
|  | Normative – Stage 2 |
|  | Normative – Stage 3 |
|  | Normative – Other* |

* Other = e.g. testing

2.2	Parent Work Item

For a brand-new topic, use “N/A” in the table below. Otherwise indicate the parent Work Item.

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |

2.3	Other related Work Items and dependencies

| Other related Work /Study Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |
| 950004 | Study on Ambient power-enabled Internet of Things | Rel-19 stage-1 study on Ambient power-enabled Internet of Things |
| 1020030 | Stage 1 of Ambient power-enabled Internet of Things | Rel-19 stage-1 Normative Work on Ambient power-enabled Internet of Things |
| 1020071 | Study on Architecture support of Ambient power-enabled Internet of Things | Rel-19 stage-2 study on Ambient power-enabled Internet of Things in SA2 |
| 1070010 | Architecture support of Ambient power-enabled Internet of Things | Rel-19 stage-2 Normative Work on Ambient power-enabled Internet of Things in SA2 |
| 1020085 | Study on solutions for Ambient IoT (Internet of Things) in NR | Study on Ambient IoT (Internet of Things) in RAN in Rel-19 |
| 1030031 | Study on Security Aspect of Ambient IoT Services in 5G | Study on Architecture support of Ambient power-enabled Internet of Things in SA3 |

3	Justification

3GPP SA1 (via TS 22.369) has specified the service and performance requirements for Ambient IoT since Rel-19, which is defined as an IoT device powered by energy harvesting, being either battery-less or with limited energy storage capability (e.g. using a capacitor), and the energy is provided through the harvesting of radio waves, light, motion, heat, or any other power source that could be seen suitable. An Ambient IoT device has low complexity, small size and lower capabilities and lower power consumption than previously defined 3GPP IoT devices (e.g. NB-IoT/eMTC devices).

Based on SA1 functional service requirements, some potential SA6 work maybe as follows:

- Collected information and network capability exposure:

- Subject to user consent, operator policy and 3rd party request, the 5G system shall be able to obtain data from Ambient IoT devices (e.g. sensor data) and provide it to a trusted 3rd party via the 5G network.

- Subject to user consent, operator’s policy and 3rd party request, the 5G system shall provide information about an Ambient IoT device or a group of Ambient IoT devices (e.g. position) to the trusted 3rd party via the 5G network.

- The 5G system shall enable an authorized 3rd party to instruct the 5G network to trigger a group of Ambient IoT devices in an specific area and which action the Ambient IoT devices need to perform when triggered (e.g. send ID, receive further information, send measurement value).

Meanwhile, RAN WGs (via TR 38.769) have investigated RAN-level solutions for Ambient IoT, and the scope includes:

- Traffic types:

  - DT: Device-terminated;

  - DO-DTT: Device-originated – device-terminated triggered.

- Connectivity topologies:

  - Topology 1: BS Reader ↔ Ambient IoT Device;

  - Topology 2: RAN ↔ UE Reader ↔ Ambient IoT Device

- Use cases focusing on:

  - indoor inventory

  - indoor command

Furthermore, aligned with the same scope as RAN study and considering the SA1 requirements, 3GPP SA2 (SP-250377) has started to specify the following functions which are focusing on Device 1 in Topology 1 in Rel-19 and plans to finalize the normative work in Q2, 2025.

- Architecture to support Ambient IoT:

  - Architecture to Support Device 1 in Topology 1 (Direct interface option and Indirect interface option).

- Ambient IoT Device management:

  - Identifier of Ambient IoT Device (i.e. permanent device ID, temporary device ID).

  - AIoT Device Data Profile Management

- Ambient IoT Service enabler for Inventory and Command:

  - Exposure for AIOT services to AF.

  - Data transfer between AIoT Device/RAN reader and AIOTF.

At present, SA2 could provide the requested AIoT Devices ID(s) to the AF. However, these AIoT Devices ID(s) are so large and not being consolidated/aggregated before sending to the AF. But the 3rd parties (e.g. application server) are more interested in direct and rich results. E.g., the 3rd parties expect to receive the changed AIoT Devices ID(s) compared to last time, the aggregated AIoT Devices ID(s) per different services/application events, or the AIoT Devices ID(s) combined with the e.g. history data, etc. And how to provide this value-added information to the 3rd parties should be in the scope of SA6 study.

What’s more, SA2 just defined the Permanent ID format for the AIoT Devices. But how to associate these IDs with different services should also be studied, especially when there is a service level agreement (SLA) between the operators and AIoT service providers. And monitoring the AIoT devices’ status (e.g. enabled, disabled) and reporting the status to the AIoT service providers as soon as possible to minimize the service impact are also valued to be studied.

In a word, based on SA1 service requirements and SA2 related normative work, there are some potential work should be done in SA6 to further meet the requirements of AIoT service providers and they are may be as follows:

- Exposure of AIoT services to AF: How application enablers utilize the exposure of AIoT device information from SA2 and exposes to the application server for the value-added information.

- Support of AIoT service requests: How application enablers support the AIoT complex service requests (e.g. automatic inventory). Application servers may have requirements on the automatic inventory which application enablers can provision some policy/triggers to achieve that if they have service level agreement (SLA).

- AIoT devices data handling: How application enablers further process (e.g. aggregate) the reported Ambient IoT devices data and generate the value-added data per different granularity (e.g. service/scenario/application events).

- AIoT devices status management: If application enablers acting as an Ambient IoT devices management platform, how to manage the Ambient IoT devices, including associating the device ID with the service ID, monitoring the status of devices, etc.

What’s more, SA2 may start the further study on Ambient IoT in Rel-20 since Q2 2025. And SA6 work of Ambient IoT in Rel-20 should also consider the work tasks of Rel-20 SA2 when its scope is available.

In summary, Ambient IoT has the potential to benefit a large number of vertical industries, so it is important for SA6 to study how to support the Ambient IoT services in the application enabled layer, aligned with SA1 requirements and SA2 architecture and functionalities, to open up new markets and opportunities for 3GPP systems.

4	Objective

This SID studies the support of ambient power-enabled IoT devices in the application enabled layer, based on the services requirements defined in SA1 and architecture and functions defined in SA2.

The SA6 objectives of this work item include the following:

- Study new architecture or enhancements on existing SA6 application enablers for supporting Ambient IoT services in application enabled layer.

- Provide the value-added service/information of Ambient IoT devices to the consumer

- Study how to utilize the exposure from other working group (e.g. SA2) and provide the value-added Ambient IoT device information to the consumer (e.g. VAL server) in an application-oriented way (e.g. per service).

- Study how to support the application server’s requests for the Ambient IoT devices (e.g. provision some policy/triggers to achieve automatic inventory) considering Ambient IoT capabilities exposed by SA2.

- Handle of Ambient IoT devices data

- Study how to enrich the processing (e.g. classification, duplication removal, aggregation, format unification, history data) of the collected Ambient IoT device information.

- Management of Ambient IoT devices

- Study how to manage the Ambient IoT devices (e.g. associate device ID with kinds of services, monitoring the status of Ambient IoT devices) when the application enablers acting as a management platform for the Ambient IoT devices.

- Identify potential key issues and related solutions, including the information flows and application enablement APIs where necessary, to satisfy the architectural requirements and enhancements identified in bullet 1.

NOTE 1:	The objectives a) and b) depend on the progress and output from SA2 in Rel-19.

NOTE 2:	The objectives in SA6 will not overlap with SA2.

5	Expected Output and Time scale

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Rapporteur |
| TR | 23.xxx | Study on application enablement for Ambient IoT services | SA#110 (Dec 2025) | SA#111 (Mar 2026) | Liping Wu, CATT, <wuliping@cictmobile.com> |

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |

6	Work item Rapporteur(s)

Liping Wu, CATT, wuliping@cictmobile.com

7	Work item leadership

SA6

8	Aspects that involve other WGs

SA2 for core network architecture aspects, SA3 for security aspects, and SA5 for management and charging aspects.

9	Supporting Individual Members

Supporting IM name

CATT

Huawei

ZTE

China Mobile

KPN

Interdigital

NTT DOCOMO