3GPP TSG-SA WG6 Meeting #67	S6-252036

Fukuoka City, Fukuoka , JP, 19th – 23th May 2025	(revision of S6-252036)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.283 | CR | 0095 | rev | - | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add Iner-MCX PerGroup Call Setup procedures |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | TD Tech, Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | enhMC_Ph2-MC |  |  |  |  |  | Date: |  |  | 2025-03-19 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-20 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | According to the 23.283 10.1.2.1,10.1.2.2,10.1.2.3,10.3.3.2,10.3.3.3 protocol, the IWF can use the IWF MCPTT ID to access the MCPTT system affiliation/de-affiliation interworking group. The IWF MCPTT ID represents all users from the legacy system. The MC service server only handles the IWF MCPTT ID and does not need to obtain the each detailed LMR users information. This solution has the following advantages: 1. For a interworking group call, only one session is required between the MCX server and the IWF, which minimizes the interworking bandwidth and concurrency requirements. 2. The MCX server and IWF do not need to share the detailed user information of group members, which ensures the security of user data. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Update the affiliation procedure to allow the per group per system granularity interaction from IWF to MC service server - Update the group call procedure to allow the per group per system granularity call request/response from IWF to MC service server. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The performance and complexity of the MCX-3 interface cannot be optimized. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.1.2.1,10.1.2.4,10.1.2.5,10.3.3.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# **************************** Next Changes *******************

10.1.2	Affiliation procedures

10.1.2.1	General

When an interworking group is defined in the MCPTT system, the LMR system (via the IWF) informs the MCPTT system of group affiliations in one of the following ways:

-	Every group affiliation in the LMR system results in an affiliation sent to the MCPTT system, which contains the identity (with appropriate translation by the IWF) of the affiliating group member; or

-	A group affiliation is sent on behalf of the group's LMR users (via the IWF) to the MCPTT system when the first group member affiliates to the designated group in the LMR system, and a group de-affiliation is sent on behalf of the group's LMR users (via the IWF) to the MCPTT system when the last group member de-affiliates, and no other group affiliation signalling is sent.

The first and second options may be used at the same time, such that some group members may explicitly affiliate while the IWF may affiliate on behalf of other group members.

In the second option, when the IWF is configured to affiliate on behalf of the group's LMR members then:

a)	the group list in the MCPTT system contains the IWF's MCPTT ID. This ID is recognized (through configuration) as having the ability to affiliate on behalf of the group's LMR users associated with this IWF;

b)	the IWF affiliates with its MCPTT ID to the group defined in the MCPTT system;

c)	the MCPTT system recognizes the affiliation as being from an IWF on behalf of the group's LMR users;

d)	when the IWF has affiliated to the group, the MCPTT system:

i)	considers any LMR user associated with the IWF to be affiliated to the group on which the IWF has affiliated. The IWF's users need not be listed ahead of time in the group list for this group in the MCPTT system;

ii)	allows requests such as call setup or floor request, from MCPTT IDs,with or without functional alias, representing LMR users associated with the IWF for actions on the group to which the IWF has affiliated;

iii)	does not carry out an additional affiliation on behalf of LMR users when those users make call requests, and therefore does not send additional messages to those users (e.g. release messages to both the IWF affiliated identity and the LMR user identity performing the action), via the IWF, during call processing;

iv)	recognizes which LMR users are associated with the IWF because their MCPTT IDs belong to the same system as the IWF; and,

v)	uses special rules for the IWF for limits such as "Limitation of number of affiliations per user (N2)".

e)	requests from LMR users to the MCPTT system are identified with their individual MCPTT IDs (as translated by the IWF):

i)	a user in the LMR system can affiliate on its own (via the IWF) as long as the user is a group member (i.e. in the group list), even if the IWF has affiliated to the group.

f)	the IWF may make requests on behalf of a group's LMR users using the IWF MCPTT ID like a normal group member including, for example, group join requests for groups using the chat model;

g)	the IWF is not allowed to affiliate to a group that is not configured with the IWF's MCPTT ID in the group member list; and,

h)	if the IWF has not affiliated to an MCPTT group, then call requests to this group from LMR users on the system associated with the IWF, can only be accepted if the LMR user's MCPTT ID is in the group list, and has already affiliated.

When an interworking group is defined in the LMR system, the MCPTT system (via the IWF) informs the LMR system of group affiliations in one of the following ways:

-	Every group affiliation in the MCPTT system results in an affiliation sent to the LMR system, which contains the identity (with appropriate translation by the IWF) of the affiliating group member; or

-	A group affiliation is sent on behalf of the group's MCPTT users (via the IWF) to the LMR system when the first group member affiliates to the designated group in the MCPTT system, and a group de-affiliation is sent on behalf of the group's MCPTT users (via the IWF) to the LMR system when the last group member de-affiliates, and no other group affiliation signalling is sent.

In the second option, when the MCPTT server is configured to affiliate on behalf of the group's MCPTT members then:

a)	the group list in the LMR system contains the proxy user's MCPTT ID. This ID is recognized (through configuration) as having the ability to affiliate on behalf of the group's MCPTT users associated with this MCPTT server;

b)	the MCPTT server affiliates with proxy user's MCPTT ID to the group defined in the LMR system;

c)	the IWF recognizes the affiliation as being from an MCPTT server on behalf of the group's MCPTT users;

d)	when the MCPTT server has affiliated to the group, the IWF:

i)	considers any MCPTT user associated with the MCPTT server to be affiliated to the group on which the MCPTT server has affiliated. The MCPTT users need not be listed ahead of time in the group list for this group in the LMR system;

ii)	allows requests such as call setup or floor request, from proxy user's MCPTT ID,with or without functional alias, representing MCPTT users associated with the MCPTT server for actions on the group to which the MCPTT server has affiliated;

iii)	does not carry out an additional affiliation on behalf of MCPTT users when those users make call requests, and therefore does not send additional messages to those users (e.g. release messages to both the proxy user's MCPTT ID. and theMCPTT user identity performing the action), via the MCPTT server, during call processing;

iv)	recognizes which MCPTT users are associated with the MCPTT server because their MCPTT IDs belong to the same system as the MCPTT server; and,

v)	uses special rules for the MCPTT server for limits such as "Limitation of number of affiliations per user (N2)".

e)	requests from MCPTT users to the LMR system are identified with their individual MCPTT IDs (as translated by the IWF):

i)	a user in the MCPTT system can affiliate on its own (via the MCPTT server) as long as the user is a group member (i.e. in the group list), even if the MCPTT server has affiliated to the group.

f)	the MCPTT server may make requests on behalf of a group's MCPTT users using the proxy user’s MCPTT ID like a normal group member including, for example, group join requests for groups using the chat model;

g)	the MCPTT server is not allowed to affiliate to a group that is not configured with the proxy user’s MCPTT ID in the group member list; and,

h)	if the MCPTT server has not affiliated to an group, then call requests to this group from MCPTT users on the system associated with the MCPTT server can only be accepted if the MCPTT user's MCPTT ID is in the group list, and has already affiliated.

MC service group affiliation and de-affiliation can be achieved using explicit or implicit methods as defined in TS 23.280 [5]. When the MC service server uses implicit affiliation/de-affiliation for an interworking group defined in the LMR system, the MC service server informs the IWF of the affiliation/de-affiliation.

# **************************** Next Changes *******************

10.1.2.4	Group affiliation to group defined in the LMR system

The MC system may affiliate its group members to an interworking group defined in the LMR system via the IWF.

The signalling procedure of group affiliation via the IWF is described in figure 10.1.2.4-1.

Pre-conditions:

1.	The group to be affiliated to is defined in the LMR system.

2.	The IWF is connected to and is authorized to work with the MC system.

3.	The mapping relationship of group and user identities between the MC system and the LMR system has been configured at the IWF.

NOTE 1:	For all signalling messages passing through the IWF between the MC system and the LMR system, the IWF performs the identity conversion and protocol translation.

Figure 10.1.2.4-1: Group affiliation to group defined in the LMR system

1.	The MC service client sends a MC service group affiliation request, including the MC service group ID(s), to the MC service server.

2.	The MC service server checks if the MC service group ID(s) is an interworking group defined in the LMR system.

3.	The MC service server sends an IWF group affiliation request to the IWF.

NOTE 2:	The IWF can forward the request to the LMR system that could check whether the MC service client is authorized to affiliate to this interworking group.

NOTE 3:	The IWF can reject the affiliation if the MC service group ID is either unknown to the IWF or not mapped to an LMR group identity in the IWF configuration.

NOTE 4: In the Option2 scenario indicated in clause 10.1.2.1, the MCPTT system initiates an affiliation procedure to the IWF only when the first user in the MCPTT system of this group initiates an affiliation procedure.

4.	The IWF returns an IWF group affiliation response to the MC service server, informing the successful affiliation to the LMR group.

5.	The MC service server stores the group affiliation status of the MC service client for the requested interworking group.

6.	The MC service server sends an MC service group affiliation response to the MC service client.

NOTE 5:	How the affiliation is conducted on the LMR system is outside the scope of the present document.

NOTE 6:	If an MC service client is implicitly affiliated to an MC service group defined in the LMR system the MC service server only performs steps 3, 4, and 5.

10.1.2.5	Group de-affiliation from a group defined in the LMR system

The signalling procedure of interworking group de-affiliation from a group defined in the LMR system is described in figure 10.1.2.5-1.

The MC system manages the individual de-affiliation requests from the MC service users. The MC system may de-affiliate its group members from the interworking group via the IWF.

Pre-conditions:

1.	The mapping relationship of group and user identities between the MC system and the LMR system has been configured at the IWF.

2.	The affiliation procedure described in subclause 10.1.2.4 was previously performed.

NOTE 1:	For all the signalling messages passing through the IWF between the MC system and the LMR system, the IWF performs the identity conversion and protocol translation.

Figure 10.1.2.5-1: Group de-affiliation from a group defined in the LMR system

1.	The MC service client of the MC service user sends an MC service group de-affiliation request to the MC service server. The MC service client shall provide the initiating MC service ID and the MC service group ID(s) being de-affiliated from.

2.	Based on the user subscription and stored group policy, the MC service server checks if the user of the MC service client is affiliated to the requested MC service group(s). The MC service server checks if the MC service group(s) is an interworking group.

3a.	If the MC service group(s) is an interworking group, the MC service server sends an IWF group de-affiliation request to the IWF.

NOTE 2: In Option 2 scenario indicated in clause 10.1.2.1, the MCPTT system initiates a de-affiliation procedure to the IWF only when the last user in the MCPTT system of this group initiates a de-affiliation procedure.

3b.	The IWF returns an IWF group de-affiliation response to the MC service server.

4.	If the user of the MC service client is authorized to de-affiliate from the requested MC service group(s), the MC service server removes the affiliation status of the user for the requested MC service group(s).

5.	The MC service server returns an MC service group de-affiliation response to the MC service client.

NOTE 3:	If an MC service client is implicitly de-affiliated from an MC service group defined in the LMR system the MC service server only performs steps 3a, 3b, and 4.

# **************************** Next Changes *******************

10.3	Group call

10.3.3.5	Group call setup initiated by LMR user on an interworking group defined in the LMR system.

In this procedure, an LMR user is initiating a group call on an interworking group defined in the LMR system. The signalling procedure is described in figure 10.3.3.5-1.

This subclause is based upon subclause for Pre-arranged group call setup in 3GPP TS 23.379 [7], subclause 10.6.2.3.1.1.2.

Pre-conditions:

1.	The interworking group information is known at the MCPTT Server and the IWF by configuration or group creation. The interworking group has been defined in the LMR system.

2.	MCPTT client 1 and MCPTT client 2 are registered and their respective users are authenticated and authorized to use the MCPTT service.

3.	The users in this interworking group have been affiliated to the group.

4.	The mapping relationship of group and user identities between the MCPTT system and the LMR system has been configured at the IWF.

5.	LMR user initiates a group call.

NOTE 1:	For all the signalling messages passing through the IWF between the MCPTT system and the LMR system, the IWF performs the identity conversion and protocol translation.

Figure 10.3.3.5-1: Group call initiated by LMR user on an interworking group defined in the LMR system

1.	The IWF sends an IWF group call request(s) to the MCPTT server for call establishment. An individual IWF group call request is sent to the MCPTT server for each affiliated MCPTT user in the group, in this example scenario to the users in MCPTT clients 1 and 2. If floor control is requested by the calling LMR user, an indication of implicit floor request is included. If the group call request contains an implicit floor request it may also include location information.  In the Option2 scenario indicated in clause 10.1.2.1, the IWF sends only one group call request to the MCPTT system where the called party is the MCPTT proxy user.

2.	MCPTT server sends a group call request(s) to the target MCPTT user(s) as described in 3GPP TS 23.379 [7].

3.	MCPTT client(s) receiving the group call request, acknowledge towards the MCPTT server by sending a group call response.

4.	The MCPTT server acknowledges the IWF group call request(s) by sending a IWF group call response(s) to the IWF. If E2EE is specified, then the MCPTT users and the LMR users shall use the same codec. If E2EE is not specified, the MCPTT users and the LMR users can use different codecs and transcoding is needed at the IWF.

NOTE 2:	How the IWF group call response(s) is handled in the IWF / LMR system and how the other LMR users are being called is outside the scope of the present document.

5.	The interworking group call has successfully established media plane for communication and any user can transmit media. The LMR system where the interworking group is defined is the controlling system of the group call and manages the floor control. .

NOTE 3:	In Option2 scenario indicated in clause 10.1.2.1, only one session is required between the IWF and MCPTT system. The subsequent media plane of the group between two systems are beared by this session.