3GPP TSG-SA WG6 Meeting #67	S6-252060

Fukuoku, Japan, 17th – 21st May 2025	revision of S6-251173

Source:	Ericsson

Title:	Enhance AIMLE Data Management Assistance for Data Drift Detection

Spec:	3GPP TR 23.700-83 v0.1.0

Agenda item:	9.4

Document for:	Approval

Contact:	Jing Yue <jing.yue@ericsson.com>

1. Introduction

As described in Key Issue#2, the study aspects on support enhancements related to ML model performance monitoring and correctness evaluation include:

- Whether and how to monitor (e.g., trigger, detect and report) ML model performance and correctness degradation for different ML-enabled operations (e.g., inferencing, training, split AIML operation, HFL, VFL, transfer learning) and considering data managements assistance.

The AIMLE data management assistance service is the AIMLE server assisting AIMLE service consumers with managing data operations performed by VAL clients. The data operations include data preparation and data analysis. In the context of AI/ML, it is important that the data used by the ML model is not significantly different from the data used to train the model. If the difference is higher enough, the model's performance tends to decrease. This discrepancy is known as data drift. Therefore, it is essential that the data management system procedure provides the capability to analyze whether data drift has occurred to assist in decision-making if the model needs to be updated due to a decrease in its performance. To support ML model management, e.g., ML model maintenance, this pCR proposes to enhance the AIMLE data management assistance service by adding drift detection functionality.

2. Reason for Change

To support ML model management, e.g., ML model maintenance, this pCR proposes to enhance the AIMLE data management assistance service by adding drift detection functionality.

3. Conclusions

This paper proposes solution on enhance AIMLE data management assistance for data drift detection.

4. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-83 v0.1.0.

* * * First Change * * * *

## 7.X	Solution#X: Enhance AIMLE Data Management Assistance for Data Drift Detection

### 7.X.1	Solution Descripiton

As introduced in 3GPP TS 23.482, the AIMLE data management assistance service is the AIMLE server assisting AIMLE service consumers with managing data operations performed by VAL clients. The data operations include data preparation and data analysis. In the context of AI/ML, it is important that the data used by the ML model is not significantly different from the data used to train the model. If the difference is higher enough, the model's performance tends to decrease. This discrepancy is known as data drift. Therefore, it is essential that the data management system procedure provides the capability to analyze whether data drift has occurred to assist in decision-making if the model needs to be updated due to a decrease in its performance.

To support ML model management, e.g., ML model maintenance, the AIMLE data management assistance service need be enhanced by adding drift detection functionality.

The following clauses specify the impact to existing AIMLE procedures for the enhancements of AIMLE data management assistance service.

### 7.X.1.1	Impact to existing AIMLE procedures

The AIMLE procedures and information flows in 3GPP TS 23.482 can be enhanced (highlighted in bold italics) as follows.

## 8.15 AIMLE data management assistance ### 8.15.1 General AIMLE data management assistance is the process of the AIMLE server assisting AIMLE service consumers with managing data operations performed by VAL clients. The data operations include data preparation, and data analysis, and data drift detection. The AIMLE server offloads the AIMLE service consumer from interacting with AIMLE clients to manage the data operations. The VAL clients perform the actual data preparation, and data analysis, and data drift detection operations and send the outputs to the AIMLE server for aggregation. The following clauses specify procedures, information flows, and APIs for AIMLE data management assistance. ### 8.15.2 Procedure Pre-conditions: 1. AIMLE clients have registered with AIMLE server. 2. UE application data for AI/ML operations have been collected and dataset identifier have been assigned to each dataset. EVEX mechanism can be reused for data collection as described in 3GPP TS 26.531 [10]. Figure 8.15.2-1: AIMLE data management assistance 1. An AIMLE service consumer (e.g., VAL server) makes a request for AIMLE data management assistance. The request includes information as described in Table 8.15.3.1-1. 2. The AIMLE server authenticates and authorizes the request. If authorized, the AIMILE server assigns an identifier for the subscription. 3. The AIMLE server sends an AIMLE data management assistance subscription response to the AIMLE service consumer. The response includes information as described in Table 8.15.3.2-1. 4. The AIMLE server sends Client data processing trigger requests to AIMLE clients. The request can be for data preparation or data analysis. The request includes information as described in Table 8.15.3.4-1. 5. Each AIMLE client sends the requirements to trigger data management for the VAL client to perform the requested data operation. The VAL client performs the operation locally. NOTE: The data preparation and data analysis functions operate in a similar manner as ML models. The AIMLE layer is able to transport the functions to the VAL clients and the VAL clients performs the associated function on the application data as part of data preparation or data analysis in a similar manner as ML training. 6. After the data operation completes, each AIMLE client sends a response to the AIMLE server with information as described in Table 8.15.3.5-1. 7. The AIMLE server aggregates the output of each AIMLE client. Aggregation includes combining or performing a statistical operation on the received data. Data received from the AIMLE clients can be numerical or categorical, which allows the AIMLE server to aggregate the output data from the AIMLE clients. If necessary, steps 4–7 is repeated to complete all the required data operations. 8. The AIMLE server sends an AIMLE data management assistance notification to the AIMLE service consumer with information as described in Table 8.15.3.3-1. ### 8.15.3 Information flows #### 8.15.3.1 AIMLE data management assistance subscription request Table 8.15.3.1-1 shows the request sent by an AIMLE service consumer to an AIMLE server for the AIMLE data management assistance subscription procedure. Table 8.15.3.1-1: AIMLE data management assistance subscription request Information element; Status; Description. Requestor identifier; M; The identifier of the requestor.. Data management operations; M; An indicator showing what data management type is being requested: data preparation, data analysis.. Data management requirements; M; Requirements for the data management request:. >Data preparation requirements; O (NOTE 1); Data preparation requirements as detailed in Table 8.15.3.1-2.. >Data analysis requirements; O (NOTE 1); Data analysis requirements as detailed in Table 8.15.3.1-3.. > Data drift detection requirements; O (NOTE 1); Data drift detection requirements in Table 8.15.3.1-x.. AIMLE clients list; O (NOTE 2); A list of AIMLE clients for which data management should be performed. The list may be specified by AIMLE client set identifier.. AIMLE client selection criteria; O (NOTE 2); Selection criteria for finding suitable AIMLE clients for AI/ML operations as detailed in Table 8.8.3.1-2.. NOTE 1: At least one of the information elements shall be provided. NOTE 2: At least one of the information elements shall be provided.. Table 8.15.3.1-2: Data preparation requirements Information element; Status; Description. Dataset identifier; M; An identifier for the dataset.. Data preparation requirements; M; Requirements for data preparation.. > Dataset ID; M; The identifier for the dataset.. > Dataset feature ID; M; Identifier or name of dataset feature to process.. > Data preparation function; M; This indicates the function which prepares the data, and it could be an identifier of a function if the function is available locally at the UE or an executable included in the request.. Table 8.15.3.1-3: Data analysis requirements Information element; Status; Description. Dataset identifier; M; An identifier for the dataset.. Dataset analysis requirements; M; Requirements for data analysis.. > Dataset ID; M; The identifier for the dataset.. > Dataset feature ID; M; Identifier or name of dataset feature.. > Data analysis function; M; This indicates the function which performs the data analysis, and it could be an identifier of a function if the function is available locally at the UE or an executable included in the request.. >Data drift analysis; O; Requirements for data drift analysis.. Table 8.15.3.1-x: Data drift detection requirements Information element; Status; Description. Dataset identifier; M; An identifier for the dataset.. Data drift detection requirements; M; Requirements for data drift detection.. > Dataset ID; M; The identifier for the dataset.. > Dataset feature ID; M; Identifier or name of dataset feature.. > Data drift detection function; M; This indicates the function which performs the drift detection, and it could be an identifier of a function if the function is available locally at the UE or an executable included in the request.. #### 8.15.3.3 AIMLE data management assistance notify Table 8.15.3.3-1 shows the notification sent by the AIMLE server to the AIMLE service consumer for the AIMLE data management assistance subscription procedure. Table 8.15.3.3-1: AIMLE data management assistance notify Information element; Status; Description. Status; M; The status for the data management operation. Aggregated data preparation outputs; O (NOTE 1, NOTE 2); Provides outputs for data preparation: dataset identifier, dataset features, and prepared data output.. Aggregated data analysis outputs; O (NOTE 1, NOTE 2); Provides outputs for data analysis: dataset identifier, statistical outputs for each feature, list of outlier and anomaly values, and feature correlation information.. Aggregated drift detection outputs; O (NOTE 1, NOTE 2); Provides outputs for data drift detection: dataset identifier, data drift detection status (e.g. detected data drift of the dataset features).. Timestamp; O; Timestamp of the data management notification. NOTE 1: At least one of the information elements shall be provided in the output. NOTE 2: The output format can be numerical or categorical. If categorical, the format can be nominal or ordinal.. #### 8.15.3.4 Client data processing trigger request Table 8.15.3.4-1 shows the request sent by the AIMLE server to AIMLE clients for the AIMLE client data processing trigger procedure. Table 8.15.3.4-1: Client data processing trigger request Information element; Status; Description. Requestor identifier; M; The identifier of the requestor.. Data management type; M; An indicator showing what data management type is being requested: data preparation, data analysis.. Data management requirements; M; Requirements for the data management request:. >Data preparation requirements; O (NOTE); Data collection requirements as detailed in Table 8.15.3.1-2.. >Data analysis requirements; O (NOTE); Data collection requirements as detailed in Table 8.15.3.1-3.. > Data drift detection; O (NOTE); Data drift detection requirements as defined in Table 8.15.3.1-x.. Operational schedule; O; A schedule to perform the requested data management operation.. NOTE: At least one of the information elements shall be provided.. #### 8.15.3.5 Client data processing trigger response Table 8.15.3.5-1 shows the response sent by AIMLE clients to the AIMLE server for the Client data processing trigger procedure. Table 8.15.3.5-1: Client data processing trigger response Information element; Status; Description. Status; M; The status for the data management operation. Data preparation outputs; O (NOTE 1); The output data after performing data preparation. One output data is generated for each requirement. (NOTE 2). Data analysis outputs; O (NOTE 1); The output data generated by data analysis. One output data is generated for each requirement. (NOTE 2). Data drift detection outputs; O; The output of data drift detection status (e.g. detected data drift of the dataset features). (NOTE 2). Timestamp; O; Timestamp of the data management operation. NOTE 1: At least one of the information elements shall be provided in the output. NOTE 2: The output format can be numerical or categorical. If categorical, the format can be nominal or ordinal. The AIMLE server is able to process either type of data..

### 7.X.2	Architecture Impacts

Editor’s Note:	The architecture impacts of the solution is FFS.

### 7.X.3	Corresponding APIs

Editor’s Note:	The corresponding APIs for supporting the solution is FFS.

### 7.X.4	Solution evaluation

Editor’s Note:	The evaluation of the solution is FFS.

# *** End of Changes ***